3GPP TSG-RAN5 Meeting #96-e	R5-225210

# Electronic Meeting, 15th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5421 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Reference Sensitivity for CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_CA_Rel13-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test requirements for 4Rx are missing for band 3 in several band combinations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test requirements for band 3 have been added in Table 7.3A.5.5-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Refsens cannot be tested for UEs with 4Rx antenna connectors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### 7.3A.5	Reference sensitivity level for 3DL CA

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	TR 36.905 test point analysis for CA_28A-40C is missing, meaning it is not yet determined if testing of fallback configuration CA_28A_40A can be skipped.

-	Band 70 CA configurations are missing test point analysis.

< Unchanged sections omitted >

#### 7.3A.5.5	Test requirement

For intra-band contiguous carrier aggregation the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-1. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-1: Reference sensitivity QPSK PREFSENS for Intra-band contiguous

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| CA_40D | 40 | - | - | - | -96.3 | -94.5 | -93.3 | TDD |
|  |  | - | - | - | -99.04 | -97.24 | -96.04 |  |
| CA_41D | 41 | - | - | - | -94.3 | -92.5 | -91.3 | TDD |
|  |  | - | - | - | -97.04 | -95.24 | -94.04 |  |
| CA_42D | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.24 | -97.24 | -95.44 | -94.24 |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.5.5-2) Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 7.3A.5.5-2: Void

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-3 (originated from single carrier Tables 7.3.5-1). The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-3: Reference sensitivity QPSK PREFSENS for inter-band

| Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Band | 1.4 MHz (dBm) |  | 3 MHz (dBm) |  | 5 MHz (dBm) |  | 10 MHz (dBm) |  | 15 MHz (dBm) |  | 20 MHz (dBm) | Duplex Mode |
| CA_1A-3A-5A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  |  |  |  |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3 (Note 4) | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -88.3 |  |
|  |  |  |  |  |  | -96.020 |  | -93.520 |  | -92.020 |  | -91.020 |  |
|  | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - |  |
| CA_1A-3A-7A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  |  |  |  |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3 (Note 4) | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -88.3 |  |
|  |  |  |  |  |  | -96.020 |  | -93.520 |  | -92.020 |  | -91.020 |  |
|  | 7 | - |  | - |  | - |  | -94.3 |  | -92.5 |  | -91.3 |  |
|  |  | - |  | - |  | - |  | -97.020 |  | -95.220 |  | -94.020 |  |
| CA_1A-3A-8A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  |  |  |  |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3 (Note 4) | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -88.3 |  |
|  |  |  |  |  |  | -96.020 |  | -93.520 |  | -92.020 |  | -91.020 |  |
|  | 8 | - |  | -98.5 |  | -96.3 |  | -93.3 |  | - |  | - |  |
| CA_1A-3A-11A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 (Note 5) | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3 (Note 4) | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -88.3 |  |
|  |  |  |  |  |  | -96.020 |  | -93.520 |  | -92.020 |  | -91.020 |  |
|  | 11 | - |  | - |  | -99.3 |  | -96.3 |  | - |  | - |  |
| CA_1A-3A-18A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 (Note 5) | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3 (Note 4) | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -88.3 |  |
|  | 18 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
| CA_1A-3A-18A (NOTE 27) | 1 | - |  | - |  | -76.3 |  | - |  | - |  | - | FDD |
|  |  | - |  | - |  | -76.320 |  | - |  | - |  | - |  |
|  | 3 | - |  | - |  | -96.3 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -99.020 |  | - |  | - |  | - |  |
|  | 18 | - |  | - |  | -99.3 |  | - |  | - |  | - |  |
| CA_1A-3A-19A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  |  |  |  |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3 (Note 4) | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -88.3 |  |
|  | 19 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
| CA_1A-3A-21A12 | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -88.3 |  |
|  | 21 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
| CA_1A-3A-21A13 | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 21 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
| CA_1A-3A-41A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  |  |  |  |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3(Note 12) | - |  | - |  | -93.7 |  | -90.8 |  | -89.7 |  | -88.3 |  |
|  | 3(Note 13) |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 41(Note 14) | - |  | - |  | -92.6 |  | -90.0 |  | -88.5 |  | -91.3 | TDD |
| CA_1A-3A-42A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  |  |  |  |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3 (Note 4) | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -88.3 |  |
|  | 42 | - |  | - |  | -98.0 |  | -95.0 |  | -93.2 |  | -92.0 | TDD |
|  |  |  |  |  |  | -100.220 |  | -97.220 |  | -95.420 |  | -94.220 |  |
|  | 42 (Note 9) | - |  | - |  | -71.2 |  | -71.2 |  | -71.2 |  | -71.2 |  |
|  | 42 (Note 10) | - |  | - |  | -96.6 |  | -94.2 |  | -92.7 |  | -92.0 |  |
| CA_1A-3A-42A (NOTE 27) | 1 | - |  | - |  | -76.3 |  | - |  | - |  | - | FDD |
|  |  | - |  | - |  | -76.320 |  | - |  | - |  | - |  |
|  | 3 | - |  | - |  | -96.3 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -99.020 |  | - |  | - |  | - |  |
|  | 42 | - |  | - |  | -98 |  | - |  | - |  | - | TDD |
|  |  | - |  | - |  | -100.220 |  | - |  | - |  | - |  |
|  | 1 | - |  | - |  | -99.3 |  | - |  | - |  | - | FDD |
|  |  | - |  | - |  | -102.020 |  | - |  | - |  | - |  |
|  | 3 | - |  | - |  | -96.3 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -99.020 |  | - |  | - |  | - |  |
|  | 42 | - |  | - |  | -87 |  | - |  | - |  | - | TDD |
|  |  | - |  | - |  | -89.220 |  | - |  | - |  | - |  |
| CA_1A-3A-42A (NOTE 28) | 1 | - |  | - |  | -99.3 |  | - |  | - |  | - | FDD |
|  |  |  |  |  |  | -102.020 |  | - |  | - |  | - |  |
|  | 3 | - |  | - |  | -67.0 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -67.020 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -88.8 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -88.220 |  | - |  | - |  | - |  |
|  | 42 | - |  | - |  | -98 |  | - |  | - |  | - | TDD |
|  |  | - |  | - |  | -100.220 |  | - |  | - |  | - |  |
| CA_1A-3A-20A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  |  |  |  |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3 (Note 4) | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -88.3 |  |
|  | 20 | - |  | - |  | -96.3 |  | -93.3 |  | -90.5 |  | -89.3 |  |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -93.220 |  | -92.020 |  |
| CA_1A-3A-26A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3 (Note 4) | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -88.3 |  |
|  | 26 | - |  | - |  | -96.86 |  | -93.86 |  | -92.06 |  |  |  |
| CA_1A-3A-26A (NOTE 27) | 1 | - |  | - |  | -76.3 |  | - |  | - |  | - | FDD |
|  |  | - |  | - |  | -76.320 |  | - |  | - |  | - |  |
|  | 3 | - |  | - |  | -92.3 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -92.320 |  | - |  | - |  | - |  |
|  | 26 | - |  | - |  | -96.8 |  | - |  | - |  | - |  |
| CA_1A-3A-26A (NOTE 28) | 1 | - |  | - |  | -99.3 |  | - |  | - |  | - | FDD |
|  |  | - |  | - |  | -102.020 |  | - |  | - |  | - |  |
|  | 3 | - |  | - |  | -96.3 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -99.020 |  | - |  | - |  | - |  |
|  | 26 | - |  | - |  | -96.8 |  | - |  | - |  | - |  |
|  | 1 | - |  | - |  | -99.3 |  | - |  | - |  | - | FDD |
|  |  | - |  | - |  | -102.020 |  | - |  | - |  | - |  |
|  | 3 | - |  | - |  | -96.3 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -99.020 |  | - |  | - |  | - |  |
|  | 26 | - |  | - |  | -70.8 |  | - |  | - |  | - |  |
| CA_1A-3A-28A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 1(Note 21) | - |  | - |  | -89.1 |  | -88.7 |  | -88.3 |  | -88.0 |  |
|  | 3(Note 4) | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -88.3 |  |
|  | 3(Note 5) | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3(Note 21) | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 28 | - |  | - |  | -97.6 |  | -94.6 |  | -92.8 |  | -90.1 |  |
| CA_1A-3A-28A (NOTE 27) | 1 | - |  | - |  | -76.3 |  | - |  | - |  | - | FDD |
|  |  | - |  | - |  | -76.320 |  | - |  | - |  | - |  |
|  | 3 | - |  | - |  | -96.3 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -99.020 |  | - |  | - |  | - |  |
|  | 28 | - |  | - |  | -97.6 |  | - |  | - |  | - |  |
| CA_1A-3A-28A (NOTE 29) | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 28 | - |  | - |  | -97.6 |  | -94.6 |  | -92.8 |  | -90.1 |  |
|  | 1 | - |  | - |  | -99.3 |  | - |  | - |  | - | FDD |
|  |  | - |  | - |  | -102.020 |  | - |  | - |  | - |  |
|  | 3 | - |  | - |  | -92.3 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -95.020 |  | - |  | - |  | - |  |
|  | 28 | - |  | - |  | -97.6 |  | - |  | - |  | - |  |
| CA_1A-3A-28A (NOTE 28) | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 28 | - |  | - |  | -97.6 |  | -94.6 |  | -92.8 |  | -90.1 |  |
|  | 1 | - |  | - |  | -88.3 |  | - |  | - |  | - | FDD |
|  |  | - |  | - |  | -91.020 |  | - |  | - |  | - |  |
|  | 3 | - |  | - |  | -96.3 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -99.020 |  | - |  | - |  | - |  |
|  | 28 | - |  | - |  | -97.6 |  | - |  | - |  | - |  |
| CA_1A-3A-32A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3(NOTE4) | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -90.3 |  |
|  | 32 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 |  |
| CA_1A-3A-40A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 1(NOTE 25) | - |  | - |  | -91.0 |  | [-88.8] |  | [-87.2] |  | [-86.2] |  |
|  | 3 |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 |  |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3 (Note 4) | - |  | - |  | -93.3 |  | -90.8 |  | -89.3 |  | -88.3 |  |
|  | 3(NOTE 25) | - |  | - |  | -93.5 |  | -90.5 |  | -88.8 |  | -87.6 |  |
|  | 40 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | TDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 40(Note 19) |  |  |  |  | -99.3 |  | -92.2 |  | -90.6 |  | -89.5 |  |
|  | 40(Note 26) | - |  | - |  | [-92.6] |  | -90.6 |  | -89.3 |  | -88.2 |  |
|  |  | - |  | - |  | [-95.3]20 |  | -93.320 |  | -92.020 |  | -90.920 |  |
| CA_1A-5A-7A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - |  |
|  | 7 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 |  |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
| CA_1A-7A-8A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 7 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 |  |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 7(Note 8) | - |  | - |  | -87.3 |  | -86.7 |  | -86.3 |  | -86.0 |  |
|  |  | - |  | - |  | -90.020 |  | -89.420 |  | -89.020 |  | -88.720 |  |
|  | 8 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 8(Note 8) |  |  |  |  | -96.1 |  | -93.1 |  | - |  | - |  |
| CA_1A-7A-20A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 7 | - |  | - |  | - |  | -94.3 |  | -92.5 |  | -91.3 |  |
|  |  | - |  | - |  | - |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 20 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | - |  | - |  |
| CA_1A-7A-28A BW set2 | 1 |  |  |  |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 |  |
|  | 1(Note 8) |  |  |  |  | -89.1 |  | -87.7 |  | -88.3 |  | -88 |  |
|  | 7 |  |  |  |  |  |  | -94.3 |  | -92.5 |  | -91.3 |  |
|  |  |  |  |  |  |  |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 28 |  |  |  |  | -97.6 |  | -94.6 |  | -92.8 |  | -90.1 |  |
| CA_1A-8A-11A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 8 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 11 | - |  | - |  | -99.3 |  | -96.3 |  | - |  | - |  |
| CA_1A-8A-28A (Note 15) | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 8 | - |  | -98.5 |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 28 | - |  | - |  | -97.6 |  | -94.6 |  | -92.8 |  | -90.1 |  |
| CA_1A-8A-38A | 1 |  | - | - | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 |  | FDD |
|  |  |  | - | - | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |  |
|  | 8 |  | - | - | -96.3 |  | -93.3 |  | - |  | - |  |  |
|  | 38 |  | - | - | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 |  | TDD |
| CA_1A-8A-40A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 1(Note 25) | - |  | - |  | -91.0 |  | [-88.8] |  | [-87.2] |  | [-86.2] |  |
|  | 8 | - |  | -98.5 |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 40 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | TDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 40(Note 26) | - |  | - |  | [-92.7] |  | -91.2 |  | -89.7 |  | -88.7 |  |
| CA_1A-11A-18A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 11 | - |  | - |  | -99.3 |  | -96.3 |  | - |  | - |  |
|  | 18 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
| CA_1A-11A-28A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.7 | FDD |
|  | 1(Note 8) | - |  | - |  | -89.1 |  | -87.7 |  | -88.3 |  | -88 |  |
|  | 11 | - |  |  |  | -99.3 |  | -96.3 |  | - |  | - |  |
|  | 11(Note 9) | - |  | - |  | -74.5 |  | -74.5 |  | - |  | - |  |
|  | 28 | - |  | - |  | -97.6 |  | -94.6 |  | -92.8 |  | -90.1 |  |
| CA_1A-18A-28A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 18 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
|  | 28 | - |  | - |  | -97.8 |  | -94.8 |  | - |  | - |  |
|  | 28 (Note 7) | - |  | - |  | -93.3 |  | -91.8 |  | - |  | - |  |
| CA_1A-18A-42A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 18 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
|  | 42 | - |  | - |  | -98.0 |  | -95.0 |  | -93.2 |  | -92.0 | TDD |
|  |  | - |  | - |  | -100.220 |  | -97.220 |  | -95.420 |  | -94.220 |  |
| CA_1A-19A-21A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 19 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
|  | 21 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | - |  |
| CA_1A-19A-28A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 19 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
|  | 28 | - |  | - |  | -97.8 |  | -94.8 |  | - |  | - |  |
|  | 28 (Note 11) | - |  | - |  | -93.3 |  | -91.3 |  | - |  | - |  |
| CA_1A-19A-42A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 19 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
|  | 42 | - |  | - |  | -98.0 |  | -95.0 |  | -93.2 |  | -92.0 | TDD |
|  |  | - |  | - |  | -100.220 |  | -97.220 |  | -95.420 |  | -94.220 |  |
| CA_1A-21A-42A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 21 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | - |  |
|  | 42 | - |  | - |  | -98.0 |  | -95.0 |  | -93.2 |  | -92.0 | TDD |
|  |  | - |  | - |  | -100.220 |  | -97.220 |  | -95.420 |  | -94.220 |  |
| CA_1A-41A-42A | 1 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 41 | - |  | - |  | - |  | -94.3 |  | -92.5 |  | -91.3 | TDD |
|  |  | - |  | - |  | - |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 42 | - |  | - |  | - |  | -95.0 |  | -93.2 |  | -92.0 |  |
|  |  | - |  | - |  | - |  | -97.220 |  | -95.420 |  | -94.220 |  |
| CA_2A-4A-5A | 2 | - |  | - |  | -97,3 |  | -94,3 |  | -92.5 |  | -91,3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 |  |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - |  |
| CA_2A-4A-7A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 |  |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 7 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 |  |
| CA_2A-4A-7A (Note 24) | 2 | - |  | - |  | - |  | - |  | - |  | -86.3 | FDD |
|  | 2 | - |  | - |  | - |  | - |  | - |  | -89.020 |  |
|  | 4 | - |  | - |  | -99.3 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -10220 |  | - |  | - |  | - |  |
|  | 7 | - |  | - |  | - |  | - |  | - |  | -91.3 |  |
|  | 2 | - |  | - |  | -97.3 |  | - |  | - |  | - | FDD |
|  | 4 | - |  | - |  | -94.3 |  | - |  | - |  | - |  |
|  |  | - |  | - |  | -97.020 |  | - |  | - |  | - |  |
|  | 7 | - |  | - |  | - |  | - |  | - |  | -91.3 |  |
| CA_2A-4A-12A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  |  |  |  |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 2 (Note 8) | - |  | - |  | -97.0 |  | -94.0 |  | -92.2 |  | -91.0 |  |
|  |  |  |  |  |  | -99.720 |  | -96.720 |  | -94.920 |  | -93.720 |  |
|  | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 |  |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 4 (Note 8) | - |  | - |  | -89.3 |  | -88.8 |  | -88.3 |  | -87.8 |  |
|  |  | - |  | - |  | -92.020 |  | -91.520 |  | -91.020 |  | -90.520 |  |
|  | 12 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 12 (Note8) | - |  | - |  | -95.8 |  | -92.8 |  | - |  | - |  |
| CA_2A-4A-13A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 |  |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 13 | - |  | - |  | - |  | -93.3 |  | - |  | - |  |
| CA_2A-4A-29A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 |  |
|  |  | - |  | - |  | -10220 |  | -9920 |  | -97.220 |  | -9620 |  |
|  | 29 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
| CA_2A-4A-30A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 |  |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 30 | - |  | - |  | -98.3 |  | -95.3 |  | - |  | - |  |
|  |  | - |  | - |  | -101.020 |  | -98.020 |  | - |  | - |  |
| CA_2A-4A-71A | 2 | - |  | - |  | -97,3 |  | -94,3 |  | -92.5 |  | -91,3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 2 (NOTE 30) | - |  | - |  | -92.4 |  | -93.0 |  | -91.5 |  | -90.4 |  |
|  |  | - |  | - |  | -95.1 |  | -95.7 |  | -94.2 |  | -93.1 |  |
|  | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 |  |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 71 | - |  | - |  | -96.5 |  | -93.5 |  | -91.3 |  | -86.8 |  |
|  | 71 (NOTE 31) | - |  | - |  | - |  | - |  | - |  | -69.7 |  |
| CA_2A-5A-12A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - |  |
|  | 12 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
| CA_2A-5A-13A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - |  |
|  | 13 | - |  | - |  | - |  | -93.3 |  | - |  | - |  |
| CA_2A-5A-29A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - |  |
|  | 29 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
| CA_2A-5A-30A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - |  |
|  | 30 | - |  | - |  | -98.3 |  | -95.3 |  | - |  | - |  |
|  |  | - |  | - |  | -101.020 |  | -98.020 |  | - |  | - |  |
| CA_2A-5A-66A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - |  |
|  | 66 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | -92.8 |  |
|  |  | - |  | - |  | -101.520 |  | -98.520 |  | -96.720 |  | -95.520 |  |
| CA_2A-7A-12A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 7 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 |  |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 12 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
| CA_2A-12A-30A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 12 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 30 | - |  | - |  | -98.3 |  | -95.3 |  | - |  | - |  |
|  |  | - |  | - |  | -101.020 |  | -98.020 |  | - |  | - |  |
| CA_2A-12A-66A | 2 | - |  | - |  | -97.0 |  | -94.0 |  | -92.2 |  | -91.0 | FDD |
|  |  | - |  | - |  | -99.720 |  | -96.720 |  | -94.920 |  | -93.720 |  |
|  | 12 | - |  | - |  | -95.8 |  | -92.8 |  | - |  | - |  |
|  | 66 | - |  | - |  | -88.8 |  | -88.3 |  | -87.8 |  | -87.3 |  |
|  |  | - |  | - |  | -91.520 |  | -91.020 |  | -90.520 |  | -90.020 |  |
| CA_2A-13A-66A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 13 | - |  | - |  | - |  | -93.3 |  | - |  | - |  |
|  | 66 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | -92.8 |  |
|  |  | - |  | - |  | -101.520 |  | -98.520 |  | -96.720 |  | -95.520 |  |
| CA_2A-14A-30A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 14 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 30 | - |  | - |  | -98.3 |  | -95.3 |  | - |  | - |  |
|  |  | - |  | - |  | -101.020 |  | -98.020 |  | - |  | - |  |
| CA_2A-14A-66A | 2 | - |  | - |  | -97.0 |  | -94.0 |  | -92.2 |  | -91.0 | FDD |
|  |  | - |  | - |  | -99.720 |  | -96.720 |  | -94.920 |  | -93.720 |  |
|  | 14 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 66 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | -92.8 |  |
|  |  | - |  | - |  | -101.520 |  | -98.520 |  | -96.720 |  | -95.520 |  |
| CA_2A-29A-30A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 29 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 30 | - |  | - |  | -98.3 |  | -95.3 |  | - |  | - |  |
|  |  | - |  | - |  | -101.020 |  | -98.020 |  | - |  | - |  |
| CA_2A-30A-66A | 2 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 30 | - |  | - |  | -98.3 |  | -95.3 |  | - |  | - |  |
|  |  | - |  | - |  | -101.020 |  | -98.020 |  | - |  | - |  |
|  | 66 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | -92.8 |  |
|  |  | - |  | - |  | -101.520 |  | -98.520 |  | -96.720 |  | -95.520 |  |
| CA_2A-66A-71A | 2 | - |  | - |  | -97,3 |  | -94,3 |  | -92.5 |  | -91,3 | FDD |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 66 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | -92.8 |  |
|  |  | - |  | - |  | -101.520 |  | -98.520 |  | -96.720 |  | -95.520 |  |
|  | 71 | - |  | - |  | -96.5 |  | -93.5 |  | -91.3 |  | -86.8 |  |
| CA_3A-7A-8A | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 7 | - |  | - |  | - |  | -94.3 |  | -92.5 |  | -91.3 |  |
|  | 7 (Note8) | - |  | - |  | - |  | -86.7 |  | -86.3 |  | -86.0 |  |
|  | 8 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 8 (Note8) | - |  | - |  | -96.1 |  | -93.1 |  | - |  | - |  |
| CA_3A-7A-20A | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 7 | - |  | - |  | - |  | -94.3 |  | -92.5 |  | -91.3 |  |
|  |  | - |  | - |  | - |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 20 | - |  | - |  | -96.3 |  | -93.3 |  | -90.5 |  | -89.3 |  |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -93.220 |  | -92.020 |  |
| CA_3A-7A-28A | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 7 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 |  |
|  |  | - |  | - |  | -10020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 28 | - |  | - |  | -97.6 |  | -94.6 |  | -92.8 |  | -90.1 |  |
| CA_3A-7A-38A | 3 |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 7 (Note 19) |  |  |  |  |  |  | [-91.2] |  | [-89.7] |  | [-88.6] | TDD |
|  | 3 |  |  |  |  | [-93.6] |  | [-91.0] |  | [-89.5] |  | [-88.4] |  |
| CA_3A-7A-42A | 3 |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 7 |  |  |  |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 |  |
|  |  |  |  |  |  | -10020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 42 |  |  |  |  | -98.0 |  | -95.0 |  | -93.2 |  | -92.0 | TDD |
|  |  |  |  |  |  | -100.220 |  | -97.220 |  | -95.420 |  | -94.220 |  |
|  | 42 (Note 9) |  |  |  |  | -71.2 |  | -71.2 |  | -71.2 |  | -71.2 |  |
|  | 42 (Note 10) |  |  |  |  | -96.6 |  | -94.2 |  | -92.7 |  | -92.0 |  |
| CA_3A-8A-11A | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 8 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 11 | - |  | - |  | -99.3 |  | -96.3 |  | - |  | - |  |
| CA_3A-8A-28A (Note 16) | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 8 | - |  | -98.5 |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 28 | - |  | - |  | -97.8 |  | -94.8 |  | -93.0 |  | -90.3 |  |
| CA_3A-8A-40A | 3 |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 3 (Note 25) | - |  | - |  | -93.5 |  | -90.5 |  | -88.8 |  | -87.6 |  |
|  | 8 | - |  | -98.5 |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 40 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | TDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 40(Note 19) | - |  | - |  | -94.7 |  | -92.2 |  | -90.6 |  | -89.5 |  |
| CA_3A-11A-28A | 3 | - |  | - |  | -96.0 |  | -93.0 |  | -91.2 |  | -90.0 | FDD |
|  |  | - |  | - |  | -98.720 |  | -95.720 |  | -93.920 |  | -92.720 |  |
|  | 11 | - |  | - |  | -98.8 |  | -95.8 |  | - |  | - |  |
|  | 11 (Note 9) | - |  | - |  | -74.5 |  | -74.5 |  | - |  | - |  |
|  | 28 | - |  | - |  | -97.6 |  | -94.6 |  | -92.8 |  | -90.1 |  |
| CA_3A-18A-42A | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 18 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
|  | 42 | - |  | - |  | -98.0 |  | -95.0 |  | -93.2 |  | -92.0 | TDD |
|  |  | - |  | - |  | -100.220 |  | -97.220 |  | -95.420 |  | -94.220 |  |
|  | 42 (Note 9) | - |  | - |  | -71.2 |  | -71.2 |  | -71.2 |  | -71.2 |  |
|  | 42 (Note 10) | - |  | - |  | -96.6 |  | -94.2 |  | -92.7 |  | -92.0 |  |
| CA_3A-19A-42A | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 19 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
|  | 42 | - |  | - |  | -98.0 |  | -95.0 |  | -93.2 |  | -92.0 | TDD |
|  |  | - |  | - |  | -100.220 |  | -97.220 |  | -95.420 |  | -94.220 |  |
|  | 42 (Note 9) | - |  | - |  | -71.2 |  | -71.2 |  | -71.2 |  | -71.2 |  |
|  | 42 (Note 10) | - |  | - |  | -96.6 |  | -94.2 |  | -92.7 |  | -92.0 |  |
| CA_3A-20A-32A | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  | 20 | - |  | - |  | -96.3 |  | -93.3 |  | -90.5 |  | -89.3 |  |
|  | 32 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | -92.8 |  |
| CA_3A-20A-42A | 3 |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 20 |  |  |  |  | -96.3 |  | -93.3 |  | -90.5 |  | -89.3 |  |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -93.220 |  | -92.020 |  |
|  | 42 |  |  |  |  | -98.0 |  | -95.0 |  | -93.2 |  | -92.0 | TDD |
|  |  |  |  |  |  | -100.220 |  | -97.220 |  | -95.420 |  | -94.220 |  |
|  | 42 (Note 9) |  |  |  |  | -71.2 |  | -71.2 |  | -71.2 |  | -71.2 |  |
|  | 42 (Note 10) |  |  |  |  | -96.6 |  | -94.2 |  | -92.7 |  | -92.0 |  |
| CA_3A-21A-42A | 3 | - |  | - |  | -96.0 |  | -93.0 |  | -91.2 |  | -90.0 | FDD |
|  |  | - |  | - |  | -98.720 |  | -95.720 |  | -93.920 |  | -92.720 |  |
|  | 21 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | - |  |
|  | 42 | - |  | - |  | -70.7 |  | -70.7 |  | -70.7 |  | -70.7 | TDD |
| CA_3A-21A-42A | 3 | - |  | - |  | -96.0 |  | -93.0 |  | -91.2 |  | -90.0 | FDD |
|  |  | - |  | - |  | -98.720 |  | -95.720 |  | -93.920 |  | -92.720 |  |
|  | 21 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | - |  |
|  | 42 | - |  | - |  | -96.1 |  | -93.7 |  | -92.2 |  | -91.5 | TDD |
| CA_3A-28A-38A | 3 |  |  |  |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  |  |  |  |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 28 |  |  |  |  | -97.8 |  | -94.8 |  | -93.0 |  | -90.3 |  |
|  | 38 |  |  |  |  | -99.1 |  | -96.1 |  | -94.3 |  | -93.1 | TDD |
| CA_3A-28A-41A (Note 18) | 3 | - |  | - |  | [-93.3] |  | [-90.3] |  | [-88.5] |  | [-87.2] | FDD |
|  | 28 | - |  | - |  | -97.8 |  | -94.8 |  | - |  | - |  |
|  | 41 | - |  | - |  | -96.817 |  | -93.817 |  | -9217 |  | -90.817 | TDD |
|  |  | - |  | - |  | -99.517,20 |  | -96.517,20 |  | -94.717,20 |  | -93.517,20 |  |
| CA_3A-28A-41A (Note 19) | 3 | - |  | - |  | -96.3 |  | -93.3 |  | -91.5 |  | -90.3 | FDD |
|  |  | - |  | - |  | -99.020 |  | -96.020 |  | -94.220 |  | -93.020 |  |
|  | 28 | - |  | - |  | -97.8 |  | -94.8 |  | - |  | - |  |
|  | 41 | - |  | - |  | [-92.6]17 |  | [-90]17 |  | [-88.5]17 |  | [-87.4]17 | TDD |
| CA_3A-41A-42A (Note 19) | 3 | - |  | - |  | -95.8 |  | -92.8 |  | -91.0 |  | -89.8 | FDD |
|  | 41 (Note 17) | - |  | - |  | [-92.6] |  | [90.0] |  | [-88.5] |  | [-87.4] | TDD |
|  | 42 |  |  |  |  | -97.5 |  | -94.5 |  | -92.7 |  | -91.5 |  |
|  | 42 (Note 9) |  |  |  |  | -70.7 |  | -70.7 |  | -70.7 |  | -70.7 |  |
|  | 42 (Note 10) |  |  |  |  | -96.1 |  | -93.7 |  | -92.2 |  | -91.5 |  |
| CA_3A-41A-42A (Note 18, Note 23) | 3 |  |  |  |  | [-92.8] |  | [-89.8] |  | [-88] |  | [-86.7] | FDD |
|  | 41 (Note 17) |  |  |  |  | -96.8 |  | -93.8 |  | -92.0 |  | -90.8 | TDD |
|  | 42 |  |  |  |  | -97.5 |  | -94.5 |  | -92.7 |  | -91.5 |  |
| CA_3A-41A-42A (Note 22, Note 23) | 3 |  |  |  |  | -95.8 |  | -92.8 |  | -91.0 |  | -89.8 | FDD |
|  | 41 (Note 17) |  |  |  |  | -96.8 |  | -93.8 |  | -92.0 |  | -90.8 | TDD |
|  | 42 |  |  |  |  | -97.5 |  | -94.5 |  | -92.7 |  | -91.5 |  |
| CA_4A-5A-12A | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - |  |
|  | 12 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
| CA_4A-5A-13A | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -10220 |  | -9920 |  | -97.220 |  | -9620 |  |
|  | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - |  |
|  | 13 | - |  | - |  | - |  | -93.3 |  | - |  | - |  |
| CA_4A-5A-29A | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - |  |
|  | 29 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
| CA_4A-5A-30A | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - |  |
|  | 30 | - |  | - |  | -98.3 |  | -95.3 |  | - |  | - |  |
|  |  | - |  | - |  | -101.020 |  | -98.020 |  | - |  | - |  |
| CA_4A-7A-12A | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 7 | - |  | - |  | -97.3 |  | -94.3 |  | -92.5 |  | -91.3 |  |
|  |  | - |  | - |  | -100.020 |  | -97.020 |  | -95.220 |  | -94.020 |  |
|  | 12 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
| CA_4A-12A-30A | 4 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | -93.3 | FDD |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | -96.020 |  |
|  | 4 (Note 8) | - |  | - |  | -89.7 |  | -89.2 |  | -88.7 |  | -88.2 |  |
|  |  | - |  | - |  | -92.420 |  | -91.920 |  | -91.420 |  | -90.920 |  |
|  | 12 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 30 | - |  | - |  | -98.3 |  | -95.3 |  | - |  | - |  |
|  |  | - |  | - |  | -101.020 |  | -98.020 |  | - |  | - |  |
| CA_4A-29A-30A | 4 | - |  | - |  | -98.9 |  | -95.9 |  | -94.1 |  | -92.9 | FDD |
|  |  | - |  | - |  | -101.620 |  | -98.620 |  | -96.820 |  | -95.620 |  |
|  | 29 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 30 | - |  | - |  | -97.8 |  | -94.8 |  | - |  | - |  |
|  |  | - |  | - |  | -100.520 |  | -97.520 |  | - |  | - |  |
| CA_5A-30A-66A | 5 | - |  | - |  | -97.3 |  | -94.3 |  | - |  | - | FDD |
|  | 30 | - |  | - |  | -98.3 |  | -95.3 |  | - |  | - |  |
|  |  | - |  | - |  | -101.020 |  | -98.020 |  | - |  | - |  |
|  | 66 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | -92.8 |  |
|  |  | - |  | - |  | -101.520 |  | -98.520 |  | -96.720 |  | -95.520 |  |
| CA_7A-8A-20A | 7 | - |  | - |  | - |  | -94.3 |  | -92.5 |  | -91.3 | FDD |
|  | 8 | - |  | -98.5 |  | -96.3 |  | -93.3 |  | - |  | - |  |
|  | 20 | - |  | - |  | [-96.3] |  | [-93.3] |  | - |  | - |  |
|  |  | - |  | - |  | [-99.0]20 |  | [-96.0]20 |  | - |  | - |  |
| CA_8A-11A-28A | 8 | - |  | - |  | -96.1 |  | -93.1 |  | - |  | - | FDD |
|  | 11 | - |  | - |  | -99.3 |  | -96.3 |  | - |  | - |  |
|  | 28 | - |  | - |  | -97.6 |  | -94.6 |  | -92.8 |  | -90.1 |  |
| CA_8A-20A-28A | 8 |  |  |  |  | -96.3 |  | -93.3 |  |  |  |  | FDD |
|  | 20 |  |  |  |  |  |  | -93.3 |  | -90.5 |  | -89.3 |  |
|  |  |  |  |  |  |  |  | -96.020 |  | -93.220 |  | -92.020 |  |
|  | 28 |  |  |  |  | -97.8 |  | -94.8 |  | -93.0 |  | -90.3 |  |
| CA_12A-30A-66A | 12 | - |  | - |  | -95.8 |  | -92.8 |  | - |  | - | FDD |
|  | 30 | - |  | - |  | -97.8 |  | -94.8 |  | - |  | - |  |
|  |  | - |  | - |  | -100.520 |  | -97.520 |  | - |  | - |  |
|  | 66 | - |  | - |  | -88.8 |  | -88.3 |  | -87.8 |  | -87.3 |  |
|  |  | - |  | - |  | -91.520 |  | -91.020 |  | -90.520 |  | -90.020 |  |
| CA_14A-30A-66A | 14 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - | FDD |
|  | 30 | - |  | - |  | -97.8 |  | -94.8 |  | - |  | - |  |
|  |  | - |  | - |  | -100.520 |  | -97.520 |  | - |  | - |  |
|  | 66 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | -92.8 |  |
|  |  | - |  | - |  | -101.520 |  | -98.520 |  | -96.720 |  | -95.520 |  |
| CA_19A-21A-42A | 19 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - | FDD |
|  | 21 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
|  |  | - |  | - |  | -102.020 |  | -99.020 |  | -97.220 |  | - |  |
|  | 42 | - |  | - |  | -98.0 |  | -95.0 |  | -93.2 |  | -92.0 | TDD |
|  |  | - |  | - |  | -100.220 |  | -97.220 |  | -95.420 |  | -94.220 |  |
| CA_29A-46A-66A | 29 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - | FDD |
|  | 46 | - |  | - |  | - |  | - |  | - |  | -88.5 | TDD |
|  | 66 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | -92.8 | FDD |
|  |  | - |  | - |  | -101.520 |  | -98.520 |  | -96.720 |  | -95.520 |  |
| CA_29A-66A-70A | 29 | - |  | - |  | -96.3 |  | -93.3 |  | - |  | - | FDD |
|  | 66 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | -92.8 |  |
|  |  | - |  | - |  | -101.520 |  | -98.520 |  | -96.720 |  | -95.520 |  |
|  | 70 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
| CA_66A-70A-71A | 66 | - |  | - |  | -98.8 |  | -95.8 |  | -94.0 |  | -92.8 | FDD |
|  |  | - |  | - |  | -101.520 |  | -98.520 |  | -96.720 |  | -95.520 |  |
|  | 70 | - |  | - |  | -99.3 |  | -96.3 |  | -94.5 |  | - |  |
|  | 70(Note 8) | - |  | - |  | -89.3 |  | -88.8 |  | -88.5 |  | - |  |
|  | 71 | - |  | - |  | -96.5 |  | -93.5 |  | -91.2 |  | -86.8 |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.5.5-4) Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: Reference sensitivity for the case that the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. Note 5: Reference sensitivity for the case that the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. Note 6: 6 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 7: These requirements apply when the uplink is active in Band 18 and the downlink channels in Band 28 are confined within the restricted frequency range specified for this CA configuration (Table 5.2A-2). Note 8: Reference sensitivity for the high band for which the 3rd harmonic of the low band is within transmission bandwidth, as specified Note 5 and 6 in Table 7.3A.0-0a. Note 9: Reference sensitivity for the high band for which the 2nd harmonic of the low band is within transmission bandwidth, as specified Note 9 and 10 in Table 7.3A.0-0a. Note 10: Reference sensitivity for the high band for which the 2nd harmonic of the low band is within transmission bandwidth, as specified Note 11 in Table 7.3A.0-0a. Note 11: These requirements apply when the uplink is active in Band 19 and the downlink channels in Band 28 are allocated at the middle of the restricted frequency range specified for this CA configuration (Table 5.2A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 19. NOTE 12: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 13: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 14: The B41 requirements also apply to the supported CA_1A-41A. NOTE 15: These requirements apply when the uplink is active in Band 1 or Band 8. NOTE 16: These requirements apply when the uplink is active in Band 3 or Band 8. NOTE 17: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690MHz. NOTE 18: These requirements apply when the uplink is active in Band 41. NOTE 19: These requirements apply when the uplink is active in Band 3. NOTE 20: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 21: These requirements apply when the uplink is active in a lower-frequency band and is within a specified frequency range such that transmitter harmonics fall within the downlink transmission bandwidth assigned in a higher band. NOTE 22: These requirements apply when the uplink is active in Band 42. NOTE 23: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 24: These requirements apply when UL CA is configured and the uplink is active in Band 2 and Band 4. NOTE 25: These requirements apply when the uplink is active in Band 40. NOTE 26: These requirements apply when the uplink is active in Band 1. NOTE 27: These requirements apply when UL CA is configured and the uplink is active in Band X and Band Y in CA_XA-YA-ZA. NOTE 28: These requirements apply when UL CA is configured and the uplink is active in Band Y and Band Z in CA_XA-YA-ZA. NOTE 29: These requirements apply when UL CA is configured and the uplink is active in Band Z and Band X in CA_XA-YA-ZA. NOTE 30: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. These requirements also apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 31: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band carrier aggregation configurations the ΔRIB,c in Tables 7.3.3-1A, 7.3.3-1B and 7.3.3-1C shall be applied for applicable bands.

Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and an E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then:

-	When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied

-	When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations

The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration.

The reference receive sensitivity (REFSENS) requirement for inter-band specified in Table 7.3A.5.5-3 shall be met for an uplink CA configurations than or equal to that specified in Table 7.3.5-2.

NOTE:	Table 7.3A.5.5-4 does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors.

Table 7.3A.5.5-4: Void

For combinations of intra-band contiguous and inter-band carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-5. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-5: Reference sensitivity QPSK PREFSENS for intra-band contiguous + inter-band

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| CA_1A-3C (Note 9) | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 3 | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
| CA_1A-3C (Note 10) | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
| CA_1A-3C (Note 9) | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 3 | - | - | -93.3 | -90.8 | -89.3 | -88.3 |  |
| CA_1A-3C (Note 10) | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
| CA_1A-41C | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_1A-42C | 1 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
| CA_2A-5B | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
| CA_2A-7C | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_2A-12B | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 12 | - | - | -96.3 | -93.3 | - | - |  |
| CA_2A-66C | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
| CA_2C-5A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
| CA_2C-29A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 29 | - | - | -96.3 | -93.3 | - | - |  |
| CA_2C-30A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 30 | - | - | -98.3 | -95.3 | - | - |  |
|  |  | - | - | -101.016 | -98.016 | - | - |  |
| CA_2C-66A | 2 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
| CA_3C-5A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 5 | - | - | -97.3 | -94.3 | - | - |  |
| CA_3C-8A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  | 8 | - | -98.5 | -96.3 | -93.3 | - | - |  |
| CA_3A-7B | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 7 | - | - | -97.3 | - | -92.5 | - |  |
|  |  | - | - | -100.016 | - | -95.216 | - |  |
| CA_3A-7C | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_3A-40C (Note 12) | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 40 | - | - | -94.7 | -92.2 | -90.6 | -89.5 | TDD |
| CA_3A-40C (Note 13) | 3 | - | - | -93.5 | -90.5 | -88.8 | -87.6 | FDD |
|  | 40 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| CA_3A-41C (Note 12) | 3 | - | - | [-93.3] | [-90.3] | [-88.5] | [-87.2] | FDD |
|  | 41 | - | - | -96.811 | -93.811 | -9211 | -90.811 | TDD |
|  |  | - | - | -99.511,16 | -96.511,16 | -94.711,16 | -93.511,16 |  |
| CA_3A-41C (Note 14) | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 41 | - | - | [-92.6]11 | [-90]11 | [-88.5]11 | [-87.4]11 | TDD |
| CA_3A-42C | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
|  | 42 (Note 4) | - | - | -71.2 | -71.2 | -71.2 | -71.2 |  |
|  | 42 (Note 5) | - | - | -96.6 | -94.2 | -92.7 | -92.0 |  |
| CA_3A-42C (Note 17) | 3 | - | - | -67.0 | - | - | - | FDD |
|  |  | - | - | -67.06 | - | - | - |  |
|  | 42 | - | - | -71.7 | - | - | - | TDD |
|  |  | - | - | -88.8 | - | - | - |  |
|  | 3 | - | - | -88.26 | - | - | - | FDD |
|  |  | - | - | -97.1 | - | - | - |  |
|  | 42 | - | - | -67.0 | - | - | - | TDD |
|  |  | - | - | -67.06 | - | - | - |  |
|  | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
| CA_3C-7A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_3C-20A | 3 |  |  | -96.3 | -93.3 | -91.5 | -90.3 |  |
|  |  |  |  | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 20 |  |  | -96.3 | -93.3 | -90.5 | -89.3 |  |
|  |  |  |  | -99.016 | -96.016 | -93.216 | -92.016 |  |
| CA_3C-28A | 3 | - | - | -96.3 | -93.3 | -91.5 | -90.3 | FDD |
|  |  | - | - | -99.016 | -96.016 | -94.216 | -93.016 |  |
|  | 28 | - | - | -97.8 | -94.8 | -93.0 | -90.3 |  |
| CA_4A-7C | 4 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_5A-12B | 5 | - | - | -97.3 | -94.3 | - | - | FDD |
|  | 12 | - | - | -96.3 | -93.3 | - | - |  |
| CA_5A-40C | 5 | - | -99.5 | -97.3 | -94.3 | - | - | FDD |
|  | 40 | - | - | - | -96.3 | -94.5 | -93.3 | TDD |
|  |  | - | - | - | -99.016 | -97.216 | -96.016 |  |
| CA_5B-30A | 5 | - | - | -97.3 | -94.3 | - | - | FDD |
|  | 30 | - | - | -98.3 | -95.3 | - | - |  |
|  |  | - | - | -101.016 | -98.016 | - | - |  |
| CA_5B-66A | 5 | - | - | -97.3 | -94.3 | - | - | FDD |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
| CA_7B-28A | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 28 | - | - | -97.6 | -94.6 | -92.8 | -90.1 |  |
| CA_7C-28A | 7 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | FDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
|  | 28 | - | - | -97.6 | -94.6 | -92.8 | -90.1 |  |
| CA_8A-40C | 8 |  |  | -96.3 | -93.3 |  |  |  |
|  | 40 |  |  |  | -96.3 | -94.5 | -93.3 |  |
|  |  |  |  |  | -99.016 | -97.216 | -96.016 |  |
| CA_8A-42C | 8 | -101.3 | -98.3 | -96.1 | -93.1 | - | - | FDD |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
|  | 42 (Note 8) | - | - | -83.8 | -83.7 | -83.6 | -83.5 |  |
| CA_11A-41C | 11 | - | - | -100.0 | -97.0 | - | - | FDD |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_11A-42C | 11 | - | - | -100.0 | -97.0 | - | - | FDD |
|  | 42 | - | - | -97.5 | -94.5 | -92.7 | -91.5 | TDD |
|  |  | - | - | -99.716 | -96.716 | -94.916 | -93.716 |  |
| CA_18A-42C | 18 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
| CA_19A-42C | 19 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
| CA_21A-42C | 21 | - | - | -99.3 | -96.3 | -94.5 | - | FDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | - |  |
|  | 42 | - | - | -98.0 | -95.0 | -93.2 | -92.0 | TDD |
|  |  | - | - | -100.216 | -97.216 | -95.416 | -94.216 |  |
| CA_25A-41C | 25 | - | - | -95.8 | -92.8 | -91.0 | -89.8 | FDD |
|  |  | - | - | -98.516 | -95.516 | -93.716 | -92.516 |  |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_26A-41C | 26 | - | - | -96.86 | -93.86 | -92.06 | - | FDD |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_28A-40C (Note 15) | 28 | - | - | -97.8 | -94.8 | -93 | -90.3 | FDD |
|  | 40 | - | - | -94.4 | -92.2 | -90.7 | -89.8 | TDD |
| CA_28A-40C (Note 13) | 28 | - | - | -96.1 | -93.4 | -91.8 | -89.1 | FDD |
|  | 40 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
| CA_28A-41C | 28 | - | - | -97.8 | -94.8 | - | - | FDD |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 | TDD |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_28A-42C | 28 | - | - | -97.3 | -94.3 | -92.5 | -89.8 | FDD |
|  | 42 | - | - | -96.5 | -93.5 | -91.7 | -90.5 | TDD |
|  | 42 (Note 7) | - | - | -84.9 | -84.6 | -84.3 | -84.1 | TDD |
| CA_29A-66C | 29 | - | - | -96.3 | -93.3 | - | - | FDD |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 |  |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
| CA_29A-70C | 29 | - | - | -96.3 | -93.3 | - | - | FDD |
|  | 70 | - | - | -99.3 | -96.3 | -94.5 | -93.3 |  |
| CA_38A-40C | 38 | - | - | - | -96.3 | -94.5 | -93.3 | TDD |
|  | 40 | - | - | - | -96.3 | -94.5 | -93.3 |  |
|  |  | - | - | - | -99.016 | -97.216 | -96.016 |  |
| CA_39A-41C | 39 | - | - | - | -96.3 | -94.5 | -93.3 | TDD |
|  |  | - | - | - | -99.016 | -97.216 | -96.016 |  |
|  | 41 | - | - | -97.3 | -94.3 | -92.5 | -91.3 |  |
|  |  | - | - | -100.016 | -97.016 | -95.216 | -94.016 |  |
| CA_39C-41A | 39 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | TDD |
|  |  | - | - | -102.016 | -99.016 | -97.216 | -96.016 |  |
|  | 41 | - | - | - | - | - | -91.3 |  |
|  |  | - | - | - | - | - | -94.016 |  |
| CA_41A-42C | 41 | - | - | - | -93.9 | -92.1 | -90.9 | TDD |
|  |  | - | - | - | -96.616 | -94.816 | -93.616 |  |
|  | 42 | - | - | -97.5 | -94.5 | -92.7 | -91.5 |  |
|  |  | - | - | -99.716 | -96.716 | -94.916 | -93.716 |  |
| CA_41C-42A | 41 | - | - | - | -93.9 | -92.1 | -90.9 | TDD |
|  |  | - | - | - | -96.616 | -94.816 | -93.616 |  |
|  | 42 | - | - | -97.5 | -94.5 | -92.7 | -91.5 |  |
|  |  | - | - | -99.716 | -96.716 | -94.916 | -93.716 |  |
| CA_41A-48C | 41 |  |  | - | -94.3 | -92.5 | -91.3 | TDD |
|  | 48 |  |  | -97.5 | -94.5 | -92.7 | -91.5 |  |
| CA_41C-48A | 41 |  |  |  | -94.3 | -92.5 | -91.3 | TDD |
|  | 48 |  |  | -97.5 | -94.5 | -92.7 | -91.5 |  |
| CA_46C-66A | 46 | - | - | - | - | - | -88.5 | TDD |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
| CA_46A-66C | 46 | - | - | - | - | - | -88.5 | TDD |
|  | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
| CA_66A-70A | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
|  | 70 | - | - | -99.3 | -96.3 | -94.5 | - |  |
| CA_66A-70C | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
|  | 70 | - | - | -99.3 | -96.3 | -94.5 | -93.3 |  |
| CA_66C-70A | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
|  | 70 | - | - | -99.3 | -96.3 | -94.5 | - |  |
| CA_66C-71A | 66 | - | - | -98.8 | -95.8 | -94.0 | -92.8 | FDD |
|  |  | - | - | -101.516 | -98.516 | -96.716 | -95.516 |  |
|  | 71 | - | - | -96.5 | -93.5 | -91.2 | -86.8 |  |
| CA_70C-71A | 70 | - | - | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
|  | 70 | - | - | -89.318 | -88.818 | -88.518 | -88.318 |  |
|  | 71 | - | - | -96.5 | -93.5 | -91.2 | -86.8 |  |
| Note 1: The transmitter shall be set to maximum output power level (Table 7.3A.5.5-4) Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 Note 3: The signal power is specified per port Note 4: Reference sensitivity for the high band for which the 2nd harmonic of the low band is within transmission bandwidth, as specified Note 9 and 10 in Table 7.3A.0-0a. Note 5: Reference sensitivity for the high band for which the 2nd harmonic of the low band is within transmission bandwidth, as specified Note 11 in Table 7.3A.0-0a. Note 6: 6indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. Note 7: Reference sensitivity for the high band for which the 5th harmonic of the low band is within transmission bandwidth, as specified Note 15 and 16 in Table 7.3A.0-0a. Note 8: Reference sensitivity for the high band for which the 4th harmonic of the low band is within transmission bandwidth, as specified Note 12 and 13 in Table 7.3A.0-0a. Note 9: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. Note 10: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. Note 11: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. Note 12: These requirements apply when the uplink is active in Band 3. Note 13: These requirements apply when the uplink is active in Band 40. Note 14: These requirements apply when the uplink is active in Band 41. Note 15: These requirements apply when the uplink is active in Band 28. Note 16: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 17: These requirements apply when UL CA is configured and the uplink is active in Band X and Band Y in CA_XA-YC. Note 18: Reference sensitivity for the high band for which the 3rd harmonic of the low band is within transmission bandwidth, as specified Note 5 and 6 in Table 7.3A.0-0a. |  |  |  |  |  |  |  |  |

For combinations of intra-band non-contiguous and inter-band carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-6. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-6: Reference sensitivity QPSK PREFSENS for intra-band non-contiguous + inter-band

| CA Configuration | Testpoint ID | PCC |  |  |  | SCC1 |  |  | SCC2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Band | NRB | Requirement (dBm) |  | Band | NRB | Requirement (dBm) | Band | NRB |  |  | Requirement (dBm) |
| CA_1A-1A-7A | 1 | 1 | 100 | -93.3 -96.08 |  | 1 | 100 | -92.81 -95.51,8 | 7 | 100 |  |  | -91.3 -94.08 |
|  | 2 | 7 | 100 | -91.3 -94.08 |  | 1 | 100 | -93.3 -96.08 | 1 | 100 |  |  | -93.3 -96.08 |
|  | 3 | 1 | 100 | -93.3 -96.08 |  | 1 | 25 | -98.81 -101.51,8 | 7 | 100 |  |  | -91.3 -94.08 |
|  | 4 | 1 | 100 | -93.3 -96.08 |  | 1 | 100 | -92.81 -95.51,8 | 7 | 100 |  |  | -91.3 -94.08 |
|  | 5 | 1 | 100 | -93.3 -96.08 |  | 1 | 25 | -98.81 -101.51,8 | 7 | 100 |  |  | -91.3 -94.08 |
| CA_1A-7A-7A | 1 | 1 | 100 | -93.3 -96.08 |  | 7 | 100 | -91.3 -94.08 | 7 | 100 |  |  | -91.3 -94.08 |
|  | 2 | 7 | 100 | -91.3 -94.08 |  | 7 | 100 | -91.3 -94.08 | 1 | 100 |  |  | -93.3 -96.08 |
|  | 3 | 7 | 100 | -91.3 -94.08 |  | 7 | 100 | -91.3 -94.08 | 1 | 100 |  |  | -93.3 -96.08 |
|  | 4 | 7 | 100 | -91.3 -94.08 |  | 7 | 100 | -91.3 -94.08 | 1 | 100 |  |  | -93.3 -96.08 |
|  | 5 | 7 | 100 | -91.3 -94.08 |  | 7 | 25 | -97.3 -100.08 | 1 | 100 |  |  | -93.3 -96.08 |
|  | 6 | 7 | 100 | -91.3 -94.08 |  | 7 | 25 | -97.3 -100.08 | 1 | 100 |  |  | -93.3 -96.08 |
| CA_2A-2A-XA or CA_XA-2A-2A | 1 | 2 | 100 | -91.3 |  | 2 | 25 | -90.11 | X | Highest NRB |  |  | PREFSENS |
|  | 2 | 2 | 100 | -94.0 |  | 2 | 25 | -90.11 | X | Highest NRB |  |  | PREFSENS |
|  | 3 | 2 | 100 | -91.3 -94.08 |  | 2 | 100 | -86.71 | X | Highest NRB |  |  | PREFSENS |
|  | 4 | X | Highest NRB | PREFSENS |  | 2 | 100 | -91.3 | 2 | 100 |  |  | -91.3 |
| CA_2A-2A-71A | 1 | 2 | 100 | -91.3 |  | 2 | 25 | -90.11 | 71 | Highest NRB |  |  | -86.6 |
|  | 2 | 2 | 100 | -94.0 |  | 2 | 25 | -90.11 | 71 | Highest NRB |  |  | -86.6 |
|  | 3 | 2 | 100 | -91.3 -94.08 |  | 2 | 100 | -86.71 | 71 | Highest NRB |  |  | -86.6 |
|  | 4 | 71 | Highest NRB | -86.8 |  | 2 | 100 | -91.3 | 2 | 100 |  |  | -91.3 |
| CA_2A-2A-71A(NOTE 9, 10) | 1 | 2 | 100 | -91.3 |  | 2 | 25 | -90.11 | 71 | 100 |  |  | -86.8 |
|  | 2 | 2 | 100 | -94.0 |  | 2 | 25 | -90.11 | 71 | 100 |  |  | -86.8 |
|  | 3 | 2 | 100 | -91.3 -94.08 |  | 2 | 100 | -86.71 | 71 | 100 |  |  | -69.7 |
|  | 4 | 2 | 100 | -91.3 -94.08 |  | 2 | 100 | -86.71 | 71 | 100 |  |  | -86.8 |
|  | 5 | 2 | 100 | -91.3 -94.08 |  | 2 | 100 | -86.71 | 71 | 100 |  |  | -86.8 |
|  | 6 | 71 | 25 | -96.5 |  | 2 | 25 | -92.7 | 2 | 100 |  |  | -91.3 |
|  | 7 | 71 | 50 | -93.5 |  | 2 | 50 | -93.3 | 2 | 100 |  |  | -91.3 |
|  | 8 | 71 | 75 | -91.3 |  | 2 | 75 | -91.8 | 2 | 100 |  |  | -91.3 |
|  | 9 | 71 | 100 | -86.8 |  | 2 | 100 | -90.7 | 2 | 100 |  |  | -91.3 |
|  | 10 | 71 | 100 | -86.8 |  | 2 | 100 | -91.3 | 2 | 100 |  |  | -91.3 |
| CA_3A-3A-XA or CA_XA-3A-3A | 1 | 3 | 100 | -90.3 -93.08 |  | 3 | 25 | -89.81 | X | Highest NRB |  |  | PREFSENS |
|  | 2 | 3 | 100 | -90.3 -93.08 |  | 3 | 25 | -96.3 | X | Highest NRB |  |  | PREFSENS |
|  | 3 | 3 | 100 | -90.3 -93.08 |  | 3 | 100 | -86.26 | X | Highest NRB |  |  | PREFSENS |
|  | 4 | 3 | 100 | -90.3 |  | 3 | 50 | -88.21 | X | Highest NRB |  |  | PREFSENS |
|  | 5 | 3 | 100 | -90.3 |  | 3 | 50 | -93.3 | X | Highest NRB |  |  | PREFSENS |
|  | 6 | X | Highest NRB | PREFSENS |  | 3 | 100 | -90.3 -93.08 | 3 | 100 |  |  | -90.3 -93.08 |
|  | 7 | X | Highest NRB | PREFSENS |  | 3 | 100 | -90.3 | 3 | 50 |  |  | -93.3 |
| CA_3A-3A-8A | 1 | 3 | 100 | -90.3 -93.08 | 3 |  | 25 | -89.8 -92.58 | 8 | 50 |  |  | -93.3 -96.08 |  |
|  | 2 | 3 | 100 | -90.3 -93.08 | 3 |  | 25 | -96.3 -99.08 | 8 | 50 |  |  | -93.3 -96.08 |  |
|  | 3 | 3 | 100 | -90.3 -93.08 | 3 |  | 50 | -88.2 -90.98 | 8 | 50 |  |  | -93.3 -96.08 |  |
|  | 4 | 3 | 100 | -90.3 -93.08 | 3 |  | 50 | -93.3 -96.08 | 8 | 50 |  |  | -93.3 -96.08 |  |
|  | 5 | 3 | 100 | -90.3 -93.08 | 3 |  | 100 | -86.2 -88.98 | 8 | 50 |  |  | -93.3 -96.08 |  |
|  | 6 | 3 | 50 | -93.3 -93.08 | 3 |  | 50 | -89.0 -91.78 | 8 | 50 |  |  | -93.3 -96.08 |  |
|  | 7 | 8 | 50 | -93.3 | 3 |  | 50 | -93.3 | 3 | 100 |  |  | -90.3 -93.08 |  |
|  | 8 | 8 | 50 | -93.3 | 3 |  | 100 | -90.3 | 3 | 100 |  |  | -90.3 -93.08 |  |
| CA_4A-4A-XA or CA_XA-4A-4A | 1 | 4 | 100 | -93.3 -968 |  | 4 | 25 | -99.3 -1028 | X | Highest NRB |  |  | PREFSENS |
|  | 2 | 4 | 100 | -93.3 -968 |  | 4 | 100 | -93.3 -968 | X | Highest NRB |  |  | PREFSENS |
|  | 3 | X | Highest NRB | PREFSENS |  | 4 | 100 | -93.3 -968 | 4 | 100 |  |  | -93.3 -968 |
| CA_4A-4A-71A | 1 | 4 | 100 | -93.3 -968 |  | 4 | 25 | -99.3 -1028 | 71 | Highest NRB |  |  | -86.8 |
|  | 2 | 4 | 100 | -93.3 -968 |  | 4 | 100 | -93.3 -968 | 71 | Highest NRB |  |  | -86.8 |
|  | 3 | 71 | Highest NRB | -86.8 |  | 4 | 100 | -93.3 -968 | 4 | 100 |  |  | -93.3 -968 |
| CA_4A-4A-12A | 1 | 4 | 100 | -93.3 -968 |  | 4 | 25 | -99.3 -1028 | 12 | 50 |  |  | -93.5 |
|  | 2 | 4 | 100 | -93.3 -968 |  | 4 | 100 | -93.3 -968 | 12 | 50 |  |  | -93.5 |
|  | 3 | 12 | 50 | -93.5 |  | 4 | 100 | -93.3 -968 | 4 | 25 |  |  | -99.3 -1028 |
|  | 4 | 12 | 50 | -93.5 |  | 4 | 100 | -87.87 -90.57,8 | 4 | 100 |  |  | -87.87 -90.57,8 |
| CA_4A-4A-13A | 1 | 4 | 100 | -93.3 -968 |  | 4 | 25 | -99.3 -1028 | 13 | 50 |  |  | -93.3 |
|  | 2 | 4 | 100 | -93.3 -968 |  | 4 | 100 | -93.3 -968 | 13 | 50 |  |  | -93.3 |
|  | 3 | 13 | 50 | -93.3 |  | 4 | 100 | -93.3 -968 | 4 | 100 |  |  | -93.3 -968 |
|  | 4 | 4 | 50 | -96.3 -998 |  | 4 | 100 | -93.3 -968 | 13 | 50 |  |  | -93.3 |
| CA_5A-5A-66A | 1 | 5 | 50 | -94.3 |  | 5 | 25 | -97.3 | 66 |  | 100 |  | -92.8 -95.58 |
|  | 2 | 5 | 50 | -94.3 |  | 5 | 50 | -94.3 | 66 |  | 100 |  | -92.8 -95.58 |
|  | 3 | 66 | 100 | -92.8 -95.58 |  | 5 | 50 | -94.3 | 5 |  | 50 |  | -94.3 |
| CA_7A-7A-XA or CA_XA-7A-7A | 1 | 7 | 100 | -91.3 |  | 7 | 25 | -97.3 | X | Highest NRB |  |  | PREFSENS |
|  | 2 | 7 | 100 | -91.3 |  | 7 | 25 | -97.3 | X | Highest NRB |  |  | PREFSENS |
|  | 3 | 7 | 100 | -91.3 |  | 7 | 50 | -94.3 | X | Highest NRB |  |  | PREFSENS |
|  | 4 | 7 | 100 | -91.3 |  | 7 | 50 | -94.3 | X | Highest NRB |  |  | PREFSENS |
|  | 5 | 7 | 100 | -91.3 |  | 7 | 75 | -92.5 | X | Highest NRB |  |  | PREFSENS |
|  | 6 | 7 | 100 | -91.3 |  | 7 | 75 | -92.5 | X | Highest NRB |  |  | PREFSENS |
|  | 7 | 7 | 100 | -91.3 |  | 7 | 100 | -91.3 | X | Highest NRB |  |  | PREFSENS |
|  | 8 | 7 | 100 | -91.3 |  | 7 | 100 | -91.3 | X | Highest NRB |  |  | PREFSENS |
|  | 9 | 7 | 50 | -94.3 |  | 7 | 100 | -91.3 | X | Highest NRB |  |  | PREFSENS |
|  | 10 | 7 | 50 | -94.3 |  | 7 | 100 | -91.3 | X | Highest NRB |  |  | PREFSENS |
|  | 11 | 7 | 50 | -94.3 |  | 7 | 25 | -97.3 | X | Highest NRB |  |  | PREFSENS |
|  | 12 | 7 | 50 | -94.3 |  | 7 | 25 | -97.3 | X | Highest NRB |  |  | PREFSENS |
|  | 13 | X | Highest NRB | PREFSENS |  | 7 | 50 | -94.3 | 7 | 100 |  |  | -91.3 |
|  | 14 | X | Highest NRB | PREFSENS |  | 7 | 100 | -91.3 | 7 | 100 |  |  | -91.3 |
| CA_7A-42A-42A | 1 | 42 | 100 | -91.5 -93.78 |  | 42 | 25 | -97.5 -99.78 | 7 | 100 |  |  | -89.6 |
|  | 2 | 42 | 100 | -91.5 -93.78 |  | 42 | 100 | -91.5 -93.78 | 7 | 100 |  |  | -89.6 |
|  | 3 | 7 | 100 | -91.3 |  | 42 | 100 | -89.4 | 42 | 100 |  |  | -89.4 |
| CA_12A-66A-66A | 1 | 66 | 100 | -92.8 -95.58 |  | 66 | 25 | -98.8 -101.58 | 12 | 50 |  |  | -93.5 |
|  | 2 | 66 | 100 | -92.8 -95.58 |  | 66 | 100 | -92.8 -95.58 | 12 | 50 |  |  | -93.5 |
|  | 3 | 12 | 25 | -96.5 |  | 66 | 100 | -92.8 -95.58 | 66 | 25 |  |  | -98.8 -101.58 |
|  | 4 | 12 | 25 | -96.5 |  | 66 | 100 | -92.8 -95.58 | 66 | 50 |  |  | -95.8 -98.58 |
|  | 5 | 12 | 50 | -93.5 |  | 66 | 100 | -92.8 -95.58 | 66 | 75 |  |  | -94.0 -96.78 |
|  | 6 | 12 | 50 | -93.5 |  | 66 | 100 | -87.37 -90.07,8 | 66 | 100 |  |  | -87.37 -90.07,8 |
| CA_20A-42A-42A | 1 | 20 | 100 | -89.3 -92.08 |  | 42 | 100 | -92 -94.28 | 42 | 100 |  |  | -83.5 -85.78 |
|  | 2 | 20 | 100 | -89.3 -92.08 |  | 42 | 100 | -92 -94.28 | 42 | 100 |  |  | -92 -94.28 |
|  | 3 | 42 | 100 | -92 -94.28 |  | 42 | 25 | -98 -100.28 | 20 | 100 |  |  | -89.3 -92.08 |
|  | 4 | 42 | 100 | -92 -94.28 |  | 42 | 100 | -92 -94.28 | 20 | 100 |  |  | -89.3 -92.08 |
| CA_29A-66A-66A | 1 | 66 | 100 | -92.8 -95.58 |  | 66 | 100 | -92.8 -95.58 | 29 | 50 |  |  | -93.3 |
|  | 2 | 66 | 100 | -92.8 -95.58 |  | 66 | 25 | -98.8 -101.58 | 29 | 50 |  |  | -93.3 |
| CA_38A-40A-40A | 1 | 38 | 100 | -93.3 |  | 40 | 100 | -93.3 -96.08 | 40 | 100 |  |  | -93.3 -96.08 |
|  | 2 | 40 | 100 | -93.3 -96.08 |  | 40 | 50 | -96.3 -99.08 | 38 | 100 |  |  | -93.3 |
|  | 3 | 40 | 100 | -93.3 -96.08 |  | 40 | 100 | -93.3 -96.08 | 38 | 100 |  |  | -93.3 |
| CA_42A-42A-XA or CA_XA-42A-42A | 1 | 42 | 100 | -92 -94.28 |  | 42 | 25 | -98 -100.28 | X | Highest NRB |  |  | PREFSENS |
|  | 2 | 42 | 100 | -92 -94.28 |  | 42 | 100 | -92 -94.28 | X | Highest NRB |  |  | PREFSENS |
|  | 3 | X | Highest NRB | PREFSENS |  | 42 | 100 | -92 -94.28 | 42 | 100 |  |  | -92 -94.28 |
| CA_46A-46A-66A | 1 | 66 | 100 | -92.8 -95.58 |  | 46 | 100 | -88.5 | 46 | 100 |  |  | -88.5 |
|  | 2 | 66 | 100 | -92.8 -95.58 |  | 46 | 100 | -88.5 | 46 | 100 |  |  | -88.5 |
| CA_66A-66A-XA or CA_XA-66A-66A | 1 | 66 | 100 | -92.8 -95.58 |  | 66 | 25 | -98.8 -101.58 | X | Highest NRB |  |  | PREFSENS |
|  | 2 | 66 | 100 | -92.8 -95.58 |  | 66 | 100 | -92.8 -95.58 | X | Highest NRB |  |  | PREFSENS |
|  | 3 | X | Highest NRB | PREFSENS |  | 66 | 100 | -92.8 -95.58 | 66 | 100 |  |  | -92.8 -95.58 |
| Note 1: Requirements with additional relaxation (ΔRIBNC) as per Table 7.3A.0-3. Note 2: “Testpoint ID” refers to IDs in Table 7.3A.5.4.1-4. Note 3: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. Note 4: The signal power is specified per port. Note 5: PREFSENS refers to the PCC band’s single carrier reference sensitivity according to table 7.3.5-1. Note 6: Void Note 7: Reference sensitivity for the high band for which the 3rd harmonic of the low band is within transmission bandwidth, as specified Note 5 in Table 7.3A.0-0a. Note 8: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. Note 9: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. These requirements also apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. Note 10: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

For combinations of intra-band non-contiguous and intra-band contiguous carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-7. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-7: Reference sensitivity QPSK PREFSENS for intra-band non-contiguous + intra-band contiguous

| CA Configuration | Testpoint ID | PCC |  | SCC1 |  | SCC2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NRB | Requirement (dBm) | NRB | Requirement (dBm) | NRB | Requirement (dBm) |
| CA_41A-41C | 1 | 100 | -91.3 | 100 | -91.3 | 25 | -97.3 |
|  |  |  | 94.06 |  | 94.06 |  | 100.06 |
|  | 2 | 100 | -91.3 | 100 | -91.3 | 100 | -91.3 |
|  |  |  | 94.06 |  | 94.06 |  | 94.06 |
| CA_41C-41A | 1 | 100 | -91.3 | 100 | -91.3 | 25 | -97.3 |
|  |  |  | 94.06 |  | 94.06 |  | 100.06 |
|  | 2 | 100 | -91.3 | 100 | -91.3 | 100 | -91.3 |
|  |  |  | 94.06 |  | 94.06 |  | 94.06 |
| CA_42A-42C | 1 | 100 | -92.3 | 100 | -92.3 | 25 | -98.3 |
|  |  |  | 94.56 |  | 94.56 |  | 100.56 |
|  | 2 | 100 | -92.3 | 100 | -92.3 | 100 | -92.3 |
|  |  |  | 94.56 |  | 94.56 |  | 94.56 |
| CA_66A-66C | 1 | 100 | -92.8 | 100 | -92.8 | 25 | -98.8 |
|  |  |  | -95.56 |  | -95.56 |  | -101.56 |
|  | 2 | 100 | -92.8 | 100 | -92.8 | 100 | -92.8 |
|  |  |  | -95.56 |  | -95.56 |  | -95.56 |
| Note 1: Requirements with additional relaxation (ΔRIBNC) as per Table 7.3A.0-3. Note 2: “Testpoint ID” refers to IDs in Table 7.3A.5.4.1-5. Note 3: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. Note 4: The signal power is specified per port. Note 5: PREFSENS refers to the PCC band’s single carrier reference sensitivity according to table 7.3.5-1. Note 6: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |

For combinations of intra-band non-contiguous carrier aggregation with three downlink carriers and one uplink carrier assigned to one E-UTRA band the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3A.5.5-8. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active.

Table 7.3A.5.5-8: Reference sensitivity QPSK PREFSENS for intra-band non-contiguous

| CA Configuration | Testpoint ID | PCC |  | SCC1 |  | SCC2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NRB | Requirement (dBm) | NRB | Requirement (dBm) | NRB | Requirement (dBm) |
| CA_66A-66A-66A | 1 | 100 | -92.8 | 100 | -92.8 | 100 | -92.8 |
|  |  |  | -95.54 |  | -95.54 |  | -95.54 |
|  | 2 | 100 | -92.8 | 100 | -92.8 | 25 | -98.8 |
|  |  |  | -95.54 |  | -95.54 |  | -101.54 |
| Note 1: “Testpoint ID” refers to IDs in Table 7.3A.5.4.1-5. Note 2: The reference measurement channel is specified in A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. Note 3: The signal power is specified per port. Note 4: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |

< End of changes >