3GPP TSG-RAN5 Meeting #96-e	R5-225299

# Electronic meeting,  15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2448 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Table 4.6.3-16: CellAccessRelatedInfo for NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies, Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SNPN TC’s in TS 38.523-1 have all the cells configured as a SNPN only cell. Similarly, the CAG TC’s in TS 38.523-1 have the CAG cells configured as a CAG only cell. As per TS 38.331 Cl. 3.1 NPN-only Cell: A cell that is only available for normal service for NPNs' subscriber. An NPN-capable UE determines that a cell is NPN-only Cell by detecting that the cellReservedForOtherUse IE is set to true while the npn-IdentityInfoList IE is present in CellAccessRelatedInfo. NPN-IdentityInfoList-r16 IE is missing the condition CAG which is required to define the NPN-Identity in SIB1 for CAG TC’s. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated Table 4.6.3-16: CellAccessRelatedInfo in TS 38.508-1 to set the IE ‘cellReservedForOtherUse’ as TRUE with condition SNPN, CAG. Added the ‘CAG’ condition to the IE ‘npn-IdentityInfoList-r16’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect Spec implementation in TTCN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-224326r1: - Fixed 3GU issues with Tdoc number and CR number. - Default value will remain as ‘not present’ for the field ‘cellReservedForOtherUse’. R5-224326r2: - Editorial error fixed: For cellReservedForOtherUse, instead of merging rows , the border is made invisible. R5-225299: Is the final Tdoc for R5-224326r2 |  |  |  |  |  |  |  |  |

### 4.6.3	Radio resource control information elements

### –	4.6.3	Radio resource control information elements

#### –	AdditionalSpectrumEmission

Table 4.6.3-1: AdditionalSpectrumEmission

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| AdditionalSpectrumEmission | 0 |  |  |

#### <Skipped Contents>

##### <Start of changes>

#### –	CellAccessRelatedInfo

Table 4.6.3-16: CellAccessRelatedInfo

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellAccessRelatedInfo ::= SEQUENCE { |  |  |  |
| plmn-IdentityInfoList | PLMN-IdentityInfoList |  |  |
| cellReservedForOtherUse | Not present |  |  |
|  | true |  | SNPN, CAG |
| cellReservedForFutureUse-r16 | Not present |  |  |
| npn-IdentityInfoList-r16 | Not present |  |  |
|  | NPN-IdentityInfoList-r16 |  | SNPN, CAG |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SNPN | Standalone NPN cell |
| CAG | PNI-NPN |

##### <End of Changes>