3GPP TSG-RAN5 Meeting #96-e	R5-225425

# Electronic Meeting, 15th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0432 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to IMS emergency over 5G test case 10.15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. TP2 is incorrectly reflected in steps 15-19. 2. Steps for TP1 and TP2 are not worded with the "Check:" statement as required by RAN5 PRD13. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Steps 15-19 were corrected to align with TP2. 2. "Check:" statement has been added for TP1 and TP2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 CR 2483 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Some typos corrected. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the outcome of 2 revisions of R5-224279. |  |  |  |  |  |  |  |  |

## 10.15	Deregistration upon emergency registration expiration / 5GS

10.15.1	Test Purpose (TP)

(1)

with { UE being registered to IMS }

ensure that {

when { UE is made to start an emergency call }

then { UE performs IMS emergency registration and sets up an emergency call }

}

(2)

with { Emergency call ongoing }

ensure that {

when { Network sends BYE }

then { UE sends 200 OK for BYE }

}

(3)

with { Emergency call having ended }

ensure that {

when { half of the emergency registration expiration time has passed }

then { UE does not attempt IMS emergency re-registration }

}

10.15.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229 clause 5.1.6.4]:

The UE shall perform user-initiated emergency reregistration as specified in subclause 5.1.1.4 if half of the time for the emergency registration has expired and:

-	the UE has emergency related ongoing dialog; or

-	standalone transactions exist; or

-	the user initiates an emergency call.

The UE shall not perform user-initiated emergency reregistration in any other cases.

[TS 24.229 clause 5.1.6.6]:

Once the UE registers a public user identity and an associated contact address via emergency registration, the UE shall not perform user-initiated deregistration of the respective public user identity and the associated contact address.

NOTE:	The UE will be deregistered when the emergency registration expires.

10.15.3	Test description

10.15.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	UE contains either ISIM and USIM applications or only USIM application on UICC.

-	UE is configured to register for IMS after switch on.

Preamble:

-	The UE is in test state 1N-A (TS 38.508-1 [21]) and registered to IMS by executing the generic test procedure in Annex A.2 up to the last step.

10.15.3.2	Test procedure sequence

Table 10.15.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to make an emergency call | - | - | - | - |
| 2-14 | Check: Are steps 1-13 in test procedure TS 38.508-1 [21] Table 4.9.11.2.2-1 performed? And are steps in A.3 and A.6 performed in the generic test procedure, to initiate IMS emergency registration and IMS emergency voice call? | - | - | 1 | P |
| 15 | Void. | - | - | - | - |
| 16 | The SS releases the emergency call with BYE. (Step 1 in annex A.8) | ß | BYE | - | - |
| 17 | Check: Does the UE send 200 OK for BYE? (Step 2 in annex A.8) | à | 200 OK | 2 | P |
| 17A-19 | Steps 3b1-3b3 in test procedure in TS 38.508-1 [21] Table 4.9.12B.2.2-1 are performed. | - | - | - | - |
| 20 | The SS waits half of the IMS Emergency Reregistration time. | - | - | - | - |
| 21 | Check: Does the UE send REGISTER message for IMS emergency Reregistration? | à | REGISTER | 3 | F |

10.15.3.3	Specific message content

Table 10.15.3.3-1: REGISTER (Step 2 and step 19, table 10.15.3.2-1)

Derivation Path: TS 34.229-1 [2], Annex A.1.7 with conditions A1 and A7.

Table 10.15.3.3-2: 200OK for REGISTER (Step 5, table 10.15.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.1.3 with condition A3. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| expires |  | 30 |  |  |