3GPP TSG-RAN5 Meeting #96-e	R5-225734

# Electronic Meeting,  15th – 26th August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1846 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition many 4CA NR combinations to reference sensitivity test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | WE Certification, DISH Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 13 4CA combinations are introduced in the specification but missing the reference sensitivity test |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the combinations to the reference sensitivity test |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 13 4CA combinations cannot be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Corrected the meeting date on the cover page |  |  |  |  |  |  |  |  |

7.3A.3.5	Test requirement

For 4DL carrier aggregation, test parameters are specified in table 7.3A.3.4.1-1. For the CA configurations listed in table 7.3A.3.5-1, the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with reference sensitivity power level specified in table 7.3.2.5-1 for each non-SDL carrier for 2 Rx antenna port, in table 7.3.2.5-2 for each non-SDL carrier for 4 Rx antenna port and in table 7.3A.1.5-2 for SDL carrier with following additional requirements:

For the UE which supports inter-band carrier aggregation, the test requirement for reference sensitivity shall be increased by the amount given by ΔRIB,c defined in clause 7.3A.0.3.2 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

For intra-band non-contiguous CA with one uplink carrier and two or more downlink sub-blocks, the test requirement for SCC(s) shall be increased by ΔRIBNC  given in Table 7.3A.0.2.2-1. Unless given by Table 7.3.2.3-4, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.3.5-1: Reference sensitivity requirement for 4DL CA

| Carrier aggregation type | DL CA configuration | UL CA configuration |
| --- | --- | --- |
| Inter-band 4DL CA | CA_n26A-n66(2A)-n70A | - |
|  | CA_n48A-n66A-n71(2A) | - |
|  | CA_n48A-n66(2A)-n70A | - |
|  | CA_n48A-n66(2A)-n71A | - |
|  | CA_n48A-n70A-n71(2A) | - |
|  | CA_n48B-n66A-n70A | - |
|  | CA_n48B-n66A-n71A | - |
|  | CA_n48B-n70A-n71A | - |
|  | CA_n48(2A)-n66A-n70A | - |
|  | CA_n48(2A)-n66A-n71A | - |
|  | CA_n48(2A)-n66(2A) | - |
|  | CA_n48(2A)-n70A-n71A | - |
|  | CA_n48(2A)-n71(2A) | - |
|  | CA_n66A-n70A-n71(2A) | - |
|  | CA_n66B-n70A-n71A | - |
|  | CA_n66(2A)-n70A-n71A | - |
|  | CA_n66(2A)-n71(2A) | - |
| SDL configuration | CA_n29A-n66B-n70A | - |
|  | CA_n29A-n66(2A)-n70 | - |