3GPP TSG-RAN5 Meeting #96-e	R5-225869

# Online,  , 15th Aug 2022 - 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1809 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EVM measurement point for DFTs-OFDM DM-RS Type 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Core specification alignment with TS 38.101-1 DFTs-OFDM DMRS Type-2 to achieve Low-PAPR by Pi/2 shift BPSK is newly defined from 38.211 Release 16. Conventional DMRS has its modulated symbols mapped directly to the frequency domain in a UE, but DFTs-OFDM DM-RS Type 2 has its modulated symbols mapped to the frequency domain after DFT instead. This is the same process as DFT-s-OFDM PUSCH and PUCCH. Calculating the EVM of DFTs-OFDM DM-RS Type 2 according to the current EVM measurement block diagram in Figure E.2.5-1 cannot provide a correct EVM result, the constellation of modulated symbols (Pi/2 shift BPSK) being incorrectly calculated prior to IDFT. It is appropriate to correct the EVM measurement block diagram to calculate EVM by the constellation after IDFT, similarly to DFT-s-OFDM PUSCH. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Figure E.2.5-1 was corrected as follows: - “DFT-s-OFDM DM-RS Type 2” was added to the input of DFT in DUT section. - “DM-RS” at the input of Tone map in DUT section was replaced by “CP-OFDM DM-RS, DFT-s-OFDM DM-RS Type 1”. - “DFT-s-OFDM DM-RS Type 2” was added to the output of IDFT in Test equipment section. - “DM-RS” at the output of Tx-Rx chain equalizer in Test equipment section was replaced by “CP-OFDM DM-RS, DFT-s-OFDM DM-RS Type 1”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No proper EVM can be calculated as used symbols are fully corrupted (due to missing IDFT block in the TE section of the block diagram in Figure E.2.5-1). May lead to the wrong interpretation by readers that a DFT block for DFTs-OFDM DM-RS Type 2 is not used by DUTs (as missing in the DUT section of the block diagram in Figure E.2.5-1). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | E.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depending on RAN4 CR R4-2212537 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-224618 and the outcome of 1 revisions to this document. r1: Tdoc number of related RAN4 CR was added. |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

E.2.5	Measurement points

The unwanted emission falling into non-allocated RB(s) is calculated directly after the FFT as described below. In contrast to this, the EVM for the allocated RB(s) is calculated after the IDFT for DFT-s-OFDM or after the Tx-Rx chain equalizer for CP-OFDM. The samples after the TX-RX chain equalizer are used to calculate EVM equalizer spectrum flatness. Carrier frequency error and carrier leakage is calculated in the block “RF correction”.

In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE (as defined in TS 38.331 [6]), carrier leakage measurement in the RF correction block shall be omitted. All statements from Annex E.3 onwards shall

DFT

IFFT

TX

Front

-

-end

Channel

RF

correction

FFT

Tx

-

Rx chain

equalizer

In

-

band

emissions

meas.

0

0

IDFT

DUT

Test equipment

EVM meas. of

CP-OFDM PUSCH, PUCCH, and CP-OFDM DM-RS, DFT-s-OFDM DM-RS Type 1

CP-OFDM PUSCH, PUCCH, and CP-OFDM DM-RS, DFT-s-OFDM DM-RS Type 1

Tone  map

DFT-s-OFDM PUSCH, PUCCH, and DFT-s-OFDM DM-RS Type 2

EVM meas. of DFT-s-OFDM PUSCH, PUCCH, and DFT-s-OFDM DM-RS Type 2

be read assuming that no carrier leakage has been measured.

Figure E.2.5-1: EVM measurement points

<<End of change>>