3GPP TSG-RAN5 Meeting #97	R5-227422

# Toulouse, France, 14th – 18th November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3415 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections to TC 8.1.2.1.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The clause numbering in TC 8.1.2.1.5.1 is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The clause numbering in TC 8.1.2.1.5.1 was updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 8.1.2.1.5.1 will not be correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-227267 and the outcome of 2 revisions to this document. |  |  |  |  |  |  |  |  |

##### 8.1.2.1.5	NR CA / RRC reconfiguration / SCell addition / modification / release / Success

###### 8.1.2.1.5.1	NR CA / RRC reconfiguration / SCell addition / modification / release / Success / Intra-band Contiguous CA

8.1.2.1.5.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state }

ensure that {

when { UE receives an RRCReconfiguration message containing sCellToAddModList for SCell addition }

then { UE adds the new SCell and sends an RRCReconfigurationComplete message }

}

(2)

with { UE in NR RRC_CONNECTED state with SCell configured}

ensure that {

when { UE receives an RRCReconfiguration message containing sCellToAddModList for SCell modification }

then { UE modifies the SCell configuration and sends an RRCReconfigurationComplete message }

}

(3)

with { UE in NR RRC_CONNECTED state with SCell configured}

ensure that {

when { UE receives an RRCReconfiguration message containing sCellToReleaseList }

then { UE sends an RRCReconfigurationComplete message }

}

8.1.2.1.5.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 5.3.5.3, 5.3.5.5.1 and 5.3.5.5.9.

[TS 38.331 clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration:

…

1>	if the RRCReconfiguration includes the masterCellGroup:

2>	perform the cell group configuration for the received masterCellGroup according to 5.3.5.5;

…

1>	else:

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

[TS 38.331 clause 5.3.5.5.1]

The UE performs the following actions based on a received CellGroupConfig IE:

…

1>	if the CellGroupConfig contains the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.5.5.8;

…

1>	if the CellGroupConfig contains the sCellToAddModList:

2>	perform SCell addition/modification as specified in 5.3.5.5.9.

[TS 38.331 clause 5.3.5.5.9]

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	configure lower layers to consider the SCell to be in deactivated state;

…

1>	for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification):

2>	modify the SCell configuration in accordance with the sCellConfigDedicated.

8.1.2.1.5.1.3	Test description

8.1.2.1.5.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell, NR Cell 3 is the SCell.

None.

Preamble:

-	The UE is in state 3N-A, and with UE test loop mode B active if pc_IP_Ping is set to FALSE, as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.2.1.5.1.3.2	Test procedure sequence

Table 8.1.2.1.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfigurationmessage containing an sCellToAddModList with SCell NR Cell 3 addition, | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | Check: Does the UE transmits an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationtComplete | 1 | P |
| 3 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB1 and Cell 3? | - | - | 1 | P |
| 4 | The SS transmits an RRCReconfigurationmessage containing an sCellToAddModList with SCell NR Cell 3 modification. | <-- | NR RRC: RRCReconfiguration | - | - |
| 5 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationtComplete | 2 | P |
| 6 | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB1 and Cell 3? | - | - | 2 | P |
| 7 | The SS transmits an RRCReconfigurationmessage containing an sCellToReleaseList with SCell NR Cell 3. | <-- | NR RRC: RRCReconfiguration | - | - |
| 8 | Check: Does the UE transmit an RRCReconfigurationComplete message? | --> | NR RRC: RRCReconfigurationtComplete | 3 | P |

8.1.2.1.5.1.3.3	Specific message contents

Table 8.1.2.1.5.1.3.3-1: RRCReconfiguration (step 1, Table 8.1.2.1.5.1.3.2-1)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition SCell_add

Table 8.1.2.1.5.1.3.3-2: RRCReconfiguration (step 4, Table 8.1.2.1.5.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.5.1.3.3-3: CellGroupConfig (Table 8.1.2.1.5.1.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigCommon | Not present |  |  |
| sCellConfigDedicated | ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.5.1.3.3-4: ServingCellConfig (Table 8.1.2.1.5.1.3.3-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| firstActiveDownlinkBWP-Id | Not present |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| firstActiveUplinkBWP-Id | Not present |  |  |
| } |  |  |  |
| pdsch-ServingCellConfig CHOICE { |  |  |  |
| setup | PDSCH-ServingCellConfig |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.5.1.3.3-5: PDSCH-ServingCellConfig (Table 8.1.2.1.5.1.3.3-4)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-102 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n16 |  |  |
| } |  |  |  |

Table 8.1.2.1.5.1.3.3-6: RRCReconfiguration (step 7, Table 8.1.2.1.5.1.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.2.1.5.1.3.3-7: CellGroupConfig (Table 8.1.2.1.5.1.3.3-6)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-19 with condition SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList | Not present |  |  |
| sCellToReleaseList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellIndex { | 1 entry |  |  |
| SCellIndex[1] | 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |