3GPP TSG-RAN5 Meeting #98	R5-230095

Athens, Greece, 27th Feb 2023 – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3435 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test case 8.1.1.4.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To update the test case 8.1.1.4.7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case 8.1.1.4.7 have been updated for - Preamble to NR-DC. - Test procedure sequence to RRC_INACTIVE and with IP data check. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

##### 8.1.1.4.7	RRC resume / Suspend-Resume / RRC setup / Active SCG SCell addition / Intra-band Contiguous CA

8.1.1.4.7.1	Test Purpose (TP)

(1)

with { UE in NR RRC_INACTIVE state }

ensure that {

when { UE receives an RRCSetup message containing sCellToAddModList and sCellState for SCell addition }

then { UE adds the new SCell and sends an RRCSetupComplete message }

}

8.1.1.4.7.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331 clauses 5.3.5.5.9 and 5.3.13.7.

[TS 38.331 clause 5.3.5.5.9]

The UE shall:

1>	for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition):

2>	add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated;

2>	if the sCellState is included:

3>	configure lower layers to consider the SCell to be in activated state;

…

[TS 38.331 clause 5.3.13.7]

The UE shall:

1>	perform the RRC connection setup procedure as specified in 5.3.3.4.

8.1.1.4.7.3	Test description

8.1.1.4.7.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the PCell, NR Cell 3 is the PSCell, NR Cell 12 is the SCell

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (NR-DC) with Bearers (MCG(s) and SCG) according to TS 38.508-1 [4], Table 4.5.1-1.

8.1.1.4.7.3.2	Test procedure sequence

Table 8.1.1.4.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS transmits an RRCRelease message with suspend. | <-- | NR RRC: RRCRelease | - | - |
| 1 | The SS transmits a Paging message including a matched identity (correct fullI-RNTI). | <-- | NR RRC: Paging | - | - |
| 2 | The UE transmit an RRCResumeRequest message to resume RRC Connection by setting resumeIdentity to the stored shortI-RNTI value? | --> | NR RRC: RRCResumeRequest | - | - |
| 3 | The SS transmits an RRCSetup message in response to RRCResumeRequest message. | <-- | NR RRC: RRCSetup | - | - |
| 4 | Check: Does the UE transmit an RRCSetupComplete message and a REGISTRATION REQUEST message by discarding any stored UE Inactive AS context and suspendConfig, setting the ng-5G-S-TMSI-Value to ng-5G-S-TMSI? | --> | NR RRC: RRCSetupComplete 5GMM: REGISTRATION REQUEST | 1 | P |
| 5-6 | Steps 4-5 as defined in TS 38.508-1 [4] Table 4.9.5.2.2-1. | - | - | - | - |
| 6A | Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB1 and Cell 3? | - | - | 1 | - |
| 7 | The SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. | <-- | NR RRC: RRCRelease | - | - |

8.1.1.4.7.3.3	Specific message contents

Table 8.1.1.4.7.3.3-1: Paging (step 1, Table 8.1.1.4.7.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-9 with condition NR_RRC_RESUME

Table 8.1.1.4.7.3.3-2: RRCSetup (step 3, Table 8.1.1.4.7.3.2-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition RESUME |  |  |
| masterCellGroup |  | CellGroupConfig with condition RESUME and SCell_add |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.1.1.4.7.3.3-3: CellGroupConfig (Table 8.1.1.4.7.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellState-r16 | activated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section>