3GPP TSG-RAN WG5 Meeting #98	R5-230337

Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0404 | rev | - | Current version: | 16.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of accuracy requiremets for UE Rx-Tx time difference |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UE Rx-Tx time difference test cases were already added in the specification, but the accuracy requirements for these measurements are still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added accuracy requiremets for UE Rx-Tx time difference in the test condition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test conditions for UE Rx-Tx time difference will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.14.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

4.14.5	Measurement Accuracy Requirements

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall not apply, if:

NTA_offset defined in TS 38.133 Table 7.1.2-2 changes during the UE Rx-Tx measurement period or

if the uplink transmission timing changes during the UE Rx-Tx measurement period due to the network-configured Timing Advance.

The UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that:

- The UE transmits SRS within [-160, 160] msec of at least one DL PRS resource of each of the TRPs in the assistance data.

If the uplink transmission timing changes during the UE Rx-Tx measurement period due to the autonomous timing adjustment defined in TS 38.133 clause 7.1.2 then:

-	UE Rx-Tx measurement accuracy requirements shall apply for a cell, which is also the downlink reference cell (defined in TS 38.133 section 7.1.1) for SRS transmission even if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

-	UE Rx-Tx measurement accuracy requirements shall not apply for a cell, which is not the downlink reference cell (defined in TS 38.133 section 7.1.1) for SRS transmission, if the uplink transmission timing changes during the UE Rx-Tx measurement period due to autonomous adjustment.

When a serving cell change occurs during the UE Rx-Tx measurement period, the UE Rx-Tx time difference measurement accuracy requirements in this clause shall apply provided that the serving cell change does not impact SRS configuration for the UE Rx-Tx measurement.

Note: The requirements for fading channel in this clause are derived based on TDL-A (30 ns delay spread, 5Hz) and TDL-C (60 ns delay spread, 300 Hz) channel models for FR1 and FR2 respectively.

The accuracy requirements in Table 4.14.5-1 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [54] for reference sensitivity are fulfilled.

PRP|dBm according to TS 38.133 Annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 4.14.5-1: UE Rx-Tx time difference measurement accuracy in FR1 in AWGN

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± 78+ | -3 | ≥24 | 15 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -118 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
| ± 59+ |  | ≥52 |  | ≥1 | Note 6 | Note 6 | Note 6 |
| ± 30+ |  | >104 |  | ≥1 | Note 6 | Note 6 | Note 6 |
| ± 57+ |  | ≥24 | 30 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -117.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -117 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -115.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -115 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -114.5 |  |
| ± 30+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 15+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 29+ |  | ≥24 | 60 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -114.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -114 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -113.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -113 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -111.5 |  |
| ± 15+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 101+ | -13 | ≥24 | 15 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 75+ |  | ≥52 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 37+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 58+ |  | ≥24 | 30 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 39+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 36+ |  | ≥24 | 60 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 16+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 8+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in TS 38.133 Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [49]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [53]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 4.14.5-5. |  |  |  |  |  |  |  |

The accuracy requirements in Table 4.14.5-2 for FR1 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-1 [54] for reference sensitivity are fulfilled.

PRP|dBm according to TS 38.133 Annex B.2.14 for a corresponding Band.

Fading propagation condition.

Table 4.14.5-2: UE Rx-Tx time difference measurement accuracy in FR1 in fading

| Accuracy | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition (TrepPRS*LPRS/KcombPRSNote 3 | NR operating band groupsNote 2 | IoNote 4 range |  |
|  |  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  |  | dBm / SCSPRS | dBm/BW |
| ± 137+ | -3 | ≥24 | 15 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -120 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -118 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 |  |
| ± 96+ |  | ≥52 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 62+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 87+ |  | ≥24 | 30 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -118 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -117.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -117 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -116.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -116 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -115.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -115 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -114.5 |  |
| ± 68+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 44+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 59+ |  | ≥24 | 60 | ≥4 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -115 | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -114.5 |  |
|  |  |  |  |  | NR_TDD_FR1_C | -114 |  |
|  |  |  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -113.5 |  |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -113 |  |
|  |  |  |  |  | NR_FDD_FR1_F | -113.5 |  |
|  |  |  |  |  | NR_FDD_FR1_G | -113 |  |
|  |  |  |  |  | NR_FDD_FR1_H | -111.5 |  |
| ± 42+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 36+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 180+ | -13 | ≥24 | 15 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 98+ |  | ≥52 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 68+ |  | >104 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 87+ |  | ≥24 | 30 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 85+ |  | ≥48 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 44+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 139+ |  | ≥24 | 60 | ≥4 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 46+ |  | ≥ 64 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| ± 30+ |  | ≥ 132 |  | ≥1 | NOTE 6 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in TS 38.133 Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [49]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [53]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 4.14.5-5. |  |  |  |  |  |  |  |

The accuracy requirements in Table 4.14.5-3 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [55] for reference sensitivity are fulfilled.

PRP|dBm according to TS 38.133 Annex B.2.14 for a corresponding Band.

AWGN propagation condition.

Table 4.14.5-3: UE Rx-Tx time difference measurement accuracy in FR2 in AWGN

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ± 22+ | -3 | ≥24 | 60 | ≥4 | Same value as PRP in TS 38.133 Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 15+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 12+ |  | ≥32 | 120 | ≥1 | Same value as PRP in TS 38.133 Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 7+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 4+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 35+ | -13 | ≥24 | 60 | ≥1 | NOTE 6 | NOTE 6 |
| ± 15+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 7+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 14+ |  | ≥32 | 120 | ≥1 | NOTE 6 | NOTE 6 |
| ± 9+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 4+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in TS 38.133 Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [49]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [53]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 4.14.5-6. |  |  |  |  |  |  |

The accuracy requirements in Table 4.14.5-4 for FR2 are valid under the following conditions:

Conditions defined in clause 7.3 of TS 38.101-2 [55] for reference sensitivity are fulfilled.

PRP|dBm according to TS 38.133 Annex B.2.14 for a corresponding Band.

Fading propagation condition.

Table 4.14.5-4: UE Rx-Tx time difference measurement accuracy in FR2 in fading

| Accuracy | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | PRS Ês/Iot | Minimum PRS bandwidth | PRS SCS | PRS resource repetition(TrepPRS*LPRS/KcombPRSNote 3 | IoNote 4 range |  |
|  |  |  |  |  | Minimum IoNote 1 | Maximum Io |
| TcNote 5 | dB | RB | kHz |  | dBm / SCSPRS | dBm/BWChannel |
| ± 75+ | -3 | ≥24 | 60 | ≥4 | Same value as PRP in TS 38.133 Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 72+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 57+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 61+ |  | ≥32 | 120 | ≥1 | Same value as PRP in TS 38.133 Table B.2.14-2, according to UE Power class, operating band and angle of arrival | -50 |
| ± 64+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 55+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 92+ | -13 | ≥24 | 60 | ≥4 | NOTE 6 | NOTE 6 |
| ± 70+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 57+ |  | ≥132 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 60+ |  | ≥32 | 120 | ≥1 | NOTE 6 | NOTE 6 |
| ± 66+ |  | ≥64 |  | ≥1 | NOTE 6 | NOTE 6 |
| ± 62+ |  | ≥128 |  | ≥1 | NOTE 6 | NOTE 6 |
| NOTE 1: This minimum Io condition is expressed as the average Io per RE over all REs in an OFDM symbol. NOTE 2: NR operating band groups are as defined in TS 38.133 Section 3.5. NOTE 3: TrepPRS, LPRS ,KcombPRS are configured by higher layer parameter dl-PRS-ResourceRepetitionFactor, dl-PRS-NumSymbols and dl-PRS-CombSizeNdefined in TS 37.355 [49]. NOTE 4: The Io is defined in PRS slots. The same Io range applies to PRS and non-PRS symbols. Io levels are different in PRS and non-PRS symbols within the same slot. NOTE 5: Tc is the basic timing unit defined in TS 38.211 [53]. NOTE 6: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding requirement with the PRS bandwidth of the smallest RB number for the corresponding SCS. NOTE 7: is the margin determined from Table 4.14.5-6. |  |  |  |  |  |  |

Table 4.14.5-5: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(PRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | 160 |
| ≥ 52 | ≥ 24 | N/A | 80 |
| ≥ 104 | ≥ 48 | ≥ 24 | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [53]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

Table 4.14.5-6: Margin for UE Rx-Tx time difference measurement accuracy in FR2

| Min(PRS BW, SRS BW) (MHz) |  | Margin (Tc Note 1) |
| --- | --- | --- |
| SCS = 60 kHz | SCS = 120 kHz |  |
| ≥ 24 | N/A | 76 |
| ≥ 64 | ≥ 32 | 32 |
| ≥ 132 | ≥ 64 | 24 |
| N/A | ≥ 128 | 20 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [53]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |