3GPP TSG-RAN5 Meeting #98	R5-230762

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0254 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of abbreviation of CCA and clarification on FR1 band selection with CCA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM JAPAN LLC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Abbreviation for CCA is missing and FR1 band selection with CCA is not clear |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added - Abreviation for CCA in section 3.3, - FR1 band selection criteria in Section 4 - Applicability of new performance test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Meaning for CCA, FR1 band selection with CCA, test applicability will be ambigious |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.0, 4.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of Changes >>

<<Some Unchanged Sections>>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [9] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [9].

For the purposes of the present document, the following abbreviations apply:

5GS	5G System

ACLR	Adjacent Channel Leakage Ratio

AWGN	Additive White Gaussian Noise

BPSK	Binary Phase Shift Keying

BWP	Bandwidth Part

CA	Carrier Aggregation

CBW	Channel Bandwidth

CC	Component Carrier

CCA	Clear Channel Assessment

CMR	Channel Measurement Resource

CQI	Channel Quality Indicator

CSI	Channel State Information

DAPS	Dual Active Protocol Stack

DC	Dual Connectivity

DCI	Downlink Control Information

DL	Downlink

DRX	Discontinuous Reception

EIRP	Effective Isotropic Radiated Power

E-UTRA	Evolved UTRA

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

EN-DC	E-UTRA/NR Dual Connectivity

FR1	Frequency Range 1 (410 MHz - 7125 MHz)

FR2	Frequency Range 2 (24250 MHz - 52600 MHz)

HST	High Speed Train

ICS	Implementation Conformance Statement

IMR	Interference Measurement Resource

IXIT	Implementation eXtra Information for Testing

L1	Layer 1

MAC	Medium Access Control

MCG	Master Cell Group

MPR	Allowed maximum power reduction

NR	New Radio

NSA	Non-Standalone, a mode of operation where operation of another radio is assisted with another radio

PCell	Primary Cell

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PIXIT	Protocol Implementation eXtra Information for Testing

PMI	Pre-coding Matrix Indicator

PRACH	Physical Random Access Channel

PSCell	Primary SCG Cell

QAM	Quadrature Amplitude Modulation

RF	Radio Frequency

RLM	Radio Link Monitoring

RRC	Radio Resource Control

RRM	Radio Resource Management

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

SA	Standalone

SC	Single Carrier

SCC	Secondary Component Carrier

SCell	Secondary Cell

SCG	Secondary Cell Group

SCS	System Conformance Statement / Subcarrier Spacing

SDL	Supplementary Downlink

SFN	System Frame Number

SFTD	SFN and Frame Timing Difference

SINR	Signal to Interference plus Noise Ratio

SL	Sidelink

SL-MIMO	Sidelink-Multiple Antenna Transmission

SRS	Sounding Reference Signal

SSB	Synchronization Signal Block

SS-RSRP	Synchronization Signal based RSRP

SS-RSRQ	Synchronization Signal based RSRQ

SS-SINR	Synchronization Signal based SINR

SUL	Supplementary UpLink

TC	Test Case

TDD	Time Division Duplex

TRP	Total Radiated Power

TxD	Tx Diversity

UEUT	User Equipment Under Test

UL	Uplink

UL MIMO	Uplink Multiple Antenna transmission

UTRA	UMTS Terrestrial Radio Access

V2X	Vehicle to Everything

<<Some Unchanged Sections>>

Table 4.0-2: Tested Bands Selection Criteria

| Code | Tested Bands Selection Criteria |  | Comment |
| --- | --- | --- | --- |
| D001 | A.4.3.1-1 OR A.4.3.1-2 |  | All supported FDD or TDD FR1 bands |
| D002 | Void |  |  |
| D003 | A.4.3.1-5 |  | All supported FR1 SUL Bands |
| D004 | {n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n28, n30, n34, n38, n39, n40, n41, n50, n51, n65, n66, n70, n71, n74, n75, n76} |  | All supported bands among n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n28, n30, n34, n38, n39, n40, n41, n50, n51, n65, n66, n70, n71, n74, n75, n76 |
| D005 | A.4.3.1-3 |  | All supported FR2 Bands |
| D006 | Void |  |  |
| D007 | Void |  |  |
| D008 | ANY((A.4.3.1-1) AND 10MHz) |  | Any FDD FR1 band within the set supporting 10 MHz UE Channel BW |
| D009 | ANY((A.4.3.1-2) AND 20MHz) |  | Any TDD FR1 band within the set supporting 20 MHz UE Channel BW |
| D010 | ANY((A.4.3.1-2) AND 40MHz) |  | Any TDD FR1 band within the set supporting 40 MHz UE Channel BW |
| D011 | A.4.3.9-4a OR A.4.3.9-4b |  | All supported 4 Rx antenna ports Bands |
| D012 | A.4.3.9-12 AND FDD |  | All supported FDD FR1 band with UL MIMO capabilities |
| D013 | ANY((A.4.3.1-3) AND 50MHz) |  | Any TDD FR2 band within the set supporting 50 MHz UE Channel BW |
| D014 | ANY((A.4.3.1-3) AND 100MHz) |  | Any TDD FR2 band within the set supporting 100 MHz UE Channel BW |
| D015 | ANY((A.4.3.1-3) AND 200MHz) |  | Any TDD FR2 band within the set supporting 200 MHz UE Channel BW |
| D016 | A.4.3.1-9 |  | All supported FR1 sidelink bands |
| D017 | {n40, n41, n77, n78, n79} |  | All supported TDD bands among n40, n41, n77, n78, n79 |
| D018 | A.4.3.1-2/2e OR A.4.3.1-2/12 |  | All supported FR1 Bands for operation with shared spectrum channel access |
| D019 |  | {n34, n38, n39, n48, n90} AND 10MHz | All supported TDD FR1 bands among n34, n38, n39, n48, n90 supporting 10MHz UE Channel BW |
| D020 |  | Void |  |
| D021 |  | Void |  |
| D022 |  | A.4.3.9-12 | All supported FDD or TDD FR1 Bands with UL MIMO capabilities |
| D023 |  | A.4.3.9-13 | All supported FR2 Bands with UL MIMO capabilities |
| D024 |  | A.4.3.9-12 AND A.4.3.1-5 | All supported FR1 Bands with UL MIMO capabilities and SUL bands |
| D025 |  | A.4.3.9-xx AND CCA | All supported TDD FR1 bands with CCA |
| D026 |  | A.4.3.9-xx AND CCA | All supported FDD FR1 bands with CCA |
| NOTE 1: Band Selection is based on set theory. For each feature, item number shall correspond to the Band number. The result is the set of bands for which the test shall be conducted. The following operators are used: AND: Set intersection ( ). {n1,n2} AND {n2,n3} = {n2} OR: Set union ( ). {n1,n2} OR {n2,n3} = {n1,n2,n3} NOT: Set complement (\), full set being all bands. NOT{n1} = {n2 ...n256} Also note that this is set without repetitions so {n1} AND {n1} = {n1} The following basic sets are used: {n1,n2}: Explicitly given band set 10MHz: All bands supporting 10 MHz FDD: All bands in FDD mode |  |  |  |

<<Some Unchanged Sections>>

### 4.1.4	Performance conformance test cases

Table 4.1.4-1: Applicability of performance test cases, ref. TS 38.521-4 [4]

| Clause | TC Title | Release | Applicability |  | Tested Bands Selection | Additional Information |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |
| 5 | Demodulation performance requirements (Conducted requirements) |  |  |  |  |  |
| 5.2 | PDSCH demodulation requirements |  |  |  |  |  |
| 5.2.1.1.1 | 1Rx FDD FR1 PDSCH performance for RedCap | Rel-17 | C177a | RedCap UEs supporting 5GS FDD FR1 | D008 | NOTE 1 |
| 5.2.2.1.1_1 | 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.1_2 | 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 | C015x | UEs supporting 5GS FDD FR1 and Enhanced Receiver Type 1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.1_3 | 2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for DL1024QAM for both SA and NSA | Rel-17 | C200 | UEs supporting 5GS FDD FR1 and DL1024Qam but not supporting FDD bands with 4Rx UE capability | D008 | NOTE 1 |
| 5.2.2.1.2_1 | 2Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.3_1 | 2Rx FDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015b | UEs supporting 5GS FDD FR1 and PDSCH mapping Type B but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.4_1 | 2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C015y | UEs supporting 5GS FDD FR1 and additional DMRS for coexistence with LTE CRS but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.5_1 | 2Rx FDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C074 | UEs supporting 5GS FDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5, but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.6_1 | 2Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C120 | UEs supporting 5GS FDD FR1 and aggregationFactorDL > 1 for PDSCH repetition multislots but not supporting FDD bands with 4Rx UE capability | D008 | NOTE 1 |
| 5.2.2.1.7_1 | 2Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C116 | UEs supporting 5GS FDD FR1 and PDSCH processing capability 2 and PDSCH mapping type B, but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.8_1 | 2Rx FDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C121 | UEs supporting 5GS FDD FR1 and PDSCH pre-emption indication but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.9_1 | 2Rx FDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C099 | UEs supporting 5GS FDD FR1 and enhanced demodulation processing for HST-SFN joint transmission scheme but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.10_1 | 2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C152 | UEs supporting 5GS FDD FR1 and number of active TCI states per BWP per CC but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.11_1 | 2Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C070 | UEs supporting 5GS FDD FR1 and single DCI based spatial division multiplexing scheme, but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.12_1 | 2Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C113 | UEs supporting 5GS FDD FR1 and multi-DCI based multi-TRP, but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.13_1 | 2Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | Rel-16 | C114 | UEs supporting 5GS FDD FR1 and single DCI based FDMSchemeA, but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.1.14_1 | 2Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C115 | UEs supporting 5GS FDD FR1 and single-DCI based inter-slot TDM, but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.2.2.2.1_1 | 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D009 D010 |  |
| 5.2.2.2.1_2 | 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 | C016x | UEs supporting 5GS TDD FR1 and Enhanced Receiver Type 1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.2.2.2.1_3 | 2Rx TDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for DL1024QAM for both SA and NSA | Rel-17 | C201 | UEs supporting 5GS TDD FR1 and DL 1024QAM | D010 | NOTE 1 |
| 5.2.2.2.2_1 | 2Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.2.2.2.3_1 | 2Rx TDD FR1 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C016b | UEs supporting 5GS TDD FR1 and PDSCH mapping Type B but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.2.2.2.4_1 | 2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C016y | UEs supporting 5GS TDD FR1 and additional DMRS for coexistence with LTE CRS but not supporting TDD bands with 4Rx UE capability | D019 |  |
| 5.2.2.2.5_1 | 2Rx TDD FR1 PDSCH 0.001% BLER performance - 1x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C075 | UEs supporting 5GS TDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5, but not supporting TDD bands with 4Rx UE capability | D009 |  |
| 5.2.2.2.6_1 | 2Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C122 | UEs supporting 5GS TDD FR1 and aggregationFactorDL > 1 for PDSCH repetition multislots but not supporting TDD bands with 4Rx UE capability | D010 | NOTE 1 |
| 5.2.2.2.7_1 | 2Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C117 | UEs supporting 5GS TDD FR1 and PDSCH processing capability 2 and PDSCH mapping type B, but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.2.2.2.8_1 | 2Rx TDD FR1 PDSCH pre-emption performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-16 | C123 | UEs supporting 5GS TDD FR1 and PDSCH pre-emption indication but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.2.2.2.9_1 | 2Rx TDD FR1 HST-SFN performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.2.2.2.10_1 | 2Rx TDD FR1 HST DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.2.2.2.11_1 | 2Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C071 | UEs supporting 5GS TDD FR1 and single DCI based spatial division multiplexing scheme, but not supporting TDD bands with 4Rx UE capability | D009 |  |
| 5.2.2.2.12_1 | 2Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C113a | UEs supporting 5GS TDD FR1 and multi-DCI based multi-TRP, but not supporting TDD bands with 4Rx UE capability | D009 |  |
| 5.2.2.2.13_1 | 2Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x2 MIMO for both SA and NSA | Rel-16 | C114a | UEs supporting 5GS TDD FR1 and single DCI based FDMSchemeA, but not supporting TDD bands with 4Rx UE capability | D009 |  |
| 5.2.2.2.14_1 | 2Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C115a | UEs supporting 5GS TDD FR1 and single-DCI based inter-slot TDM, but not supporting TDD bands with 4Rx UE capability | D009 |  |
| 5.2.2.2.15 | 2Rx TDD FR1 PDSCH mapping type A performance on band with shared spectrum access | Rel-16 | C204 | UEs supporting 5GS TDD FR1 and NR-U | D025 | NOTE 1 |
| 5.2.3.1.1_1 | 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 5.2.3.1.1_2 | 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 5.2.3.1.1_4 | 4Rx FDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 | C017x | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and Enhanced Receiver Type 1 | D008 |  |
| 5.2.3.1.1_5 | 4Rx FDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for DL1024QAM for both SA and NSA | Rel-17 | C202 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and DL1024QAM | D008 | NOTE 1 |
| 5.2.3.1.2_1 | 4Rx FDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 5.2.3.1.3_1 | 4Rx FDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C017b | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and PDSCH mapping Type B | D008 D011 |  |
| 5.2.3.1.4_1 | 4Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C017y | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and LTE-NR coexistence | D008 |  |
| 5.2.3.1.5_1 | 4Rx FDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA | Rel-16 | C076 | UEs supporting 5GS FDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 4Rx antenna ports | D008 |  |
| 5.2.3.1.6_1 | 4Rx FDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-16 | C124 | UEs supporting 5GS FDD FR1 and aggregationFactorDL > 1 for PDSCH repetition multislots and 4Rx antenna ports | D008 | NOTE 1 |
| 5.2.3.1.7_1 | 4Rx FDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-16 | C118 | UEs supporting 5GS FDD FR1 and PDSCH processing capability 2 and PDSCH mapping type B and 4Rx antenna ports | D008 |  |
| 5.2.3.1.8_1 | 4Rx FDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-16 | C169 | UEs supporting 5GS FDD FR1 and PDSCH pre-emption indication and 4Rx antenna ports | D008 |  |
| 5.2.3.1.9_1 | 4Rx FDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C127 | UEs supporting 5GS FDD FR1 and enhanced demodulation processing for HST-SFN joint transmission scheme and 4Rx antenna ports | D008 |  |
| 5.2.3.1.10_1 | 4Rx FDD FR1 HST-DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C154 | UEs supporting 5GS FDD FR1 and number of active TCI states per BWP per CC and 4Rx antenna ports | D008 |  |
| 5.2.3.1.11_1 | 4Rx FDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x4 MIMO for both SA and NSA | Rel-16 | C072 | UEs supporting 5GS FDD FR1 and single DCI based spatial division multiplexing scheme and 4Rx antenna ports | D008 |  |
| 5.2.3.1.12_1 | 4Rx FDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x4 MIMO for both SA and NSA | Rel-16 | C113b | UEs supporting 5GS FDD FR1 and multi-DCI based multi-TRP and 4Rx antenna ports | D008 |  |
| 5.2.3.1.13_1 | 4Rx FDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x4 MIMO for both SA and NSA | Rel-16 | C114b | UEs supporting 5GS FDD FR1 and single DCI based FDMSchemeA and 4Rx antenna ports | D008 |  |
| 5.2.3.1.14_1 | 4Rx FDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x4 MIMO for both SA and NSA | Rel-16 | C115b | UEs supporting 5GS FDD FR1 and single-DCI based inter-slot TDM and 4Rx antenna ports | D008 |  |
| 5.2.3.2.1_1 | 4Rx TDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D009 D010 |  |
| 5.2.3.2.1_2 | 4Rx TDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 5.2.3.2.1_4 | 4Rx TDD FR1 PDSCH mapping Type A performance - 4x4 MIMO with enhanced receiver type 1 for both SA and NSA | Rel-15 | C019x | UEs supporting 5GS TDD FR1 and Enhanced Receiver Type 1 and 4Rx antenna ports | D010 |  |
| 5.2.3.2.1_5 | 4Rx TDD FR1 PDSCH mapping Type A performance - 2x4 MIMO with baseline receiver for DL1024QAM for both SA and NSA | Rel-17 | C203 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and DL 1024QAM | D010 | NOTE 1 |
| 5.2.3.2.2_1 | 4Rx TDD FR1 PDSCH mapping Type A and CSI-RS overlapped with PDSCH performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D009 D010 |  |
| 5.2.3.2.3_1 | 4Rx TDD FR1 PDSCH mapping Type B performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C019b | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and PDSCH mapping Type B | D009 D011 |  |
| 5.2.3.2.4_1 | 4Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C017z | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and LTE-NR coexistence | D009 |  |
| 5.2.3.2.5_1 | 4Rx TDD FR1 PDSCH 0.001% BLER performance - 1x4 MIMO with baseline receiver for both SA and NSA | Rel-16 | C077 | UEs supporting 5GS TDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 4Rx antenna ports | D009 |  |
| 5.2.3.2.6_1 | 4Rx TDD FR1 PDSCH repetitions over multiple slots performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-16 | C125 | UEs supporting 5GS TDD FR1 and aggregationFactorDL > 1 for PDSCH repetition multislots and 4Rx antenna ports | D010 | NOTE 1 |
| 5.2.3.2.7_1 | 4Rx TDD FR1 PDSCH Mapping Type B and UE processing capability 2 performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-16 | C119 | UEs supporting 5GS TDD FR1 and PDSCH processing capability 2 and PDSCH mapping type B and 4Rx antenna ports | D010 |  |
| 5.2.3.2.8_1 | 4Rx TDD FR1 PDSCH pre-emption performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-16 | C170 | UEs supporting 5GS TDD FR1 and PDSCH pre-emption indication and 4Rx antenna ports | D010 |  |
| 5.2.3.2.9_1 | 4Rx TDD FR1 HST-SFN performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C019y | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and enhanced demodulation processing for HST-SFN joint transmission scheme | D010 D011 |  |
| 5.2.3.2.10_1 | 4Rx TDD FR1 HST DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 D011 | NOTE 1 |
| 5.2.3.2.11_1 | 4Rx TDD FR1 PDSCH Single-DCI based SDM scheme performance - 2x2 MIMO for both SA and NSA | Rel-16 | C073 | UEs supporting 5GS TDD FR1 and single DCI based spatial division multiplexing scheme and 4Rx antenna ports | D009 |  |
| 5.2.3.2.12_1 | 4Rx TDD FR1 PDSCH Multiple-DCI based transmission scheme performance - 2x4 MIMO for both SA and NSA | Rel-16 | C113c | UEs supporting 5GS TDD FR1 and multi-DCI based multi-TRP and 4Rx antenna ports | D009 |  |
| 5.2.3.2.13_1 | 4Rx TDD FR1 PDSCH Single-DCI based FDM scheme A performance - 2x4 MIMO for both SA and NSA | Rel-16 | C114c | UEs supporting 5GS TDD FR1 and single DCI based FDMSchemeA and 4Rx antenna ports | D009 |  |
| 5.2.3.2.14_1 | 4Rx TDD FR1 PDSCH Single-DCI based Inter-slot TDM scheme performance - 2x4 MIMO for both SA and NSA | Rel-16 | C115c | UEs supporting 5GS TDD FR1 and single-DCI based inter-slot TDM and 4Rx antenna ports | D009 |  |
| 5.2.3.2.15 | 2Rx FDD FR1 PDSCH mapping type A performance on band with shared spectrum access | Rel-16 | C205 | UEs supporting 5GS FDD FR1 and NR-U | D026 | NOTE 1 |
| 5.2A.2.1.1 | 2Rx Normal PDSCH Demodulation Performance for CA (2DL CA) for both SA and NSA | Rel-15 | C017g | UEs supporting 5GS FR1 AND 2DLCA but not supporting 4Rx UE capability on any CCs | E016 |  |
| 5.2A.2.1.2 | 2Rx Normal PDSCH Demodulation Performance for CA (3DL CA) for both SA and NSA | Rel-15 | C017h | UEs supporting 5GS FR1 AND 3DLCA but not supporting 4Rx UE capability on any CCs | E017 |  |
| 5.2A.2.1.3 | 2Rx Normal PDSCH Demodulation Performance for CA (4DL CA) for both SA and NSA | Rel-15 | C017i | UEs supporting 5GS FR1 AND 4DLCA but not supporting 4Rx UE capability on any CCs | E018 |  |
| 5.2A.2.2.1 | 2Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) | Rel-15 | C017j | UEs supporting 5GS FR1 AND 2DLCA but not supporting 4Rx UE capability on any CCs | E003a |  |
| 5.2A.2.2.2 | 2Rx PDSCH Demodulation Performance for CA with power imbalance (3DL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 AND 3DLCA but not supporting 4Rx UE capability on any CCs | E033 | NOTE 1 |
| 5.2A.2.2.3 | 2Rx PDSCH Demodulation Performance for CA with power imbalance (4DL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and 4DLCA but not supporting 4Rx UE capability on any 4CCs | E034 | NOTE 1 |
| 5.2A.2.3 | 2Rx TDD FR1 PDSCH mapping type A performance on band with shared spectrum access | Rel-16 | C204 | UEs supporting 5GS TDD FR1 and NR-U | D0265 | NOTE 1 |
| 5.2A.2.4.1 | 2RX PDSCH Demodulation Performance for HST-SFN CA | Rel-16 | C152b | UEs supporting 5GS FR1 AND enhanced demodulation processing for carrier aggregation for HST-SFN joint transmission but not supporting 4Rx UE capability on any CCs | E016 |  |
| 5.2A.2.5.1 | 2RX PDSCH Demodulation Performance for HST-DPS CA | Rel-15 | C152a | UEs supporting 5GS FR1 AND 2DLCA AND number of active TCI | E016 |  |
| 5.2A.3.1.1 | 4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | Rel-15 | C017g | UEs supporting 5GS FR1 AND 2DLCA AND supporting 4Rx antenna ports on all CCs | E0016 |  |
| 5.2A.3.1.2 | 4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | Rel-15 | C017h | UEs supporting 5GS FR1 AND 3DLCA AND supporting 4Rx antenna ports on all CCs | E017 |  |
| 5.2A.3.1.3 | 4Rx Normal PDSCH Demodulation Performance for CA (4DL CA) | Rel-15 | C017i | UEs supporting 5GS FR1 AND 4DLCA AND supporting 4Rx antenna ports on all CCs | E018 |  |
| 5.2A.3.2.1 | 4Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA) | Rel-15 | C017g | UEs supporting 5GS FR1 AND 2DLCA AND supporting 4Rx antenna ports on all CCs | E003a |  |
| 5.2A.3.3 | 4Rx TDD FR1 PDSCH mapping type A performance on band with shared spectrum access | Rel-16 | C2054 | UEs supporting 5GS TDD FR1 and NR-U | D025 | NOTE 1 |
| 5.2A.3.4.1 | 4RX PDSCH Demodulation Performance for HST-SFN CA | Rel-16 | C127a | UEs supporting 5GS FR1 AND enhanced demodulation processing for carrier aggregation for HST-SFN joint transmission AND supporting 4Rx TDD and FDD UE capability on any CCs | E016 | NOTE 1 |
| 5.2A.3.5.1 | 4RX PDSCH Demodulation Performance for HST-DPS CA | Rel-15 | C154a | UEs supporting 5GS FR1 AND 2DLCA AND number of active TCI AND supporting 4Rx TDD and FDD UE capability on any CCs | E016 | NOTE 1 |
| 5.2A.3A.1.1 | 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (2DL CA) | Rel-15 | C017g | UEs supporting 5GS FR1 and 2DLCA AND supporting 4Rx UE capability on some of the CCs | E016 |  |
| 5.2A.3A.1.2 | 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (3DL CA) | Rel-15 | C017h | UEs supporting 5GS FR1 and 3DLCA AND supporting 4Rx UE capability on some of the CCs | E017 |  |
| 5.2A.3A.1.3 | 2Rx-4Rx Normal PDSCH Demodulation Performance for CA (4DL CA) | Rel-15 | C017i | UEs supporting 5GS FR1 and 4DLCA AND supporting 4Rx UE capability on some of the CCs | E018 |  |
| 5.3.2.1.1 | 2Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.3.2.1.2 | 2Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.3.2.1.3 | 2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving | Rel-16 | C088 | UEs supporting 5GS FDD FR1 and Long DRX Cycle and DRX adaptation but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 5.3.2.2.1 | 2Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.3.2.2.2 | 2Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.3.2.2.3 | 2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving | Rel-16 | C089 | UEs supporting 5GS TDD FR1 and Long DRX Cycle and DRX adaptation but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 5.3.3.1.1 | 4Rx FDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 5.3.3.1.2 | 4Rx FDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 5.3.3.1.3 | 4Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving | Rel-16 | C090 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports and Long DRX Cycle and DRX adaptation | D008 |  |
| 5.3.3.2.1 | 4Rx TDD FR1 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 5.3.3.2.2 | 4Rx TDD FR1 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 5.3.3.2.3 | 4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving | Rel-16 | C091 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports and Long DRX Cycle and DRX adaptation | D010 |  |
| 5.5.1 | FR1 Sustained downlink data rate performance for single carrier | Rel-15 | C001 | UEs supporting 5GS FDD FR1 or TDD FR1 (SA) | D008 D009 D010 |  |
| 5.5A.1.1 | FR1 Sustained downlink data rate performance for CA (2DLCA) | Rel-15 | C001e | UEs supporting 5GS FDD FR1 or TDD FR1 (SA) and supporting 2DLCA | E016 |  |
| 5.5A.1.2 | FR1 Sustained downlink data rate performance for CA (3DL CA) | Rel-15 | C001i | UEs supporting 5GS FDD FR1 or TDD FR1 (SA) and supporting 3DLCA | E017 |  |
| 5.5A.1.3 | FR1 Sustained downlink data rate performance for CA (4DL CA) | Rel-15 | C001j | UEs supporting 5GS FDD FR1 or TDD FR1 (SA) and supporting 4DLCA | E018 |  |
| 5.5A.1.4 | FR1 Sustained downlink data rate performance for CA (5DL CA) | Rel-15 | C001k | UEs supporting 5GS FDD FR1 or TDD FR1 (SA) and supporting 5DLCA | E019 |  |
| 6 | CSI reporting requirements (Conducted requirements) |  |  |  |  |  |
| 6.2.1.1.1.1 | 1Rx FDD FR1 periodic CQI reporting under AWGN conditions for RedCap | Rel-17 | C177a | RedCap UEs supporting 5GS FDD FR1 | D008 | NOTE 1 |
| 6.2.2.1.1.1 | 2Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.2.2.1.1.2 | 2Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | Rel-16 | C074 | UEs supporting 5GS FDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5, but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.2.2.1.2.1 | 2Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.2.2.1.2.2 | 2Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.2.2.2.1.1 | 2Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.2.2.2.1.2 | 2Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | Rel-16 | C075 | UEs supporting 5GS TDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5, but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.2.2.2.2.1 | 2Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.2.2.2.2.2 | 2Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.2.3.1.1.1 | 4Rx FDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 6.2.3.1.1.2 | 4Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | Rel-16 | C076 | UEs supporting 5GS FDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 4Rx antenna ports | D008 |  |
| 6.2.3.1.2.1 | 4Rx FDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 6.2.3.1.2.2 | 4Rx FDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 |  |
| 6.2.3.2.1.1 | 4Rx TDD FR1 periodic CQI reporting under AWGN conditions for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 6.2.3.2.1.2 | 4Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA | Rel-16 | C077 | UEs supporting 5GS TDD FR1 and alternative 64QAM MCS table for PDSCH and CQI table with target BLER of 10^-5 and 4Rx antenna ports | D010 |  |
| 6.2.3.2.2.1 | 4Rx TDD FR1 periodic wideband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 6.2.3.2.2.2 | 4Rx TDD FR1 aperiodic subband CQI reporting under fading conditions for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 6.2A.3.1.1 | 2Rx CQI reporting accuracy under AWGN conditions for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 | Test execution not necessary if 6.2A.3.1.2 is executed. |
| 6.2A.3.1.2 | 2Rx CQI reporting accuracy under AWGN conditions for CA (3DL CA) | Rel-15 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 | Test execution not necessary if 6.2A.3.1.3 is executed. |
| 6.2A.3.1.3 | 2Rx CQI reporting accuracy under AWGN conditions for CA (4DL CA) | Rel-15 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |
| 6.3.2.1.1 | 2Rx FDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.3.2.1.2 | 2Rx FDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.3.2.1.3 | 2Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.3.2.1.4 | 2Rx FDD FR1 Single PMI with 32Tx Type I – SinglePanel Codebook for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.3.2.1.5 | 2Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | Rel-15 | C015c | UEs supporting 5GS FDD FR1 and supporting Type II codebook but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.3.2.1.6 | 2Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | Rel-16 | C128 | UEs supporting 5GS FDD FR1 and Enhanced Type II codebook with at least 16 ports per CSI-RS resource, but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.3.2.2.1 | 2Rx TDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.3.2.2.2 | 2Rx TDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.3.2.2.3 | 2Rx TDD FR1 Multiple PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.3.2.2.4 | 2Rx TDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.3.2.2.5 | 2Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | Rel-15 | C016c | UEs supporting 5GS TDD FR1 and supporting Type II codebook but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.3.2.2.6 | 2Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | Rel-16 | C129 | UEs supporting 5GS TDD FR1 and Enhanced Type II codebook with at least 16 ports per CSI-RS resource, but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.3.3.1.1 | 4Rx FDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 D011 |  |
| 6.3.3.1.2 | 4Rx FDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 D011 |  |
| 6.3.3.1.3 | 4Rx FDD FR1 Multiple PMI with 16Tx Type I – SinglePanel Codebook for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 D011 |  |
| 6.3.3.1.4 | 4Rx FDD FR1 Single PMI with 32Tx Type I – SinglePanel Codebook for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 D011 |  |
| 6.3.3.1.5 | 4Rx FDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | Rel-15 | C017c | UEs supporting 5GS FDD FR1 and supporting Type II codebook and 4Rx antenna ports | D008 D011 |  |
| 6.3.3.1.6 | 4Rx FDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | Rel-16 | C130 | UEs supporting 5GS FDD FR1 and Enhanced Type II codebook with at least 16 ports per CSI-RS resource, and 4Rx antenna ports | D008 D011 |  |
| 6.3.3.2.1 | 4Rx TDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D010 D011 |  |
| 6.3.3.2.2 | 4Rx TDD FR1 Single PMI with 8TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D010 D011 |  |
| 6.3.3.2.3 | 4Rx TDD FR1 Multiple PMI with 16Tx Type1 - SinglePanel codebook for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 6.3.3.2.4 | 4Rx TDD FR1 Single PMI with 32Tx Type1 - SinglePanel codebook for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 |  |
| 6.3.3.2.5 | 4Rx TDD FR1 Multiple PMI with 16Tx TypeII codebook for both SA and NSA | Rel-15 | C019c | UEs supporting 5GS TDD FR1 and supporting Type II codebook and 4Rx antenna ports | D010 |  |
| 6.3.3.2.6 | 4Rx TDD FR1 Multiple PMI with 16Tx Enhanced TypeII codebook for both SA and NSA | Rel-16 | C131 | UEs supporting 5GS TDD FR1 and Enhanced Type II codebook with at least 16 ports per CSI-RS resource, and 4Rx antenna ports | D010 |  |
| 6.4.2.1_1 | 2Rx FDD FR1 RI reporting for both SA and NSA | Rel-15 | C015 | UEs supporting 5GS FDD FR1 but not supporting FDD bands with 4Rx UE capability | D008 |  |
| 6.4.2.2_1 | 2Rx TDD FR1 RI reporting for both SA and NSA | Rel-15 | C016 | UEs supporting 5GS TDD FR1 but not supporting TDD bands with 4Rx UE capability | D010 |  |
| 6.4.3.1_1 | 4Rx FDD FR1 RI reporting for both SA and NSA | Rel-15 | C017 | UEs supporting 5GS FDD FR1 and 4Rx antenna ports | D008 D011 |  |
| 6.4.3.2_1 | 4Rx TDD FR1 RI reporting for both SA and NSA | Rel-15 | C019 | UEs supporting 5GS TDD FR1 and 4Rx antenna ports | D010 D011 |  |
| 7 | Demodulation performance requirements (Radiated requirements) |  |  |  |  |  |
| 7.2.2.2.1_1 | 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for SA and NSA | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D013 D014 D015 |  |
| 7.2.2.2.1_2 | 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with enhanced type 1 receiver for SA and NSA | Rel-15 | C062 | UEs supporting 5GS TDD FR2 and Enhanced Receiver Type 1 | D014 |  |
| 7.2.2.2.1_3 | 2Rx TDD FR2 PDSCH mapping Type A performance - 2x2 MIMO with 256QAM for SA and NSA (Rel-16 and forward) | Rel-16 | C126 | UEs supporting 5GS TDD FR2 and PDSCH 256QAM for FR2 | D013 |  |
| 7.2.2.2.2_1 | 2Rx TDD FR2 PDSCH repetitions over multiple slots - 2x2 MIMO with baseline receiver for SA and NSA | Rel-16 | C171 | UEs supporting 5GS TDD FR2 and aggregationFactorDL > 1 for PDSCH repetition multislots | D014 | NOTE 1 |
| 7.2.2.2.3_1 | 2Rx TDD FR2 PDSCH mapping Type B performance - 2x2 MIMO with baseline receiver for SA and NSA | Rel-16 | C172 | UEs supporting 5GS TDD FR2 and aggregationFactorDL > 1 for PDSCH repetition multislots | D014 |  |
| 7.2A.2.1 | 2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (2DLCA) | Rel-15 | C061a | UEs supporting 5GS TDD FR2 AND 2DLCA | E032 |  |
| 7.2A.2.2 | 2Rx TDD FR2 CA requirements for normal PDSCH Demodulation Performance for both SA and NSA (3DLCA) | Rel-15 | C061b | UEs supporting 5GS TDD FR2 AND 3DLCA | E033 |  |
| 7.3.2.2.1 | 2Rx TDD FR2 PDCCH 1 Tx antenna performance for both SA and NSA | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D014 |  |
| 7.3.2.2.2 | 2Rx TDD FR2 PDCCH 2 Tx antenna performance for both SA and NSA | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D014 |  |
| 7.3.2.2.3 | 2Rx TDD FR2 PDCCH 1 Tx antenna performance for power saving | Rel-16 | C092 | UEs supporting 5GS TDD FR2 and Long DRX Cycle and DRX adaptation | D014 |  |
| 7.5.1 | FR2 Sustained downlink data rate performance for single carrier | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D014 |  |
| 7.5.1.1 | FR2 SDR performance for CA (2DL CA) | Rel-15 | C061a | UEs supporting 5GS TDD FR2 and CA (2DL CA) | E032 |  |
| 7.5.1.2 | FR2 SDR performance for CA (3DL CA) | Rel-15 | C061b | UEs supporting 5GS TDD FR2 and CA (3DL CA) | E033 | NOTE 1 |
| 7.5.1.3 | FR2 SDR performance for CA (4DL CA) | Rel-15 | TBD | UEs supporting 5GS TDD FR2 and CA (4DL CA) | E034 | NOTE 1 |
| 7.5.1.4 | FR2 SDR performance for CA (5DL CA) | Rel-15 | TBD | UEs supporting 5GS TDD FR2 and CA (5DL CA) | E035 | NOTE 1 |
| 7.5.1.5 | FR2 SDR performance for CA (6DL CA) | Rel-15 | TBD | UEs supporting 5GS TDD FR2 and CA (6DL CA) | E036 | NOTE 1 |
| 7.5.1.6 | FR2 SDR performance for CA (7DL CA) | Rel-15 | TBD | UEs supporting 5GS TDD FR2 and CA (7DL CA) | E037 | NOTE 1 |
| 7.5.1.7 | FR2 SDR performance for CA (8DL CA) | Rel-15 | TBD | UEs supporting 5GS TDD FR2 and CA (8DL CA) | E038 |  |
| 8 | CSI reporting requirements (Radiated requirements) |  |  |  |  |  |
| 8.2.2.2.1.1 | 2Rx TDD FR2 periodic CQI reporting under AWGN performance for both SA and NSA | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D014 |  |
| 8.2.2.2.2.1 | 2Rx TDD FR2 aperiodic wideband CQI reporting under fading performance for both SA and NSA | Rel-15 | C061F | UEs supporting 5GS TDD FR2 | D014 | Skip TC 8.2.2.2.2.1 if TS 38.521-4 TC 8.2.2.2.2.1_1 has been executed and passed. |
| 8.2.2.2.2.1_1 | 2Rx TDD FR2 aperiodic CQI reporting under fading performance for both SA and NSA – 256QAM | Rel-16 | C126 | UEs supporting 5GS TDD FR2 and DL 256QAM | D013 | NOTE 1 |
| 8.2A.3.1.1 | 2Rx CQI reporting accuracy under AWGN conditions for CA (2DL CA) | Rel-15 | C006c | UEs supporting 5GS FR2 and CA (2DL CA) | E032 | Test execution not necessary if 8.2A.3.1.2 is executed. |
| 8.2A.3.1.2 | 2Rx CQI reporting accuracy under AWGN conditions for CA (3DL CA) | Rel-15 | C006d | UEs supporting 5GS FR2 and CA (3DL CA) | E033 | Test execution not necessary if 8.2A.3.1.3 is executed. |
| 8.2A.3.1.3 | 2Rx CQI reporting accuracy under AWGN conditions for CA (4DL CA) | Rel-15 | C006e | UEs supporting 5GS FR2 and CA (4DL CA) | E034 |  |
| 8.3.2.2.1 | 2Rx TDD FR2 Single PMI with 2TX TypeI-SinglePanel codebook for both SA and NSA | Rel-15 | C061 | UEs supporting 5GS TDD FR2 | D014 |  |
| 8.4.2.2.1 | 2Rx TDD FR2 RI reporting for both SA and NSA | FFS | FFS | FFS | FFS | NOTE 1 |
| 9 | Demodulation performance requirements for interworking |  |  |  |  |  |
| 9.4B.1.1 | Sustained downlink data rate performance for EN-DC within FR1 | Rel-15 | C020 | UEs supporting 5GS FDD FR1 or TDD FR1 (NSA) | D008 D009 D010 |  |
| 9.4B.1.2 | Sustained downlink data rate performance for EN-DC including FR2 NR carrier | FFS | FFS | FFS | FFS | NOTE 1 |
| 10 | CSI reporting requirements for interworking |  |  |  |  |  |
| 11 | V2X requirements |  |  |  |  |  |
| 11.1.2.1.1_1 | 2Rx FR1 PSSCH performance - single active PSSCH link | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |
| 11.1.3.1.1_1 | 2Rx FR1 PSCCH performance - single active PSSCH link | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |
| 11.1.5.1.1_1 | 2Rx FR1 PSCCH performance - single active PSSCH link | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |
| 11.1.6.1.1_1 | 2Rx FR1 Power imbalance performance - two active PSSCH link | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |
| 11.1.7.1.1_1 | 2Rx FR1 HARQ buffer soft combining performance - maximum number of HARQ processes | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |
| 11.1.8.1.1_1 | 2Rx FR1 PSCCH decoding capability - maximum number of received PSCCHs | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |
| 11.1.9.1.1_1 | 2Rx FR1 PSFCH decoding capability - maximum number of received PSFCHs | Rel-16 | C079 | UEs supporting 5GS FR1 and NR sidelink | D016 |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-4. NOTE 2: Void. NOTE 3: Void. |  |  |  |  |  |  |

<< End of Changes >>