3GPP TSG-RAN5 Meeting #98	R5-230863

# <Athens, Greece, 27th Feb - 3rd March, 2023>

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2248 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | add test case for TS38.533 clause 5.5.5.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_feMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2023-2-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Add new test case for the TRP specific BFD and RLM |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new ‘EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode’ test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Depends on the RAN5 CR R5-230860 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.5.5.7	EN-DC FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX

Editor's Note: 	This test case is incomplete for Test frequency f > 40.8 GHz

-	This test case is incomplete for UE power class other than PC3.

5.5.5.7.1	Test purpose

The purpose of this test is to verify that the UE properly detects CSI-RS-based beam failure in the set q0 configured for an active SCell and that the UE performs correct CSI-RS-based link recovery based on beam candidate set q1. The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the SCell with schedulingRequestID-BFR-SCell-r16 configuration, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 SCell requirements in TS 38.133 [6] clause 8.5.

5.5.5.7.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from release 16 onwards supporting SCell BFR.

5.5.5.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 5.5.5.0.2 and 5.5.5.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.5.5.5.7.

5.5.5.7.4	Test description

There are three cells configured in this test. Cell 1 is the E-UTRAN PCell, Cell 2 is the PSCell and Cell 3 is the SCell. This test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 5.5.5.7.4-1 shows the variation of the downlink SNR of the active SCell and the SNR of the CSI-RS in set q0 in the active SCell to emulate CSI-RS based beam failure. Figure 5.5.5.7.4-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery.

Figure 5.5.5.7.4-1: SNR and L1-RSRP variation for CSI-RS based beam failure detection and link recovery testing for SCell in DRX mode

5.5.5.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 5.5.5.7.4.1-1.

Table 5.5.5.7.4.1-1: Supported test configurations for EN-DC FR2 SCell beam failure detection and link recovery testing in DRX

| Configuration | Description |
| --- | --- |
| 5.5.5.7-1 | LTE FDD, TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |
| 5.5.5.7-2 | LTE TDD, TDD duplex mode, 120 kHz SSB SCS, 100 MHz bandwidth |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 5.5.5.7.4.1-2.

Table 5.5.5.7.4.1-2: Initial conditions for EN-DC FR2 SCell beam failure detection and link recovery testing in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 5.5.5.7.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 5.5.5.7.4.1-3.

2.	Message contents are defined in clause 5.5.5.7.4.3.

3.	There are one E-UTRAN cell and two NR cells specified in the test. E-UTRAN Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table 5.5.5.7.4.1-3: General test parameters for FR2 SCell for beam failure detection and link recovery testing in DRX mode

| Parameter |  | Test Config. | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active E-UTRA PCell |  | 1-2 |  | Cell 1 |  |
| E-UTRA RF Channel Number |  | 1-2 |  | 1 |  |
| Active PCell |  | 1-2 |  | Cell 2 |  |
| RF Channel Number for PSCell |  | 1-2 |  | 2 |  |
| Active SCell |  | 1-2 |  | Cell 3 |  |
| RF Channel Number for SCell |  | 1-2 |  | 3 |  |
| Duplex mode |  | 1-2 |  | TDD |  |
| TDD Configuration |  | 1-2 |  | TDDConf.3.1 |  |
| BWchannel |  | 1-2 | MHz | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1-2 |  | 66 |  |
| PDSCH/PDCCH subcarrier spacing |  | 1-2 | kHz | 120 |  |
| DL initial BWP configuration |  | 1-2 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | 1-2 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | 1-2 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | 1-2 |  | ULBWP.1.1 |  |
| PDSCH Reference Channel |  | 1-2 |  | SR.3.2 TDD |  |
| RMSI CORESET Reference Channel |  | 1-2 |  | CR.3.1 TDD | A.1.2.2 |
| Dedicated CORESET Reference Channel |  | 1-2 |  | CCR.3.1 TDD |  |
| OCNG parameters |  | 1-2 |  | OP.1 | A.2.1 |
| CP length |  | 1-2 |  | Normal |  |
| PDSCH/PDCCH TCI state |  | 1-2 |  | TCI.State.0 |  |
| CSI-RS for tracking |  | 1-2 |  | TRS.2.1 TDD |  |
| SSB Configuration |  | 1-2 |  | SSB.3 FR2 | A.3 |
| SMTC Configuration |  | 1-2 |  | SMTC.3 | A.4 |
| PRACH Configuration |  | 1-2 |  | FR2 PRACH configuration 4 | Table A.7.2-1 |
| DRX configuration |  | 1-2 |  | DRX.3 | A.5 |
| CSI-RS configuration for BFD/CBD in activated SCell |  | 1-2 |  | CSI-RS.3.2 TDD | A.1.4.2 |
| CSI-RS index assigned as BFD RS (q0) in activated SCell |  | 1-2 |  | 0 |  |
| CSI-RS index assigned as CBD RS (q1) in activated SCell |  | 1-2 |  | 1 |  |
| CSI-RS configuration for RLM in PSCell |  | 1-2 |  | CSI-RS.3.2 TDD | A.1.4.2 |
| Beam failure detection transmission parameters | DCI format | 1-2 |  | 1-0 |  |
|  | Number of Control OFDM symbols | 1-2 |  | 2 |  |
|  | Aggregation level | 1-2 | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 1-2 | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 1-2 | dB | 0 |  |
|  | DMRS precoder granularity | 1-2 |  | REG bundle size |  |
|  | REG bundle size | 1-2 |  | 6 |  |
| Gap pattern ID |  | 1-2 |  | N/A |  |
| schedulingRequestID-BFR-SCell-r16 |  | 1-2 |  | Configured |  |
| Periodicity of PUCCH for SR configuration for BFR on SCell |  | 1-2 | slot | 40 | 5ms |
| Offset of PUCCH for SR configuration for BFR on SCell |  | 1-2 | slot | 4 |  |
| PUCCH parameters for SR configuration for BFR on SCell |  | 1-2 |  | Table 8.3.3.1.2-1 in [28] |  |
| rlmInSyncOutOfSyncThreshold |  | 1-2 |  | absent | Value 0 is applied. (Table 8.1.1-1 in TS 38.133 [6]). |
| rsrp-ThresholdSSB |  | 1-2 | dBm/SCS | -109Note 2 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  | 1-2 |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  | 1-2 |  | n1 | see TS 38.321 [12], clause 5.17 |
| beamFailureDetectionTimer |  | 1-2 |  | pbfd4 | see TS 38.321 [12], clause 5.17 |
| CSI-RS configuration for CSI reporting |  | 1-2 |  | CSI-RS.3.1 TDD | A.1.4.2 |
| reportConfigType |  | 1-2 |  | periodic |  |
| reportQuantity |  | 1-2 |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity |  | 1-2 | slot | 40 |  |
| CSI reporting offset |  | 1-2 | slot | 4 |  |
| T310 |  | 1-2 | ms | 1000 |  |
| N310 |  | 1-2 |  | 2 |  |
| T1 |  | 1-2 | s | 1 | The UE shall be fully synchronized to cell 1 during T1 |
| T2 |  | 1-2 | s | 5.43 |  |
| T3 |  | 1-2 | s | 5.16 |  |
| T4 |  | 1-2 | s | 0 |  |
| T5 |  | 1-2 | s | 0.31 |  |
| D1 |  | 1-2 | s | 0.27 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. Note 2: Including test toelerance given in Annex F.1.3.2 |  |  |  |  |  |

5.5.5.7.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to Cell 1, Cell 2 and Cell 3. The UE shall be configured for periodic CSI reporting with a reporting periodicity defined in CSI-RS configuration. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test. During the test the UE is scheduled to transmit continuously in UL.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.4.

2.	The SS shall configure SCell (Cell 3) on the SCC as per TS 38.508-1 [14] clause 7.5.2, with the message content exceptions defined in clause 5.5.5.7.4.3. NR RRCReconfiguration message is contained in RRCConnectionReconfiguration and NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete.

3.	The SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [12], clauses 5.9, 6.1.3.10).

4.	Set the parameters of NR Cell 2 and Cell 3 according to T1 in Table 5.5.5.7.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

5.	When T1 expires the SS shall change the SNR value to T2 as specified in Table 5.5.5.7.5-1. T2 starts.

6.	When T2 expires the SS shall change the SNR value to T3 as specified in Table 5.5.5.7.5-1. T3 starts.

7.	When T3 expires the SS shall change the SNR value to T4 as specified in Table 5.5.5.7.5-1. T4 starts.

8.	When T4 expires the SS shall change the SNR value to T5 as specified in Table 5.5.5.7.5-1. T5 starts.

9.	If the SS:

a)	detects uplink power on NR carrier in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B; and

b)	does not detect PUCCH with LRR before time point B, and

c)	detects PUCCH with LRR, followed by BFR MAC CE containing a beam associated with the candidate beam set q1 before time point F (D1 after the start of T5),

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

10.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 5.5.5.7.5-1.

11.	If the iteration fails, the SS shall first attempt to release and add the FR2 SCell. If that also fails, then the UE is switched OFF/ON to proceed with the next iteration, and ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.4.

12.	Repeat steps 2-11 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

5.5.5.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 5.5.5.7.4.3-1: Common Exception messages for EN-DC FR2 SCell for beam failure detection and link recovery testing in DRX mode

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-8 with Condition CSI-RS BFD Table H.3.1-12 on Cell 3 Table H.3.1-13 on Cell 3 Table H.3.7-1 with condition DRX.3 |

Table 5.5.5.7.4.3-2: MAC-CellGroupConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestID-BFR-SCell-r16 | SchedulingRequestId |  |  |
| } |  |  |  |

Table 5.5.5.7.4.3-3: SchedulingRequestResourceConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl40 | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

5.5.5.7.5	Test requirement

Tables 5.5.5.7.4.1-3 and 5.5.5.7.5-1 define the primary level settings including test tolerances for EN-DC FR2 SCell for beam failure detection and link recovery testing in DRX.

Table 5.5.5.7.5-1: NR Cell specific test parameters for EN-DC FR2 SCell for beam failure detection and link recovery testing in DRX mode

| Parameter |  | Unit | Cell2 | Cell3 Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 to T5 | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9.1 | Setup 1 defined in A.9.1 |  |  |  |  |
| Assumption for UE beamsNote 10 |  |  | Rough | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 | 13.7 | 5.7 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | 0.2 | 0.2 | 0.2 | 20 | 20 | 20 |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS kHz | -104.5 | -104.5 | -104.5 | -84.7 | -84.7 | -84.7 |
| Noc | Config 1 | dBm/120kHz | -104.7 | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 5.5.5.7.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause D.4. Note 10: Information about types of UE beam is given in B.2.1.3 in TS 38.133 [6], and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 2.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 2 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 2.

During T3 the UE shall detect beam failure and initial link recovery. During T4 and T5 the UE measures and evaluates beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 260+10 ms after the start of T5, the UE shall transmit PUCCH with LRR, followed by BFR MAC CE containing a beam associated with the candidate beam set q1. The UE shall not transmit PUCCH with an LRR with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

<< Start of changes >>

5.5.5.8	EN-DC TRP specific Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode

Editor's Note: 	This test case is incomplete for Test frequency f > 40.8 GHz

-	This test case is incomplete for UE power class other than PC3.

5.5.5.8.1	Test purpose

The purpose of this test is to verify that the UE properly detects the TRP specific CSI-RS-based beam failure in the set (q0,0), (q0,1) configured for a serving PSCell and a cell with PCID different from the serving cell, and that the UE performs correct CSI-RS-based link recovery based on beam candidate set (q1,0) and (q1,1). The purpose is to test the downlink monitoring for beam failure detection within the UEs active DL BWP of the PSCell with schedulingRequestID-BFR-r17 configured, during the evaluation period, and link recovery, when DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in TS 38.133 [6] clause 8.5.

5.5.5.8.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from release 17 onwards supporting PSCell BFR.

5.5.5.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 5.5.5.0.2 and 5.5.5.0.4.

The normative reference for this requirement is TS 38.133 [6] clause A.5.5.5.8.

5.5.5.8.4	Test description

There are three cells, cell 1 is the E-UTRAN PCell, cell 2 is the PSCell and cell 3 is the cell with different PCID in the test. The test consists of five successive time periods, with time duration of T1, T2, T3, T4 and T5 respectively. Figure 5.5.5.8.4-1 shows the variation of the downlink SNR of the PSCell and the SNR of the CSI-RS in set q0 in the active PSCell to emulate CSI-RS based beam failure. Figure 5.5.5.8.4-1 additionally shows the variation of the downlink L1-RSRP of the CSI-RS in set q1 of the candidate beam used for link recovery.

Figure 5.5.5.8.4-1: SNR and L1-RSRP variation for CSI-RS-based beam failure detection and link recovery testing in DRX mode

5.5.5.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 5.5.5.8.4.1-1.

Table 5.5.5.8.4.1-1: Supported test configurations for FR2 PSCell

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SCS, 100 MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 5.5.5.8.4.1-2.

Table 5.5.5.8.4.1-2: Initial conditions for EN-DC FR2 PSCell beam failure detection and link recovery testing in DRX

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.5-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 5.5.5.8.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 5.5.5.8.4.1-3.

2.	Message contents are defined in clause 5.5.5.8.4.3.

3.	There are one E-UTRAN cell and one NR cell specified in the test. E-UTRAN Cell 1 is the cell used for connection setup with the power level set according to Annex C.1.1 and C.1.2 for this test.

Table A.5.5.5.8.4.1-3: General test parameters for FR2 PSCell for CSI-RS-based beam failure detection and link recovery testing in DRX mode

| Parameter |  | Test Config. | Unit | Value | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Test 1 |  |
| Active E-UTRA PCell |  | 1-2 |  | Cell 1 |  |
| E-UTRA RF Channel Number |  | 1-2 |  | 1 |  |
| Active PSCell |  | 1-2 |  | Cell 2 |  |
| RF Channel Number |  | 1-2 |  | 2 |  |
| Duplex mode |  | 1-2 |  | TDD |  |
| TDD Configuration |  | 1-2 |  | TDDConf.3.1 |  |
| BWchannel |  | 1-2 |  | 100: NRB,c = 66 |  |
| Data RBs allocated |  | 1-2 |  | 66 |  |
| PDSCH/PDCCH subcarrier spacing |  | 1-2 | kHz | 120 |  |
| DL initial BWP configuration |  | 1-2 |  | DLBWP.0.1 |  |
| DL dedicated BWP configuration |  | 1-2 |  | DLBWP.1.1 |  |
| UL initial BWP configuration |  | 1-2 |  | ULBWP.0.1 |  |
| UL dedicated BWP configuration |  | 1-2 |  | ULBWP.1.1 |  |
| PDSCH Reference Channel |  | 1-2 |  | SR.3.2 TDD |  |
| RMSI CORESET Reference Channel |  | 1-2 |  | CR.3.1 TDD | A.1.2.2 |
| Dedicated CORESET Reference Channel |  | 1-2 |  | CCR.3.1 TDD |  |
| OCNG parameters |  | 1-2 |  | OP.1 | A.2.1 |
| CP length |  | 1-2 |  | Normal |  |
| PDSCH/PDCCH TCI state |  | 1-2 |  | TCI.State.0 |  |
| CSI-RS for tracking |  | 1-2 |  | TRS.2.1 TDD |  |
| SSB Configuration |  | 1-2 |  | SSB.1 FR2 | A.3 |
| SMTC Configuration |  | 1-2 |  | SMTC.3 | A.4 |
| PRACH Configuration |  | 1-2 |  | FR2 PRACH configuration 4 | Table A.7.2-1 |
| DRX configuration |  | 1-2 |  | DRX.3 | A.5 |
| CSI-RS configuration for TRP0 |  | 1-2 |  | CSI-RS.3.2 TDD | A.1.4.2 |
| CSI-RS configuration for TRP1 |  |  |  | CSI-RS.3.6 TDD | A.1.4.2 |
| CSI-RS index assigned as BFD RS (q0,0) |  | 1-2 |  | CSI-RS#0 |  |
| CSI-RS index assigned as CBD RS (q1,0) |  | 1-2 |  | CSI-RS#1 |  |
| CSI-RS index assigned as BFD RS (q0,1) |  | 1-2 |  | CSI-RS#2 |  |
| CSI-RS index assigned as CBD RS (q1,1) |  | 1-2 |  | CSI-RS#3 |  |
| CSI-RS index assigned as RLM RS |  | 1-2 |  | 0,1,2,3 | A.1.4.2 |
| Beam failure detection transmission parameters | DCI format | 1-2 |  | 1-0 |  |
|  | Number of Control OFDM symbols | 1-2 |  | 2 |  |
|  | Aggregation level | 1-2 | CCE | 8 |  |
|  | Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 1-2 | dB | 0 |  |
|  | Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 1-2 | dB | 0 |  |
|  | DMRS precoder granularity | 1-2 |  | REG bundle size |  |
|  | REG bundle size | 1-2 |  | 6 |  |
| Gap pattern ID |  | 1-2 |  | N/A |  |
| schedulingRequestID-BFR- r17 |  | 1-2 |  | Configured |  |
| Periodicity of PUCCH for SR configuration for BFR on PSCell |  | 1-2 | slot | 40 | 5ms |
| rlmInSyncOutOfSyncThreshold |  | 1-2 |  | absent | Value 0 is applied. (Table 8.1.1-1 in TS 38.133 [6]). |
| rsrp-ThresholdSSB |  | 1-2 | dBm/SCS | -95 | Threshold used for Qin_LR_SSB |
| powerControlOffsetSS |  | 1-2 |  | db0 | Used for deriving rsrp-ThresholdCSI-RS |
| beamFailureInstanceMaxCount |  | 1-2 |  | n1 | see TS 38.321 [12], clause 5.17 |
| beamFailureDetectionTimer |  | 1-2 |  | pbfd4 | see TS 38.321 [12], clause 5.17 |
| CSI-RS configuration for CSI reporting |  | 1-2 |  | CSI-RS.3.1 TDD | A.1.4.2 |
| reportConfigType |  | 1-2 |  | periodic |  |
| reportQuantity |  | 1-2 |  | cri-RI-PMI-CQI |  |
| CSI reporting periodicity |  | 1-2 | slot | 40 |  |
| CSI reporting offset |  | 1-2 | slot | 4 |  |
| T310 |  | 1-2 | ms | 1000 |  |
| N310 |  | 1-2 |  | 2 |  |
| T1 |  | 1-2 | s | 1 | The UE shall be fully synchronized to cell 1 during T1 |
| T2 |  | 1-2 | s | 10.81 |  |
| T3 |  | 1-2 | s | 10.28 |  |
| T4 |  | 1-2 | s | 0 |  |
| T5 |  | 1-2 | s | 0.57 |  |
| D1 |  | 1-2 | s | 0.53 |  |
| Note 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |  |  |

5.5.5.8.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to cell 1 and cell 2. The UE shall be configured for periodic CSI reporting with a reporting periodicity of 5 ms. In the test, DRX configuration is enabled in PCell and DRX inactivity timer has already been expired, i.e. UE tries to decode PDCCH and to send periodic CQI during the period when On-duration timer is running. Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.4.

2.	NR RRCReconfiguration message is contained in RRCConnectionReconfiguration and NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete.

3.	Set the parameters of NR Cell 2 according to T1 in Table 5.5.5.8.5-1. Propagation conditions are set according to Annex C.2.3. T1 starts.

4.	When T1 expires the SS shall change the SNR1 and SNR2 values to T2 as specified in Table 5.5.5.8.5-1. T2 starts.

5.	When T2 expires the SS shall change the SNR1 and SNR2 values to T3 as specified in Table 5.5.5.8.5-1. T3 starts.

6.	When T3 expires the SS shall change the SNR1 and SNR2 values to T4 as specified in Table 5.5.5.8.5-1. T4 starts.

7.	When T4 expires the SS shall change the SNR1 and SNR2 values to T5 as specified in Table 5.5.5.8.5-1. T5 starts.

8.	If the SS:

a)	detects uplink power on NR carrier in each slot configured for CSI transmission (according CSI reporting on PUCCH) during the period from time point A to time point B; and

b)	does not detect PUCCH with LRR before time point B, and

c)	detects PUCCH with LRR, followed by BFR MAC CE containing a beam associated with the candidate beam set q10 and q11 before time point F (D1 after the start of T5),

the number of successful tests is increased by one.

Otherwise the number of failed tests is increased by one.

9.	When T5 expires the SS shall change the SNR value to T1 as specified in Table 5.5.5.8.5-1.

10.	If the iteration fails, the SS shall first attempt to release and add the FR2 PSCell. If that also fails, then the UE is switched OFF/ON to proceed with the next iteration, and ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.4.

11.	Repeat steps 2-10 for all subtests until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

5.5.5.8.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 5.5.5.8.4.3-1: Common Exception messages for EN-DC FR2 PSCell for beam failure detection and link recovery testing in DRX mode

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-8A with Condition CSI-RS BFD Table H.3.1-12A on Cell 2 Table H.3.1-13 on Cell 2 with condition SpCell Table H.3.7-1 with condition DRX.3 |

Table 5.5.5.8.4.3-2: MAC-CellGroupConfig for R17 BFR

| Derivation Path: TS 38.508-1, Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |
| schedulingRequestID-BFR-r17 | Not present |  |  |
| schedulingRequestID-BFR2-r17 | Not present |  |  |
| schedulingRequestConfig-v1700 | SchedulingRequest-Config |  |  |
| } |  |  |  |

Table 5.5.5.8.4.3-3: SchedulingRequestResourceConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| periodicityAndOffset CHOICE { |  |  |  |
| sl40 | 4 |  |  |
| } |  |  |  |
| } |  |  |  |

5.5.5.8.5	Test requirement

Tables 5.5.5.8.4.1-3 and 5.5.5.8.5-1 define the primary level settings including test tolerances for EN-DC FR2 PSCell for beam failure detection and link recovery testing in DRX.

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.9.1 |  |  |  |  |
| Assumption for UE beamsNote 10 |  |  | Rough |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  | dB |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  | dB |  |  |  |  |  |
| SNR_CSI-RS of set q0,0 | Config 1-2 | dB | 5+TT | -3+TT | -12+TT | -12+TT | -12+TT |
| SNR_CSI-RS of set q1,0 | Config 1-2 | dB | 5+TT | 5+TT | 5+TT | 5+TT | 5+TT |
| SNR_CSI-RS of set q0,1 | Config 1-2 | dB | 0.2+TT | 0.2+TT | 20.2+TT | 20.2+TT | 20.2+TT |
| SNR_CSI-RS of set q1,1 | Config 1-2 | dB | 0.2+TT | 0.2+TT | 20.2+TT | 20.2+TT | 20.2+TT |
| CSI-RS_RP of set q1,0 | Config 1-2 | dBm/SCS | -104.5+TT | -104.5+TT | -84.5+TT | -84.5+TT | -84.5+TT |
| CSI-RS_RP of set q1,1 | Config 1-2 | dBm/SCS | -104.5+TT | -104.5+TT | -84.5+TT | -84.5+TT | -84.5+TT |
|  | Config 1-2 | dBm/120 KHz | -104.7+TT |  |  |  |  |
| Propagation condition |  |  | TDL-A 30ns 75Hz |  |  |  |  |
| Note 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. Note 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. Note 4: Void Note 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. Note 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. Note 7: SNR levels correspond to the signal to noise ratio over the SSS REs. Note 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure 5.5.5.8.4-1. Note 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. Note 10: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation Note 11: This value allows up to 1dB degradation from applied SNR to UE baseband |  |  |  |  |  |  |  |

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the UE shall detect beam failure and initiate link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 260+10 ms after the start of T5, the UE shall transmit PUCCH with LRR, followed by BFR MAC CE containing a beam associated with the candidate beam set q1,0. The UE shall not transmit PUCCH with an LRR with the candidate beam set q1,0 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90%.

<<end of change>>