3GPP TSG-RAN5 Meeting #98	R5-231346

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0652 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to URLLC CQI test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Int |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-UEConTest |  |  |  |  |  | Date: |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 10^-5 BLER test cases are currently incomplete due to early pass decision rules requiring more analysis. The reason for the need for early pass fail decision for these 10^-5 BLER cases is because waiting for the statistical significance requires evaluation of atleast 2 order magnitudes greater than the BLER limit. Waiting for 10^7 samples to declare pass/fail will require several hours which puts unnecessary burden on the test time. Annex G.4.4 provides the simulation to derive the pass-fail limits for 0.001% BLER. Since CQI test cases are all for AWGN which is static propagation condition, as seen in other typical PDSCH test cases, there is no minimum test time needed to account for propagation condition. So we can conclude that early pass fail decision rules as per table G.4.3-1 can be adopted. Test procedure requires update to account for the early pass decision rules [Check pending on additional [0.5 dB] applied to the SNR under test to speed up pass/fail decision for these test cases. Check the RAN4 LS for the exact suggested TT value and update the CR.] |  |  |  |  |  |  |  |  |
| Summary of change: |  | -- Added text under Annex G.4.3a to elaborate on how to interpret the early pass fail limits and apply the rules. -- Test procedure needs to be updated as per early pass/fail decision rules. -- Editor’s note can be removed since early pass fail decision rules are now established for the test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1.1.2, 6.2.2.2.1.2, 6.2.3.1.1.2, 6.2.3.2.1.2, Annex G.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

###### 6.2.2.1.1.2	2Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA

6.2.2.1.1.2.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10-5 falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.2.1.1.2.2	Test Applicability

This test applies to all types of NR UE release 16 supporting cqi-TableAlt.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and cqi-TableAlt.

6.2.2.1.1.2.3	Minimum requirement for periodic CQI reporting with Table 3

For the parameters specified in Table 6.2.2.1.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5.

c)	The reported CQI value according to the reference channel shall be ≥ 1.

Table 6.2.2.1.1.2.3-1: CQI reporting test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |
| Duplex Mode |  |  |  | FDD |  |
| Subcarrier spacing |  |  | kHz | 15 |  |
| SNR |  |  | dB | 1 | 2 |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1×2 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-4, TBS.4-1 |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.2.1.1.2.

6.2.2.1.1.2.4	Test Description

6.2.2.1.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 6.1.2-1 and 6.2.2.1.1.2.3-1 as appropriate.

3.	Downlink signals for the NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 , and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions for the NR cell are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.2.2.1.1.2.4.3.

6.2.2.1.1.2.4.2	Test Procedure

1.	Set the parameters of bandwidth, SCS, reference Channel, the propagation condition, antenna configuration and the SNR according to Table 6.2.2.1.1.2.3-1.

2.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to CQI value 2 and keep it regardless of the wideband CQI value sent by the UE. The SS sends downlink MAC padding bits on the DL RMC. Continue transmission of the PDSCH until 5000 wideband CQI reports have been gathered. In this process the SS collects wideband CQI reports every 5 ms and also cases where UE transmits nothing in its CQI timing are also counted as wideband CQI reports.

3.	Set up a relative frequency distribution for the reported wideband CQI-values, Calculate the median value (wideband Median CQI is the wideband CQI that is at or crosses 50% distribution from the lower wideband CQI side). This CQI-value is declared as wideband Median CQI value.

4.	If Median CQI is not equal to 1 or 15 and 4500 or more of the wideband CQI values are in the range (Median CQI - 1) ≤ Median CQI ≤ ( Median CQI + 1) then continue with step 5, otherwise go to step 8.

5.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH transmitted by the SS, record the associated ACK, NACK and statDTX responses. The responses are then filtered as follows: for the sequence of responses for each HARQ process, discard all the statDTX responses. Measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4 and early pass fail decision rules as per Annex G.4.3a.

For the filtered ACK and NACK responses if the ratio (NACK / (ACK + NACK)) ≤ 10-5 then go to step 6, otherwise go to step 7.

6.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI+1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4 and early pass fail decision rules as per Annex G.4.3a..

If the ratio (NACK /(ACK + NACK)) > 10-5

then pass the UE for this test, otherwise go to step 8.

7.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI-1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4 and early pass fail decision rules as per Annex G.4.3a..

If the ratio (NACK /ACK + NACK) ≤10-5

then pass the UE for this test, otherwise go to step 8.

8.	If both SNR points of the test have not been tested, then repeat the same procedure (steps 1 to 7) for the other SNR point as appropriate. Otherwise fail the UE.

<Several sections skipped>

###### 6.2.2.2.1.2	2Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA

6.2.2.2.1.2.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10-5 falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.2.2.1.2.2	Test Applicability

This test applies to all types of NR UE release 16 supporting cqi-TableAlt.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and cqi-TableAlt.

6.2.2.2.1.2.3	Minimum requirement for periodic CQI reporting with Table 3

For the parameters specified in Table 6.2.2.2.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5.

c)	The reported CQI value according to the reference channel shall be ≥ 1.

Table 6.2.2.2.1.2.3-1: CQI reporting test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |
| Subcarrier spacing |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  | TDD |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |
| SNR |  |  | dB | 1 | 2 |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1×2 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 1 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-4, TBS.4-2 |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.2.2.1.2.

6.2.2.2.1.2.4	Test Description

6.2.2.2.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.2 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 6.1.2-1 and 6.2.2.2.1.2.3-1 as appropriate.

3.	Downlink signals for the NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 , and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions for the NR cell are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.2.2.2.1.2.4.3.

6.2.2.2.1.2.4.2	Test Procedure

1.	Set the parameters of bandwidth, SCS, reference Channel, the propagation condition, antenna configuration and the SNR according to Table 6.2.2.2.1.2.3-1.

2.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to CQI value 2 and keep it regardless of the wideband CQI value sent by the UE. The SS sends downlink MAC padding bits on the DL RMC. Continue transmission of the PDSCH until 5000 wideband CQI reports have been gathered. In this process the SS collects wideband CQI reports every 10 ms and also cases where UE transmits nothing in its CQI timing are also counted as wideband CQI reports.

3.	Set up a relative frequency distribution for the reported wideband CQI-values, Calculate the median value (wideband Median CQI is the wideband CQI that is at or crosses 50% distribution from the lower wideband CQI side). This CQI-value is declared as wideband Median CQI value.

4.	If Median CQI is not equal to 1 or 15 and 4500 or more of the wideband CQI values are in the range (Median CQI - 1) ≤ Median CQI ≤ ( Median CQI + 1) then continue with step 5, otherwise go to step 8.

5.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH transmitted by the SS, record the associated ACK, NACK and statDTX responses. The responses are then filtered as follows: for the sequence of responses for each HARQ process, discard all the statDTX responses. Measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4 and early pass fail decision rules as per Annex G.4.3a.

For the filtered ACK and NACK responses if the ratio (NACK / (ACK + NACK)) ≤ 10-5 then go to step 6, otherwise go to step 7.

6.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI+1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4 and early pass fail decision rules as per Annex G.4.3a .

If the ratio (NACK / (ACK + NACK)) > 10-5

then pass the UE for this test, otherwise go to step 8.

7.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI-1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4 and early pass fail decision rules as per Annex G.4.3a.

If the ratio (NACK / ACK + NACK) ≤10-5

then pass the UE for this test, otherwise go to step 8.

8.	If both SNR points of the test have not been tested, then repeat the same procedure (steps 1 to 7) for the other SNR point as appropriate. Otherwise fail the UE.

<Several sections skipped>

###### 6.2.3.1.1.2	4Rx FDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA

6.2.3.1.1.2.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10-5 falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.3.1.1.2.2	Test Applicability

This test applies to all types of NR UE release 16 supporting cqi-TableAlt.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and cqi-TableAlt.

6.2.3.1.1.2.3	Minimum requirement for periodic CQI reporting with Table 3

For the parameters specified in Table 6.2.3.1.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5.

c)	The reported CQI value according to the reference channel shall be ≥ 1.

Table 6.2.3.1.1.2.3-1: CQI reporting test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |  |
| Subcarrier spacing |  |  | kHz | 15 |  |
| Duplex Mode |  |  |  | FDD |  |
| SNR |  |  | dB | -2 | -1 |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1×4 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 1 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 8 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 5/0 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 8 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-4, TBS.4-1 |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.3.1.1.2.

6.2.3.1.1.2.4	Test Description

6.2.3.1.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 6.1.2-1 and 6.2.3.1.1.2.3-1 as appropriate.

3.	Downlink signals for the NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 , and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions for the NR cell are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.2.3.1.1.2.4.3.

6.2.3.1.1.2.4.2	Test Procedure

1.	Set the parameters of bandwidth, SCS, reference Channel, the propagation condition, antenna configuration and the SNR according to Table 6.2.3.1.1.2.3-1.

2.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to CQI value 2 and keep it regardless of the wideband CQI value sent by the UE. The SS sends downlink MAC padding bits on the DL RMC. Continue transmission of the PDSCH until 5000 wideband CQI reports have been gathered. In this process the SS collects wideband CQI reports every 5 ms and also cases where UE transmits nothing in its CQI timing are also counted as wideband CQI reports.

3.	Set up a relative frequency distribution for the reported wideband CQI-values, Calculate the median value (wideband Median CQI is the wideband CQI that is at or crosses 50% distribution from the lower wideband CQI side). This CQI-value is declared as wideband Median CQI value.

4.	If Median CQI is not equal to 1 or 15 and 4500 or more of the wideband CQI values are in the range (Median CQI - 1) ≤ Median CQI ≤ ( Median CQI + 1) then continue with step 5, otherwise go to step 8.

5.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH transmitted by the SS, record the associated ACK, NACK and statDTX responses. The responses are then filtered as follows: for the sequence of responses for each HARQ process, discard all the statDTX responses. Measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4 and early pass fail decision rules as per Annex G.4.3a.

For the filtered ACK and NACK responses if the ratio (NACK / (ACK + NACK)) ≤ 10-5 then go to step 6, otherwise go to step 7.

6.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI+1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4 and early pass fail decision rules as per Annex G.4.3a.

If the ratio (NACK /(ACK + NACK)) > 10-5

then pass the UE for this test, otherwise go to step 8.

7.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI-1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4 and early pass fail decision rules as per Annex G.4.3a.

If the ratio (NACK /ACK + NACK) ≤10-5

then pass the UE for this test, otherwise go to step 8.

8.	If both SNR points of the test have not been tested, then repeat the same procedure (steps 1 to 7) for the other SNR point as appropriate. Otherwise fail the UE.

<Several sections skipped>

###### 6.2.3.2.1.2	4Rx TDD FR1 periodic CQI reporting with Table 3 under AWGN conditions for both SA and NSA

6.2.3.2.1.2.1	Test Purpose

The purpose of this test is to verify the variance of the wideband CQI reports is within the limits defined and a PDSCH BLER of 10-5 falls between the transport format based median CQI-1 and median CQI or the transport format based median CQI and median CQI +1.

6.2.3.2.1.2.2	Test Applicability

This test applies to all types of NR UE release 16 supporting cqi-TableAlt.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and cqi-TableAlt.

6.2.3.2.1.2.3	Minimum requirement for periodic CQI reporting with Table 3

For the parameters specified in Table 6.2.3.2.1.2.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified by the following:

a)	The reported CQI value according to the reference channel shall be in the range of ±1 of the reported median more than 90% of the time.

b)	If the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 10-5, then the BLER using the transport format indicated by the (median CQI+1) shall be greater than 10-5. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 10-5, then the BLER using transport format indicated by (median CQI-1) shall be less than or equal to 10-5.

c)	The reported CQI value according to the reference channel shall be ≥ 1.

Table 6.2.3.2.1.2.3-1: CQI reporting test parameters

| Parameter |  |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 40 |  |
| Subcarrier spacing |  |  | kHz | 30 |  |
| Duplex Mode |  |  |  | TDD |  |
| TDD UL-DL pattern |  |  |  | FR1.30-1 |  |
| SNR |  |  | dB | -2 | -1 |
| Propagation channel |  |  |  | AWGN |  |
| Antenna configuration |  |  |  | 1×4 with static channel specified in Annex B.1 |  |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 4 |  |
|  | CDM Type |  |  | FD-CDM2 |  |
|  | Density (ρ) |  |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,4 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |  |
|  | CSI-RS periodicity and offset |  | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  |  | 1 |  |
|  | CDM Type |  |  | No CDM |  |
|  | Density (ρ) |  |  | 3 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | Row 1,(0,-) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 1 |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 10/1 |  |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |  |
|  | CSI-IM RE pattern |  |  | 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |  |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 10/1 |  |
| ReportConfigType |  |  |  | Periodic |  |
| CQI-table |  |  |  | Table 3 |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |
| Sub-band Size |  |  | RB | 16 |  |
| Csi-ReportingBand |  |  |  | 1111111 |  |
| CSI-Report periodicity and offset |  |  | slot | 10/9 |  |
| aperiodicTriggeringOffset |  |  |  | Not configured |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |
|  |  | Codebook Mode |  | 1 |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |
|  |  | CodebookSubsetRestriction |  | 000001 |  |
|  |  | RI Restriction |  | N/A |  |
| Physical channel for CSI report |  |  |  | PUCCH |  |
| CQI/RI/PMI delay |  |  | ms | 9.5 |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |
| Measurement channel |  |  |  | As specified in Table A.4-4, TBS.4-2 |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.2.3.2.1.2.

6.2.3.2.1.2.4	Test Description

6.2.3.2.1.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the NR cell are set up according to Table 6.1.2-1 and 6.2.3.2.1.2.3-1 as appropriate.

3.	Downlink signals for the NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 , and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions for the NR cell are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.2.3.2.1.2.4.3.

6.2.3.2.1.2.4.2	Test Procedure

1.	Set the parameters of bandwidth, SCS, reference Channel, the propagation condition, antenna configuration and the SNR according to Table 6.2.3.2.1.2.3-1.

2.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to CQI value 2 and keep it regardless of the wideband CQI value sent by the UE. The SS sends downlink MAC padding bits on the DL RMC. Continue transmission of the PDSCH until 5000 wideband CQI reports have been gathered. In this process the SS collects wideband CQI reports every 10 ms and also cases where UE transmits nothing in its CQI timing are also counted as wideband CQI reports.

3.	Set up a relative frequency distribution for the reported wideband CQI-values, Calculate the median value (wideband Median CQI is the wideband CQI that is at or crosses 50% distribution from the lower wideband CQI side). This CQI-value is declared as wideband Median CQI value.

4.	If Median CQI is not equal to 1 or 15 and 4500 or more of the wideband CQI values are in the range (Median CQI - 1) ≤ Median CQI ≤ ( Median CQI + 1) then continue with step 5, otherwise go to step 8.

5.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH transmitted by the SS, record the associated ACK, NACK and statDTX responses. The responses are then filtered as follows: for the sequence of responses for each HARQ process, discard all the statDTX responses. Measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4 and early pass fail decision rules as per Annex G.4.3a.

For the filtered ACK and NACK responses if the ratio (NACK / (ACK + NACK)) ≤ 10-5 then go to step 6, otherwise go to step 7.

6.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI+1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4 and early pass fail decision rules as per Annex G.4.3a.

If the ratio (NACK / (ACK + NACK)) > 10-5

then pass the UE for this test, otherwise go to step 8.

7.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to the wideband median-CQI-1 value and shall not react to the UE’s wideband CQI reports. The SS sends downlink MAC padding bits on the DL RMC. For any PDSCH, transmitted by the SS, record and filter the ACK, NACK and statDTX responses as in step 5, and measure the BLER for a duration sufficient to achieve statistical significance according to Annex G.4 and early pass fail decision rules as per Annex G.4.3a.

If the ratio (NACK / ACK + NACK) ≤10-5

then pass the UE for this test, otherwise go to step 8.

8.	If both SNR points of the test have not been tested, then repeat the same procedure (steps 1 to 7) for the other SNR point as appropriate. Otherwise fail the UE.

<Several sections skipped>

# G.4	Statistical testing of Performance Requirements with BLER limit

## G.4.1	General

The test of receiver performance characteristics is twofold.

1.	A signal or a combination of signals is offered to the RX port(s) of the receiver.

2.	The ability of the receiver to demodulate /decode this signal is verified by measuring the throughput.

In (2) is the statistical aspect of the test and is treated here.

The minimum requirement for several receiver performance test cases is specified in regards of BLER: 0.001%.

## G.4.2	Design of the test

The test is defined by the following design principles (see clause G.2, Theory):

1.	The early decision concept is applied.

2.	A second limit is introduced: Bad DUT factor M>1.

3.	To decide the test pass:

Supplier risk is applied based on the Bad DUT quality

To decide the test fail:

Customer Risk is applied based on the specified DUT quality

## G.4.3	Numerical definition of the pass fail limits for 0.001% BLER

The numerical pass/fail limit is derived by the following parameters:

1a) Limit Error Ratio = 0.001%

2a) Bad DUT factor M=1.5 (selectivity)

justification see: TS 34.121 Clause F.6.3.3

3) Confidence level CL = 99.999%

Table G.4.3-1: Pass fail limits

| ne | nsp | nsf | ne | nsp | nsf | ne | nsp | nsf |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1074532 | 1067 | 215 | 20006574 | 14871394 | 430 | 36441701 | 33298651 |
| 1 | 1074532 | 1067 | 216 | 20085020 | 14954177 | 431 | 36516711 | 33386452 |
| 2 | 1274645 | 1067 | 217 | 20163439 | 15036999 | 432 | 36591711 | 33474268 |
| 3 | 1444583 | 1067 | 218 | 20241831 | 15119861 | 433 | 36666702 | 33562097 |
| 4 | 1599072 | 4727 | 219 | 20320196 | 15202761 | 434 | 36741683 | 33649940 |
| 5 | 1743641 | 12160 | 220 | 20398535 | 15285701 | 435 | 36816654 | 33737797 |
| 6 | 1881111 | 23683 | 221 | 20476847 | 15368679 | 436 | 36891616 | 33825668 |
| 7 | 2013164 | 39190 | 222 | 20555133 | 15451695 | 437 | 36966568 | 33913553 |
| 8 | 2140902 | 58403 | 223 | 20633393 | 15534749 | 438 | 37041511 | 34001452 |
| 9 | 2265092 | 81000 | 224 | 20711628 | 15617841 | 439 | 37116445 | 34089364 |
| 10 | 2386297 | 106667 | 225 | 20789836 | 15700971 | 440 | 37191369 | 34177291 |
| 11 | 2504945 | 135116 | 226 | 20868019 | 15784137 | 441 | 37266283 | 34265231 |
| 12 | 2621369 | 166089 | 227 | 20946177 | 15867341 | 442 | 37341189 | 34353184 |
| 13 | 2735834 | 199360 | 228 | 21024309 | 15950581 | 443 | 37416085 | 34441151 |
| 14 | 2848557 | 234730 | 229 | 21102417 | 16033858 | 444 | 37490972 | 34529132 |
| 15 | 2959718 | 272025 | 230 | 21180499 | 16117172 | 445 | 37565849 | 34617126 |
| 16 | 3069467 | 311091 | 231 | 21258557 | 16200521 | 446 | 37640718 | 34705134 |
| 17 | 3177931 | 351792 | 232 | 21336590 | 16283906 | 447 | 37715577 | 34793155 |
| 18 | 3285220 | 394009 | 233 | 21414599 | 16367326 | 448 | 37790427 | 34881189 |
| 19 | 3391428 | 437636 | 234 | 21492584 | 16450782 | 449 | 37865268 | 34969237 |
| 20 | 3496637 | 482577 | 235 | 21570545 | 16534273 | 450 | 37940100 | 35057298 |
| 21 | 3600921 | 528746 | 236 | 21648482 | 16617799 | 451 | 38014923 | 35145372 |
| 22 | 3704343 | 576068 | 237 | 21726395 | 16701360 | 452 | 38089737 | 35233459 |
| 23 | 3806960 | 624473 | 238 | 21804284 | 16784955 | 453 | 38164542 | 35321560 |
| 24 | 3908823 | 673898 | 239 | 21882150 | 16868585 | 454 | 38239338 | 35409673 |
| 25 | 4009977 | 724286 | 240 | 21959993 | 16952248 | 455 | 38314125 | 35497800 |
| 26 | 4110465 | 775585 | 241 | 22037812 | 17035945 | 456 | 38388903 | 35585939 |
| 27 | 4210324 | 827748 | 242 | 22115608 | 17119676 | 457 | 38463672 | 35674092 |
| 28 | 4309587 | 880730 | 243 | 22193382 | 17203440 | 458 | 38538432 | 35762258 |
| 29 | 4408285 | 934492 | 244 | 22271133 | 17287238 | 459 | 38613184 | 35850436 |
| 30 | 4506448 | 988997 | 245 | 22348861 | 17371068 | 460 | 38687927 | 35938627 |
| 31 | 4604101 | 1044211 | 246 | 22426567 | 17454931 | 461 | 38762661 | 36026831 |
| 32 | 4701268 | 1100101 | 247 | 22504250 | 17538827 | 462 | 38837386 | 36115048 |
| 33 | 4797972 | 1156638 | 248 | 22581911 | 17622755 | 463 | 38912102 | 36203278 |
| 34 | 4894232 | 1213795 | 249 | 22659550 | 17706716 | 464 | 38986810 | 36291520 |
| 35 | 4990069 | 1271547 | 250 | 22737168 | 17790708 | 465 | 39061510 | 36379774 |
| 36 | 5085500 | 1329869 | 251 | 22814763 | 17874733 | 466 | 39136200 | 36468042 |
| 37 | 5180542 | 1388740 | 252 | 22892337 | 17958789 | 467 | 39210882 | 36556322 |
| 38 | 5275209 | 1448137 | 253 | 22969889 | 18042876 | 468 | 39285556 | 36644614 |
| 39 | 5369517 | 1508043 | 254 | 23047420 | 18126994 | 469 | 39360221 | 36732919 |
| 40 | 5463478 | 1568438 | 255 | 23124929 | 18211144 | 470 | 39434877 | 36821237 |
| 41 | 5557107 | 1629304 | 256 | 23202418 | 18295325 | 471 | 39509525 | 36909566 |
| 42 | 5650414 | 1690627 | 257 | 23279885 | 18379536 | 472 | 39584165 | 36997908 |
| 43 | 5743410 | 1752389 | 258 | 23357331 | 18463778 | 473 | 39658796 | 37086263 |
| 44 | 5836108 | 1814577 | 259 | 23434757 | 18548050 | 474 | 39733419 | 37174629 |
| 45 | 5928516 | 1877177 | 260 | 23512162 | 18632353 | 475 | 39808033 | 37263008 |
| 46 | 6020643 | 1940175 | 261 | 23589546 | 18716685 | 476 | 39882639 | 37351399 |
| 47 | 6112500 | 2003560 | 262 | 23666910 | 18801047 | 477 | 39957237 | 37439803 |
| 48 | 6204094 | 2067319 | 263 | 23744254 | 18885439 | 478 | 40031826 | 37528218 |
| 49 | 6295434 | 2131442 | 264 | 23821577 | 18969861 | 479 | 40106407 | 37616645 |
| 50 | 6386526 | 2195916 | 265 | 23898880 | 19054311 | 480 | 40180980 | 37705085 |
| 51 | 6477380 | 2260734 | 266 | 23976164 | 19138791 | 481 | 40255545 | 37793536 |
| 52 | 6568000 | 2325884 | 267 | 24053427 | 19223300 | 482 | 40330102 | 37882000 |
| 53 | 6658395 | 2391358 | 268 | 24130671 | 19307838 | 483 | 40404650 | 37970475 |
| 54 | 6748569 | 2457146 | 269 | 24207895 | 19392404 | 484 | 40479190 | 38058963 |
| 55 | 6838530 | 2523241 | 270 | 24285099 | 19476999 | 485 | 40553722 | 38147462 |
| 56 | 6928283 | 2589634 | 271 | 24362284 | 19561623 | 486 | 40628246 | 38235973 |
| 57 | 7017834 | 2656318 | 272 | 24439450 | 19646274 | 487 | 40702762 | 38324496 |
| 58 | 7107187 | 2723285 | 273 | 24516597 | 19730954 | 488 | 40777270 | 38413030 |
| 59 | 7196348 | 2790528 | 274 | 24593724 | 19815662 | 489 | 40851770 | 38501576 |
| 60 | 7285321 | 2858041 | 275 | 24670832 | 19900397 | 490 | 40926262 | 38590134 |
| 61 | 7374112 | 2925816 | 276 | 24747922 | 19985160 | 491 | 41000746 | 38678704 |
| 62 | 7462724 | 2993848 | 277 | 24824993 | 20069950 | 492 | 41075222 | 38767285 |
| 63 | 7551162 | 3062130 | 278 | 24902045 | 20154768 | 493 | 41149690 | 38855878 |
| 64 | 7639430 | 3130657 | 279 | 24979078 | 20239613 | 494 | 41224150 | 38944482 |
| 65 | 7727532 | 3199424 | 280 | 25056093 | 20324485 | 495 | 41298602 | 39033098 |
| 66 | 7815471 | 3268424 | 281 | 25133089 | 20409383 | 496 | 41373047 | 39121725 |
| 67 | 7903252 | 3337653 | 282 | 25210068 | 20494309 | 497 | 41447483 | 39210364 |
| 68 | 7990878 | 3407105 | 283 | 25287028 | 20579261 | 498 | 41521912 | 39299014 |
| 69 | 8078352 | 3476777 | 284 | 25363970 | 20664239 | 499 | 41596333 | 39387675 |
| 70 | 8165677 | 3546663 | 285 | 25440893 | 20749244 | 500 | 41670746 | 39476348 |
| 71 | 8252857 | 3616759 | 286 | 25517799 | 20834275 | 501 | 41745152 | 39565032 |
| 72 | 8339894 | 3687060 | 287 | 25594687 | 20919332 | 502 | 41819550 | 39653727 |
| 73 | 8426792 | 3757563 | 288 | 25671558 | 21004415 | 503 | 41893940 | 39742434 |
| 74 | 8513553 | 3828263 | 289 | 25748411 | 21089524 | 504 | 41968323 | 39831151 |
| 75 | 8600181 | 3899156 | 290 | 25825246 | 21174658 | 505 | 42042698 | 39919880 |
| 76 | 8686677 | 3970239 | 291 | 25902063 | 21259818 | 506 | 42117065 | 40008620 |
| 77 | 8773044 | 4041508 | 292 | 25978864 | 21345003 | 507 | 42191424 | 40097371 |
| 78 | 8859286 | 4112960 | 293 | 26055647 | 21430213 | 508 | 42265777 | 40186133 |
| 79 | 8945403 | 4184590 | 294 | 26132413 | 21515449 | 509 | 42340121 | 40274907 |
| 80 | 9031399 | 4256396 | 295 | 26209162 | 21600709 | 510 | 42414458 | 40363691 |
| 81 | 9117276 | 4328375 | 296 | 26285893 | 21685995 | 511 | 42488788 | 40452486 |
| 82 | 9203035 | 4400523 | 297 | 26362608 | 21771305 | 512 | 42563110 | 40541292 |
| 83 | 9288680 | 4472838 | 298 | 26439306 | 21856639 | 513 | 42637425 | 40630109 |
| 84 | 9374212 | 4545316 | 299 | 26515987 | 21941999 | 514 | 42711732 | 40718937 |
| 85 | 9459633 | 4617954 | 300 | 26592652 | 22027382 | 515 | 42786032 | 40807776 |
| 86 | 9544944 | 4690751 | 301 | 26669300 | 22112790 | 516 | 42860324 | 40896625 |
| 87 | 9630149 | 4763702 | 302 | 26745931 | 22198222 | 517 | 42934609 | 40985485 |
| 88 | 9715249 | 4836806 | 303 | 26822546 | 22283678 | 518 | 43008887 | 41074356 |
| 89 | 9800245 | 4910060 | 304 | 26899145 | 22369157 | 519 | 43083157 | 41163238 |
| 90 | 9885139 | 4983461 | 305 | 26975727 | 22454661 | 520 | 43157420 | 41252131 |
| 91 | 9969933 | 5057007 | 306 | 27052293 | 22540188 | 521 | 43231676 | 41341034 |
| 92 | 10054629 | 5130696 | 307 | 27128843 | 22625739 | 522 | 43305924 | 41429947 |
| 93 | 10139228 | 5204526 | 308 | 27205377 | 22711313 | 523 | 43380165 | 41518872 |
| 94 | 10223731 | 5278493 | 309 | 27281895 | 22796910 | 524 | 43454399 | 41607806 |
| 95 | 10308141 | 5352597 | 310 | 27358398 | 22882531 | 525 | 43528626 | 41696752 |
| 96 | 10392459 | 5426835 | 311 | 27434884 | 22968175 | 526 | 43602846 | 41785708 |
| 97 | 10476685 | 5501204 | 312 | 27511355 | 23053842 | 527 | 43677058 | 41874674 |
| 98 | 10560822 | 5575703 | 313 | 27587810 | 23139531 | 528 | 43751263 | 41963651 |
| 99 | 10644871 | 5650331 | 314 | 27664249 | 23225243 | 529 | 43825462 | 42052638 |
| 100 | 10728833 | 5725084 | 315 | 27740673 | 23310978 | 530 | 43899653 | 42141635 |
| 101 | 10812709 | 5799961 | 316 | 27817081 | 23396736 | 531 | 43973837 | 42230643 |
| 102 | 10896501 | 5874961 | 317 | 27893475 | 23482516 | 532 | 44048014 | 42319662 |
| 103 | 10980210 | 5950082 | 318 | 27969852 | 23568318 | 533 | 44122183 | 42408690 |
| 104 | 11063837 | 6025321 | 319 | 28046215 | 23654143 | 534 | 44196346 | 42497729 |
| 105 | 11147384 | 6100677 | 320 | 28122563 | 23739989 | 535 | 44270502 | 42586778 |
| 106 | 11230851 | 6176149 | 321 | 28198895 | 23825858 | 536 | 44344651 | 42675837 |
| 107 | 11314239 | 6251735 | 322 | 28275212 | 23911748 | 537 | 44418793 | 42764907 |
| 108 | 11397550 | 6327434 | 323 | 28351515 | 23997661 | 538 | 44492928 | 42853986 |
| 109 | 11480785 | 6403243 | 324 | 28427803 | 24083595 | 539 | 44567056 | 42943076 |
| 110 | 11563945 | 6479161 | 325 | 28504075 | 24169550 | 540 | 44641177 | 43032176 |
| 111 | 11647030 | 6555187 | 326 | 28580333 | 24255527 | 541 | 44715291 | 43121286 |
| 112 | 11730042 | 6631320 | 327 | 28656577 | 24341526 | 542 | 44789399 | 43210406 |
| 113 | 11812982 | 6707558 | 328 | 28732806 | 24427546 | 543 | 44863499 | 43299535 |
| 114 | 11895850 | 6783899 | 329 | 28809020 | 24513587 | 544 | 44937593 | 43388675 |
| 115 | 11978648 | 6860343 | 330 | 28885220 | 24599649 | 545 | 45011680 | 43477825 |
| 116 | 12061377 | 6936887 | 331 | 28961405 | 24685732 | 546 | 45085760 | 43566985 |
| 117 | 12144037 | 7013532 | 332 | 29037577 | 24771836 | 547 | 45159833 | 43656155 |
| 118 | 12226629 | 7090274 | 333 | 29113734 | 24857961 | 548 | 45233900 | 43745334 |
| 119 | 12309155 | 7167114 | 334 | 29189876 | 24944107 | 549 | 45307960 | 43834523 |
| 120 | 12391614 | 7244050 | 335 | 29266005 | 25030273 | 550 | 45382013 | 43923723 |
| 121 | 12474008 | 7321081 | 336 | 29342119 | 25116460 | 551 | 45456059 | 44012932 |
| 122 | 12556338 | 7398206 | 337 | 29418220 | 25202668 | 552 | 45530099 | 44102150 |
| 123 | 12638604 | 7475422 | 338 | 29494306 | 25288896 | 553 | 45604132 | 44191379 |
| 124 | 12720808 | 7552731 | 339 | 29570379 | 25375144 | 554 | 45678159 | 44280617 |
| 125 | 12802949 | 7630129 | 340 | 29646438 | 25461412 | 555 | 45752178 | 44369865 |
| 126 | 12885029 | 7707617 | 341 | 29722483 | 25547700 | 556 | 45826192 | 44459123 |
| 127 | 12967048 | 7785194 | 342 | 29798514 | 25634009 | 557 | 45900198 | 44548390 |
| 128 | 13049007 | 7862857 | 343 | 29874532 | 25720337 | 558 | 45974198 | 44637667 |
| 129 | 13130907 | 7940606 | 344 | 29950536 | 25806685 | 559 | 46048192 | 44726953 |
| 130 | 13212749 | 8018441 | 345 | 30026527 | 25893053 | 560 | 46122179 | 44816249 |
| 131 | 13294533 | 8096360 | 346 | 30102504 | 25979441 | 561 | 46196159 | 44905555 |
| 132 | 13376259 | 8174362 | 347 | 30178468 | 26065848 | 562 | 46270133 | 44994870 |
| 133 | 13457929 | 8252446 | 348 | 30254418 | 26152274 | 563 | 46344100 | 45084194 |
| 134 | 13539543 | 8330612 | 349 | 30330355 | 26238721 | 564 | 46418061 | 45173528 |
| 135 | 13621102 | 8408859 | 350 | 30406279 | 26325186 | 565 | 46492016 | 45262871 |
| 136 | 13702605 | 8487185 | 351 | 30482190 | 26411671 | 566 | 46565964 | 45352224 |
| 137 | 13784055 | 8565589 | 352 | 30558087 | 26498174 | 567 | 46639906 | 45441586 |
| 138 | 13865452 | 8644072 | 353 | 30633972 | 26584697 | 568 | 46713841 | 45530958 |
| 139 | 13946795 | 8722632 | 354 | 30709843 | 26671239 | 569 | 46787770 | 45620339 |
| 140 | 14028086 | 8801268 | 355 | 30785702 | 26757800 | 570 | 46861692 | 45709729 |
| 141 | 14109325 | 8879979 | 356 | 30861547 | 26844380 | 571 | 46935608 | 45799128 |
| 142 | 14190513 | 8958765 | 357 | 30937380 | 26930979 | 572 | 47009518 | 45888537 |
| 143 | 14271650 | 9037625 | 358 | 31013200 | 27017596 | 573 | 47083422 | 45977955 |
| 144 | 14352737 | 9116558 | 359 | 31089007 | 27104232 | 574 | 47157319 | 46067382 |
| 145 | 14433775 | 9195563 | 360 | 31164802 | 27190886 | 575 | 47231210 | 46156818 |
| 146 | 14514763 | 9274640 | 361 | 31240584 | 27277559 | 576 | 47305094 | 46246264 |
| 147 | 14595702 | 9353788 | 362 | 31316353 | 27364250 | 577 | 47378973 | 46335719 |
| 148 | 14676593 | 9433006 | 363 | 31392110 | 27450959 | 578 | 47452845 | 46425182 |
| 149 | 14757437 | 9512294 | 364 | 31467854 | 27537687 | 579 | 47526711 | 46514655 |
| 150 | 14838233 | 9591650 | 365 | 31543586 | 27624433 | 580 | 47600570 | 46604137 |
| 151 | 14918983 | 9671074 | 366 | 31619306 | 27711197 | 581 | 47674424 | 46693628 |
| 152 | 14999686 | 9750566 | 367 | 31695013 | 27797979 | 582 | 47748271 | 46783128 |
| 153 | 15080344 | 9830124 | 368 | 31770708 | 27884779 | 583 | 47822113 | 46872637 |
| 154 | 15160956 | 9909749 | 369 | 31846390 | 27971597 | 584 | 47895948 | 46962155 |
| 155 | 15241523 | 9989439 | 370 | 31922061 | 28058432 | 585 | 47969777 | 47051682 |
| 156 | 15322045 | 10069194 | 371 | 31997719 | 28145286 | 586 | 48043599 | 47141218 |
| 157 | 15402524 | 10149014 | 372 | 32073365 | 28232157 | 587 | 48117416 | 47230762 |
| 158 | 15482959 | 10228896 | 373 | 32149000 | 28319045 | 588 | 48191227 | 47320316 |
| 159 | 15563350 | 10308842 | 374 | 32224622 | 28405951 | 589 | 48265031 | 47409879 |
| 160 | 15643699 | 10388851 | 375 | 32300232 | 28492875 | 590 | 48338830 | 47499450 |
| 161 | 15724005 | 10468921 | 376 | 32375831 | 28579815 | 591 | 48412622 | 47589030 |
| 162 | 15804270 | 10549052 | 377 | 32451417 | 28666774 | 592 | 48486409 | 47678619 |
| 163 | 15884492 | 10629245 | 378 | 32526992 | 28753749 | 593 | 48560190 | 47768217 |
| 164 | 15964673 | 10709497 | 379 | 32602555 | 28840741 | 594 | 48633964 | 47857823 |
| 165 | 16044814 | 10789809 | 380 | 32678107 | 28927751 | 595 | 48707733 | 47947438 |
| 166 | 16124913 | 10870180 | 381 | 32753646 | 29014778 | 596 | 48781495 | 48037062 |
| 167 | 16204973 | 10950610 | 382 | 32829175 | 29101821 | 597 | 48855252 | 48126695 |
| 168 | 16284993 | 11031098 | 383 | 32904691 | 29188882 | 598 | 48929003 | 48216336 |
| 169 | 16364973 | 11111643 | 384 | 32980196 | 29275959 | 599 | 49002747 | 48305986 |
| 170 | 16444914 | 11192245 | 385 | 33055690 | 29363053 | 600 | 49076486 | 48395644 |
| 171 | 16524817 | 11272904 | 386 | 33131172 | 29450164 | 601 | 49150219 | 48485312 |
| 172 | 16604680 | 11353619 | 387 | 33206643 | 29537291 | 602 | 49223946 | 48574987 |
| 173 | 16684506 | 11434390 | 388 | 33282102 | 29624435 | 603 | 49297668 | 48664671 |
| 174 | 16764294 | 11515215 | 389 | 33357550 | 29711596 | 604 | 49371383 | 48754364 |
| 175 | 16844045 | 11596095 | 390 | 33432987 | 29798773 | 605 | 49445093 | 48844065 |
| 176 | 16923758 | 11677030 | 391 | 33508413 | 29885966 | 606 | 49518797 | 48933775 |
| 177 | 17003435 | 11758018 | 392 | 33583827 | 29973176 | 607 | 49592495 | 49023493 |
| 178 | 17083075 | 11839059 | 393 | 33659230 | 30060402 | 608 | 49666187 | 49113220 |
| 179 | 17162679 | 11920153 | 394 | 33734623 | 30147644 | 609 | 49739874 | 49202955 |
| 180 | 17242247 | 12001299 | 395 | 33810004 | 30234902 | 610 | 49813554 | 49292699 |
| 181 | 17321779 | 12082497 | 396 | 33885374 | 30322176 | 611 | 49887229 | 49382451 |
| 182 | 17401276 | 12163747 | 397 | 33960734 | 30409467 | 612 | 49960899 | 49472211 |
| 183 | 17480738 | 12245048 | 398 | 34036082 | 30496773 | 613 | 50034562 | 49561980 |
| 184 | 17560165 | 12326400 | 399 | 34111419 | 30584095 | 614 | 50108220 | 49651757 |
| 185 | 17639558 | 12407801 | 400 | 34186746 | 30671433 | 615 | 50181872 | 49741542 |
| 186 | 17718917 | 12489253 | 401 | 34262062 | 30758787 | 616 | 50255519 | 49831335 |
| 187 | 17798241 | 12570754 | 402 | 34337367 | 30846156 | 617 | 50329160 | 49921137 |
| 188 | 17877532 | 12652304 | 403 | 34412662 | 30933541 | 618 | 50402795 | 50010947 |
| 189 | 17956790 | 12733903 | 404 | 34487945 | 31020942 | 619 | 50476425 | 50100765 |
| 190 | 18036015 | 12815550 | 405 | 34563218 | 31108358 | 620 | 50550049 | 50190592 |
| 191 | 18115206 | 12897245 | 406 | 34638481 | 31195790 | 621 | 50623667 | 50280427 |
| 192 | 18194366 | 12978988 | 407 | 34713733 | 31283237 | 622 | 50697280 | 50370270 |
| 193 | 18273492 | 13060777 | 408 | 34788974 | 31370699 | 623 | 50770887 | 50460120 |
| 194 | 18352587 | 13142614 | 409 | 34864205 | 31458177 | 624 | 50844489 | 50549980 |
| 195 | 18431650 | 13224497 | 410 | 34939426 | 31545670 | 625 | 50918085 | 50639847 |
| 196 | 18510681 | 13306426 | 411 | 35014636 | 31633178 | 626 | 50991676 | 50729722 |
| 197 | 18589681 | 13388401 | 412 | 35089836 | 31720702 | 627 | 51065261 | 50819605 |
| 198 | 18668650 | 13470421 | 413 | 35165025 | 31808240 | 628 | 51138840 | 50909497 |
| 199 | 18747588 | 13552486 | 414 | 35240204 | 31895794 | 629 | 51212414 | 50999396 |
| 200 | 18826495 | 13634596 | 415 | 35315373 | 31983362 | 630 | 51285983 | 51089304 |
| 201 | 18905372 | 13716750 | 416 | 35390532 | 32070946 | 631 | 51359546 | 51179219 |
| 202 | 18984219 | 13798949 | 417 | 35465680 | 32158544 | 632 | 51433104 | 51269143 |
| 203 | 19063035 | 13881191 | 418 | 35540819 | 32246157 | 633 | 51506656 | 51359074 |
| 204 | 19141822 | 13963476 | 419 | 35615947 | 32333785 | 634 | 51580203 | 51449013 |
| 205 | 19220579 | 14045805 | 420 | 35691065 | 32421428 | 635 | 51653744 | 51538961 |
| 206 | 19299307 | 14128176 | 421 | 35766173 | 32509085 | 636 | 51727280 | 51628916 |
| 207 | 19378006 | 14210590 | 422 | 35841272 | 32596757 | 637 | 51800811 | 51718879 |
| 208 | 19456676 | 14293046 | 423 | 35916360 | 32684443 | 638 | 51874336 | 51808850 |
| 209 | 19535318 | 14375544 | 424 | 35991438 | 32772144 | 639 | 51947856 | 51898828 |
| 210 | 19613930 | 14458083 | 425 | 36066507 | 32859859 | 640 | 52021370 | 51988815 |
| 211 | 19692515 | 14540664 | 426 | 36141565 | 32947589 | 641 | 52094880 | 52078809 |
| 212 | 19771071 | 14623286 | 427 | 36216614 | 33035333 | 642 | 52168384 | 52168811 |
| 213 | 19849600 | 14705948 | 428 | 36291653 | 33123092 |  |  |  |
| 214 | 19928101 | 14788651 | 429 | 36366682 | 33210864 |  |  |  |

NOTE 1:	The first column is the number of errors (ne = number of NACK).

NOTE 2:	The second column is the number of samples for the pass limit (nsp, ns=Number of Samples= number of NACK + ACK).

NOTE 3:	The third column is the number of samples for the fail limit (nsf).

NOTE 4:	An ideal DUT passes after 1074532 samples. The maximum test time is 52171625 samples. A DUT passes, if the maximum number of samples is reached and it did not fail before.

## G.4.3a Pass fail decision rules

The pass fail decision rules apply for a single test.

Having observed   1 error, pass the test at   1074532+ samples, fail the test at 216- samples, otherwise continue

Having observed   2 errors, pass the test at  1274645+ samples, fail the test at 217- samples, otherwise continue

Having observed   3 errors, pass the test at  1444583+ samples, fail the test at 218- samples,  otherwise continue

Etc. etc.

Having observed 641 errors,  pass the test at  52094880+  samples, fail the test at 52078809- samples,  otherwise continue

Having observed 642 errors,  pass the test at  52168384+  samples, fail the test at 52168811- samples.

Where x+ means: x or more,  x- means x or less

NOTE 1:	an ideal DUT passes after 216 samples. The maximum test time is 52168384 samples.

## G.4.4	Simulation to derive the pass-fail limits for 0.001% BLER

The binomial distribution and its inverse are used to design the pass and fail limits. Note that this method is not unique and that other methods exist.

Where

-	fail(..) is the error ratio for the fail limit.

-	pass(..) is the error ratio for the pass limit.

-	ER is the specified error ratio 1e-5.

-	ne is the number of bad results. This is the variable in both equations.

-	M is the Bad DUT factor M=1.5.

-	df is the wrong decision probability of a single (ne, ns) co-ordinate for the fail limit.
It is found by simulation to be df = 2e-7.

-	clp is the confidence level of a single (ne, ns) co-ordinate for the pass limit.
It is found by simulation to be clp = 0.9999999.

-	qnbinom(..): The inverse cumulative function of the negative binomial distribution.

The simulation works as follows:

-	A large population of limit DUTs with true ER = 1e-5 is decided against the pass and fail limits.

-	clp and df are tuned such that CL (99.999 %) of the population passes and D (0.001 %) of the population fails.

-	A population of Bad DUTs with true ER = M*1e-5 is decided against the same pass and fail limits.

-	clp and df are tuned such that CL (99.999 %) of the population fails and D (0.001 %) of the population passes.

-	The number of DUTs decrease during the simulation, as the decided DUTs leave the population. That number decreases with an approximately exponential characteristics. After 642 bad results all DUTs of the population are decided.

NOTE:	The exponential decrease of the population is an optimal design goal for the decision co-ordinates (ne, ns), which can be achieved with other formulas or methods as well.