3GPP TSG RAN WG5 Meeting #98                                                 R5-231375

Athens, Greece, 27th Feb – 3rd Mar 2023

Agenda item:	5.3.41.1

Source:   Apple Inc.

Title:       On FR2 RF Enhanced Test Methods work plan updates

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

3GPP RAN5 has, in Rel-17, been working on various topics involving Enhanced Test Methods for FR2 UEs. The ongoing work incorporates identified improvements to current FR2 test methodologies as well as new enhanced FR2 test methods across a range of topics. This includes some issues previously reported by RAN5 such as:

- Test requirement relaxation due to High DL/Low UL signal power limitations

- Mitigation of polarization basis mismatch

- Extreme Temperature Condition testing methodology.

At RAN5#93 in November 2021, a way forward [1] was agreed to create an internal work plan to track the progress of this item in the RAN5 WG. A framework for such a work plan is discussed in this paper.

# 2	Discussion

## 2.1 High DL Power/Low UL Power impacted tests

One key area of focus of the Enhanced FR2 RF test methods plan has been to address testability issues with several FR2 RF tests that were impacted by the low/high PSD issues as detailed below in the original scope of the internal RAN5 work plan.

- High DL power and low UL power test cases: improvement of current permitted methods (e.g. IFF) has been identified in the relaxation of requirements for the specific tests reported by RAN5.

- High DL power and low UL power test cases: new methodologies based on near-field (i.e. Direct Near Field (DNF), Combined Far-Field/Direct Near Field (CFFDNF), Combined Far-Field/Delta Near Field (CFFdeltaNF) and Combined Far-Field/Near Field (CFFNF)) have been analyzed and the study outcome is included in TR 38.884.

Table 2.2.2.1-1: Summary of test cases and testability issues

| Clause | Requirement | Testability issue | Test Metric |
| --- | --- | --- | --- |
| 6.3.1 | Minimum output power | Low UL power | EIRP (Link=TX beam peak direction, Meas=Link angle). |
| 6.3.2 | Transmit OFF power | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid) |
| 6.5.1 | Occupied bandwidth | Low UL power | OBW (Link=TX beam peak direction, Meas=Link angle) |
| 6.5.2.3 | Adjacent channel leakage ratio | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). |
| 6.5.3.2 | Additional spurious emissions | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). |
| 7.4 | Maximum input power | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle). |
| 7.5 | Adjacent channel selectivity (case 1) | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) |
| 7.5 | Adjacent channel selectivity (case 2) | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) |
| 7.6.2 | In-band blocking | High DL power | EIS (Link=RX beam peak direction, Meas=Link angle) |
| 7.9 | Receiver spurious emissions | Low UL power | TRP (Link=TX beam peak direction, Meas=TRP grid). |

A detailed analysis was presented in [4] at RAN5#97 which suggested that several TCs do no longer require relaxations due to improvements in permitted test methods and thus should be removed from the list of TCs with high/low PSD issues.

A similar analysis was also shared by another TE vendor in [8] at RAN#96e.

Observation 1: Extensive analysis has been carried out across two contributions by TE vendors to demonstrate that the below tests no longer require relaxations and therefore can be removed from the list of TCs with low/high PSD issues:

6.3.1, 6.5.1, 6.5.3.2, 7.5 (case1), 7.6.2.

While the proposal in [6] was not agreed due to request for more time to evaluate, offline discussions confirmed that there is no strong objection to updating the work plan to make progress on this topic. Therefore, the suggestion in Observation 1 has been incorporated in the pre-RAN5#98 Work Plan for FR2 Enhanced Test Methods for NR UEs.

Observation 2: The Work plan has already been updated to reflect the list of tests identified as no longer impacted the low/high PSD issues.

Proposal 1: Agree to the current Work Plan which has removed the test cases that are no longer impacted by low/high PSD issues removed.

# Summary

Observation 1: Extensive analysis has been carried out across two contributions by TE vendors to demonstrate that the below tests no longer require relaxations and therefore can be removed from the list of TCs with low/high PSD issues:

6.3.1, 6.5.1, 6.5.3.2, 7.5 (case1), 7.6.2.

Observation 2: The Work plan has already been updated to reflect the list of tests identified as no longer impacted the low/high PSD issues.

Proposal 1: Agree to the current Work Plan which has removed the test cases that are no longer impacted by low/high PSD issues due to improvements in permitted test methods.

# References

- R5-218202 – Incorporating TR 38.884 outcome in RAN5 Test Specifications, Apple Portugal, AT&T, China Telecom, Dish Network, DOCOMO Communications Lab, Keysight Technologies, Nokia, Nokia Shanghai Bell, Oppo, Orange, QUALCOMM Europe Inc. - Spain, Rohde & Schwarz, Samsung, Telecom Italia, Verizon Switzerland AG, ZTE Corporation;  3GPP TSG RAN WG5 Meeting #93-e, November 2021

- TR 38.810 - NR: Study on Test Methods

- TR 38.884 - FR2 Enhanced Test Methods for NR UEs

- R5-213839: CR to 38.521-2 on Optional 4x2 PC3 Antenna Array Configuration, Keysight, 3GPP TSG RAN WG5 Meeting #92-e, August 2021

- TS 38.521-2 – FR2 RF Test Specification

- R5-221783, “Discussion on improvement of permitted test methods

- R5-231376, “Work Plan for Rel17 FR2 RF Enhanced Test Methods”, Apple, RAN5#98, Athens, Greece, Feb 27th to Mar 3rd, 2023

- R5-225222, “On achievable improvements of permitted methods”, Keysight Technologies, RAN5#96-e, August 2022