3GPP TSG-RAN5 Meeting # 98	R5-231805

# Athens, Greece,  27th Feb 2023 – 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 38.551 | CR | n/a | rev |  | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of channel models and Base Station beam configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_OTA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-08 |
| Category: |  |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 core specification has added channel models and Base station beam configurations defined for FR1 MIMO OTA measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added channel model definitions and Base station beam confiruations required for FR1 MIMO OTA measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing the definitions of channel models and Base station beam configuration in the test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

B.1	FR1 Channel models

The following channel models are required for FR1 MIMO OTA measurement.

The generic models are Table B.1-1 FR1 UMi CDL-C and Table B.1-2 FR1 UMa CDL-C, which do not include base station antenna filtering. UMi CDL-C and UMa CDL-C are selected to define 2x2 and 4x4 MIMO OTA requirements, respectively.

Therefore, in addition, the BS beam filtering effect defined in Annex B.2 also apply when emulating the channel models.

Table B.1-1: Channel model parameters for UMi CDL-C at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -36.1891 | -122.2815 | 98.9242 | 90 |
| 2 | 20.99 | -1.25 | -21.5937 | 125.831 | 99.1915 | 90 |
| 3 | 22.19 | -3.4684 | -21.5937 | 125.831 | 99.1915 | 90 |
| 4 | 23.29 | -5.2294 | -21.5937 | 125.831 | 99.1915 | 90 |
| 5 | 21.76 | -2.5215 | -32.5709 | -143.6126 | 99.5732 | 90 |
| 6 | 63.66 | 0 | -7.4275 | 166.4003 | 99.306 | 90 |
| 7 | 64.48 | -2.2185 | -7.4275 | 166.4003 | 99.306 | 90 |
| 8 | 65.6 | -3.9794 | -7.4275 | 166.4003 | 99.306 | 90 |
| 9 | 65.84 | -7.4215 | 37.2175 | 73.8315 | 100.4513 | 90 |
| 10 | 79.35 | -7.1215 | -47.1664 | 82.7664 | 98.5616 | 90 |
| 11 | 82.13 | -10.7215 | 41.5716 | -79.6999 | 100.6231 | 90 |
| 12 | 93.36 | -11.1215 | -67.1585 | 66.9895 | 98.218 | 90 |
| 13 | 122.85 | -5.1215 | -41.5244 | 84.0543 | 100.165 | 90 |
| 14 | 130.83 | -6.8215 | -47.0437 | -96.2818 | 100.2604 | 90 |
| 克劳福gojhiutfy16 | 217.04 | -8.7215 | -55.7519 | 94.8406 | 98.1225 | 90 |
| 16 | 271.05 | -13.2215 | 55.3698 | 53.9494 | 100.2604 | 90 |
| 17 | 425.89 | -13.9215 | 53.2234 | 16.0364 | 98.4852 | 90 |
| 18 | 460.03 | -13.9215 | 46.8456 | 32.2963 | 98.1416 | 90 |
| 19 | 549.02 | -15.8215 | -70.1021 | 18.2098 | 97.9698 | 90 |
| 20 | 560.77 | -17.1215 | 48.9306 | 37.0455 | 100.7376 | 90 |
| 21 | 630.65 | -16.0215 | 49.6052 | 33.7452 | 98.1225 | 90 |
| 22 | 663.74 | -15.7215 | 57.7615 | 29.801 | 98.1034 | 90 |
| 23 | 704.27 | -21.6215 | 65.6725 | 11.6092 | 100.4513 | 90 |
| 24 | 865.23 | -22.8215 | -83.5324 | 56.2837 | 100.9476 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.2265 | 12.0742 | 0.5726 | 0 | 7 |  |

Table B.1-2: Channel model parameters for UMa CDL-C at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -4.4215 | -37.4195 | -96.4031 | 96.7645 | 90 |
| 2 | 76.6135 | -1.25 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 3 | 80.9935 | -3.4684 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 4 | 85.0085 | -5.2294 | -21.7362 | 118.7405 | 98.4506 | 90 |
| 5 | 79.424 | -2.5215 | -33.5316 | -124.0196 | 100.8594 | 90 |
| 6 | 232.359 | 0 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 7 | 235.352 | -2.2185 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 8 | 239.44 | -3.9794 | -6.5142 | 171.2639 | 99.1732 | 90 |
| 9 | 240.316 | -7.4215 | 41.4581 | 51.4188 | 106.3995 | 90 |
| 10 | 289.6275 | -7.1215 | -49.2149 | 62.9864 | 94.4761 | 90 |
| 11 | 299.7745 | -10.7215 | 46.1367 | -41.2744 | 107.4834 | 90 |
| 12 | 340.764 | -11.1215 | -70.697 | 42.5606 | 92.3083 | 90 |
| 13 | 448.4025 | -5.1215 | -43.1524 | 64.6538 | 104.5929 | 90 |
| 14 | 477.5295 | -6.8215 | -49.0831 | -62.7423 | 105.1951 | 90 |
| 15 | 792.196 | -8.7215 | -58.4403 | 78.6184 | 91.7061 | 90 |
| 16 | 989.3325 | -13.2215 | 60.9633 | 25.6781 | 105.1951 | 90 |
| 17 | 1554.4985 | -13.9215 | 58.6569 | -23.4063 | 93.9944 | 90 |
| 18 | 1679.1095 | -13.9215 | 51.8037 | -2.3553 | 91.8265 | 90 |
| 19 | 2003.923 | -15.8215 | -73.86 | -20.5926 | 90.7426 | 90 |
| 20 | 2046.8105 | -17.1215 | 54.0442 | 3.7933 | 108.2061 | 90 |
| 21 | 2301.8725 | -16.0215 | 54.7691 | -0.4794 | 91.7061 | 90 |
| 22 | 2422.651 | -15.7215 | 63.5332 | -5.5859 | 91.5856 | 90 |
| 23 | 2570.5855 | -21.6215 | 72.0338 | -29.1381 | 106.3995 | 90 |
| 24 | 3158.0895 | -22.8215 | -88.2912 | 28.7003 | 109.5309 | 90 |
| Per-Cluster Parameters |  |  |  |  |  |  |
| Parameter | CASD in [°] | CASA in [°] | CZSD in [°] | CZSA in [°] | XPR in [dB] |  |
| Value | 1.3179 | 15.632 | 3.6131 | 0 | 7 |  |

B.2	FR1 Base Station beam configuration

The emulated BS beam configuration to be used for all emulation of channel models defined in Annex B.1 is specified in this clause.

The Base Station beam configuration includes basic antenna parameters and beamforming characteristic. The basic BS antenna parameters is defined in Table B.2-1.

Table B.2-1: BS Antenna Parameters

| Parameter description | Symbol | Parameter value |  |
| --- | --- | --- | --- |
|  |  | FR1 ≤2.5GHz | FR1 >2.5GHz |
| Antenna panels in vertical dimension | Mg | 1 | 1 |
| Antenna panels in horizontal dimension | Ng | 1 | 1 |
| Elements per panel in vertical dimension | Me | 4 | 8 |
| Elements per panel in horizontal dimension | Ne | 8 | 8 |
| Number of polarizations per panel | P | 2 | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 | 0.5 |

Antenna element radiation patterns, including orientation of the element main polarization components as well as orientation of the antenna array are as in the example pattern in Table 7.3-1 of TR38.901. The antenna element has ±45 polarization components and the radiation pattern parameters are 3dB = 65, 3dB = 65, Amax = 30dB, SLAv = 30dB, GE,max =8 dBi.

The beamforming characteristic of the FR1 BS pattern is defined as follow:

-	A code book of 60 fixed beams is constructed to a grid of five elevation angles from –20 to +20 with 10 steps and 12 azimuth angles from –80 to +80 with ~15 steps；

-	For 4x4 MIMO OTA, two strongest transmitting beams are selected from the pre-defined beam grid based on their proximity to the strong clusters of each FR1 channel model. These beams should have different azimuth directions and can provide the highest receive power for UE.

-	For 2x2 MIMO OTA, 1 strongest transmitting beam is selected from the pre-defined beam grid which provides the highest received power for UE based on the FR1 channel model.

-	Beam directions for channels model given in Annex B.1 are:

-	For UMa CDL-C, the beam directions are:

-	Strongest beam: AoD: -7.27°, ZoD: 100°

-	2nd strongest beam: AoD: -21.82°, ZoD: 100°

-	For UMi CDL-C, the strongest beam direction is: AoD: -7.27°, ZoD: 100°.

<End of Changes>