3GPP TSG-RAN5 Meeting #98	R5-231922

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5187 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of IoT NTN TC 7.1.4.43 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Coverage of “upon configuration or reconfiguration of offsetThresholdTA by upper layers” is needed. - Cell configuration is the System Simulator needs to be updated according to the cell to be used. - UE position needs to be pre-configured in the preamble. - Value of Differential Koffset needs to be defined and the reported TA needs to be checked in step 20. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New TP2 and steps 16A-16F were added. - Cell configuration was updated - UE position was pre-configured in the preamble. - Differential Koffset was defined and checked in step 20. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test coverage and configuration parameters will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.4.43 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-231031 with one revision. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-231031 -> R5-231922 |  |  |  |  |  |  |  |  |

[Start of change]

7.1.4.43	eMTC / NTN / UE specific TA report / UE specific Koffset

7.1.4.43.1	Test Purpose (TP)

(1)

with { UE in RRC_IDLE state and ta-Report is configured with value enabled in SystemInformationBlockType2 }

ensure that {
  when { during the Random Access procedure }

then { UE performs the Timing Advance reporting procedure }

}

(2)

with { UE in RRC_CONNECTED state and ta-Report is not configured in SystemInformationBlockType2 }

ensure that {
  when { RRCConnectionReconfiguration with offsetThresholdTA is received }

then { UE performs the Timing Advance reporting procedure }

}

(3)

with { UE in RRC_CONNECTED state and k-Offset-r17 is broadcasted in SystemInformationBlockType31 }

ensure that {
  when { Differential Koffset MAC CE is received }

then { UE indicates to lower layers the Differential Koffset for TA reporting }

}

7.1.4.43.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.331 clauses 5.3.3.2, TS 36.321 clauses 5.4.9, 5.26, 6.1.3.20 and 6.3.1.21. Unless otherwise stated these are Rel-17 requirements.

[TS 36.331, clause 5.3.3.2]

Except for NB-IoT, upon initiating the procedure, if connected to EPC or 5GC, the UE shall:

…

1>	if UE supports timing advance reporting and ta-Report is included in SystemInformationBlockType2:

2>	instruct the associated MAC entity to trigger Timing Advance reporting;

[TS 36.321, clause 5.4.9]

The UE may be configured to report information about UE specific timing advance during a Random Access procedure and in RRC_CONNECTED Mode.

The Timing Advance reporting procedure is used in a non-terrestrial network to provide the eNB with an estimate of the UE's Timing Advance, see TTA in TS 36.211 [7] clause 8.1.

Timing Advance reporting shall be triggered if any of the following events occur:

-	if triggered by upper layers;

-	upon configuration or reconfiguration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell;

-	if the variation between current information about Timing Advance and the last reported information about Timing Advance is equal to or larger than offsetThresholdTA, if configured.

If the Timing Advance reporting procedure determines that at least one Timing Advance Report has been triggered and not cancelled:

-	if the MAC entity has UL resources allocated for new transmission for this TTI, and;

-	if the allocated UL resources can accommodate the Timing Advance Report MAC control element plus its subheader, as a result of logical channel prioritization:

-	instruct the Multiplexing and Assembly procedure to generate the Timing Advance report MAC control element as defined in clause 6.1.3.20.

A MAC PDU shall contain at most one Timing Advance Report MAC CE, even when multiple events have triggered a Timing Advance report.

All triggered Timing Advance reports shall be cancelled when a Timing Advance Report MAC CE is included in a MAC PDU for transmission.

[TS 36.321, clause 5.26]

The network may provide and update the Differential Koffset of a Serving Cell in a non-terrestrial network by sending the Differential Koffset MAC CE described in clause 6.1.3.21.

The MAC entity shall:

-	if the MAC entity receives a Differential Koffset MAC CE on a Serving Cell:

-	indicate to lower layers the information regarding the Differential Koffset MAC CE.

[TS 36.321, clause 6.1.3.20]

The Timing Advance MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-2.

It has a fixed size and consists of a single field defined as follows (Figure 6.1.3.20-1):

-	R: Reserved bit, set to 0;

-	Timing Advance: The Timing Advance field indicates the least integer number of subframes greater than or equal to the Timing Advance value (see TS 36.211 [7] clause 8.1). The length of the field is 14 bits.

Figure 6.1.3.20-1: Timing Advance MAC CE

[TS 36.321, clause 6.1.3.21]

The Differential Koffset MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-1.

It has a fixed size and consists of a single field defined as follows (Figure 6.1.3.21-1):

-	R: Reserved bit, set to 0;

-	Differential Koffset: This field contains the differential Koffset. The length of the field is 6 bits.

Figure 6.1.3.21-1: Differential Koffset MAC CE

7.1.4.43.3	Test description

7.1.4.43.3.1	Pre-test conditions

System Simulator:

-	Cell 1 as specified in TS 36.508 [18] clause 4.4.1.1 is configured.

-	System information combination 32 as defined in TS 36.508 [18] clause 4.4.3.1.1 is used.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble

-	The UE is in state Switched OFF (State 1) according to TS 36.508 [18] clause 4.5.1.

7.1.4.43.3.2	Test procedure sequence

Table 7.1.4.43.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on UE. | - | - | - | - |
| 2 | The UE transmits preamble on PRACH. | --> | PRACH Preamble | - | - |
| 3 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 2 | <-- | Random Access Response | - | - |
| 4 | Check: Does the UE transmit an RRCConnectionRequest message with Timing Advance Report MAC CE? | --> | MAC PDU (Timing Advance Report MAC CE) | 1 | P |
| 5-17 | Steps 3 to 15 of the registration procedure described in TS 36.508 [18] subclause 4.5.2.3 are performed on Cell 1. | - | - | - | - |
| 17A | Power off UE. | - | - | - | - |
| 17B | The UE transmits a DETACH REQUEST message with the Detach type IE indicating “switch off”. | --> | RRC: ULInformationTransfer-NB NAS: DETACH REQUEST | - | - |
| 17C | The SS transmits an RRCConnectionRelease message to release RRC connection. | <-- | RRC: RRCConnectionRelease | - | - |
| 17D | SS broadcasts SystemInformationBlockType2 with ta-Report-r17 not present. | <-- | RRC: SystemInformationBlockType2 | - | - |
| 17E | Power on UE. | - | - | - | - |
| 17F | Steps 1 to 15 of the Attach, connected mode procedure described in TS 36.508 [18] subclause 4.5.2.3 are performed on Ncell 1. | - | - | - | - |
| 18 | The SS transmits an RRCConnectionReconfiguration message to configure the offsetThresholdTA. | <-- | RRC: RRCConnectionReconfiguration | - | - |
| 19 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message with Timing Advance Report MAC CE? | --> | RRC: RRCConnectionReconfigurationComplete | 2 | P |
| 20 | The SS transmits a MAC PDU containing a Differential Koffset MAC CE with differential Koffset set to 5 ms. | <-- | MAC PDU (Differential Koffset MAC CE) | - | - |
| 21 | Check: Does the UE transmit an MAC PDU with Timing Advance Report MAC CE? Note: The reported TA shall be 5 ms longer than the one reported in step 19. | --> | MAC PDU (Timing Advance Report MAC CE) | 3 | P |

7.1.4.43.3.3	Specific message contents

Table 7.1.4.43.3.3-1: SystemInformationBlockType2 (preamble and step 17D, Table 7.1.4.43.3.2-1)

| Derivation Path: TS 36.508 [18], Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| radioResourceConfigCommon SEQUENCE { |  |  |  |
| ntn-ConfigCommon-r17 SEQUENCE { |  |  |  |
| ta-Report-r17 | enabled |  | Preamble |
|  | Not present |  | Step 17D |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.4.43.3.3-2 RRCConnectionReconfiguration (step 18, Table 7.1.4.43.3.2-1)

| Derivation Path: TS 36.508 [18], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |  |
| radioResourceConfigDedicated SEQUENCE { |  |  |  |  |
| mac-MainConfig CHOICE { |  |  |  |  |
| explicitValue SEQUENCE { |  |  |  |  |
| timeAlignmentTimerDedicated |  | infinity |  |  |
| offsetThresholdTA-r17 CHOICE { |  |  |  |  |
| setup |  | ms3 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

[End of change]