3GPP TSG-RAN5 Meeting #99	R5-232059

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3631 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Idle mode TC 6.1.1.4a and 6.1.2.15a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specific message contents of SIB1 need to align with TS 38.508-1 and TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The message contents of SIB1 were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specific message contents will exist in these cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.4a.3 and 6.1.2.15a.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

6.1.1.4a	PLMN selection in shared network environment / Automatic mode / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities

6.1.1.4a.1	Test Purpose (TP)

Same as TC 6.1.1.4.1

6.1.1.4a.2	Conformance requirements

Same as TC 6.1.1.4.2

6.1.1.4a.3	Test description

6.1.1.4a.3.1	Pre-test conditions

Same as TC 6.1.1.4.3.1

6.1.1.4a.3.2	Test procedure sequence

Same as TC 6.1.1.4.3.2

6.1.1.4a.3.3	Specific message contents

Table 6.1.1.4a.3.3-1: SIB1 for NR Cell 1 (all steps, Table 6.1.1.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityInfoList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo { | 2 entries |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { |  |  |  |
| PLMN-Identity[1] | PLMN15 |  |  |
| } |  |  |  |
| trackingAreaCode | '00000000 00000000 00000001'B |  |  |
| ranac | 1 |  |  |
| cellIdentity | '00000000 00000000 00000001 10000000 1011'B |  |  |
| } |  |  |  |
| PLMN-IdentityInfo[2] SEQUENCE { |  | entry 2 |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { |  |  |  |
| PLMN-Identity[1] | PLMN1 |  |  |
| } |  |  |  |
| trackingAreaCode | '00000000 00000000 00000010'B |  |  |
| ranac | 2 |  |  |
| cellIdentity | '00000000 00000000 00000001 01000000 1010'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.1.4a.3.3-2: SIB1 for NR Cell 2 (all steps, Table 6.1.1.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityInfoList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo { | 3 entries |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { |  | entry 1 |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { |  |  |  |
| PLMN-Identity[1] | PLMN15 |  |  |
| } |  |  |  |
| trackingAreaCode | '00000000 00000000 00000011'B |  |  |
| ranac | 3 |  |  |
| cellIdentity | '00000000 00000000 00000001 11000000 1101'B |  |  |
| } |  |  |  |
| PLMN-IdentityInfo[2] SEQUENCE { |  | entry 2 |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { |  |  |  |
| PLMN-Identity[1] | PLMN1 |  |  |
| } |  |  |  |
| trackingAreaCode | '00000000 00000000 00000100'B |  |  |
| ranac | 4 |  |  |
| cellIdentity | '00000000 00000000 00000010 00000000 1110'B |  |  |
| } |  |  |  |
| PLMN-IdentityInfo[3] SEQUENCE { |  | entry 3 |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { |  |  |  |
| PLMN-Identity[1] | PLMN16 |  |  |
| } |  |  |  |
| trackingAreaCode | '00000000 00000000 00000101'B |  |  |
| ranac | 5 |  |  |
| cellIdentity | '00000000 00000000 00000010 01000000 1111'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.1.4a.3.3-3: RRCSetupComplete (step 5 and 27, Table 6.1.1.4.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| selectedPLMN-Identity | 2 | PLMN1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

[Skip unchanged text]

6.1.2.15a	Cell reselection in shared network environment / Cells broadcasting multiple PLMN IDs with unique TAC's, RAN areas, and cell identities

6.1.2.15a.1	Test Purpose (TP)

Same as TC 6.1.2.15.1

6.1.2.15a.2	Conformance requirements

Same as TC 6.1.2.15.2

6.1.2.15a.3	Test description

6.1.2.15a.3.1	Pre-test conditions

Same as TC 6.1.2.15.3.1

6.1.2.15a.3.2	Test procedure sequence

Same as TC 6.1.2.15.3.2

6.1.2.15a.3.3	Specific message contents

Table 6.1.2.15a.3.3-1: SIB1 for NR Cell 1 (Preamble and all steps, Table 6.1.2.15.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |  |
| plmn-IdentityInfoList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo { |  |  |  |  |
| PLMN-IdentityInfo[1] |  |  |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { |  |  |  |  |
| PLMN-Identity[1] | Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.1.2.15a.3.3-2: SIB1 for NR Cell 11 (Preamble and all steps, Table 6.1.2.15.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| cellAccessRelatedInfo SEQUENCE { |  |  |  |
| plmn-IdentityInfoList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo { | 2 entries |  |  |
| PLMN-IdentityInfo[1] SEQUENCE { | entry 1 |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { |  |  |  |
| PLMN-Identity[1] | Set to the same Mobile Country Code stored in EFIMSI on the test USIM, MNC=02 | Same MCC like PLMN for NR Cell 1 but different MNC |  |
| } |  |  |  |
| trackingAreaCode | '00000000 00000000 00000010'B |  |  |
| ranac | 2 |  |  |
| cellIdentity | '00000000 00000000 00000001 10000000 1011'B |  |  |
| } |  |  |  |
| PLMN-IdentityInfo[2] SEQUENCE { | entry 2 |  |  |
| plmn-IdentityList SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity { |  |  |  |
| PLMN-Identity[1] | Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card | This is the same PLMN as NR Cell 1 |  |
| } |  |  |  |
| trackingAreaCode | ''00000000 00000000 00000011'B |  |  |
| ranac | 1 |  |  |
| cellIdentity | ''00000000 00000000 00000001 01000000 1010'B |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.15a.3.3-3: RRCSetupComplete (step 4, Table 6.1.2.15.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetupComplete::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity |  | 2 | HPLMN |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

[End of change]