3GPP TSG-RAN5 Meeting #99	R5-233376

Incheon, Korea (Republic Of), 22nd May 2023 – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1526 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Annex A.2.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MT INVITE sent at step 1 of TS 34.229-5 test case 7.21 to add video to the call uses the default message as specified in Annex A.2.9 of TS 34.229-1 with conditions A1(IMS security) and A5(re-INVITE within a dialog). The conditions A1 and A5 for MT INVITE have different values for Contact Uri as per default configuration which might cause the SS to send a contact uri different than the one that is sent in the 183 Session Progress message(px_IMS_CalleeContactUri) during establishment of initial MO dialog in preamble. This in turn might cause a UE to update the Request Uri of consequent messages (e.g. in MO INVITE for removal of video component at step 9) to a value different than px_IMS_CalleeContactUri. In R5-233153, it is proposed to contain only condition A5 in Specific message content of MT INVITE at step 1(cl.7.21.3.3) but with that proposal, IMS security condition A1 may not be applied at intended places while executing TS 34.229-1 A.2.9 procedure. Hence, current CR proposes changes in A.2.9 of TS 34.229-1 to remove the ambiguity in Contact header address to be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Contact header for MT INVITE, addr-spec conditions are modified to ‘A1 AND A4’ and ‘A2 and A4’ instead of only A1 and A2 so that for re-INVITE scenarios(condition A5), MT INVITE includes the Contact address sent in the previous dialog. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.9 |  |  |  |  |  |  |  |  |
| sss |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

## A.2.9	INVITE for MT Call

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | INVITE |  |  |
| Request-URI | A4 | UE’s registered contact address in SIP URI form, as provided in the Contact header of the REGISTER message |  |  |
|  | A5 | UE’s contact address in SIP URI form, as provided in the Contact header within any response or request within the dialog |  |  |
|  | A10 | UE’s registered contact address in SIP URI form, as provided in the Contact header of the REGISTER message with the tag user = phone |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  |  |  | RFC 3261 [15] |
| sent-protocol |  | SIP/2.0/UDP (when using UDP) or SIP/2.0/TCP (when using TCP) |  |  |
| sent-by | A1 | The SS P-CSCF address and the SS protected server port |  |  |
|  | A2 | The SS P-CSCF address and the SS unprotected server port (optional) |  |  |
| via-branch |  | Value starting with ‘z9hG4bK’ |  |  |
| Via |  | In addition to the via-parm entry for the SS, the following via-parm entries are included: |  | RFC 3261 [15] |
| via-parm |  | SIP/2.0/UDP scscf1.3gpp.org;branch=z9hG4bK…, SIP/2.0/UDP scscf2.3gpp.org;branch=z9hG4bK..., SIP/2.0/UDP pcscf2.3gpp.org;branch=z9hG4bK..., SIP/2.0/UDP caller.3gpp.org:6543;branch=z9hG4bK... (NOTE 2) |  |  |
| Record-Route |  |  |  | RFC 3261 [15] |
| rec-route | A1 AND A4 | <sip: SS P-CSCF address: protected server port of SS ;lr> |  |  |
|  | A2 AND A4 | <sip: SS P-CSCF address: unprotected server port of SS (optional);lr> |  |  |
| Record-Route |  | In addition to the rec-route entry for the SS, the following rec-route entries are included: |  | RFC 3261 [15] |
| rec-route |  | <sip:term@scscf1.3gpp.org;lr>, <sip:orig@scscf2.3gpp.org;lr>, <sip:pcscf2.3gpp.org;lr> |  |  |
| Record-Route |  |  |  | RFC 3261 [15] |
| rec-route | A1 AND A5 | MO call established: same value as in 183 Session Progress for INVITE, condition A1, in reverse order MT call established: <sip: SS P-CSCF address: protected server port of SS ;lr> |  |  |
| Record-Route |  |  |  | RFC 3261 [15] |
| rec-route | A2 AND A5 | MO call established: same value as in 183 Session Progress for INVITE, condition A3, in reverse order MT call established: <sip: SS P-CSCF address: unprotected server port of SS (optional);lr> |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec | A4 | SIP URI of the SS representing the calling UE |  |  |
| tag | A4 | any value |  |  |
| addr-spec | A5 | SIP URI of the SS representing the calling UE as used in any previous request in the same dialog (In the earlier requests within the same dialog this URI appears in To header within requests sent by the UE and in From header within requests sent by the SS) |  |  |
| tag | A5 | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec | A4 | SIP URI of the UE’s default public user id |  |  |
| tag | A4 | not present |  |  |
| addr-spec | A5 | SIP URI of the UE as used in any previous request in the same dialog (In the earlier requests within the same dialog this URI appears in From header within requests sent by the UE and in To header within requests sent by the SS) |  |  |
| tag | A5 | remote tag of the dialog ID |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid | A4 | a random text string generated by the SS |  |  |
|  | A5 | value of Call-ID as in any previous request in the same dialog |  |  |
| CSeq |  |  |  | RFC 3261 [15] |
| value | A4 | any value (e.g. 4711) |  |  |
|  | A5 | value of CSeq sent by the SS within its previous request in the same dialog but increased by one |  |  |
| method |  | INVITE |  |  |
| Supported |  | The option tags defined below shall be included additionally to any option tags defined in any specific message content, unless specified otherwise in this specific message content. |  | RFC 3261 [15] |
| option-tag |  | 100rel |  |  |
|  |  | timer |  | RFC 4028 [146] |
| P-Called-Party-ID |  | One of the UE’s registered, non-barred public ID |  | RFC 7315 [132] |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec | A1 AND A4 | SIP URI with IP address or FQDN and protected server port of the calling UE, for example “sip:caller@3gpp.org:6543” |  |  |
|  | A2 AND A4 | SIP URI with IP address or FQDN and unprotected server port of the calling UE |  |  |
|  | A5 | same contact information for the SS as used before in this dialog |  |  |
| feature-param | A3 | +g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (NOTE 1) |  |  |
|  | A7 | video |  |  |
|  |  | audio |  |  |
| Feature-Caps |  |  |  |  |
| feature-param | A9 | g.3gpp.srvcc-alerting |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | non-zero value |  |  |
| Accept |  |  | Rel-7 | RFC 3261 [15] |
| media-range | A4 | application/sdp, application/3gpp-ims+xml |  |  |
| P-Asserted-Service |  |  |  | RFC 6050 [68] |
| Service-ID | A3 AND A4 | urn:urn-7:3gpp-service.ims.icsi.mmtel |  |  |
| Accept-Contact |  |  |  | RFC 3841 [64] |
| ac-value | A3 AND A4 | *;+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel" (NOTE 1) |  | TS 24.173 [65] |
| ac-value | A8 | video |  | IR.94 [134] NG.114 [157] |
| Content-Type |  |  |  | RFC 3261 [15] |
| media-type |  | application/sdp |  |  |
| Content-Length |  |  |  | RFC 3261 [15] |
| value |  | length of message-body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) |
| A2 | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) |
| A3 | UE supports MTSI (A.3A/50 3GPP TS 34.229-2 [5]) |
| A4 | INVITE creating a dialog |
| A5 | re-INVITE within a dialog |
| A6 | Void |
| A7 | UE supports video feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A8 | INVITE for creating a video call and UE supports video media feature tag (A.12/32 3GPP TS 34.229-2 [5]) |
| A9 | INVITE for creating a voice or video call and UE supports g.3gpp.srvcc-alerting media feature tag (A.12/34 3GPP TS 34.229-2 [5]) |
| A10 | SIP Digest without TLS for Fixed Broadband Access (SIP Digest without TLS, A.6a/5 3GPP TS 34.229-2 [5]) |

NOTE 1:	URN is the outcome of the URL encoding (“Percent-Encoding” according to RFC 3986 [129]) of urn:urn-7:3gpp-service.ims.icsi.mmtel.

NOTE 2:	Branch parameter values sent by SS are different within a test case execution.

<End of modified section>