3GPP TSG-RAN5 Meeting #99			R5-233481

Incheon, Korea, May 22th - 26th, 2023

Title:	LS on frequencyInfo for NR SL RSRP measurements

Response to:	N/A

Release:	Rel-16

Work Item:	5G_V2X_NRSL_eV2XARC-UEConTest

Source:	TSG RAN WG5

To:	TSG RAN WG2

Cc:	-

Contact Person:

Name:	Xuesong Wang

Tel. Number:

E-mail Address:	cedar.wang@hisilicon.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN5 has the following observations impacting NR SL-RSRP measurement test cases in TS 38.523-1 clause 12.1.3.1, 12.1.3.2, 12.1.3.3, 12.2.5.1, 12.2.5.2 and 12.2.5.3 and message content default setting in TS 38.508-1 Table 4.6.6-16:

- In IE SL-MeasObject-r16 there is a mandatory field frequencyInfoSL-r16 with IE type ARFCN-ValueNR. However, there is no description for this field in 38.331. Then its exact meaning is unclear.

In RAN5 understanding, this field is not the indication of the central frequency of measurement resources since NR SL-RSRP measurement is based on PSSCH-DMRS. It’s impossible for SL UE to keep sending PSSCH-DMRS in a fixed frequency location, especially in SL transmission mode 2.

It’s also RAN5 understanding this field seems to be the indication of the carrier on which SL-RSRP measurement is performed. However, there is still ambiguity about the interpretation of this field. For example, this field may be interpreted as the indication to the central/lower edge/upper edge of carrier/SL-BWP/resource pool, etc.. This ambiguity can lead to unpredictable UE behaviour during testing.

Considering The test cases mentioned above cannot be deliverable until the meaning of frequencyInfoSL-r16 is clarified, RAN5 would like RAN2 to kindly answer below questions.

Q1: What is the exact meaning of frequencyInfoSL-r16 in SL-MeasObject-r16?

2. Actions:

To RAN2 group.

ACTION:

RAN5 respectfully asks RAN2 to provide feedback on question Q1 above.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN5 Meeting#100	 21st – 25th August 2023	Toulouse, France

TSG-RAN5 Meeting#101	 13nd – 17th November 2023	Chicago, United States

4. Reference:

[1] R5-232496 Discussion on frequencyInfo for NR SL RSRP measurements