3GPP TSG-RAN5 Meeting #99	R5-232036r1

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | 0081 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MUSIM UAI test function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During RAN5#98-e meeting,a discussion paper R5-230807 was endorsed to introduce a new UAI test function to TS 38.509 for MUSIM NR devices to verify the MUSIM assistance information related test case, this CR will introduce the MUSIM UAI test function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new MUSIM UAI test function for NR MUSIM devices. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WP of MUSIM can not be completed and the related UAI test cases can not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.13(new),6.13(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR .38.523-1.. CR .3624.. |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1:tick the ME box in the coversheet of change affects |  |  |  |  |  |  |  |  |

<Start of modified section 1>

5.13	Set MUSIM UAI test function

5.13.1	General

The SS uses the Set MUSIM UAI test procedure to trigger a change of preference in the MUSIM UE that will cause the indication of UE Assistance Information (UAI) by the UE, see figure 5.8.1-1. The different types of information that can be sent in a SET MUSIM UAI REQUEST is preferred RRC state and gap preference for MUSIM, see clause 6.13.

Figure 5.13.1-1: Set MUSIM UAI test procedure

5.13.2	Initiation

The SS requests the UE to set the MUSIM related UE Assistance Information by transmitting a SET MUSIM UAI REQUEST message and the UE confirms by responding with a SET MUSIM UAI RESPONSE message.

### 5.13.3	Reception of SET MUSIM UAI REQUEST message by UE

When UE receives SET MUSIM UAI REQUEST message then the UE shall:

1>	if the MUSIM UE is operating in RRC_CONNECTED state:

2>	set its preferred RRC state within the release preference, to the equivalent value as received in the musim-PreferredRRC-State-r17 of the SET MUSIM UAI REQUEST message.

2>	transmit SET MUSIM UAI RESPONSE message.

1>	if the UE has a preference for MUSIM periodic gap(s):

2>	set its preferred gap preference, to the equivalent value as received in the musim-GapPreferenceList of the SET MUSIM UAI REQUEST message.

2>	transmit SET MUSIM UAI RESPONSE message.

else:

2>	the UE behaviour is unspecified.

<End of modified section 1>

<Start of modified section 2>

6.13	SET MUSIM UAI REQUEST

6.13.1	SET MUSIM UAI REQUEST

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| musim-PreferredRRC-State |  | M | V | 1 |
| musim-GapPreferenceList |  | CV-musimGap | TLV | 5*(NC+1)+2 |

| Condition | Explanation |
| --- | --- |
| CV-musimGap | 0 ≤ NC ≤ 3. |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | octet 1 |

where musim-PreferredRRC-State is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reserved |  |  |  |  |  | E1 | E0 | octet 3 |

E1=0 and E0=0; idle.

E1=0 and E0=1; inactive.

E1=1 and E0=0: outOfConnected.

E1=1 and E0=1: Reserved.

where musim-GapPreferenceList is:

| 8 7 6 5 4 3 2 1 |  |
| --- | --- |
| musim-GapPreferenceList Value type | octet 1 |
| Length of musim-GapPreferenceList Value contents in bytes | octet 2 |
| musim-GapPreferenceList IE #0 | octet 3 octet 4 octet 5 octet 6 octet 7 |
| … |  |
| musim-GapPreferenceList IE #NC | octet 3+5 * NC octet 4+5 * NC octet 5+5 * NC octet 6+5 * NC octet 7+5 * NC |

And where musim-GapPreferenceList Value type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | octet 1 |

And where musim-GapPreferenceList IE #n is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 |  | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A9 | A8 | A7 | A6 | A5 | A4 |  | A3 | A2 | octet 1 |
| A1 | A0 | B3 | B2 | B1 | B0 |  | Reserved |  | octet 2 |
| C2 | C1 | C0 | D3 | D2 | D1 |  | D0 | Reserved | octet 3 |
| E13 | E12 | E11 | E10 | E9 | E8 |  | E7 | E6 | octet 4 |
| E5 | E4 | E3 | E2 | E1 | E0 |  | Reserved |  | octet 5 |

A9..A0 = 0..1023 Indicates gap starting SFN number for the aperiodic MUSIM gap preference (binary coded, A9 is most significant bit and A0 least significant bit).

B3..B0 = 0..9 Indicates gap starting subframe number for the aperiodic MUSIM gap(binary coded, B3 is most significant bit and B0 least significant bit).

C2..C0 = 0..4 Indicates the length of the UE's MUSIM gap as specified in TS 38.133 [14] clause 9.1.10. This field is mandatory present for both periodic gap and aperiodic gap preference indication(binary coded, C2 is most significant bit and C0 least significant bit).

C2..C0 = 0; ms3.

C2..C0 = 1; ms4.

C2..C0 = 2; ms6.

C2..C0 = 3; ms10.

C2..C0 = 4; ms20.

D3..D0 = 0..8 Indicates the gap repetition period in ms for the periodic MUSIM gap as specified in TS 38.133 [14] clause 9.1.10 (binary coded, D3 is most significant bit and D0 least significant bit).

D3..D0 = 0; ms20.

D3..D0 = 1; ms40.

D3..D0 = 2; ms80.

D3..D0 = 3; ms160.

D3..D0 = 4; ms320.

D3..D0 = 5; ms640.

D3..D0 = 6; ms1280.

D3..D0 = 7; ms2560.

D3..D0 = 8; ms5120.

E13..E0= 0.. (gap repetition period-1) Indicates the gap offset in number of subframes for the periodic MUSIM gap as specified in TS 38.133 [14] clause 9.1.10. This field is mandatory present for the periodic MUSIM gap indication (binary coded, E13 is most significant bit and E0 least significant bit).

6.13.2	SET MUSIM UAI RESPONSE

This message is only sent in the direction UE to SS.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | octet 1 |

<End of modified section 2>