3GPP TSG-RAN5 Meeting #99	R5-232231r1

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2361 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of SA FR2 direct SCell activation test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA FR2 direct Scell activation test case is incomplete in RAN5 test specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added cell configuration mapping in E.5 - Added connection diagram - Test Applicability in TS38.522 is completed - Test procedure, message contents, test requirement are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case would be incomplete in the test specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.3.4, E.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0271 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Correction in editor’s notes |  |  |  |  |  |  |  |  |

## 7.5	Signalling characteristics

### 7.5.3	SCell activation and deactivation delay

-------- Skipped unchanged clauses --------

-------- Start of changes --------

#### 7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2

Editor’s note: This test case is incomplete. The following aspects are either missing or TBD

- TT analysis is missing

- Measurement uncertainties are missing

7.5.3.4.1		Test Purpose

The purpose of this test is to verify that the delay and interruption for direct SCell activation delay at SCell addition are within the requirements stated in TS 38.133 [6] clause 8.3.4.

7.5.3.4.2                      Test applicability

This test applies to all types of NR UE from Release 15 onwards and supporting 2DL CA.

7.5.3.4.3                      Minimum conformance requirements

The minimum conformance requirements are specified in clauses 7.5.3.0.1 and 7.5.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.7.5.3.4.

7.5.3.4.4		Test description

7.5.3.4.4.1		Initial Conditions

This test shall be tested using the test configuration in Table 7.5.3.4.4.1-1.

Table 7.5.3.4.4.1-1: Supported test configurations for NR SA FR2 SCell activation case

| Configuration | Description |
| --- | --- |
| 7.5.3.4-1 | NR 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |

Configure the test equipment and the DUT according to the parameters in Table 7.5.3.4.4.1-2 and Table 7.5.3.4.4.1-3.

Table 7.5.3.4.4.1-2: Initial conditions for NR SA FR2 SCell activation case

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 7.5.3.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.3.1.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.4.1.1 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

Table 7.5.3.4.4.1-3: General test parameters for FR2 SCell activation case with FR2 PSCell

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channels (1,2) in FR2 are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured and activated SCell |  | Cell 2 | Configured and activated SCell on NR RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| T1 | s | 7 | During this time the measurement for Cell 2 is configured, and Cell 2 is detected. |
| T2 | s | Ndirect | During this time the UE shall configure and activate Cell 2 as SCell. |
| T3 | ms | 100 | During this time the UE shall report valid CQI for both PCell and SCell. |
| THARQ | ms | k1*NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [8] |
| k | ms |  | As specified in clause 4.3 of TS38.213 [8] |

1.The general test parameter settings are set up according to Table 7.5.3.4.4.1-3.

2. Message contents are defined in clause 7.5.3.4.4.3.

3. There are 2 NR Cells specified in the test. Cell 1 is the PCell and Cell 2 is SCell. Cell 1 and Cell 2 are configured according to Annex C.1.1 and C.1.2.

7.5.3.4.4.2		Test Procedure

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are two FR2 carriers and two NR cells. Before the test starts the UE is connected to Cell 1 (PCell) on carrier #1, but is not aware of Cell 2 on NR carrier #2. Cell 1 and Cell 2 have constant signal levels throughout the test. The UE is monitoring the PCell. The UE shall be continuously scheduled in the PCell throughout the whole test.

At the beginning of T1 the UE receives an RRC message by which the Cell 2 is monitored by the UE. During T1, Cell 2 should be detected and measured by the UE such that it meets the condition for known cell defined in TS 38.133 [6] clause 8.3.4 for direct SCell activation.

Time period T2 starts when the RRCReconfiguration message for the configuration and activation of Cell 2 (the SCell), which is sent from the test equipment, is received at the UE antenna connector in a slot # denoted m. The test equipment shall set the parameter sCellState to activated for the SCell, which causes Cell 2 to become configured and activated.

Time period T3 starts at (m + Ndirect), at which point UE shall be reporting a valid CQI for both PCell and SCell.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during the activation of SCell. The test equipment verifies the activation time by counting the slots from the time when the SCell activation message is sent until a CQI report with other than CQI index 0 is received.

- Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

- Set the parameters according to T1 in Tables 7.5.3.4.4.1-3 and 7.5.3.4.5-1. Propagation conditions are set according to Annex C clauses C.2.2..

- T1 starts. Immediately after, the SS shall configure SCell (Cell 2) on the SCC as per TS 38.508-1 [14] clause 7.5.1 and provide measurement configurations.

3a.	The UE sends a MeasurementReport message.

- The SS shall configure transmission of PDSCH with a maximum number of 1 HARQ transmission.

- After having received a measurement report containing Cell 2, the SS activates SCC by sending a RRCReconfiguration message to the UE by which it configures the SCell (Cell 2) in activated state as defined in message contents table 7.5.3.4.4.3-5.

6.	The UE shall start reporting CSI in PSCell in slot (m+k+TRRC_process), where  TRRC_process is the RRC procedure delay defined in clause 12 of TS 38.331 [13], and Tx is the time to the end of the first complete the SSB burst as specified in TS 38.133 [6] clause 8.3.4. UE shall report CQI index 0 (out-of-range) until the SCell activation has been completed, and the SS shall monitor CSI reports for SCell sent from the UE according to the following criteria:

-	If the first CSI report for SCell is received by the SS no later than slot m+1+THARQ+T1+ TRRC_process +T_(CSI_Reporting)NR slot length,

-	or slot m+1+THARQ+TRRC_process+T_(CSI_Reporting)+T1+TXNR slot length+Ninterruption+1 if the slot m+1+THARQ+T1+TRRC_process+T_(CSI_Reporting)NR slot length was subject to interruption,

-	and CSI report with non-zero CQI index is received by the SS earlier than or equal to slot m+NdirectNR slot length,

-	or the next available uplink resource if there are no uplink resources for reporting the valid CSI in a slot m+NdirectNR slot length

-	and DTX is not observed by the SS outside the slot m+1+THARQNR slot length to m+1+THARQ+TRRC_process+T1+TXNR slot length+Ninterruption+THARQNR slot length up to the end of T2

-	Then the number of successes for the event "Activation" is increased by one. Otherwise, count a fail for the event "Activation" and go to step 7.

7.	When T2 expires, or Activation in step 5 was not acknowledged, or a failure was counted for the event "Activation" in step 6, the SS shall transmit a RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8.	Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 1008) for next iteration of the test procedure loop.

9.	After the RRC connection release, the SS:
- transmits in Cell 1 a Paging message (including PagingRecord with UE-Identity) for the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5 (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5),
or
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

10.	Repeat steps 2-9 until a test verdict has been achieved.

Each of the event “Activation” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved.

7.5.3.4.4.3		Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with condition SCELL_CSI_ON_SPCELL with the following exceptions:

Table 7.5.3.4.4.3-1: Common Exception messages

| Default Message Contents |  |  |
| --- | --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-7 with Condition Deactivated SCell Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 7.5.3.4.4.3-2: RRCReconfiguration: SCell addition

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig | Table 7.5.3.4.4.3-2A |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig-SCell | Table 7.5.3.4.4.3-5 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.5.3.4.4.3-2A: MeasConfig (Table 7.5.3.4.4.3-2)

| Derivation path: Table H.3.1-2 with condition Deactivated SCell |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | 2 entries |  |  |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR for SCell | entry 2 Table 7.5.3.4.4.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE (SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | ReportConfigId |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR | Table 7.5.3.4.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.5.3.4.4.3-3: MeasObjectNR for SCell

| Derivation Path: Table H.3.1-3 with condition Deactivated SCell and Synchronous cells |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| smtc1 | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| measCycleSCell-v1530 | sf160 |  |  |
| } |  |  |  |

Table 7.5.3.4.4.3-4: ReportConfigNR

| Derivation Path: Table H.3.1-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | -30 | To ensure reporting can always be triggered |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.5.3.4.4.3-5: CellGroupConfig-SCell (Table 7.5.3.4.4.3-2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition MEAS and SCell_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present | PCell always uses servCellIndex=0 |  |
| reconfigurationWithSync | Not present |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| rlmInSyncOutOfSyncThreshold | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig-SpCell | Table 7.5.3.4.4.3-6 |  |
| } |  |  |  |
| sCellToAddModList SEQUENCE (SIZE (1..maxNrofSCells)) OF SCellConfig { | 1 entry |  |  |
| SCellConfig[1] SEQUENCE { |  | entry 1 |  |
| sCellConfigDedicated | ServingCellConfig-SCell | Table 7.5.3.4.4.3-7 |  |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.1 |  |  |
| sCellState-r16 | activated |  |  |
| … |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.5.3.4.4.3-6: ServingCellConfig-SpCell (Table 7.5.3.4.4.3-5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| servingCellMO | 1 |  |  |
| } |  |  |  |

Table 7.5.3.4.4.3-7: ServingCellConfig-SCell (Table 7.5.3.4.4.3-5)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-167 with condition No_UL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| csi-MeasConfig | CSI-MeasConfig for RRM specified in TS 38.508-1 [14] Table 7.3.1-6 |  |  |
| servingCellMO | 2 |  |  |
| } |  |  |  |

7.5.3.4.5	Test requirement

Table 7.5.3.4.5-1 defines the cell specific test parameters and Table 7.5.3.4.5-2 defines OTA related test parameters.

Table 7.5.3.4.5-1: Cell specific test parameters for FR2 SCell activation case with FR2 PCell

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| SSB ARFCN |  |  | freq1 |  |  | freq2 |  |  |
| Duplex mode | Config 1 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  | TDDConf.3.1 |  |  |  |  |  |
| BWchannel | Config 1 | MHz | 100: NRB,c = 66 |  |  |  |  |  |
| DL initial BWP configuration | Config 1 |  | DLBWP.0.1 |  |  |  |  |  |
| DL dedicated BWP configuration | Config 1 |  | DLBWP.1.1 |  |  |  |  |  |
| UL initial BWP configuration | Config 1 |  | ULBWP.0.1 |  |  |  |  |  |
| UL dedicated BWP configuration | Config 1 |  | ULBWP.1.1 |  |  |  |  |  |
| Timing offset to Cell 1 |  | ms | Not Applicable |  |  | 0 |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET Reference Channel | Config 1 |  | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| RMC CORESET Reference Channel | Config 1 |  | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| TRS configuration | Config 1 |  | TRS.2.1 TDD |  |  | TRS.2.1 TDD |  |  |
| CSI-RS configuration | Config 1 |  | CSI-RS.3.1 TDD |  |  | CSI-RS.3.1 TDD |  |  |
| CSI reporting periodicity | Config 1 | ms | 5 |  |  | 5 |  |  |
| OCNG Patterns |  |  | OP.1 |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration | Config 1 |  | SSB.1 FR2 |  |  | SSB.1 FR2 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |  |  |  |

Table 7.5.3.4.5-2: OTA related test parameters for FR2 SCell activation case with FR2 PSCell

| ParameterNote 6 | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  | Setup 1 according to A.3.15.1 |  |  |  |  |  |
| Assumption for UE beamsNote 7 |  | Rough |  |  | Rough |  |  |
| Note1 | dBm/15kHzNote4 | -112+TT |  |  | -112+TT |  |  |
| Note1 | dBm/SCSNote3 | -102.97+TT |  |  | -102.97+TT |  |  |
|  | dB | 14+TT |  |  | 14+TT |  |  |
| SS-RSRPNote2 | dBm/SCS Note4 | -88.97+TT |  |  | -88.97+TT |  |  |
|  | dB | 14+TT |  |  | 14+TT |  |  |
| IoNote2 | dBm/95.04 MHz Note4 | -59.81+TT |  |  | -59.81+TT |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: All parameters apply for configuration 1 Note 7: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

The UE shall complete the direct activation of the SCell no later than at slot m + NdirectNR slot length.

The UE shall report non-zero CQI for SCell from slot m + NdirectNR slot length and onwards throughout time period T3.

The interruption on PCell during direct activation of the SCell shall occur within the interruption window specified in TS 38.133 [6] clause 8.3.4 and shall not exceed the length specified in TS 38.133 [6] clause 8.2.2.2.11.

The rate of correct events observed during repeated tests shall be at least 90%.

-------- End of changes --------

-------- Skipping unchanged clauses --------

-------- Start of changes --------

# E.5	Cell configuration mapping for SA FR2 test cases in Chapter 7

Table E.5-1 defines the cell configuration mapping for NR/5GC FR2 test cases in chapter 7 of this test specification.

Table E.5-1: Cell configuration mapping for SA FR2 RRM testing

| TC |  | Description |  | 38.533 NR Cell1 |  | 38.533 NR Cell2 |  | 38.533 NR Cell3 |  | 38.533 NR Cell4 |  | CA Type |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.1 |  | NR SA FR2 cell re-selection |  | NR Cell 1 |  | NR Cell 11 |  |  |  |  |  |  |  |  |
| 7.1.1.2 |  | NR SA FR2-FR2 cell re-selection |  | NR Cell 1 |  | NR Cell 23 |  |  |  |  |  |  |  |  |
| 7.1.1.3 |  | NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.1.1.4 |  | NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.1.1.5 |  | NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.1.1.6 |  | NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.2.1.1 |  | TA validation for CG-SDT in FR2 |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |
| 7.3.1.1 |  | NR SA FR1-FR2 Inter-frequency handover; unknown target cell |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 7.3.1.2 |  | NR SA FR2 Intra-frequency handover; unknown target cell |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.3.1.3 |  | NR SA FR2-FR2 Inter-frequency handover; unknown target cell |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 7.3.1.4 |  | NR SA FR1-FR2 synchronous DAPS handover |  | NR Cell 1 |  | NR Cell 10 |  |  |  |  |  |  |  |  |
| 7.3.1.5 |  | NR SA FR1-FR2 asynchronous DAPS handover |  | NR Cell 1 |  | NR Cell 10 |  |  |  |  |  |  |  |  |
| 7.3.2.1.1 |  | NR SA FR2 RRC re-establishment |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.1.2 |  | NR SA FR2-FR2 re-establishment |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.1.3 |  | NR SA FR2 RRC re-establishment without serving cell timing |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.3.2.2.1 |  | NR SA FR2 contention based random access |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.2.2 |  | NR SA FR2 non-contention based random access |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.2.4 |  | NR SA FR2 2-step non-contention based random access |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.3.2.3.1 |  | NR SA FR2-FR2 RRC connection release with redirection |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 7.3.3.1 |  | NR SA FR2 conditional handover |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.3.3.2 |  | NR SA FR2-FR2 conditional handover |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 7.4.1.1 |  | NR SA FR2 UE transmit timing accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.4.3.1 |  | NR SA FR2 timing advance adjustment accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.1.1 |  | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.1.2 |  | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.1.3 |  | Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.1.4 |  | Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.1.9 |  | NR SA FR2 radio link monitoring UE scheduling restrictions |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.3.4 |  | #### Direct SCell activation at SCell addition of known SCell in FR2 |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |
| 7.5.5.1 |  | NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.5.2 |  | NR SA FR2 SSB-based beam failure detection and link recovery in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.5.3 |  | NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.5.4 |  | NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.5.6 |  | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in non-DRX |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 7.5.5.7 |  | NR SA FR2 Scell CSI-RS-based beam failure detection and link recovery in DRX |  | NR Cell 1 |  | NR Cell 6 |  |  |  |  |  |  |  |  |
| 7.5.6.1 |  | NR SA FR2 DCI-based DL active BWP switch in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.5.11.1 |  | NR SA FR2 UE UL carrier RRC reconfiguration delay |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.6.1.1 |  | NR SA FR2 event-triggered reporting without gap in non-DRX |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.6.1.2 |  | NR SA FR2 event-triggered reporting without gap in DRX |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.6.1.3 |  | NR SA FR2 event-triggered reporting with gap in non-DRX |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.6.1.4 |  | NR SA FR2 event-triggered reporting with gap in DRX |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.6.2.1 |  | NR SA FR2-FR2 event-triggered reporting in non-DRX |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.2 |  | NR SA FR2-FR2 event-triggered reporting in DRX |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.3 |  | NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.4 |  | NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.5 |  | NR SA FR1-FR2 event-triggered reporting in non-DRX |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.6 |  | NR SA FR1-FR2 event-triggered reporting in DRX |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.7 |  | NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.2.8 |  | NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.6.3.1 |  | NR SA FR2 SSB-based L1-RSRP measurement in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.3.2 |  | NR SA FR2 SSB-based L1-RSRP measurement in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.3.3 |  | NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.3.4 |  | NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.6.1 |  | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.6.2 |  | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.6.3 |  | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.6.13.1 |  | UE Rx-Tx time difference measurement for propagation delay compensation using PRS in FR2 |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.7.1.1 |  | NR SA FR2 SS-RSRP measurement accuracy |  | NR Cell 489 |  | NR Cell 1 |  |  |  |  |  |  |  |  |
| 7.7.1.2 |  | NR SA FR2-FR2 SS-RSRP measurement accuracy |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.7.1.3.1 |  | NR SA FR1-FR2 SS-RSRP absolute measurement accuracy |  | NR Cell 1 |  | NR Cell 10 |  |  |  |  |  |  |  |  |
| 7.7.1.3.2 |  | Void |  |  |  |  |  |  |  |  |  |  |  |  |
| 7.7.2.1 |  | NR SA FR2 SS-RSRQ measurement accuracy |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.7.2.2 |  | NR SA FR2-FR2 SS-RSRQ measurement accuracy |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.7.3.1 |  | NR SA FR2 SS-SINR measurement accuracy |  | NR Cell 1 |  | NR Cell 2 |  |  |  |  |  |  |  |  |
| 7.7.3.2 |  | NR SA FR2-FR2 SS-SINR measurement accuracy |  | NR Cell 6 |  | NR Cell 3 |  |  |  |  |  |  |  |  |
| 7.7.6.1 |  | NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.7.6.2 |  | NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |
| 7.7.6.3 |  | NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  | NR Cell 1 |  |  |  |  |  |  |  |  |  |  |

-------- End of changes --------