3GPP TSG-RAN5 Meeting #99	R5-232359r1

# Incheon, Korea, 22th May 2023 – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2759 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of test frequency parameters for n79 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Table 4.3.1.1.1.79-1, KSSB must be an even number lower than 12. Therefore, corrections are needed for 10MHz Mid and 20MHz Mid and High. In Table 4.3.1.1.1.79-1, for 10MHz High, there is a valid higher frequency that should be selected to be closer to the High range. In Table 4.3.1.1.1.79-2, for 10MHz Low and Mid, OffsetCarrierCORESET#0 = 3 is not correct. In Table 4.3.1.1.1.79-2, for 20MHz Low, there is a valid lower GSCN that should be selected to be closer to the Low range. In Table 4.3.1.1.1.79-2 and Table 4.3.1.1.1.79-3, for 20MHz High, a smaller carrier center frequency must be selected to avoid the guardband exceeding the band limit. In Table 4.3.1.1.1.79-1, for 10MHz Mid and High and in Table 4.3.1.1.1.79-2 for 10MHz Low, it is not possible to find a valid set of frequency parameters respecting the number of steps and directions indicated in annex C.3.2. These cases should be treated as exceptions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Tables 4.3.1.1.1.79-1, 4.3.1.1.1.79-2 and 4.3.1.1.1.79-3 corrected according to the reasons explained above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.79 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: -r1: added Huawei, HiSilicon as co-source company. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

###### 4.3.1.1.1.79	Reference test frequencies for NR operating band n79

Table 4.3.1.1.1.79-1: Test frequencies for NR operating band n79, SCS 15 kHz and ΔFRaster 15 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 103 | 52 | Downlink | Low | 4405.02 | 693668 | 4400.34 | 693356 | 0 | 30 | 8475 | 693696 | 4 | 4 | 0 (4) | 8 |
|  |  | & | Mid (Note 4) | 4700.13 | 713342 | 4677.09 | 711806 | 102 |  | 8680 | 713376 | 10 | 4 | 0 (4) | 110 |
|  |  | Uplink | High (Note 4) | 4994.76 | 732984 | 4899.36 | 726624 | 504 |  | 8884 | 732960 | 0 | 0 | 0 (4) | 508 |
| 20 | 106 | Downlink | Low | 4410 | 694000 | 4400.46 | 693364 | 0 | 30 | 8476 | 693792 | 8 | 11 | 0 (4) | 15 |
|  |  | & | Mid | 4699.98 | 713332 | 4672.08 | 711472 | 102 |  | 8676 | 712992 | 8 | 0 | 0 (4) | 106 |
|  |  | Uplink | High | 4989.99 | 732666 | 4889.73 | 725982 | 504 |  | 8878 | 732384 | 6 | 5 | 0 (4) | 513 |
| 40 | 216 | Downlink | Low | 4420.02 | 694668 | 4400.58 | 693372 | 0 | 30 | 8480 | 694176 | 0 | 43 | 0 (4) | 47 |
|  |  | & | Mid | 4699.98 | 713332 | 4662.18 | 710812 | 102 |  | 8672 | 712608 | 8 | 23 | 0 (4) | 129 |
|  |  | Uplink | High | 4980 | 732000 | 4869.84 | 724656 | 504 |  | 8864 | 731040 | 0 | 4 | 0 (4) | 512 |
| 50 | 270 | Downlink | Low | 4425 | 695000 | 4400.7 | 693380 | 0 | 30 | 8480 | 694176 | 4 | 42 | 0 (4) | 46 |
|  |  | & | Mid | 4699.98 | 713332 | 4657.32 | 710488 | 102 |  | 8672 | 712608 | 8 | 50 | 0 (4) | 156 |
|  |  | Uplink | High | 4974.99 | 731666 | 4859.97 | 723998 | 504 |  | 8864 | 731040 | 10 | 58 | 0 (4) | 566 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-5 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: The High range frequency can’t cover the two RBs at the right edge of band due to the limit of GSCN range in TS 38.101-1 table 5.4.3.3-1. Note 4: This frequency is obtained applying certain exceptions over general rules defined in annex C.3.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.79-2: Test frequencies for NR operating band n79, SCS 30 kHz and ΔFRaster 30 kHz

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 | Downlink | Low (Note 3) | 4405.83 | 693722 | 4401.51 | 693434 | 0 | 30 | 8475 | 693696 | 22 | 0 | 0 (0) | 0 |
|  |  | & | Mid | 4699.92 | 713328 | 4658.88 | 710592 | 102 |  | 8679 | 713280 | 0 | 0 | 0 (0) | 204 |
|  |  | Uplink | High | 4995 | 733000 | 4809.24 | 720616 | 504 |  | 8884 | 732960 | 8 | 0 | 0 (0) | 1008 |
| 20 | 51 | Downlink | Low | 4410 | 694000 | 4400.82 | 693388 | 0 | 30 | 8475 | 693696 | 20 | 2 | 0 (0) | 4 |
|  |  | & | Mid | 4700.01 | 713334 | 4654.11 | 710274 | 102 |  | 8676 | 712992 | 6 | 1 | 0 (0) | 206 |
|  |  | Uplink | High | 4989.99 | 732666 | 4799.37 | 719958 | 504 |  | 8878 | 732384 | 18 | 3 | 0 (0) | 1014 |
| 40 | 106 | Downlink | Low | 4420.02 | 694668 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4644.21 | 709614 | 102 |  | 8672 | 712608 | 18 | 8 | 1 (4) | 228 |
|  |  | Uplink | High | 4980 | 732000 | 4779.48 | 718632 | 504 |  | 8864 | 731040 | 0 | 3 | 0 (0) | 1014 |
| 50 | 133 | Downlink | Low | 4425 | 695000 | 4401.06 | 693404 | 0 | 30 | 8480 | 694176 | 4 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4639.35 | 709290 | 102 |  | 8672 | 712608 | 6 | 22 | 1 (4) | 256 |
|  |  | Uplink | High | 4974.99 | 731666 | 4769.61 | 717974 | 504 |  | 8864 | 731040 | 10 | 26 | 1 (4) | 1068 |
| 60 | 162 | Downlink | Low | 4430.01 | 695334 | 4400.85 | 693390 | 0 | 30 | 8480 | 694176 | 18 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4634.13 | 708942 | 102 |  | 8672 | 712608 | 18 | 36 | 1 (4) | 284 |
|  |  | Uplink | High | 4969.98 | 731332 | 4759.38 | 717292 | 504 |  | 8864 | 731040 | 20 | 54 | 1 (4) | 1124 |
| 80 | 217 | Downlink | Low | 4440 | 696000 | 4400.94 | 693396 | 0 | 30 | 8480 | 694176 | 12 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4624.23 | 708282 | 102 |  | 8656 | 711072 | 6 | 0 | 1 (4) | 212 |
|  |  | Uplink | High | 4959.99 | 730666 | 4739.49 | 715966 | 504 |  | 8848 | 729504 | 2 | 46 | 1 (4) | 1108 |
| 100 | 273 | Downlink | Low | 4450.02 | 696668 | 4400.88 | 693392 | 0 | 30 | 8480 | 694176 | 16 | 18 | 1 (4) | 44 |
|  |  | & | Mid | 4700.01 | 713334 | 4614.15 | 707610 | 102 |  | 8656 | 711072 | 6 | 28 | 1 (4) | 268 |
|  |  | Uplink | High | 4950 | 730000 | 4719.42 | 714628 | 504 |  | 8832 | 727968 | 20 | 37 | 1 (4) | 1090 |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-6 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3: This frequency is obtained applying certain exceptions over general rules defined in annex C.3.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.79-3: Test frequencies for NR operating band n79, SCS 60 kHz and ΔFRaster 30 kHz without CORESET#0.

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 | Downlink | Low | 4405.02 | 693668 | 4401.06 | 693404 | 0 | 30 | - | 693644 |
|  |  | & | Mid | 4700.01 | 713334 | 4622.61 | 708174 | 102 |  | - | 713310 |
|  |  | Uplink | High | 4995 | 733000 | 4628.16 | 708544 | 504 |  | - | 732976 |
| 20 | 24 | Downlink | Low | 4410 | 694000 | 4401.36 | 693424 | 0 | 30 | - | 693664 |
|  |  | & | Mid | 4700.01 | 713334 | 4617.93 | 707862 | 102 |  | - | 712998 |
|  |  | Uplink | High | 4989.99 | 732666 | 4618.47 | 707898 | 504 |  | - | 732330 |
| 40 | 51 | Downlink | Low | 4420.02 | 694668 | 4401.66 | 693444 | 0 | 30 | - | 693684 |
|  |  | & | Mid | 4700.01 | 713334 | 4608.21 | 707214 | 102 |  | - | 712350 |
|  |  | Uplink | High | 4980 | 732000 | 4598.76 | 706584 | 504 |  | - | 731016 |
| 50 | 65 | Downlink | Low | 4425 | 695000 | 4401.6 | 693440 | 0 | 30 | - | 693680 |
|  |  | & | Mid | 4700.01 | 713334 | 4603.17 | 706878 | 102 |  | - | 712014 |
|  |  | Uplink | High | 4974.99 | 731666 | 4588.71 | 705914 | 504 |  | - | 730346 |
| 60 | 79 | Downlink | Low | 4430.01 | 695334 | 4401.57 | 693438 | 0 | 30 | - | 693678 |
|  |  | & | Mid | 4700.01 | 713334 | 4598.13 | 706542 | 102 |  | - | 711678 |
|  |  | Uplink | High | 4969.98 | 731332 | 4578.66 | 705244 | 504 |  | - | 729676 |
| 80 | 107 | Downlink | Low | 4440 | 696000 | 4401.48 | 693432 | 0 | 30 | - | 693672 |
|  |  | & | Mid | 4700.01 | 713334 | 4588.05 | 705870 | 102 |  | - | 711006 |
|  |  | Uplink | High | 4959.99 | 730666 | 4558.59 | 703906 | 504 |  | - | 728338 |
| 100 | 135 | Downlink | Low | 4450.02 | 696668 | 4401.42 | 693428 | 0 | 30 | - | 693668 |
|  |  | & | Mid | 4700.01 | 713334 | 4577.97 | 705198 | 102 |  | - | 710334 |
|  |  | Uplink | High | 4950 | 730000 | 4538.52 | 702568 | 504 |  | - | 727000 |
| Note: FR1 carrier without CORESET#0 is indicated in the MIB by setting =31, controlResourceSetZero=0 and searchSpaceZero = 0 (TS 38.213 [22], clause 13). |  |  |  |  |  |  |  |  |  |  |  |

## <<< END OF CHANGES >>>