3GPP TSG-RAN5 Meeting #99	R5-232658r2

# Incheon, Korea (Republic Of), 22nd May 2023 – 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2409 | rev | - | Current version: | 17.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to CLI tests 4.7.6.1 and 6.7.8.1 with TTs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CLI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CLI-based SRS-RSRP measurement accuracy test cases have multiple missing parts and need to be updated. One of these multiple parts are the TTs, which this CR is also adding. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated core requirements sections. - Updated test cases 4.7.6.1 and 6.7.8.1 and added test procedure and message contents. - Added TTs to test cases 4.7.6.1 and 6.7.8.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case implementation will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.6.0, 4.7.6.1, 6.7.8.0, 6.7.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: updated spec version R2: editorial update to header style |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### 4.7.6	CLI measurements

#### 4.7.6.0	Minimum conformance requirements

##### 4.7.6.0.1	Minimum conformance requirements for SRS-RSRP accuracy

The SRS-RSRP measurement reported by the UE shall fulfil the accuracy requirements defined in Table 4.7.6.0.1-1 for FR1, provided that the following conditions are met. The accuracy requirements in this clause are derived based on AWGN radio propagation conditions.

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

-	Conditions for SRS-RSRP measurements are fulfilled according to 38.133 [6] Annex B.2.7 for a corresponding Band for each relevant SRS resource configured for measurement.

-	The time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where:

-	Terror_SRS_RSRP = TC × NTA_offset + 4.67us for FR1

-	NTA_offset is defined in 38.133 [6] Table 7.1.2-2

-	TC is 0.509ns

-	The number of SRS ports in the SRS resource configured for measurement is 1,

-	The number of symbols in the SRS resource configured for measurement is 1,

-	The number of repetitions in the SRS resource configured for measurement is 1,

-	Frequency hopping, sequence group hopping, or sequence hopping is disabled in the SRS resource configured for measurement,

-	The bandwidth of the SRS resource is 48 PRBs.

-	One of the following conditions is met

-	There is no other SRS resource with the same root sequence and on the same symbol and with same comb as the relevant SRS resource.

-	If multiple SRS resources are on the same symbol and with same comb, the distance between cyclic shifts of any two resources is no less than 6 if transmissionComb = n4, and no less than 4 if transmissionComb = n2.

Table 4.7.6.0.1-1: SRS-RSRP absolute accuracy in FR1

| Accuracy |  |  |  |  |  |  |  |  |  |  | Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition |  |  |  |  | Extreme condition |  |  |  | SRS Ês/Iot |  | Io Note 1 range |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | NR operating band groups Note 2 |  |  |  | Minimum Io |  |  |  |  |  |  | Maximum Io |  |  |
| dB |  |  |  |  |  |  |  |  |  |  | dB |  |  |  |  | dBm / SCSSRS |  |  |  |  |  | dBm/BW Channel |  |  | dBm/BW Channel |
| SCSSRS (kHz) |  |  | SCSSRS (kHz) |  |  |  |  |  |  |  |  |  |
| 15 |  | 30 | 60 |  | 15 | 30 | 60 |  |  |  |  |  |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz |  | SCSSRS = 60 kHz |  |  |  |  |  |  |
| 3 |  | 3.5 | 5 |  | 7.5 | 8 | 9.5 |  | 1 |  | NR_TDD_FR1_A, |  |  |  | -120 | -117 |  | -114 |  | N/A |  | -70 |  |  |
|  |  |  |  |  |  |  |  |  |  |  | NR_TDD_FR1_C |  |  |  | -119 | -116 |  | -113 |  | N/A |  | -70 |  |  |
|  |  |  |  |  |  |  |  |  |  |  | NR_TDD_FR1_D |  |  |  | -118.5 | -115.5 |  | -112.5 |  | N/A |  | -70 |  |  |
|  |  |  |  |  |  |  |  |  |  |  | NR_TDD_FR1_E |  |  |  | -118 | -115 |  | -112 |  | N/A |  | -70 |  |  |
| 6.5 |  | 7 | 8.5 |  | 9.5 | 10 | 11.5 |  | 1 |  | NR_TDD_FR1_A, NR_TDD_FR1_C, NR_TDD_FR1_D, NR_TDD_FR1_E |  |  |  | N/A | N/A |  | N/A |  | -70 |  | -50 |  |  |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in 38.133 [6] clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.22.1.1

###### 4.7.6.0.1.1	SRS-RSRP report mapping

The reporting range of SRS-RSRP is defined from -140 dBm to -44 dBm with 1 dB resolution. The mapping of measured quantity is defined in Table 4.7.6.0.1.2-1. The range in the signalling may be larger than the guaranteed accuracy range.

Table 4.7.6.0.1.2-1: SRS-RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS-RSRP_0 | SRS-RSRP<-140 | dBm |
| SRS-RSRP_1 | -140≤ SRS-RSRP<-139 | dBm |
| SRS-RSRP_2 | -139≤ SRS-RSRP<-138 | dBm |
| SRS-RSRP_3 | -138≤ SRS-RSRP<-137 | dBm |
| SRS-RSRP_4 | -137≤ SRS-RSRP<-136 | dBm |
| .. | .. | … |
| SRS-RSRP_95 | -46≤ SRS-RSRP<-45 | dBm |
| SRS-RSRP_96 | -45≤ SRS-RSRP<-44 | dBm |
| SRS-RSRP_97 | -44≤ SRS-RSRP | dBm |
| SRS-RSRP_98 | Infinity |  |
| Note: ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.22.1.2.

##### 4.7.6.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell

The CLI-RSSI measurement reported by the UE shall fulfil the accuracy requirements defined in Table 4.7.6.0.2-1 for FR1, provided that the following conditions are met.

-	Conditions defined in clause 7.3 of TS 38.101-1 [2] for reference sensitivity are fulfilled.

Table 4.7.6.0.2-1: CLI-RSSI absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Io Note 1 range |  |  |  |  |  |
|  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB |  | dBm / SCSSRS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSRS = 15 kHz | SCSSRS = 30 kHz | SCSSRS = 60 kHz |  |  |
| 3.5 | 6.5 | NR_TDD_FR1_A, | -120 | -117 | -114 | N/A | -70 |
|  |  | NR_TDD_FR1_C | -119 | -116 | -113 | N/A | -70 |
|  |  | NR_TDD_FR1_D | -118.5 | -115.5 | -112.5 | N/A | -70 |
|  |  | NR_TDD_FR1_E | -118 | -115 | -112 | N/A | -70 |
| 5.5 | 8.5 | Note 3 | Note 3 | Note 3 | Note 3 | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in 38.133 [6] clause 3.5.2. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clause 10.1.22.2.1

###### 4.7.6.0.2.1	CLI-RSSI report mapping

The reporting range of CLI-RSSI is defined from -100 dBm to -25 dBm with 1 dB resolution. The mapping of measured quantity is defined in Table 4.7.6.0.2.1-1. The range in the signalling may be larger than the guaranteed accuracy range. UE shall scale the measured CLI-RSSI to report a nominal RSSI equivalent to 6RB measurement with 15kHz SCS.

Table 4.7.6.0.2.1-1: CLI-RSSI measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CLI-RSSI_00 | CLI-RSSI < -100 | dBm |
| CLI-RSSI_01 | -100 CLI-RSSI < -99 | dBm |
| CLI-RSSI_02 | -99 CLI-RSSI < -98 | dBm |
| … | … | … |
| CLI-RSSI_74 | -27 CLI-RSSI < -26 | dBm |
| CLI-RSSI_75 | -26 CLI-RSSI < -25 | dBm |
| CLI-RSSI_76 | -25 CLI-RSSI | dBm |

The normative reference for this requirement is TS 38.133 [6] clauses 10.1.22.2.2.

#### 4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

4.7.6.1.1	Test purpose

The purpose of this test is to verify that the SRS-RSRP measurement accuracy is within the specified limits. This test will verify the SRS-RSRP measurement requirements in clause 4.7.6.0.

4.7.6.1.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting NR EN-DC and CLI-based SRS-RSRP measurements.

4.7.6.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.7.6.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.7.6.1.

4.7.6.1.4	Test description

In this set of test cases there are two cells in the test, E-UTRAN PCell (Cell 1) and NR FR1 PSCell (Cell 2). The test parameters for Cell 1 are defined in Annex A.6. The test parameters for the Cell 2 are given in Table 4.7.6.1.5-1 below. The test parameter for the (virtual) neighbor cell UE transmitting SRS are given in Table 4.7.6.1.5-2. The SRS resource configuration is given in Table 4.7.6.1.5-3.

4.7.6.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.7.6.1.4.1-1.

Table 4.7.6.1.4.1-1: Supported test configurations for EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

| Config | Description |
| --- | --- |
| 4.7.6.1-1 | LTE FDD, NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 4.7.6.1-2 | LTE FDD, NR 30kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| 4.7.6.1-3 | LTE TDD, NR 15 kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 4.7.6.1-4 | LTE TDD, NR 30kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 4.7.6.1.4.1-2.

Table 4.7.6.1.4.1-2: Initial conditions for EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.7.6.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |

1.	Message contents are defined in clause 4.7.6.1.4.3.

2.	Cell 1 is the E-UTRAN PCell, with power levels and settings set according to Annex A.6. Cell 2 is the NR serving cell (PSCell). The power levels and settings for Cell 2 are set according to Annex C.1.2 and C.1.3. Virtual UE 1 is the target for SRS-RSRP measurements.

3.	The test parameters are given in Table 4.7.6.1.4.1-3

4.7.6.1.4.2	Test procedure

Before the test UE is configured to perform SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the DL slots according to the SRS configuration in Table 4.7.6.1.5-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to the corresponding Test in Table 4.7.6.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message, embedded in an RRCConnectionReconfiguration message, configuring a CLI measurement object with periodic reporting, as specified in section 4.7.6.1.4.3.

4. The UE shall transmit an RRCReconfigurationComplete message, embedded in an RRCConnectionReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages, embedded in ULInformationTransferMRDC messages.

6. After 10s wait from Step 5, the SS shall check the SRS-RSRP reported values in the periodic MeasurementReport. The reported SRS-RSRP value of the neighbor virtual UE is compared to the expected SRS-RSRP as specified in Table 4.7.6.1.5-4 and Table 4.7.6.1.5-5 for Test 1 and Test 2, correspondingly. If the reported value is outside the limits specified in such tables, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each Test in Table 4.7.6.1.5-1 as appropriate and repeat steps 2-7.

4.7.6.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 4.7.6.1.4.3-1: Common Exception messages for EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 Table H.3.4-1a Table H.3.4-2 Table H.3.4-3 |

Table 4.7.6.1.4.3-2: MeasObjectToAddModList for EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-77 and TS 38.331 [13], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectCLI-r16 | MeasObjectCLI-r16 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.7.6.1.4.3-3: MeasObjectCLI-r16 for EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 and TS 38.331 [13], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectCLI-r16 ::= SEQUENCE { |  |  |  |
| cli-ResourceConfig-r16 SEQUENCE { |  |  |  |
| srs-ResourceConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| SRS-ResourceListConfigCLI-r16 SEQUENCE { | 1 entry |  |  |
| srs-Resource-r16 | SRSConf.1 | entry 1 | Config 1,3 |
|  | SRSConf.2 | entry 1 | Config 2,4 |
| srs-SCS-r16 | kHz15 |  | Config 1,3 |
|  | kHz30 |  | Config 2,4 |
| refServCellIndex-r16 | 0 | PSCell |  |
| refBWP-r16 | 0 | BWP-0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.7.6.1.4.3-4: ReportConfigNR for EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 and TS 38.331 [13], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| cli-Periodical-r16 SEQUENCE { |  |  |  |
| reportInterval-r16 | ms240 |  |  |
| reportAmount-r16 | infinity |  |  |
| reportQuantityCLI-r16 | srs-rsrp |  |  |
| maxReportCLI-r16 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.7.6.1.4.3-5: MeasResultCLI-r16 for EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 and TS 38.331 [13], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultCLI-r16 ::= SEQUENCE { |  |  |  |
| measResultsListSRS-r16 CHOICE { |  |  |  |
| srs-ResourceId-r16 | SRS-ResourceId |  |  |
| srs-RSRP-Result-r16 | SRS-RSRP-Range-r16 | INTEGER (0..98) |  |
| } |  |  |  |
| } |  |  |  |

4.7.6.1.5	Test requirement

Table 4.7.6.1.5-1 and Table 4.7.6.1.5-2 define the primary level settings including test tolerances for EN-DC SRS-RSRP measurement accuracy with FR1 serving cell. Table 4.7.6.1.5-3 defines the SRS resource configurations. Table 4.7.6.1.5-4 and Table 4.7.6.1.5-5 define the absolute accuracy requirements for Tests 1, 2 and 3 for configurations (1 and 3) and (2 and 4), correspondingly.

Table 4.7.6.1.5-1: NR Cell specific test parameters for EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~4 |  | freq1 | freq1 | freq1 |
| Duplex mode |  | 1~4 |  | TDD | TDD | TDD |
| TDD configuration |  | 1,3 |  | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 2,4 |  | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1,3 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2,4 |  | 40: NRB,c = 106 | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference |  | 1,3 |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
| measurement channel |  | 2,4 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference |  | 1,3 |  | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD |
| Channel |  | 2,4 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET |  | 1,3 |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
| Reference Channel |  | 2,4 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1,3 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2,4 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~4 |  | OP.1 | OP.1 | OP.1 |
| TRS configuration |  | 1,3 |  | TRS.1.1 TDD | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 2,4 |  | TRS.1.2 TDD | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~4 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~4 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~4 |  | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system |  | 1~4 | s | 17.67 | 17.67 | 17.67 |
| EPRE ratio of PSS to SSS |  | 1~4 | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A Note3 | 1,3 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A Note3 | 2,4 |  | Not applicableNote4 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A Note3 | 1,3 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A Note3 | 2,4 |  | Not applicableNote4 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 4: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table 4.7.6.1.5-2: Neighbor UE specific test parameters for EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1,3 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2,4 |  | Not applicableNote 6 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1,3 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2,4 |  | Not applicableNote 6 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| on SRS | NR_TDD_FR1_A NOTE 5 | 1,3 | dB | 1.5 | 1.5 | 1.75 |
|  | NR_TDD_FR1_C |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_D |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_E |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_A NOTE 5 | 2,4 |  | Not applicableNote 6 | 1.5 | 2.25 |
|  | NR_TDD_FR1_C |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_D |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_E |  |  |  |  | 1.5 |
| SRS RSRP Note3 | NR_TDD_FR1_A NOTE 5 | 1,3 | dBm/SRS SCS | -104.5 | -86.5 | -112.25 |
|  | NR_TDD_FR1_C |  |  |  |  | -111.5 |
|  | NR_TDD_FR1_D |  |  |  |  | -111 |
|  | NR_TDD_FR1_E |  |  |  |  | -110.5 |
|  | NR_TDD_FR1_A NOTE 5 | 2,4 |  | Not applicableNote 6 | -86.49 | -108.74 |
|  | NR_TDD_FR1_C |  |  |  |  | -108.49 |
|  | NR_TDD_FR1_D |  |  |  |  | -107.99 |
|  | NR_TDD_FR1_E |  |  |  |  | -107.49 |
| Io Note3 | NR_TDD_FR1_A NOTE 5 | 1,3 | dBm/9.36 MHz | -74.42 | -56.42 | -82.28 |
|  | NR_TDD_FR1_C |  |  |  |  | -81.42 |
|  | NR_TDD_FR1_D |  |  |  |  | -80.92 |
|  | NR_TDD_FR1_E |  |  |  |  | -80.42 |
|  | NR_TDD_FR1_A NOTE 5 | 2,4 | dBm/38.16 MHz | Not applicableNote 6 | -54.80 | -77.49 |
|  | NR_TDD_FR1_C |  |  |  |  | -76.80 |
|  | NR_TDD_FR1_D |  |  |  |  | -76.30 |
|  | NR_TDD_FR1_E |  |  |  |  | -75.80 |
| on SRS | NR_TDD_FR1_A NOTE 5 | 1,3 | dB | 1.5 | 1.5 | 1.75 |
|  | NR_TDD_FR1_C |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_D |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_E |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_A NOTE 5 | 2,4 |  | Not applicableNote 6 | 1.5 | 2.25 |
|  | NR_TDD_FR1_C |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_D |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_E |  |  |  |  | 1.5 |
| Propagation condition |  | 1,2,3,4 |  | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1,2,3,4 |  | 1x2 | 1x2 | 1x2 |
| SRS configuration |  | 1,3 |  | SRSConf.1 | SRSConf.1 | SRSConf.1 |
|  |  | 2,4 |  | SRSConf.2 | SRSConf.2 | SRSConf.2 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of the test. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 6: Test 1 is not used when testing with 30kHz SSB SCS Note 7: For Test 3, values are specified based on NR_TDD_FR1_X but they apply also to the FDD band group counterpart, NR_FDD_FR1_X. Informatively, test parameters are defined based on band group A and then add ΔBG_offset , which is the band group dependent component defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |

Table 4.7.6.1.5-3: SRS configuration for EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

|  | Field | SRSConf.1 | SRSConf.2 |
| --- | --- | --- | --- |
| SRS- | srs-ResourceSetId | 0 | 0 |
| ResourceSet | srs-ResourceIdList | 0 | 0 |
|  | resourceType | Periodic | Periodic |
|  | Usage | Codebook | Codebook |
| SRS- | SRS-ResourceId | 0 | 0 |
| Resource | nrofSRS-Ports | Port1 | Port1 |
|  | transmissionComb | n2 | n2 |
|  | combOffset-n2 | 0 | 0 |
|  | cyclicShift-n2 | 0 | 0 |
|  | resourceMapping startPosition | 0 | 0 |
|  | resourceMapping nrofSymbols | n1 | n1 |
|  | resourceMapping repetitionFactor | n1 | n1 |
|  | freqDomainPosition | 0 | 0 |
|  | freqDomainShift | 0 | 0 |
|  | freqHopping c-SRS | 12 | 12 |
|  | freqHopping b-SRS | 0 | 0 |
|  | freqHopping b-hop | 0 | 0 |
|  | groupOrSequenceHopping | Neither | Neither |
|  | resourceType | Periodic | Periodic |
|  | periodicityAndOffset-p | sl20, 9 | sl40, 19 |
|  | sequenceId | 0 | 0 |

Table 4.7.6.1.5-4: Absolute accuracy requirements for the reported values for test configurations 1 and 3 of EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest SRS-RSRP reported value | 30 | 45 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 23 |
|  |  |  | Bands NR_TDD_FR1_C | 23 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 24 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 24 |
| Highest SRS-RSRP reported value | 41 | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  |  | Bands NR_TDD_FR1_C | 34 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 34 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 35 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |  |

Table 4.7.6.1.5-5: Absolute accuracy requirements for the reported values for test configurations 2 and 4 of EN-DC SRS-RSRP measurement accuracy with FR1 serving cell

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest SRS-RSRP reported value | N/A Note 2 | 44 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 26 |
|  |  |  | Bands NR_TDD_FR1_C | 26 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 26 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 27 |
| Highest SRS-RSRP reported value | N/A Note 2 | 63 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 37 |
|  |  |  | Bands NR_TDD_FR1_C | 37 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 38 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 38 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 Note 2: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

<< Skipped sections >>

### 6.7.8	CLI Measurements

#### 6.7.8.0	Minimum conformance requirements

##### 6.7.8.0.1	Minimum conformance requirements for SRS-RSRP accuracy

Same as in clause 4.7.6.0.1.

##### 6.7.8.0.2	Minimum conformance requirements for CLI-RSSI measurement accuracy with FR1 serving cell

Same as in clause 4.7.6.0.2.

#### 6.7.8.1	NR SA FR1 SRS-RSRP measurement accuracy

##### 6.7.8.1.1	Test purpose

The purpose of this test is to verify that the SRS-RSRP measurement accuracy is within the specified limits. This test will verify the SRS-RSRP measurement requirements in clause 6.7.8.0.

##### 6.7.8.1.2	Test applicability

This test applies to all types of NR UE release 16 and forward, supporting standalone NR and CLI-based SRS-RSRP measurements.

##### 6.7.8.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.7.8.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.7.8.1.

##### 6.7.8.1.4	Test description

##### In this set of test cases there is one cell in the test, FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in Table 6.7.8.1.5-1 below. The test parameter for the (virtual) neighbor cell UE transmitting SRS are given in Table 6.7.8.1.5-2. The SRS resource configuration is given in Table 6.7.8.1.5-3.

6.7.8.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.7.8.1.4.1-1.

Table 6.7.8.1.4.1-1: Supported test configurations for NR SA FR1 SRS-RSRP measurement accuracy

| Config | Description |
| --- | --- |
| 6.7.8.1-1 | 15kHz SRS SCS, 10 MHz bandwidth, TDD duplex mode |
| 6.7.8.1-2 | 30kHz SRS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Configure the test equipment and the DUT according to the parameters in Table 6.7.8.1.4.1-2.

Table 6.7.8.1.4.1-2: Initial conditions for NR SA FR1 SRS-RSRP measurement accuracy

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.7.8.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 2 and φ1 = 5 Hz | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 2 and φ1,1 = 5 Hz, φ1,2 = 10 Hz, φ1,3 = 15 Hz |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | - Without LTE link |  |  |

1.	Message contents are defined in clause 6.7.8.1.4.3.

2.	Cell 1 is the NR serving cell (PCell). The power levels and settings for Cell 1 are set according to Annex C.1.2 and C.1.3. Virtual UE 1 is the target for SRS-RSRP measurements.

3.	The test parameters are given in Table 6.7.8.1.4.1-3

6.7.8.1.4.2	Test procedure

Before the test UE is configured to perform SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the DL slots according to the SRS resource configuration in Table 6.7.8.1.5-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to the corresponding Test in Table 6.7.8.1.5-1 as appropriate.

3. The SS shall transmit an RRCReconfiguration message configuring a CLI measurement object with periodic reporting, as specified in section 6.7.8.1.4.3.

4. The UE shall transmit an RRCReconfigurationComplete message.

5. The UE shall transmit periodically MeasurementReport messages.

6. After 10s wait from Step 5, the SS shall check the SRS-RSRP reported values in the periodic MeasurementReport. The reported SRS-RSRP value of the neighbor virtual UE is compared to the expected SRS-RSRP as specified in Table 6.7.8.1.5-4 and Table 6.7.8.1.5-5 for Test 1 and Test 2, correspondingly. If the reported value is outside the limits specified in such tables, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7. The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8. Set the parameters according to each Test in Table 6.7.8.1.5-1 as appropriate and repeat steps 2-7.

6.7.8.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clauses 4.6.1 and 7.3 with the following exceptions:

Table 6.7.8.1.4.3-1: Common Exception messages for NR SA FR1 SRS-RSRP measurement accuracy

| Default Message Contents |  |  |
| --- | --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 Table H.3.1-5 |

Table 6.7.8.1.4.3-2: MeasObjectToAddModList for NR SA FR1 SRS-RSRP measurement accuracy

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-77 and TS 38.331 [13], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectToAddModList::= SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectCLI-r16 | MeasObjectCLI-r16 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.8.1.4.3-3: MeasObjectCLI-r16 for NR SA FR1 SRS-RSRP measurement accuracy

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 and TS 38.331 [13], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectCLI-r16 ::= SEQUENCE { |  |  |  |
| cli-ResourceConfig-r16 SEQUENCE { |  |  |  |
| srs-ResourceConfig-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| SRS-ResourceListConfigCLI-r16 SEQUENCE { | 1 entry |  |  |
| srs-Resource-r16 | SRSConf.1 | entry 1 | Config 1 |
|  | SRSConf.2 | entry 1 | Config 2 |
| srs-SCS-r16 | kHz15 |  | Config 1 |
|  | kHz30 |  | Config 2 |
| refServCellIndex-r16 | 0 | PCell |  |
| refBWP-r16 | 0 | BWP-0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.8.1.4.3-4: ReportConfigNR for NR SA FR1 SRS-RSRP measurement accuracy

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 and TS 38.331 [13], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| cli-Periodical-r16 SEQUENCE { |  |  |  |
| reportInterval-r16 | ms240 |  |  |
| reportAmount-r16 | infinity |  |  |
| reportQuantityCLI-r16 | srs-rsrp |  |  |
| maxReportCLI-r16 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.8.1.4.3-5: MeasResultCLI-r16 for NR SA FR1 SRS-RSRP measurement accuracy

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 and TS 38.331 [13], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResultCLI-r16 ::= SEQUENCE { |  |  |  |
| measResultsListSRS-r16 CHOICE { |  |  |  |
| srs-ResourceId-r16 | SRS-ResourceId |  |  |
| srs-RSRP-Result-r16 | SRS-RSRP-Range-r16 | INTEGER (0..98) |  |
| } |  |  |  |
| } |  |  |  |

##### 6.7.8.1.5	Test requirement

Table 6.7.8.1.5-1 and Table 6.7.8.1.5-2 define the primary level settings including test tolerances for NR SA FR1 SRS-RSRP measurement accuracy. Table 6.7.8.1.5-3 defines the SRS resource configurations. Table 6.7.8.1.5-4 and Table 6.7.8.1.5-5 define the absolute accuracy requirements for Tests 1, 2 and 3 for configurations 1 and 2, correspondingly.

Table 6.7.8.1.5-1: NR Cell specific test parameters for NR SA FR1 SRS-RSRP measurement accuracy

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~2 |  | freq1 | freq1 | freq1 |
| Duplex mode |  | 1~2 |  | TDD | TDD | TDD |
| TDD configuration |  | 1 |  | TDDConf.1.1 | TDDConf.1.1 | TDDConf.1.1 |
|  |  | 2 |  | TDDConf.2.1 | TDDConf.2.1 | TDDConf.2.1 |
| BWchannel |  | 1 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 | 10: NRB,c = 52 |
|  |  | 2 |  | 40: NRB,c = 106 | 40: NRB,c = 106 | 40: NRB,c = 106 |
| PDSCH Reference measurement channel |  | 1 |  | SR.1.1 TDD | SR.1.1 TDD | SR.1.1 TDD |
|  |  | 2 |  | SR.2.1 TDD | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.1.1 TDD | CR.1.1 TDD | CR.1.1 TDD |
|  |  | 2 |  | CR.2.1 TDD | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.1.1 TDD | CCR.1.1 TDD | CCR.1.1 TDD |
|  |  | 2 |  | CCR.2.1 TDD | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 |
|  |  | 2 |  | SSB.2 FR1 | SSB.2 FR1 | SSB.2 FR1 |
| OCNG Patterns |  | 1~2 |  | OP.1 | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.1 TDD | TRS.1.1 TDD | TRS.1.1 TDD |
|  |  | 2 |  | TRS.1.2 TDD | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1~2 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1~2 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1~2 |  | SMTC.1 | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system |  | 1~2 | s | 17.67 | 17.67 | 17.67 |
| EPRE ratio of PSS to SSS |  | 1~2 | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 4 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 3 | 2 |  | Not applicableNote 4 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  | -109 |
| Note 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 4: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

Table 6.7.8.1.5-2: Neighbor UE specific test parameters for NR SA FR1 SRS-RSRP measurement accuracy

| Parameter |  | Config | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- | --- |
| Note2 | NR_TDD_FR1_A NOTE 5 |  | 1 | dBm/15kHz | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 |  | 2 |  | Not applicableNote 6 | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 5 |  | 1 | dBm/SRS SCS | -106 | -88 | -114 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  |  |  | -112 |
|  | NR_TDD_FR1_A NOTE 5 |  | 2 |  | Not applicableNote 6 | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  |  |  | -109 |
| on SRS | NR_TDD_FR1_A NOTE 5 | 1 | dB | 1.5 | 1.5 | 1.75 |
|  | NR_TDD_FR1_C |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_D |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_E |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | 1.5 | 2.25 |
|  | NR_TDD_FR1_C |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_D |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_E |  |  |  |  | 1.5 |
| SRS RSRP Note3 | NR_TDD_FR1_A NOTE 5 |  | 1 | dBm/SRS SCS | -104.5 | -86.5 | -112.25 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -111.5 |
|  | NR_TDD_FR1_D |  |  |  |  |  | -111 |
|  | NR_TDD_FR1_E |  |  |  |  |  | -110.5 |
|  | NR_TDD_FR1_A NOTE 5 |  | 2 |  | Not applicableNote 6 | -86.49 | -108.74 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -108.49 |
|  | NR_TDD_FR1_D |  |  |  |  |  | -107.99 |
|  | NR_TDD_FR1_E |  |  |  |  |  | -107.49 |
| Io Note3 | NR_TDD_FR1_A NOTE 5 |  | 1 | dBm/9.36 MHz | -74.42 | -56.42 | -82.28 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -81.42 |
|  | NR_TDD_FR1_D |  |  |  |  |  | -80.92 |
|  | NR_TDD_FR1_E |  |  |  |  |  | -80.42 |
|  | NR_TDD_FR1_A NOTE 5 |  | 2 | dBm/38.16 MHz | Not applicableNote 6 | -54.80 | -77.49 |
|  | NR_TDD_FR1_C |  |  |  |  |  | -76.80 |
|  | NR_TDD_FR1_D |  |  |  |  |  | -76.30 |
|  | NR_TDD_FR1_E |  |  |  |  |  | -75.80 |
| on SRS | NR_TDD_FR1_A NOTE 5 | 1 | dB | 1.5 | 1.5 | 1.75 |
|  | NR_TDD_FR1_C |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_D |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_E |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_A NOTE 5 | 2 |  | Not applicableNote 6 | 1.5 | 2.25 |
|  | NR_TDD_FR1_C |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_D |  |  |  |  | 1.5 |
|  | NR_TDD_FR1_E |  |  |  |  | 1.5 |
| Propagation condition |  | 1~2 |  | AWGN | AWGN | AWGN |
| Antenna configuration |  | 1~2 |  | 1x2 | 1x2 | 1x2 |
| SRS configuration |  | 1 |  | SRSConf.1 | SRSConf.1 | SRSConf.1 |
|  |  | 2 |  | SRSConf.2 | SRSConf.2 | SRSConf.2 |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of the test. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification Note 6: Test 1 is not used when testing with 30kHz SSB SCS Note 7: For Test 3, values are specified based on NR_TDD_FR1_X but they apply also to the FDD band group counterpart, NR_FDD_FR1_X. Informatively, test parameters are defined based on band group A and then add ΔBG_offset , which is the band group dependent component defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |  |  |  |

Table 6.7.8.1.5-3: SRS configurations for NR SA FR1 SRS-RSRP measurement accuracy

|  | Field | SRSConf.1 | SRSConf.2 |
| --- | --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 | 0 |
|  | srs-ResourceIdList | 0 | 0 |
|  | resourceType | Periodic | Periodic |
|  | Usage | Codebook | Codebook |
| SRS-Resource | SRS-ResourceId | 0 | 0 |
|  | nrofSRS-Ports | Port1 | Port1 |
|  | transmissionComb | n2 | n2 |
|  | combOffset-n2 | 0 | 0 |
|  | cyclicShift-n2 | 0 | 0 |
|  | resourceMapping startPosition | 0 | 0 |
|  | resourceMapping nrofSymbols | n1 | n1 |
|  | resourceMapping repetitionFactor | n1 | n1 |
|  | freqDomainPosition | 0 | 0 |
|  | freqDomainShift | 0 | 0 |
|  | freqHopping c-SRS | 12 | 12 |
|  | freqHopping b-SRS | 0 | 0 |
|  | freqHopping b-hop | 0 | 0 |
|  | groupOrSequenceHopping | Neither | Neither |
|  | resourceType | Periodic | Periodic |
|  | periodicityAndOffset-p | sl20, 9 | sl40, 19 |
|  | sequenceId | 0 | 0 |

Table 6.7.8.1.5-4: Absolute accuracy requirements for the reported values for test configuration 1 of NR SA FR1 SRS-RSRP measurement accuracy

| Normal Conditions | Test 1 All bands | Test 2 All bands | Test 3 |  |
| --- | --- | --- | --- | --- |
| Lowest SRS-RSRP reported value | 30 | 45 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 23 |
|  |  |  | Bands NR_TDD_FR1_C | 23 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 24 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 24 |
| Highest SRS-RSRP reported value | 41 | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  |  | Bands NR_TDD_FR1_C | 34 |
|  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 34 |
|  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 35 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |  |

Table 6.7.8.1.5-5: Absolute accuracy requirements for the reported values for test configuration 2 of NR SA FR1 SRS-RSRP measurement accuracy

| Normal Conditions | Test 1 All bands |  | Test 2 All bands |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Lowest SRS-RSRP reported value |  | N/A Note 2 | 44 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A |  | 26 |
|  |  |  |  | Bands NR_TDD_FR1_C |  | 26 |
|  |  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D |  | 26 |
|  |  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E |  | 27 |
| Highest SRS-RSRP reported value |  | N/A Note 2 | 63 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A |  | 37 |
|  |  |  |  | Bands NR_TDD_FR1_C |  | 37 |
|  |  |  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D |  | 38 |
|  |  |  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E |  | 38 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 Note 2: Test 1 is not used when testing with 30kHz SSB SCS |  |  |  |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

### 6.7.9	L1-SINR measurement for beam reporting

<< End of Changes >>