3GPP TSG WG RAN5 meeting#99					R5-232687r1

Incheon, South Korea, 22nd May 2023 - 26th May 2023

Source:		Qualcomm Incorporated

Title:	Discussion to add coverage for the TEI15 feature - "intraFreqReselection” field in MIB message is set to "not allowed"

Document for:	Discussion and Decision

Agenda Item:	6.4.7

# Introduction

The Idle mode section of the TS38.523-1[2] does not cover the conformance requirements from TS38.304[1], clause 5.3.1, to check the UE behaviour when the field “intraFreqReselection” in MIB message is set to "not allowed”.

### [TS 38.304]

### 5.3.1	Cell status and cell reservations

[3GPP Release 15, TS 38.304-f80, clause 5.3.1]

…

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	else:

-	If the field intraFreqReselection in MIB message is set to "allowed":

-	the UE may select another cell on the same frequency if re-selection criteria are fulfilled;

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	else:

-	the UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "not allowed":

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

-	the UE shall not re-select to another cell on the same frequency as the barred cell and exclude such cell(s) as candidate(s) for cell selection/reselection for 300 seconds;

-	else:

-	the UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

NOTE 2:	If barring of a cell is triggered by the condition of trackingAreaCode not being provided, as specified in TS 38.331 [3], the barring only applies to this PLMN and the UE can re-evaluate the barring condition again due to selection of another PLMN.

The cell selection of another cell may also include a change of RAT.

# 2.	Discussion

To cover the Spec requirement described in section 1, a new test case can be defined as part of the Idle mode section “6.1.2 NG-RAN Only Cell Selection” or the existing test prose of “6.1.2.3 Cell selection / Intra NR / Serving cell becomes non-suitable (S<0, MIB Indicated barred)” can be updated to add an additional TP to cover the required conformance requirements w.r.t the UE behaviour when the field “intraFreqReselection” in MIB message is set to "not allowed”.

# 3.	Proposal

1) It is proposed that a testcase be added in TS38.523-1 as TC 6.1.2.3a. Contributions in R5-232688r3 address the new prose addition and R5-232689 addresses the applicability.

2) It is proposed that the existing testcase 6.1.2.3 be updated to add an additional TP (3) as below:

#### 6.1.2.3a	Cell selection / Intra NR / Serving cell becomes non-suitable (S<0, MIB Indicated barred)

6.1.2.3a.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state }

ensure that {

when { the serving cell becomes barred and broadcasting intraFreqReselection in MIB message set to "not allowed" and UE detects both intra-frequency and equal priority inter-frequency neighbour cells and the intra-frequency cell is the highest ranked cell  }

then { UE reselects the inter-frequency cell }

}

# 4.	References

[1] 	TS38.304	User Equipment (UE) procedures in Idle mode and RRC Inactive state, (Release 15).

[2]	TS38.523-1: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; 5GS; User Equipment (UE) conformance specification; Part 1: Protocol