3GPP TSG-RAN5 Meeting #99	R5-232763r1

# Incheon, South Korea, 22nd – 26th May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0537 | rev | - | Current version: | 16.6.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to test case 8.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To apply default EVS configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test case 8.6 has been updated to use the generic procedure A.5.1a |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The aligment to use default EVS configuration cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: updated the Tdoc number on the cover page. |  |  |  |  |  |  |  |  |

## 8.6	Terminating Identification Restriction / Signalling / 5GS

8.6.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and TIR in temporary mode being activated }

ensure that {

when { UE receives INVITE for voice call }

then { UE includes the Privacy header in its 183 Session Progress response and in its 180 Ringing response and in its final 200 OK for INVITE response }

}

8.6.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.608 clause 4.2.1]:

The Terminating Identification Restriction (TIR) is a service offered to the terminating party which enables the terminating party to prevent presentation of the terminating identity information to originating party.

[TS 24.608 clause 4.5.2.12]:

The destination UE, if the terminating user wishes to override the default setting of "presentation not restricted" of the TIR service in temporary mode, shall include a Privacy header with privacy type of "id" in any non-100 responses it sends upon receipt of a SIP request.

….NOTE:	It is assumed that TIR subscribers support IETF RFC 3325.

8.6.3	Test description

8.6.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is configured to use preconditions.

-	The UE is configured to use Terminating Identification Restriction in temporary mode.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS

8.6.3.2	Test procedure sequence

Table 8.6.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 9-10 | Steps 1-2 of Annex A.5.1a are performed. | - | - | - | - |
| 11 | Check: Does the UE send 183 Session Progress response reliably, including a Privacy header with privacy type of "id"? (Steps 3 of Annex A.5.1a)) | --> | 183 Session Progress | 1 | P |
| 12-13 | Steps 4-5 of Annex A.5.1a are performed. | - | - | - | - |
| 13A-13C | SS triggers resource reservation: Steps 10-12 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 14-15 | Step 6-7 of Annex A.5.1a are performed | - | - | - | - |
| 16 | Check: Does the UE send 180 Ringing including a Privacy header with privacy type of "id"? (Steps 8 of Annex A.5.1a) | --> | 180 Ringing | 1 | P |
| 17-19 | Steps 9-10A of Annex A.5.1a are performed. | - | - | - | - |
| 20 | Check: Does the UE send 200 OK including a Privacy header with privacy type of "id"? (Steps 11 of Annex A.5.1a) | --> | 200 OK | 1 | P |
| 21 | Steps 12 of Annex A.5.1a is performed. | <-- | ACK | - | - |

8.6.3.3	Specific message contents

Table 8.6.3.3-1: 183 Session Progress (step 11, Table 8.6.3.2-1)

| Derivation Path: Step 3 of A.5.1a |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Privacy |  | Id |  | RFC 3323[50] RFC 3325[51] |

Table 8.6.3.3-2: 180 Ringing (step 16, Table 8.6.3.2-1)

| Derivation Path: Step 8 of A.5.1a |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Privacy |  | Id |  | RFC 3323[50] RFC 3325[51] |

Table 8.6.3.3-3: 200 OK (step 11, Table 8.6.3.2-1)

| Derivation Path: Step 11 of A.5.1a |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Privacy |  | id |  | RFC 3323[50] RFC 3325[51] |