3GPP TSG-RAN5 Meeting #99	R5-232853r1

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0675 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Power Saving test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Europe Inc. Sweden |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Change the value of drx-onDurationTimer for FR1 tests to meet the MNS requirements in Table G.1.5-2. Test procedure is not clear regarding scheduling of PDSCH in DRX on period. Ps-Offset value is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addtion of DRX adaptation to Clause 5.1.1.3 - Update ps-Offset-r16 value for 5.3.2.1.3, 5.3.2.2.3, 5.3.3.1.3, 5.3.3.2.3 based on highest MinTimeGap value (sl3 for 15KHz and sl6 for 30KHz). - Update Test procedure sections 5.3.2.1.3.4.2, 5.3.3.1.3.4.2 for FDD tests 5.3.2.1.3 and 5.3.3.1.3 to align with sections 5.3.2.2.3.4.2, 5.3.3.2.3.4.2 of TDD tests 5.3.2.2.3 and 5.3.3.2.3 to ensure PDCCH DCI 1_1 is only scheduled in DRX on period. Change C_RNTI to PS_RNTI in Step 1. - Updated titles for Tables 5.3.2.2.3.3-2, 5.3.2.2.3.5-1, 5.3.3.2.3.3-2, 5.3.3.2.3.5-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case implementation will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.3, 5.3.2.1.3, 5.3.2.2.3, 5.3.3.1.3, 5.3.3.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Changed drx-InactivityTimer value from ms1 to ms0. Changed drx-onDurationTimer from ms1 to ms5. Added note to Tables A.3.3.1.1-3 and A.3.3.2.2-3 to clarify scheduling of PDCCH and PDSCH with DRX configured. |  |  |  |  |  |  |  |  |

<< Start of Changes >>

# 5	Demodulation performance requirements (Conducted requirements)

## 5.1	General

### 5.1.1	Applicability of requirements

#### 5.1.1.1	General

The minimum performance requirements are applicable to all FR1 operating bands defined in TS 38.101-1 [2].

The minimum performance requirements in Clause 5 are mandatory for UE supporting NR operation, except test cases listed in Clauses 5.1.1.3, 5.1.1.4.

#### 5.1.1.2	Applicability of requirements for different number of RX antenna ports

The number of RX antenna ports for different RF operating bands is up to UE declaration.

The UE shall support 2 or 4 RX antenna ports for different RF operating bands. The operating bands, where 4 RX antenna ports shall be the baseline, are defined in Clause 7.2 of TS 38.101-1 [3]. The UE requirements applicability for UEs with different number of RX antenna ports is defined in Table 5.1.1.2-1.

Table 5.1.1.2-1: Requirements applicability

| Supported RX antenna ports | Test type | Test list |
| --- | --- | --- |
| UE supports only 2RX | PDSCH | All tests in Clause 5.2.2 |
|  | PDCCH | All tests in Clause 5.3.2 |
|  | PBCH | All tests in Clause 5.4.2 |
| UE supports only 4RX or both 2RX and 4RX | PDSCH | All tests in Clause 5.2.3 |
|  | PDCCH | All tests in Clause 5.3.3 |
|  | PBCH | All tests in Clause 5.4.2 or 5.4.3 Note 1 |
| Note 1: Requirements for PBCH with 4Rx is up to UE declaration |  |  |

#### 5.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1) |  |
| Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 (Test 1-2) Clause 5.2.3.2.4 (Test 1-2) |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 5.5A.1 | 1) Up to 16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |
| Enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h | FR1 FDD | PDSCH | Clause 5.2.2.1.9 (Test 1-1) Clause 5.2.3.1.9 (Test 1-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.9 (Test 1-1) Clause 5.2.3.2.9 (Test 1-1) |  |
| Single DCI based SDM transmission for multi-TRxP (singleDCI-SDM-scheme-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.11 Clause 5.2.3.1.11 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.11 Clause 5.2.3.2.11 |  |
| Multi DCI based multi-TRxP support (multiDCI-MultiTRP-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.12 Clause 5.2.3.1.12 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.12 Clause 5.2.3.2.12 |  |
| Single DCI based FDM Scheme-A for multi-TRxP(supportFDM-SchemeA-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.13 Clause 5.2.3.1.13 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.13 Clause 5.2.3.2.13 |  |
| Single DCI based inter-slot TDM for multi-TRxP (supportInter-slotTDM-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |
| Maximum number of TCI states in Single-DCI based inter-slot TDM (maxNumberTCI-states-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 | The requirements apply only when maxNumberTCI-states-r16 = 2. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |
| Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |
| CQI table with target BLER of 10^-5New CQI table (cqi-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 |  |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR1 FDD | PDSCH | Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |
| UE PDSCH processing capability #2 (pdsch-ProcessingType2) | FR1 FDD | PDSCH | Clause 5.2.2.1.7 Clause 5.2.3.1.7 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.7 Clause 5.2.3.2.7 |  |
| Pre-emption indication for DL (pre-EmptIndication-DL) | FR1 FDD | PDSCH | Clause 5.2.2.1.8 Clause 5.2.3.1.8 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.8 Clause 5.2.3.2.8 |  |
| DRX Adaptation (drx-adaptation-r16) | FR1 FDD | PDCCH | Clause 5.3.2.1.3 | If the Test 1 in Clause 5.3.2.1.3 is passed, the test coverage can be considered fulfilled without executing Test 3 in clause 5.3.2.1.1. |
|  | FR1 TDD | PDCCH | Clause 5.3.2.2.3 | If the Test 1 in Clause 5.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 2 in clause 5.3.2.2.1. |
|  | FR1 FDD | PDCCH | Clause 5.3.3.1.3 | If the Test 1 in Clause 5.3.3.1.3 is passed, the test coverage can be considered fulfilled without executing Test 3 in clause 5.3.3.1.1. |
|  | FR1 TDD | PDCCH | Clause 5.3.3.2.3 | If the Test 1 in Clause 5.3.3.2.3 is passed, the test coverage can be considered fulfilled without executing Test 2 in clause 5.3.3.2.1. |

<< Sections skipped >>

## 5.3	PDCCH demodulation requirements

<< Sections skipped >>

### 5.3.2	2RX requirements

#### 5.3.2.1	FDD

<< Sections skipped >>

##### 5.3.2.1.3	2Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving

5.3.2.1.3.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.1.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.1.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting Long DRX and DRX adaptation

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and Long DRX and DRX adaptation.

5.3.2.1.3.3	Minimum conformance requirements

The parameters specified in Table 5.3.2.1.3.3-1 are valid for FDD test unless otherwise stated.

Table 5.3.2.1.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  |  | nonInterleaved |
| REG bundle size |  |  | 6 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2μ/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

For the parameters specified in Table 5.3.2.1.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.1.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.2.1.3.3-2: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 5.5 |
|  |  |  | 2 | 8 | R.PDCCH. 1-2.7 FDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.2.1.3.

5.3.2.1.3.4	Test description

5.3.2.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.2.1-1, Table 5.3.2.1.3.3-1 and Table 5.3.2.1.3.3-2 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.2.1.3.4.3.

5.3.2.1.3.4.2	Test procedure

1.	SS transmits PDCCH with DCI format2_6 as specified in PDCCH Reference Channel for PS_RNTI within DRX off state. The Wake-up indication bit in PDCCH is set to 1.

2.  SS transmits PDSCH via PDCCH with DCI format1_1 as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.2.1.3.3-2 in DRX on period. The details of PDCCH are specified in Table 5.3.1, Table 5.3.2.1-1, Table 5.3.2.1.3.3-1 and Table 5.3.2.1.3.3-2 respectively. The details of PDSCH are specified in Table A.3.3.1.1-3. The SS sends downlink MAC padding bits on the DL RMC. During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

3.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.2.1.3.4.4-1as appropriate.

4.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.2.1.3.4.4-1, pass the UE. Otherwise fail the UE.

5.	Repeat steps from 2 to 4 for each subtest in Table 5.3.2.1.3.3-2 as appropriate.

5.3.2.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.2.1.3.4.3.1	Message exceptions for SA

Table 5.3.2.1.3.4.3.1-1: DRX-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-56 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms5 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms0 |  |  |
| drx-HARQ-RTT-TimerDL | 0 |  |  |
| drx-HARQ-RTT-TimerUL | 0 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms10 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.3.4.3.1-2: DCP-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 32 |  | Scs-15kHz-r16 = sl3 |
| sizeDCI-2-6-r16 | 2 |  |  |
| ps-PositionDCI-2-6-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.1.3.4.3.1-3: PDCCH-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SEQUENCE(SIZE (1..3)) OF ControlResourceSet ::= SEQUENCE { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet |  |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace ::= SEQUENCE { | 2 entries |  |  |
| SearchSpace[1] | SearchSpace1 |  |  |
| SearchSpace[2] | SearchSpace2 |  |  |
| } |  |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| searchSpaceExt-r16[1] | SearchSpaceExt1 |  |  |
| searchSpaceExt-r16[2] | SearchSpaceExt2 |  |  |
| } |  |  |  |
| } |  |  |  |

<< Sections skipped >>

#### 5.3.2.2	TDD

<< Sections skipped >>

##### 5.3.2.2.3	2Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving

5.3.2.2.3.1	Test Purpose

This test verifies the demodulation performance of DCI format 2_6 PDCCH under 2 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.2.2.3.3-2 after receipt wake-up indication in the DCI format 2_6 PDCCH in DRX off state. The downlink physical setup is in accordance with Annex C.2.1.

5.3.2.2.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting Long DRX Cycle and DRX adaptation.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and Long DRX Cycle and DRX adaptation.

5.3.2.2.3.3	Minimum conformance requirements

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

The parameters specified in Table 5.3.2.2.3.3-1 are valid for all TDD tests for power saving unless otherwise stated.

Table 5.3.2.2.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.30-1 |
| CCE to REG mapping type |  |  | interleaved |
| Interleaver size |  |  | 3 |
| REG bundle size |  |  | 2 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2μ/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| PDCCH configuration | Slots for PDCCH monitoring |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability.. |

For the parameters specified in Table 5.3.2.2.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.2.2.3.3-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.2.2.3.3-2: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.0 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.2.2.

5.3.2.2.3.4	Test description

5.3.2.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of DRX, DCP, PDSCH and PDCCH before measurement are specified in 5.3.2.2.3.4.3.1and Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.2 for TE diagram and section A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.2.2-1 and Table 5.3.2.2.3.3-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.2.2.3.4.3.

5.3.2.2.3.4.2	Test procedure

1.	SS transmits PDCCH with DCI format2_6 as specified in PDCCH Reference Channel for PS_RNTI within DRX off state. The Wake-up indication bit in PDCCH is set to 1.

2.	SS transmits PDSCH via PDCCH with DCI format1_1 as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.2.2.3.3-2 in DRX on period. The details of PDCCH are specified in Table 5.3-1, Table 5.3.2.2-1 and Table 5.3.2.2.3.3-2 respectively. The details of PDSCH are specified in Table A.3.3.2.2-3. The SS sends downlink MAC padding bits on the DL RMC.

3.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to  Table 5.3.2.2.3.5-1 as appropriate.

4.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.2.2.3.5-1, pass the UE. Otherwise fail the UE.

5.	Repeat steps from 2 to 4 for each subtest in Table 5.3.2.2.3.3-2 as appropriate.

5.3.2.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 and 5.4.2.

5.3.2.2.3.4.3.1	Message exceptions for SA

Table 5.3.2.2.3.4.3.1-1: DRX-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-56 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms5 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms0 |  |  |
| drx-HARQ-RTT-TimerDL | 0 |  |  |
| drx-HARQ-RTT-TimerUL | 0 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms10 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.3.4.3.1-2: DCP-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 28 |  | Scs-30kHz-r16 = sl6 |
| sizeDCI-2-6-r16 | 2 |  |  |
| ps-PositionDCI-2-6-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.2.2.3.4.3.1-3: PDCCH-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SEQUENCE(SIZE (1..3)) OF ControlResourceSet ::= SEQUENCE { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet |  |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace ::= SEQUENCE { | 2 entries |  |  |
| SearchSpace[1] | SearchSpace1 |  |  |
| SearchSpace[2] | SearchSpace2 |  |  |
| } |  |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| searchSpaceExt-r16[1] | SearchSpaceExt1 |  |  |
| searchSpaceExt-r16[2] | SearchSpaceExt2 |  |  |
| } |  |  |  |
| } |  |  |  |

<< Sections skipped >>

5.3.2.2.3.4.3.2	Message exceptions for NSA

Same as 5.3.2.2.3.4.3.1.

5.3.2.2.3.5	Test requirement

Table 5.3.2.2.3.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.2.2.3.5-1.

Table 5.3.2.2.3.5-1: Test Requirements for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x2 Low | 1 | 3.9 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

### 5.3.3	4RX requirements

#### 5.3.3.1	FDD

<< Sections skipped >>

##### 5.3.3.1.3	4Rx FDD FR1 PDCCH 1 Tx antenna performance for power saving

5.3.3.1.3.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.1.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.1.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward that supporting 4 Rx antenna ports and DRX adaptation.

This test also applies to all types of EUTRA UE release 16 and forward that supporting EN-DC, 4 Rx antenna ports, Long DRX and DRX adaptation.

5.3.3.1.3.3	Minimum conformance requirements

The parameters specified in Table 5.3.3.1.3.3-1 are valid for FDD test unless otherwise stated.

Table 5.3.3.1.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| CCE to REG mapping type |  |  | nonInterleaved |
| REG bundle size |  |  | 6 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2μ/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| Slots for PDCCH monitoring |  |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

For the parameters specified in Table 5.3.3.1.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.1.3.3-2. The downlink physical setup is in accordance with Annex C.2.1.

Table 5.3.3.1.3.3-2: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x4 Low | 1 | 0.2 |
|  |  |  | 2 | 8 | R.PDCCH. 1-2.7 FDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [2] clause 5.3.3.1.3.

5.3.3.1.3.4	Test description

5.3.3.1.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.3.1-1, Table 5.3.3.1.3.3-1 and Table 5.3.3.1.3.3-2 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.1.3.4.3.

5.3.3.1.3.4.2	Test procedure

1.	SS transmits PDCCH with DCI format2_6 as specified in PDCCH Reference Channel for PS_RNTI within DRX off state. The Wake-up indication bit in PDCCH is set to 1.

2.  SS transmits PDSCH with PDCCH with DCI format1_1 as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.3.1.3.3-2 in DRX on period. The details of PDCCH are specified in Table 5.3.1, Table 5.3.3.1-1, Table 5.3.3.1.3.3-1 and Table 5.3.3.1.3.3-2 respectively. The details of PDSCH are specified in Table A.3.3.1.1-3. The SS sends downlink MAC padding bits on the DL RMC. During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

3.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.1.3.4.4-1 as appropriate.

4.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.1.3.4.4-1, pass the UE. Otherwise fail the UE.

5.	Repeat steps from 2 to 4 for each subtest in Table 5.3.3.1.3.3-2 as appropriate.

5.3.3.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.3.3.1.3.4.3.1	Message exceptions for SA

Table 5.3.3.1.3.4.3.1-1: DRX-Config

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-56 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms5 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms0 |  |  |
| drx-HARQ-RTT-TimerDL | 0 |  |  |
| drx-HARQ-RTT-TimerUL | 0 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms10 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.3.4.3.1-2: DCP-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 32 |  | Scs-15kHz-r16 = sl3 |
| sizeDCI-2-6-r16 | 2 |  |  |
| ps-PositionDCI-2-6-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.1.3.4.3.1-3: PDCCH-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SEQUENCE(SIZE (1..3)) OF ControlResourceSet ::= SEQUENCE { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet |  |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace ::= SEQUENCE { | 2 entries |  |  |
| SearchSpace[1] | SearchSpace1 |  |  |
| SearchSpace[2] | SearchSpace2 |  |  |
| } |  |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| searchSpaceExt-r16[1] | SearchSpaceExt1 |  |  |
| searchSpaceExt-r16[2] | SearchSpaceExt2 |  |  |
| } |  |  |  |
| } |  |  |  |

<< Sections skipped >>

#### 5.3.3.2	TDD

<< Sections skipped >>

##### 5.3.3.2.3	4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving

5.3.3.2.3.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.2.3.3-2 after receipt wake-up indication in the DCI format 2_6 PDCCH in DRX off state. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.2.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting 4 Rx antenna ports and Long DRX Cycle and DRX adaptation.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and 4 Rx antenna ports and Long DRX Cycle and DRX adaptation.

5.3.3.2.3.3	Minimum conformance requirements

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

For the parameters specified in Table 5.3.3.2.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.3.2.3.3-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.3.2.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.30-1 |
| CCE to REG mapping type |  |  | interleaved |
| Interleaver size |  |  | 3 |
| REG bundle size |  |  | 2 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2μ/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| Slots for PDCCH monitoring |  |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signalled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

Table 5.3.3.2.3.3-2: Minimum performance for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x4 Low | 1 | -0.9 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.2.

5.3.3.2.3.4	Test description

5.3.3.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of DRX, DCP, PDSCH and PDCCH before measurement are specified in 5.3.3.2.3.4.3.1 and Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.3 for TE diagram and section A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.3.2-1 and Table 5.3.3.2.1.3-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On) for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.2.1.4.3.

5.3.3.2.3.4.2	Test procedure

1.	SS transmits PDCCH with DCI format2_6 as specified in PDCCH Reference Channel for PS_RNTI within DRX off state. The Wake-up indication bit in PDCCH is set to 1.

2.	SS transmits PDSCH via PDCCH with DCI format1_1 as specified in PDCCH Reference Channel for C_RNTI to transmit the DL RMC according to Table 5.3.3.2.3.3-2 in DRX on period. The details of PDCCH are specified in Table 5.3-1, Table 5.3.3.2-1 and Table 5.3.3.2.3.3-2 respectively. The details of PDSCH are specified in Table A.3.3.2.2-3. The SS sends downlink MAC padding bits on the DL RMC.

3.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.2.3.5-1as appropriate.

4.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.2.3.5-1, pass the UE. Otherwise fail the UE.

5.	Repeat steps from 2 to 4 for each subtest in Table 5.3.3.2.3.3-2 as appropriate.

5.3.3.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 and 5.4.2.

5.3.3.2.3.4.3.1	Message exceptions for SA

Table 5.3.3.2.3.4.3.1-1: DRX-Config

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-56 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms5 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms0 |  |  |
| drx-HARQ-RTT-TimerDL | 0 |  |  |
| drx-HARQ-RTT-TimerUL | 0 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms10 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-2: DCP-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 28 |  | Scs-30kHz-r16 = sl6 |
| sizeDCI-2-6-r16 | 2 |  |  |
| ps-PositionDCI-2-6-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-3: PDCCH-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SEQUENCE(SIZE (1..3)) OF ControlResourceSet ::= SEQUENCE { | 1 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet |  |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace ::= SEQUENCE { | 2 entries |  |  |
| SearchSpace[1] | SearchSpace1 |  |  |
| SearchSpace[2] | SearchSpace2 |  |  |
| } |  |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpace { | 2 entries |  |  |
| searchSpaceExt-r16[1] | SearchSpaceExt1 |  |  |
| searchSpaceExt-r16[2] | SearchSpaceExt2 |  |  |
| } |  |  |  |
| } |  |  |  |

<< Sections skipped >>

5.3.3.2.3.4.3.2	Message exceptions for NSA

Same as 5.3.3.2.3.4.3.1.

5.3.3.2.3.5	Test requirement

Table 5.3.3.2.3.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.3.5-1.

Table 5.3.3.2.3.5-1: Test Requirements for PDCCH with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x4 Low | 1 | 0 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

<< Sections skipped >>

## A.3.3	Reference measurement channels for PDCCH performance requirements

### A.3.3.1	FDD

#### A.3.3.1.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.3.1.1-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-1.1 FDD | R.PDCCH.1-1.2 FDD | R.PDCCH.1-1.3 FDD |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |  |
| CORESET frequency domain allocation |  | 48 | 48 | 48 |  |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 |  |  |  |
| Aggregation level |  | 4 | 4 | 8 |  |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 |  |  |  |
| Payload (without CRC) | Bits | 39 | 52 | 52 |  |  |  |

Table A.3.3.1.1-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-2.1 FDD | R.PDCCH.1-2.2 FDD | R.PDCCH.1-2.3 FDD | R.PDCCH.1-2.4 FDD | R.PDCCH.1-2.5 FDD | R.PDCCH.1-2.6 FDD | R.PDCCH.1-2.7 FDD |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 24 | 24 | 48 | 48 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Aggregation level |  | 2 | 4 | 2 | 4 | 8 | 16 | 8 |
| DCI Format |  | 1_0 | 1_0 | 1_1 | 1_1 | 1_1 | 1_0 | 2_6 |
| Payload (without CRC) | Bits | 39 | 39 | 52 | 52 | 52 | 39 | 12 |

Table A.3.3.1.1-3: Additional PDSCH Reference Channel FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| DCI Format |  | 1_0 | 1_1 |
| Channel bandwidth | MHz | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 |
| Number of allocated resource blocks | PRBs | 52 | 52 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 4 | 4 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 0.30 | 0.30 |
| Number of MIMO layers |  | 1 | 1 |
| Number of DMRS REs |  | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 3368 | 4096 |
| Transport block CRC per Slot |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 16 | 24 |
| Number of Code Blocks per Slot |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 1 | 1 |
| Binary Channel Bits Per Slot |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |
| For Slots i = 10, 11 | Bits | 9984 | 13104 |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 11232 | 13728 |
| Max. Throughput averaged over 2 frames | Mbps | 3.1996 | 3.8912 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 2: Slot i is slot index per 2 frames. Note 3: For PDSCH & PDSCH DMRS precoding the following configuration shall be used: Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Note 4: For tests with DRX configured, PDCCH and PDSCH shall only be scheduled during the DRX on duration of the test. |  |  |  |

#### A.3.3.1.2	Reference measurement channels for SCS 30 kHz FR1

Table A.3.3.1.2-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.2-1.1 FDD | R.PDCCH.2-1.2 FDD | R.PDCCH.2-1.3 FDD |  |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |  |  |
| CORESET frequency domain allocation |  | 102 | 102 | 90 |  |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 |  |  |  |
| Aggregation level |  | 2 | 4 | 8 |  |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 |  |  |  |
| Payload (without CRC) | Bits | 41 | 53 | 53 |  |  |  |

Table A.3.3.1.2-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.2-2.1 FDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 48 |  |  |  |  |  |
| CORESET time domain allocation |  | 2 |  |  |  |  |  |
| Aggregation level |  | 16 |  |  |  |  |  |
| DCI Format |  | 1_0 |  |  |  |  |  |
| Payload (without CRC) | Bits | 41 |  |  |  |  |  |

### A.3.3.2	TDD

#### A.3.3.2.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.3.2.1-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-1.1 TDD | R.PDCCH.1-1.2 TDD | R.PDCCH.1-1.3 TDD |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |  |
| CORESET frequency domain allocation |  | 48 | 48 | 48 |  |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 |  |  |  |
| Aggregation level |  | 4 | 4 | 8 |  |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 |  |  |  |
| Payload (without CRC) | Bits | 39 | 52 | 52 |  |  |  |

Table A.3.3.2.1-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-2.1 TDD | R.PDCCH.1-2.2 TDD | R.PDCCH.1-2.3 TDD | R.PDCCH.1-2.4 TDD | R.PDCCH.1-2.5 TDD | R.PDCCH.1-2.6 TDD |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 24 | 24 | 48 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 2 | 2 | 2 | 2 |
| Aggregation level |  | 2 | 4 | 2 | 4 | 8 | 16 |
| DCI Format |  | 1_0 | 1_0 | 1_1 | 1_1 | 1_1 | 1_0 |
| Payload (without CRC) | Bits | 39 | 39 | 52 | 52 | 52 | 39 |

#### A.3.3.2.2	Reference measurement channels for SCS 30 kHz FR1

Table A.3.3.2.2-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.2-1.1 TDD | R.PDCCH.2-1.2 TDD | R.PDCCH.2-1.3 TDD | R.PDCCH.2-1.4 TDD |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |  |  |
| CORESET frequency domain allocation |  | 102 | 102 | 90 | 102 |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 | 1 |  |  |
| Aggregation level |  | 2 | 4 | 8 | 8 |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 | 2_6 |  |  |
| Payload (without CRC) | Bits | 41 | 53 | 53 | 12 |  |  |

Table A.3.3.2.2-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.2-2.1 TDD |  |  |  |  |  |
| Subcarrier spacing | kHz | 30 |  |  |  |  |  |
| CORESET frequency domain allocation |  | 48 |  |  |  |  |  |
| CORESET time domain allocation |  | 2 |  |  |  |  |  |
| Aggregation level |  | 16 |  |  |  |  |  |
| DCI Format |  | 1_0 |  |  |  |  |  |
| Payload (without CRC) | Bits | 41 |  |  |  |  |  |

Table A.3.3.2.2-3: Additional PDSCH Reference Channel TDD

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
| DCI Format |  |  | 1-0 | 1-1 |  |
| TDD UL/DL pattern |  |  | FR1.30-1 | FR1.30-1 |  |
| Channel bandwidth |  | MHz | 40 | 40 |  |
| Subcarrier spacing |  | kHz | 30 | 30 |  |
| Allocated resource blocks |  | PRBs | 106 | 106 |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  |  | 4 | 4 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  |  | 12 | 12 |  |
| Allocated slots per 2 frames |  |  | 31 | 31 |  |
| MCS table |  |  | 64QAM | 64QAM |  |
| MCS index |  |  | 4 | 4 |  |
| Modulation |  |  | QPSK | QPSK |  |
| Target Coding Rate |  |  | 0.30 | 0.3 |  |
| Number of MIMO layers |  |  | 1 | 1 |  |
| Number of DMRS rEs |  |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  |  | 6 | 6 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  |  | 12 | 12 |  |
| Overhead for TBS determination |  |  | 0 | 0 |  |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | Bits | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | Bits | 2280 | 2664 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  | Bits | 6912 | 8456 |  |
| Transport block CRC per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | Bits | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | Bits | 16 | 16 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  | Bits | 24 | 24 |  |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | CBs | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | CBs | 1 | 1 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  | CBs | 1 | 2 |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} |  | Bits | N/A | N/A |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | Bits | 7488 | 8904 |  |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  | Bits | 22896 | 27984 |  |
| Max. Throughput averaged over 2 frames |  | Mbps | 9.78 | 11.94 |  |
| Note 1: For PDSCH & PDSCH DMRS precoding the following configuration shall be used: Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Note 2: For tests with DRX configured, PDCCH and PDSCH shall only be scheduled during the DRX on duration of the test. |  |  |  |

#### A.3.3.2.3	Reference measurement channels for SCS 60 kHz FR1

<< End of Changes >>