3GPP TSG-RAN5 Meeting #99	R5-232963r1

# Incheon, Korea, 22th May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3785 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MBS Multicast TC 14.2.5.2.1-UE-requested to join MBS multicast session-accept |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_5MBS_5MBUSA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | MBS TC 14.2.5.2.1 needs to be added to test UE-requested to join two MBS session and both MBS session were accepted by NW. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of MBS TC 14.2.5.2.1 MBS Multicast/ Session management / UE-requested PDU session establishment / UE-requested PDU session modification / Join MBS multicast session / Accepted |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_MBS_5MBS_5MBUSA-UEConTest WP will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.2.5.2.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0356 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revsion: - Add wording <Check:question?> in step 1 because this step is related to TP1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

14.2.5.2	MBS Multicast/ Session management / UE-requested PDU session establishment / UE-requested PDU session modification

14.2.5.2.1	MBS Multicast/ Session management / UE-requested PDU session establishment / UE-requested PDU session modification / Join MBS multicast session / Accepted

14.2.5.2.1.1	Test Purpose (TP)

(1)

with { UE is in 5GMM-REGISTERED state}

ensure that {

when { UE requests to establish a new PDU session associated with MBS multicast sessions and the UE at the same time intends to join two MBS multicast sessions }

then { UE sends PDU SESSION ESTABLISHMENT REQUEST or PDU SESSION MODIFICATION REQUEST including the Requested MBS container IE with MBS operation setting to "Join MBS session" }

}

(2)

with { UE is in 5GMM-REGISTERED state and has sent PDU SESSION ESTABLISHMENT REQUEST or PDU SESSION MODIFICATION REQUEST including the Requested MBS container IE with MBS operation setting to "Join MBS session"}

ensure that {

when { UE receives PDU SESSION ESTABLISHMENT ACCEPT or PDU SESSION MODIFICATION COMMAND message including Received MBS container IE with MBS decision setting to "MBS join is accepted" }

then { UE shall consider that it has successfully joined the MBS session and shall store the received TMGI}

}

14.2.5.2.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 6.4.1.2, 6.4.1.3, 6.4.2.2 and 6.3.2.2. Unless otherwise stated these are Rel-17 requirements.

[TS 24.501, clause 6.4.1.2]

If the UE requests to:

a)	establish a new PDU session;

b)	perform handover of an existing PDU session from non-3GPP access to 3GPP access;

c)	transfer an existing PDN connection in the EPS to the 5GS according to subclause 4.8.2.3.1;

d)	transfer an existing PDN connection in untrusted non-3GPP access connected to the EPC to the 5GS; or

e)	establish user plane resources over 3GPP access of an MA PDU session established over non-3GPP access only;

and the UE at the same time intends to join one or more multicast MBS sessions that is associated to the PDU session, the UE should include the Requested MBS container IE in the PDU SESSION ESTABLISHMENT REQUEST message. In that case, the UE shall set the MBS operation to "Join multicast MBS session" and include the multicast MBS session information(s) and shall set the Type of multicast MBS session ID for each of the multicast MBS session information to either "Temporary Mobile Group Identity (TMGI)" or "Source specific IP multicast address" depending on the type of the multicast MBS session ID available in the UE. Then the remaining values of each of the multicast MBS session information shall be set as following:

a)	if the Type of multicast MBS session ID is set to "Temporary Mobile Group Identity (TMGI)", the UE shall set the multicast MBS session ID to the TMGI; or

b)	if the Type of multicast MBS session ID is set to "Source specific IP multicast address for IPv4" or " Source specific IP multicast address for IPv6", the UE shall set the Source IP address information and the Destination IP address information to the corresponding values.

[TS 24.501, clause 6.4.1.3]

If the PDU SESSION ESTABLISHMENT REQUEST included the Requested MBS container IE with the MBS operation set to "Join multicast MBS session", the SMF:

a)	shall include the TMGI for the multicast MBS session IDs that the UE is allowed to join, if any, in the Received MBS container IE, shall set the MBS decision to "MBS join is accepted" for each of those Received MBS information, may include the MBS start time to indicate the time when the multicast MBS session starts and shall include the MBS security container in each of those Received MBS information if security protection is applied for that multicast MBS session and the control plane security procedure is used as specified in annex W.4.1.2 in 3GPP TS 33.501 [24], and shall use separate QoS flows dedicated for multicast by including the Authorized QoS flow descriptions IE if no separate QoS flows dedicated for multicast exist or if the SMF wants to establish new QoS flows dedicated for multicast;

[TS 24.501, clause 6.4.2.2]

If the UE requests to join or leave one or more multicast MBS sessions associated with a PDU session, the UE shall include the Requested MBS container IE in the PDU SESSION MODIFICATION REQUEST message and shall set the MBS operation to "Join multicast MBS session" for the join case or to "Leave multicast MBS session" for the leave case. The UE shall include the multicast MBS session information(s) and shall set the Type of multicast MBS session ID for each of the multicast MBS session information to either "Temporary Mobile Group Identity (TMGI)" or "Source specific IP multicast address" depending on the type of the multicast MBS session ID available in the UE. Then the remaining values of each of the multicast MBS session informations shall be set as following:

a)	if the Type of multicast MBS session ID is set to "Temporary Mobile Group Identity (TMGI)", the UE shall set the multicast MBS session ID to the TMGI; or

b)	if the Type of multicast MBS session ID is set to "Source specific IP multicast address for IPv4" or " Source specific IP multicast address for IPv6", the UE shall set the Source IP address information and the Destination IP address information to the corresponding values.

[TS 24.501, clause 6.3.2.2]

If the network-requested PDU session modification procedure is triggered by a UE-requested PDU session modification procedure and the UE has included the Requested MBS container IE in the PDU SESSION MODIFICATION REQUEST message with the MBS operation set to "Join multicast MBS session", the SMF:

a)	shall include the TMGI for the multicast MBS session IDs that the UE is allowed to join, if any, in the Received MBS container IE, shall set the MBS decision to "MBS join is accepted" for each of those Received MBS information, may include the MBS start time to indicate the time when the multicast MBS session starts, and shall include the MBS security container in each of those Received MBS information if security protection is applied for that multicast MBS session and the control plane security procedure is used as specified in annex W.4.1.2 in 3GPP TS 33.501 [24], and shall use separate QoS flows dedicated for multicast by including the Authorized QoS flow descriptions IE if no separate QoS flows dedicated for multicast exist or if the SMF wants to establish new QoS flows dedicated for multicast;

14.2.5.2.1.3	Test description

14.2.5.2.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the Serving cell.

-	System information combination NR-1 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cell 1.

UE:

-	UE is made interested in receiving MBS Multicast service with MBS Service ID ‘000101’H and ‘000102’H.

Preamble:

-	The UE is in state 1N-A on NR Cell 1 (serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1 with Test Mode = on to activate UE TEST MODE C and Test Loop Function = off.

14.2.5.2.1.3.2	Test procedure sequence

Table 14.2.5.2.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1a1-1b12a1 | Check: Does steps 1a1 to 1b12a1 of the generic procedures described in TS 38.508-1 subclause 4.9.34 perform on NR Cell 1 to establish an associated PDU Session to the MBS DNN and join in two MBS Multicast session? Note: One MBS session Id is TMGI-1, and another MBS session Id is TMGI-2. | - | - | 1 | - |
| 2 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 3-17 | Check: Does UE respond to paging with TMGI-1 and receive the MRB associated with TMGI-1 as specified in steps 1 to 15 of the procedure in TS 38.508-1[4] Table 4.9.XX.2.2-1? | - | - | 2 | - |
| 18 | The SS transmits an OPEN UE TEST LOOP message. | <-- | NR RRC: DLInformationTransfer TC: OPEN UE TEST LOOP | - | - |
| 19 | The UE transmits an OPEN UE TEST LOOP COMPLETE message. | --> | NR RRC: ULInformationTransfer TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 20 | The SS transmits an RRCRelease message | <-- | NR RRC: RRCRelease | - | - |
| 21-35 | Check: Does UE respond to paging with TMGI-2 and receive the MRB associated with TMGI-2 as specified in steps 1 to 15 of the procedure in TS 38.508-1[4] Table 4.9.XX.2.2-1? | - | - | 2 | - |

14.2.5.2.1.3.3	Specific message contents

Table 14.2.5.2.1.3.3-1: ACTIVATE TEST MODE (preamble, Table 14.2.5.2.1.3.2-1)

Derivation Path: TS 36.508 [6], Table 4.7A-1, condition UE TEST LOOP MODE C

Table 14.2.5.2.1.3.3-2: PDU SESSION MODIFICATION REQUEST (step 1a14, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-7. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |

Table 14.2.5.2.1.3.3-3:  PDU SESSION MODIFICATION COMMAND (step 1a15, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |

Table 14.2.5.2.1.3.3-4: RRCReconfiguration (step 1a15, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig | m=1 Table 14.2.4.1.1.3.3-5 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm and UM_PTM | m=1 Table 14.2.4.1.1.3.3-6 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.2.1.3.3-5: RadioBearerConfig (Table 14.2.5.2.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 with condition MRBm and UM_PTM (m=1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 2 entries |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-1 | Table 14.2.5.2.1.3.3-7 |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=1 |  |
| } |  |  |  |
| MRB-ToAddMod-r17 [2] SEQUENCE { |  | entry 2 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-2 | Table 14.2.5.2.1.3.3-7 |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=2 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.1.3.3-6: CellGroupConfig (Table 14.2.5.2.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition MRBm and UM_PTM (m=1) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 2 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 1 m=1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 2 m=2 |  |
| } |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig with condition MBS_Multicast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.1.3.3-7: TMGI (Table 14.2.5.2.1.3.3-5)

| Derivation Path: TS 38.508-1 [4], Table 4.6.7-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TMGI-r17 ::= SEQUENCE { |  |  |  |
| plmn-Id-r17 CHOICE { |  |  |  |
| plmn-Index-r17 | 1 |  |  |
| } |  |  |  |
| serviceId-r17 | ‘000101’H | OCTET STRING (SIZE (3)) | TMGI-1 |
|  | ‘000102’H | OCTET STRING (SIZE (3)) | TMGI-2 |
| } |  |  |  |

Table 14.2.5.2.1.3.3-8: PDU SESSION ESTABLISHMENT REQUEST (step 1b9, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-1. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Requested MBS container |  |  |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MBS session information |  |  |  |  |
| MBS operation |  | ‘01’B | Join MBS session |  |
| Type of MBS session ID |  | Not checked |  |  |
| MBS session ID |  |  |  |  |
| MBMS Service ID |  | ‘000101’H | TMGI-2 |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |

Table 14.2.5.2.1.3.3-9: PDU SESSION ESTABLISHMENT ACCEPT (step 1b10, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.2-2. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Received MBS container |  |  |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-1 |  |
| MBMS Service ID |  | ‘000101’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |
| Received MBS information |  |  |  |  |
| Rejection cause |  | ‘000’B | No additional information provided |  |
| MSAI |  | ‘00’B | MBS service area not included |  |
| MD |  | ‘010’B | MBS join is accepted |  |
| MSCI |  | ‘0’B | MBS security container not included |  |
| MTI |  | ‘00’B | No MBS timers included |  |
| IPAE |  | ‘0’B | Source and destination IP address information not included |  |
| TMGI |  |  | TMGI-2 |  |
| MBMS Service ID |  | ‘000102’H |  |  |
| MCC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| MNC |  | See 38.508-1[4] table 4.4.2-3 |  |  |
| Source IP address information |  | Not present |  |  |
| Destination IP address information |  | Not present |  |  |
| MBS service area |  | Not present |  |  |
| MBS timers |  | Not present |  |  |
| MBS security container |  | Not present |  |  |

Table 14.2.5.2.1.3.3-10: RRCReconfiguration (step 1b10, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig | Table 14.2.5.2.1.3.3-11 |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 14.2.5.2.1.3.3-12 |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message {} |  | DedicatedNAS-Message |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.2.1.3.3-11: RadioBearerConfig (Table 14.2.5.2.1.3.3-10)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-132 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioBearerConfig ::= SEQUENCE { |  |  |  |
| drb-ToAddModList SEQUENCE (SIZE (1..maxDRB)) OF DRB-ToAddMod { | 1 entry |  |  |
| DRB-ToAddMod[1] SEQUENCE { |  | entry 1 |  |
| cnAssociation CHOICE { |  |  |  |
| sdap-Config | SDAP-Config |  |  |
| } |  |  |  |
| drb-Identity | DRB-Identity with condition DRBn | n is chosen as the next available number higher or equal to 2 |  |
| reestablishPDCP | Not present |  |  |
| recoverPDCP | Not present |  |  |
| pdcp-Config | PDCP-Config |  |  |
| } |  |  |  |
| } |  |  |  |
| mrb-ToAddModList-r17 SEQUENCE (SIZE (1..maxDRB)) OF MRB-ToAddMod-r17 { | 2 entries |  |  |
| MRB-ToAddMod-r17 [1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-1 |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=1 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=1 |  |
| } |  |  |  |
| MRB-ToAddMod-r17 [2] SEQUENCE { |  | entry 2 |  |
| mbs-SessionId-r17 | TMGI with condition TMGI-2 |  |  |
| mrb-Identity-r17 | MRB-Identity with condition MRBm | m=2 |  |
| pdcp-Config-r17 | PDCP-Config with condition MRB_Initialization and UM_MRB and MRBm | m=2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.1.3.3-12: CellGroupConfig (Table 14.2.5.2.1.3.3-11)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 1 m=1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions UM_DLonly and PTM and MRBm | entry 2 m=2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM and DRBn | entry 3 n is set to the same value as for the radioBearerConfig IE in Table 14.2.5.2.1.3.3-11 |  |
| } |  |  |  |
| mac-CellGroupConfig | MAC-CellGroupConfig with condition MBS_Multicast |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| spCellConfigDedicated | ServingCellConfig with condition MBS_Multicast |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.1.3.3-13: Paging (step 3, step 21, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-9, condition TMGI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| pagingGroupList-r17 SEQUENCE (SIZE(1..maxNrofPageGroup-r17)) OF TMGI-r17 { | 1 entry |  |  |
| TMGI-r17[1] | TMGI with condition TMGI-1 | entry 1 Table 14.2.5.2.1.3.3-7 | Step 3 |
|  | TMGI with condition TMGI-2 | entry 1 Table 14.2.5.2.1.3.3-7 | Step 21 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 14.2.5.2.1.3.3-14: RRCReconfiguration (step 11 and step 29, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4],Table 4.6.1-13 and condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | RadioBearerConfig with condition MRBm | m=1 | Step 11 |
|  |  | RadioBearerConfig with condition MRBm | m=2 | Step 29 |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig with condition MRBm and UM_PTM | m=1 | Step 11 |
|  |  | CellGroupConfig with condition MRBm and UM_PTM | m=2 | Step 29 |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.2.5.2.1.3.3-15: CLOSE UE TEST LOOP (step 13 and step 31, Table 14.2.5.2.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7A-3, condition UE TEST LOOP MODE C and Multicast MRB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode C LB setup |  |  |  |
| MRB ID | ‘ 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 ’B | MRB-Identity is 1 | Step 13 |
|  | ‘ 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 0 0 0 0 0 0 0 ’B | MRB-Identity is 2 | Step 31 |

<End of modified section 1>