3GPP TSG-RAN5 Meeting #99	R5-232977r1

# Incheon, South Korea, 22 – 26 May, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2272 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding UE additional maximum output power reduction for new NR band n13 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Missing UE additional maximum output power reduction for NR band n13 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add UE additional maximum output power reduction requirements for NR band n13. Add NS_07 requirements into clauses 6.2.3, 6.2.3.2, 6.2.3.3.1, 6.2.3.3.29, 6.2.3.4.1, 6.2.3.4.3.35 (new), 6.2.3.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE additional maximum output power reduction requirements for NR band n13 will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TR 38.905... CR 0778 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP Analysis in R5-232976 (CR 0778) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: Include Table 6.2.3.4.1-31 into clauses 6.2.3.4.1 and 6.2.3.4.2. |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same A-MPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, NS_07, NS_27 and NS_35 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and NR Power Class 1 UE release 15 and forward.

The requirements of this test apply in test case 6.5.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 3 UE release 15 and forward that don’t support Tx diversity.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_07, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_24, NS_27, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed.

6.2.3.3	Minimum conformance requirements

6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where CEIL{x, 0.5} means x rounding upwards to closest 0.5dB, NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 Table 6.2.2.3-1.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 (NOTE 8) | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.3.7 |
| NS_03U | 6.5.2.3.3.3, 6.5.2.4.2.3 | n2, n25, n66, n86 (NOTE 1) |  |  | Clause 6.2.3.3.7 |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_05 | 6.5.3.3.3.4 | n1, n65, n84 (NOTE 1) | 5, 10, 15, 20 (Note 2) |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_05U | 6.5.3.3.3.4, 6.5.2.4.2.3 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.3.4 (NOTE 7) |
| NS_06 | 6.5.2.3.3.4 | n12 | 5, 10, 15 |  | N/A |
|  |  | n14 | 5, 10 |  |  |
| NS_07 | 6.5.2.3.3.4, 6.5.3.3.5.26 | n13 | 5,10 | Table 6.2.3.3.29-1 | Table 6.2.3.3.29-2 |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3.3-1 | Table 6.2.3.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 5, 10 | Table 6.2.3.3.21-1 | Table 6.2.3.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 5 | Table 6.2.3.3.22-1 | Table 6.2.3.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10, 15, 20 | Table 6.2.3.3.23-1 | Table 6.2.3.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 5, 10, 15, 20 | Table 6.2.3.3.24-1 | Table 6.2.3.3.24-2 |
| NS_17 | 6.5.3.3.3.2 | n28, n83 | 5, 10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.3.13-1, A2 |
|  |  |  | 30 |  | Table 6.2.3.3.13-1, A3, A4, A5 |
| NS_21 | 6.5.2.3.3.9 6.5.3.3.3.12 | n30 | 5,10 |  | Clause 6.2.3.3.14 |
| NS_24 | 6.5.3.3.3.13 | n65 (Note 4) | 5, 10, 15, 20 | Table 6.2.3.3.15-1 | Subclause 6.2.3.3.15 |
| NS_27 | 6.5.2.3.3.8 6.5.3.3.3.14 | n48 | 5, 10, 15, 20, 30, 40 | Table 6.2.3.3.16-1 | Table 6.2.3.3.16-2 |
| NS_35 | 6.5.2.3.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.3.6 | n74 (Note 3) | 10, 15 | Table 6.2.3.3.8-1 | Table 6.2.3.3.8-1 |
| NS_38 | 6.5.3.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.3.9-1 | Table 6.2.3.3.9-1 |
| NS_39 | 6.5.3.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.3.10-1 | Table 6.2.3.3.10-1 |
| NS_40 | 6.5.3.3.3.9 | n51 | 5 |  | Table 6.2.3.3.5-1 |
| NS_41 | 6.5.3.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.11-1 |
| NS_42 | 6.5.3.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.12-1 |
| NS_43 | 6.5.3.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_43U | 6.5.3.3.3.5, 6.5.2.4.2.3 | n8, n81 (NOTE 1) | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25,30,40 | Table 6.2.3.3.20-1 | Table 6.2.3.3.20-1 |

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_45 | 6.5.3.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.3.25 |
| NS_46 | 6.5.3.3.3.25 | n7 | 25, 30, 40, 50 | Table 6.2.3.3.17-1 | Table 6.2.3.3.17-2 |
| NS_47 | 6.5.3.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.3.18-1 | Table 6.2.3.3.18-2 |
| NS_48 | 6.5.3.3.3.22 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.26-1, Table 6.2.3.3.26-3 | Table 6.2.3.3.26-2, Table 6.2.3.3.26-4 (NOTE 7) |
| NS_49 | 6.5.3.3.3.23 | n1 and n84 | 10, 15, 20, 25, 30, 40, 45, 50 | Table 6.2.3.3.27-1 Table 6.2.3.3.27-3 | Table 6.2.3.3.27-2, Table 6.2.3.3.27-4 (NOTE 7) |
| NS_50 | 6.5.3.3.3.16 | n39 | 10, 15, 20, 25, 30, 40 |  | Clause 6.2.3.3.19 |
| NS_55 | NOTE 6 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_56 | 6.5.3.3.3.27 | n24, n99 | 5, 10 | Table 6.2.3.3.30 | Clause 6.2.3.3.30 |
| NS_100 | 6.5.2.4.2.3 | n1, n2, n3, n5, n8, n25, n26, n66, n80, n81, n84, n86 (Note 1) |  |  | Table 6.2.3.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWchannel where the lower channel edge is ≥1930 MHz,10 MHz BWchannel where the lower channel edge is ≥1950 MHz and 15 MHz BWchannel where the lower channel edge is ≥1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. NOTE 7: The 1Tx architecture is assumed. For power class 2 UE indicating txDiversity-r16 [TS 38.306], the additional relaxation of [2] dB is applicable. NOTE 8: The NS_01 label with the field additionalPmax [7] absent is default for all NR bands. |  |  |  |  |  |

Table 6.2.3.3.1-1A: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n13 | NS_01 | NS_06 | NS_07 |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n24 | NS_01 | NS_56 |  |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n53 | NS_01 | NS_45 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U | NS_51 |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 | NS_55 |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n91 | NS_01 |  |  |  |  |  |  |  |
| n92 | NS_01 |  |  |  |  |  |  |  |
| n93 | NS_01 |  |  |  |  |  |  |  |
| n94 | NS_01 |  |  |  |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| n99 | NS_01 | NS_56 |  |  |  |  |  |  |
| n100 | NS_01 |  |  |  |  |  |  |  |
| n101 | NS_01 |  |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.1.

-------- Skipped unchanged clauses --------

6.2.3.3.28	A-MPR for NS_51

Table 6.2.3.3.28: void

6.2.3.3.29	A-MPR for NS_07

Table 6.2.3.3.29-1: A-MPR regions for NS_07

| Channel Bandwidth, MHz | Carrier Frequency, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | 782 ≤ Fc ≤ 784.5 | >0 | ≥ 1.8 | A3 |
| 5 MHz | 779.5 ≤ Fc < 782 | ≤ 0.9 | ≥ 0 | A1 |
|  |  | > 0.9, ≤ 1.26 | ≥1.26 | A2 |
|  |  | > 1.26, ≤ 3.42 | ≥ 1.8 | A3 |
|  |  | >3.42 | ≤ 0.36 | A4 |
| 10 MHz | Fc = 782 | ≤ 2.34 | ≥ 0 | A1 |
|  |  | >2.34, ≤ 3.24 | ≥ 1.44 | A2 |
|  |  | > 3.24, ≤ 6.48 | ≥ 3.24 | A3 |
|  |  | > 6.48 | ≤ 0.36 | A4 |

Table 6.2.3.3.29-2: A-MPR for NS_07

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | 12 | 9 | 6 | 3 |
| DFT-s-OFDM QPSK | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 16 QAM | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 64 QAM | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 256 QAM | 12 | 9 | 6 | 3 |
| CP-OFDM QPSK | 14 | 10 | 7 | 3 |
| CP-OFDM 16 QAM | 14 | 10 | 7 | 3 |
| CP-OFDM 64 QAM | 14 | 10 | 7 | 3 |
| CP-OFDM 256 QAM | 14 | 10 | 7 | 3 |

6.2.3.3.30	A-MPR for NS_56

-------- Skipped unchanged clauses --------

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2.3.4.1-1 to 6.2.3.4.1-31. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Network signalling value NS_10 is not tested. The relaxation specified under NS_10 is for the purpose to control / mitigate self-desensitization of the UE’s own receiver, but no corresponding REFSENS requirements are specified. There is no need to test A-MPR requirements for its own.

-------- Skipped unchanged clauses --------

Table 6.2.3.4.1-30: Test Configuration table for NS_56

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | Use uplink carrier center frequency (Fc) as specified in test parameters |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  | 5 MHz, 10 MHz as specified in test parameters |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  | Lowest, Highest unless otherwise specified in test parameters. |  |  |  |
| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 1 | Low | 5 | 15 | N/A for A-MPR testing | 14 | DFT-s-OFDM | PI/2 BPSK |  | Edge_1RB_Left | N/A | N/A |
| 2 | Low | 5 | 30 |  | 8 |  | PI/2 BPSK |  | N/A | Edge_1RB_Left | N/A |
| 3 | Low | 5 | Default |  | 6 |  | PI/2 BPSK |  | Outer_Full | Outer_Full | N/A |
| 4 | Low | 5 | Default |  | 4 |  | PI/2 BPSK |  | 20@4 | 8@3 | N/A |
| 5 | Low | 5 | Default |  | 4 |  | PI/2 BPSK |  | 1@4 | 1@3 | N/A |
| 6 | Low | 5 | 15 |  | 14 |  | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 7 | Low | 5 | 30 |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | N/A |
| 8 | Low | 5 | Default |  | 6 |  | QPSK |  | Outer_Full | Outer_Full | N/A |
| 9 | Low | 5 | Default |  | 4 |  | QPSK |  | 20@4 | 8@3 | N/A |
| 10 | Low | 5 | Default |  | 4 |  | QPSK |  | 1@4 | 1@3 | N/A |
| 11 | Low | 5 | 15 |  | 14 |  | 16 QAM |  | Edge_1RB_Left | N/A | N/A |
| 12 | Low | 5 | 30 |  | 8 |  | 16 QAM |  | N/A | Edge_1RB_Left | N/A |
| 13 | Low | 5 | Default |  | 6 |  | 16 QAM |  | Outer_Full | Outer_Full | N/A |
| 14 | Low | 5 | Default |  | 4 |  | 16 QAM |  | 20@4 | 8@3 | N/A |
| 15 | Low | 5 | Default |  | 4 |  | 16 QAM |  | 1@4 | 1@3 | N/A |
| 16 | Low | 5 | 15 |  | 14 |  | 64 QAM |  | Edge_1RB_Left | N/A | N/A |
| 17 | Low | 5 | 30 |  | 8 |  | 64 QAM |  | N/A | Edge_1RB_Left | N/A |
| 18 | Low | 5 | Default |  | 6 |  | 64 QAM |  | Outer_Full | Outer_Full | N/A |
| 19 | Low | 5 | Default |  | 4 |  | 64 QAM |  | 20@4 | 8@3 | N/A |
| 20 | Low | 5 | Default |  | 4 |  | 64 QAM |  | 1@4 | 1@3 | N/A |
| 21 | Low | 5 | 15 |  | 14 |  | 256 QAM |  | Edge_1RB_Left | N/A | N/A |
| 22 | Low | 5 | 30 |  | 8 |  | 256 QAM |  | N/A | Edge_1RB_Left | N/A |
| 23 | Low | 5 | Default |  | 6 |  | 256 QAM |  | Outer_Full | Outer_Full | N/A |
| 24 | Low | 5 | Default |  | 4 |  | 256 QAM |  | 20@4 | 8@3 | N/A |
| 25 | Low | 5 | Default |  | 4 |  | 256 QAM |  | 1@4 | 1@3 | N/A |
| 26 | Low | 10 | 15 |  | 12 |  | PI/2 BPSK |  | Edge_1RB_Left | N/A | N/A |
| 27 | Low | 10 | Highest |  | 8 |  | PI/2 BPSK |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 28 | Low | 10 | Default |  | 8 |  | PI/2 BPSK |  | Outer_Full | Outer_Full | Outer_Full |
| 29 | Low | 10 | Default |  | 6 |  | PI/2 BPSK |  | 1@3 | 1@2 | 1@1 |
| 30 | Low | 10 | Default |  | 6 |  | PI/2 BPSK |  | 40@9 | 18@5 | 8@2 |
| 31 | Low | 10 | Default |  | 4 |  | PI/2 BPSK |  | 1@35 | 1@17 | 1@8 |
| 32 | Low | 10 | Default |  | 2 |  | PI/2 BPSK |  | 16@35 | 5@19 | 3@8 |
| 33 | Low | 10 | Default |  | 5 |  | PI/2 BPSK |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 34 | Low | 10 | 15 |  | 5 |  | PI/2 BPSK |  | 1@40 | N/A | N/A |
| 35 | Low | 10 | 30 |  | 5 |  | PI/2 BPSK |  | N/A | 1@20 | N/A |
| 36 | Low | 10 | 15 |  | 3 |  | PI/2 BPSK |  | 8@44 | N/A | N/A |
| 37 | Low | 10 | 15 |  | 12 |  | QPSK |  | Edge_1RB_Left | N/A | N/A |
| 38 | Low | 10 | Highest |  | 8 |  | QPSK |  | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 39 | Low | 10 | Default |  | 8 |  | QPSK |  | Outer_Full | Outer_Full | Outer_Full |
| 40 | Low | 10 | Default |  | 6 |  | QPSK |  | 1@3 | 1@2 | 1@1 |
| 41 | Low | 10 | Default |  | 6 |  | QPSK |  | 40@9 | 18@5 | 8@2 |
| 42 | Low | 10 | Default |  | 4 |  | QPSK |  | 1@35 | 1@17 | 1@8 |
| 43 | Low | 10 | Default |  | 2 |  | QPSK |  | 16@35 | 5@19 | 3@8 |
| 44 | Low | 10 | Default |  | 5 |  | QPSK |  | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 45 | Low | 10 | 15 |  | 5 |  | QPSK |  | 1@40 | N/A | N/A |

| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 46 | Low | 10 | 30 |  | 5 |  | QPSK | N/A | 1@20 | N/A |
| 47 | Low | 10 | 15 |  | 3 |  | QPSK | 8@44 | N/A | N/A |
| 48 | Low | 10 | 15 |  | 12 |  | 16 QAM | Edge_1RB_Left | N/A | N/A |
| 49 | Low | 10 | Highest |  | 8 |  | 16 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 50 | Low | 10 | Default |  | 8 |  | 16 QAM | Outer_Full | Outer_Full | Outer_Full |
| 51 | Low | 10 | Default |  | 6 |  | 16 QAM | 1@3 | 1@2 | 1@1 |
| 52 | Low | 10 | Default |  | 6 |  | 16 QAM | 40@9 | 18@5 | 8@2 |
| 53 | Low | 10 | Default |  | 4 |  | 16 QAM | 1@35 | 1@17 | 1@8 |
| 54 | Low | 10 | Default |  | 2 |  | 16 QAM | 16@35 | 5@19 | 3@8 |
| 55 | Low | 10 | Default |  | 5 |  | 16 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 56 | Low | 10 | 15 |  | 5 |  | 16 QAM | 1@40 | N/A | N/A |
| 57 | Low | 10 | 30 |  | 5 |  | 16 QAM | N/A | 1@20 | N/A |
| 58 | Low | 10 | 15 |  | 3 |  | 16 QAM | 8@44 | N/A | N/A |
| 59 | Low | 10 | 15 |  | 12 |  | 64 QAM | Edge_1RB_Left | N/A | N/A |
| 60 | Low | 10 | Highest |  | 8 |  | 64 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 61 | Low | 10 | Default |  | 8 |  | 64 QAM | Outer_Full | Outer_Full | Outer_Full |
| 62 | Low | 10 | Default |  | 6 |  | 64 QAM | 1@3 | 1@2 | 1@1 |
| 63 | Low | 10 | Default |  | 6 |  | 64 QAM | 40@9 | 18@5 | 8@2 |
| 64 | Low | 10 | Default |  | 4 |  | 64 QAM | 1@35 | 1@17 | 1@8 |
| 65 | Low | 10 | Default |  | 2 |  | 64 QAM | 16@35 | 5@19 | 3@8 |
| 66 | Low | 10 | Default |  | 5 |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 67 | Low | 10 | 15 |  | 5 |  | 64 QAM | 1@40 | N/A | N/A |
| 68 | Low | 10 | 30 |  | 5 |  | 64 QAM | N/A | 1@20 | N/A |
| 69 | Low | 10 | 15 |  | 3 |  | 64 QAM | 8@44 | N/A | N/A |
| 70 | Low | 10 | 15 |  | 12 |  | 256 QAM | Edge_1RB_Left | N/A | N/A |
| 71 | Low | 10 | Highest |  | 8 |  | 256 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 72 | Low | 10 | Default |  | 8 |  | 256 QAM | Outer_Full | Outer_Full | Outer_Full |
| 73 | Low | 10 | Default |  | 6 |  | 256 QAM | 1@3 | 1@2 | 1@1 |
| 74 | Low | 10 | Default |  | 6 |  | 256 QAM | 40@9 | 18@5 | 8@2 |
| 75 | Low | 10 | Default |  | 4 |  | 256 QAM | 1@35 | 1@17 | 1@8 |
| 76 | Low | 10 | Default |  | 2 |  | 256 QAM | 16@35 | 5@19 | 3@8 |
| 77 | Low | 10 | Default |  | 5 |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 78 | Low | 10 | 15 |  | 5 |  | 256 QAM | 1@40 | N/A | N/A |
| 79 | Low | 10 | 30 |  | 5 |  | 256 QAM | N/A | 1@20 | N/A |
| 80 | Low | 10 | 15 |  | 3 |  | 256 QAM | 8@44 | N/A | N/A |
| 81 | Low | 5 | 15 | N/A for A-MPR testing | 14 | CP-OFDM | QPSK | Edge_1RB_Left | N/A | N/A |
| 82 | Low | 5 | 30 |  | 8 |  | QPSK | N/A | Edge_1RB_Left | N/A |
| 83 | Low | 5 | Default |  | 6 |  | QPSK | Outer_Full | Outer_Full | N/A |
| 84 | Low | 5 | Default |  | 4 |  | QPSK | 21@4 | 8@3 | N/A |
| 85 | Low | 5 | Default |  | 4 |  | QPSK | 1@4 | 1@3 | N/A |
| 86 | Low | 5 | 15 |  | 14 |  | 16 QAM | Edge_1RB_Left | N/A | N/A |
| 87 | Low | 5 | 30 |  | 8 |  | 16 QAM | N/A | Edge_1RB_Left | N/A |
| 88 | Low | 5 | Default |  | 6 |  | 16 QAM | Outer_Full | Outer_Full | N/A |
| 89 | Low | 5 | Default |  | 4 |  | 16 QAM | 21@4 | 8@3 | N/A |
| 90 | Low | 5 | Default |  | 4 |  | 16 QAM | 1@4 | 1@3 | N/A |
| 91 | Low | 5 | 15 |  | 14 |  | 64 QAM | Edge_1RB_Left | N/A | N/A |
| 92 | Low | 5 | 30 |  | 8 |  | 64 QAM | N/A | Edge_1RB_Left | N/A |
| 93 | Low | 5 | Default |  | 6 |  | 64 QAM | Outer_Full | Outer_Full | N/A |
| 94 | Low | 5 | Default |  | 4 |  | 64 QAM | 21@4 | 8@3 | N/A |
| 95 | Low | 5 | Default |  | 4 |  | 64 QAM | 1@4 | 1@3 | N/A |
| 96 | Low | 5 | 15 |  | 14 |  | 256 QAM | Edge_1RB_Left | N/A | N/A |
| 97 | Low | 5 | 30 |  | 8 |  | 256 QAM | N/A | Edge_1RB_Left | N/A |
| 98 | Low | 5 | Default |  | 6 |  | 256 QAM | Outer_Full | Outer_Full | N/A |
| 99 | Low | 5 | Default |  | 4 |  | 256 QAM | 21@4 | 8@3 | N/A |

| A-MPR test parameters for NS_56 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | ChBw (MHz) | SCS (kHz) | Downlink Config. | A-MPR | Uplink Configuration |  |  |  |  |
|  |  |  |  |  |  | Modulation (NOTE 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 100 | Low | 5 | Default |  | 4 |  | 256 QAM | 1@4 | 1@3 | N/A |
| 101 | Low | 10 | 15 |  | 12 |  | QPSK | Edge_1RB_Left | N/A | N/A |
| 102 | Low | 10 | Highest |  | 8 |  | QPSK | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 103 | Low | 10 | Default |  | 8 |  | QPSK | Outer_Full | Outer_Full | Outer_Full |
| 104 | Low | 10 | Default |  | 6 |  | QPSK | 1@3 | 1@2 | 1@1 |
| 105 | Low | 10 | Default |  | 6 |  | QPSK | 43@9 | 19@5 | 8@2 |
| 106 | Low | 10 | Default |  | 4 |  | QPSK | 1@35 | 1@17 | 1@8 |
| 107 | Low | 10 | Default |  | 2 |  | QPSK | 17@35 | 5@19 | 3@8 |
| 108 | Low | 10 | Default |  | 5 |  | QPSK | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 109 | Low | 10 | 15 |  | 5 |  | QPSK | 1@40 | N/A | N/A |
| 110 | Low | 10 | 30 |  | 5 |  | QPSK | N/A | 1@20 | N/A |
| 111 | Low | 10 | 15 |  | 3 |  | QPSK | 8@44 | N/A | N/A |
| 112 | Low | 10 | 15 |  | 12 |  | 16 QAM | Edge_1RB_Left | N/A | N/A |
| 113 | Low | 10 | Highest |  | 8 |  | 16 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 114 | Low | 10 | Default |  | 8 |  | 16 QAM | Outer_Full | Outer_Full | Outer_Full |
| 115 | Low | 10 | Default |  | 6 |  | 16 QAM | 1@3 | 1@2 | 1@1 |
| 116 | Low | 10 | Default |  | 6 |  | 16 QAM | 43@9 | 19@5 | 8@2 |
| 117 | Low | 10 | Default |  | 4 |  | 16 QAM | 1@35 | 1@17 | 1@8 |
| 118 | Low | 10 | Default |  | 2 |  | 16 QAM | 17@35 | 5@19 | 3@8 |
| 119 | Low | 10 | Default |  | 5 |  | 16 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 120 | Low | 10 | 15 |  | 5 |  | 16 QAM | 1@40 | N/A | N/A |
| 121 | Low | 10 | 30 |  | 5 |  | 16 QAM | N/A | 1@20 | N/A |
| 122 | Low | 10 | 15 |  | 3 |  | 16 QAM | 8@44 | N/A | N/A |
| 123 | Low | 10 | 15 |  | 12 |  | 64 QAM | Edge_1RB_Left | N/A | N/A |
| 124 | Low | 10 | Highest |  | 8 |  | 64 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 125 | Low | 10 | Default |  | 8 |  | 64 QAM | Outer_Full | Outer_Full | Outer_Full |
| 126 | Low | 10 | Default |  | 6 |  | 64 QAM | 1@3 | 1@2 | 1@1 |
| 127 | Low | 10 | Default |  | 6 |  | 64 QAM | 43@9 | 19@5 | 8@2 |
| 128 | Low | 10 | Default |  | 4 |  | 64 QAM | 1@35 | 1@17 | 1@8 |
| 129 | Low | 10 | Default |  | 2 |  | 64 QAM | 17@35 | 5@19 | 3@8 |
| 130 | Low | 10 | Default |  | 5 |  | 64 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 131 | Low | 10 | 15 |  | 5 |  | 64 QAM | 1@40 | N/A | N/A |
| 132 | Low | 10 | 30 |  | 5 |  | 64 QAM | N/A | 1@20 | N/A |
| 133 | Low | 10 | 15 |  | 3 |  | 64 QAM | 8@44 | N/A | N/A |
| 134 | Low | 10 | 15 |  | 12 |  | 256 QAM | Edge_1RB_Left | N/A | N/A |
| 135 | Low | 10 | Highest |  | 8 |  | 256 QAM | N/A | Edge_1RB_Left | Edge_1RB_Left |
| 136 | Low | 10 | Default |  | 8 |  | 256 QAM | Outer_Full | Outer_Full | Outer_Full |
| 137 | Low | 10 | Default |  | 6 |  | 256 QAM | 1@3 | 1@2 | 1@1 |
| 138 | Low | 10 | Default |  | 6 |  | 256 QAM | 43@9 | 19@5 | 8@2 |
| 139 | Low | 10 | Default |  | 4 |  | 256 QAM | 1@35 | 1@17 | 1@8 |
| 140 | Low | 10 | Default |  | 2 |  | 256 QAM | 17@35 | 5@19 | 3@8 |
| 141 | Low | 10 | Default |  | 5 |  | 256 QAM | Edge_1RB_Right | Edge_1RB_Right | Edge_1RB_Right |
| 142 | Low | 10 | 15 |  | 5 |  | 256 QAM | 1@40 | N/A | N/A |
| 143 | Low | 10 | 30 |  | 5 |  | 256 QAM | N/A | 1@20 | N/A |
| 144 | Low | 10 | 15 |  | 3 |  | 256 QAM | 8@44 | N/A | N/A |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-31: Test Configuration table for NS_07

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_07 |  |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | SCS 15 kHz |  | SCS 30 kHz |
| 1 | Low | Default | Default | N/A for A-MPR test cases | DFT-s OFDM | PI/2 BPSK |  | Edge_1RB_Left |  |
| 2 | Low | 5 MHz | Default |  |  | PI/2 BPSK |  | 7@6 | 4@3 |
| 3 | Low | 10 MHz | Default |  |  | PI/2 BPSK |  | 8@14 | 4@7 |
| 4 | Low | 5 MHz | Default |  |  | PI/2 BPSK |  | 10@8 | 5@4 |
| 5 | Low | 10 MHz | Default |  |  | PI/2 BPSK |  | 18@19 | 10@10 |
| 6 | High | 5 MHz | Default |  |  | PI/2 BPSK |  | 10@0 | 5@0 |
| 7 | Low | 5 MHz | Default |  |  | PI/2 BPSK |  | 2@19 | 1@10 |
| 8 | Low | 10 MHz | Default |  |  | PI/2 BPSK |  | 2@36 | 1@18 |
| 9 | Low | Default | Default |  |  | QPSK |  | Edge_1RB_Left |  |
| 10 | Low | 5 MHz | Default |  |  | QPSK |  | 7@6 | 4@3 |
| 11 | Low | 10 MHz | Default |  |  | QPSK |  | 8@14 | 4@7 |
| 12 | Low | 5 MHz | Default |  |  | QPSK |  | 10@8 | 5@4 |
| 13 | Low | 10 MHz | Default |  |  | QPSK |  | 18@19 | 10@10 |
| 14 | High | 5 MHz | Default |  |  | QPSK |  | 10@0 | 5@0 |
| 15 | Low | 5 MHz | Default |  |  | QPSK |  | 2@19 | 1@10 |
| 16 | Low | 10 MHz | Default |  |  | QPSK |  | 2@36 | 1@18 |
| 17 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |  |
| 18 | Low | 5 MHz | Default |  |  | 16 QAM |  | 7@6 | 4@3 |
| 19 | Low | 10 MHz | Default |  |  | 16 QAM |  | 8@14 | 4@7 |
| 20 | Low | 5 MHz | Default |  |  | 16 QAM |  | 10@8 | 5@4 |
| 21 | Low | 10 MHz | Default |  |  | 16 QAM |  | 18@19 | 10@10 |
| 22 | High | 5 MHz | Default |  |  | 16 QAM |  | 10@0 | 5@0 |
| 23 | Low | 5 MHz | Default |  |  | 16 QAM |  | 2@19 | 1@10 |
| 24 | Low | 10 MHz | Default |  |  | 16 QAM |  | 2@36 | 1@18 |
| 25 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |  |
| 26 | Low | 5 MHz | Default |  |  | 64 QAM |  | 7@6 | 4@3 |
| 27 | Low | 10 MHz | Default |  |  | 64 QAM |  | 8@14 | 4@7 |
| 28 | Low | 5 MHz | Default |  |  | 64 QAM |  | 10@8 | 5@4 |
| 29 | Low | 10 MHz | Default |  |  | 64 QAM |  | 18@19 | 10@10 |
| 30 | High | 5 MHz | Default |  |  | 64 QAM |  | 10@0 | 5@0 |
| 31 | Low | 5 MHz | Default |  |  | 64 QAM |  | 2@19 | 1@10 |
| 32 | Low | 10 MHz | Default |  |  | 64 QAM |  | 2@36 | 1@18 |
| 33 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |  |
| 34 | Low | 5 MHz | Default |  |  | 256 QAM |  | 7@6 | 4@3 |
| 35 | Low | 10 MHz | Default |  |  | 256 QAM |  | 8@14 | 4@7 |
| 36 | Low | 5 MHz | Default |  |  | 256 QAM |  | 10@8 | 5@4 |
| 37 | Low | 10 MHz | Default |  |  | 256 QAM |  | 18@19 | 10@10 |
| 38 | High | 5 MHz | Default |  |  | 256 QAM |  | 10@0 | 5@0 |
| 39 | Low | 5 MHz | Default |  |  | 256 QAM |  | 2@19 | 1@10 |
| 40 | Low | 10 MHz | Default |  |  | 256 QAM |  | 2@36 | 1@18 |
| 41 | Low | Default | Default |  | CP-s OFDM | QPSK |  | Edge_1RB_Left |  |
| 42 | Low | 5 MHz | Default |  |  | QPSK |  | 7@6 | QPSK |
| 43 | Low | 10 MHz | Default |  |  | QPSK |  | 8@14 | QPSK |
| 44 | Low | 5 MHz | Default |  |  | QPSK |  | 10@8 | QPSK |
| 45 | Low | 10 MHz | Default |  |  | QPSK |  | 18@19 | QPSK |
| 46 | High | 5 MHz | Default |  |  | QPSK |  | 10@0 | QPSK |
| 47 | Low | 5 MHz | Default |  |  | QPSK |  | 2@19 | QPSK |
| 48 | Low | 10 MHz | Default |  |  | QPSK |  | 2@36 | QPSK |
| 49 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |  |
| 50 | Low | 5 MHz | Default |  |  | 16 QAM |  | 7@6 | 16 QAM |
| 51 | Low | 10 MHz | Default |  |  | 16 QAM |  | 8@14 | 16 QAM |
| 52 | Low | 5 MHz | Default |  |  | 16 QAM |  | 10@8 | 16 QAM |
| 53 | Low | 10 MHz | Default |  |  | 16 QAM |  | 18@19 | 16 QAM |
| 54 | High | 5 MHz | Default |  |  | 16 QAM |  | 10@0 | 16 QAM |
| 55 | Low | 5 MHz | Default |  |  | 16 QAM |  | 2@19 | 16 QAM |
| 56 | Low | 10 MHz | Default |  |  | 16 QAM |  | 2@36 | 16 QAM |
| 57 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |  |
| 58 | Low | 5 MHz | Default |  |  | 64 QAM |  | 7@6 | 64 QAM |
| 59 | Low | 10 MHz | Default |  |  | 64 QAM |  | 8@14 | 64 QAM |
| 60 | Low | 5 MHz | Default |  |  | 64 QAM |  | 10@8 | 64 QAM |
| 61 | Low | 10 MHz | Default |  |  | 64 QAM |  | 18@19 | 64 QAM |
| 62 | High | 5 MHz | Default |  |  | 64 QAM |  | 10@0 | 64 QAM |
| 63 | Low | 5 MHz | Default |  |  | 64 QAM |  | 2@19 | 64 QAM |
| 64 | Low | 10 MHz | Default |  |  | 64 QAM |  | 2@36 | 64 QAM |
| 65 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |  |
| 66 | Low | 5 MHz | Default |  |  | 256 QAM |  | 7@6 | 256 QAM |
| 67 | Low | 10 MHz | Default |  |  | 256 QAM |  | 8@14 | 256 QAM |
| 68 | Low | 5 MHz | Default |  |  | 256 QAM |  | 10@8 | 256 QAM |
| 69 | Low | 10 MHz | Default |  |  | 256 QAM |  | 18@19 | 256 QAM |
| 70 | High | 5 MHz | Default |  |  | 256 QAM |  | 10@0 | 256 QAM |
| 71 | Low | 5 MHz | Default |  |  | 256 QAM |  | 2@19 | 256 QAM |
| 72 | Low | 10 MHz | Default |  |  | 256 QAM |  | 2@36 | 256 QAM |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2.3.4.1. As new tables are added to this section, these lines should always follow the tables

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-30.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.3.4.3.

6.2.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2.3.4.1-1 to Table 6.2.3.4.1-31. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

-------- Skipped unchanged clauses --------

6.2.3.4.3.34	Message contents exceptions for network signalled value "NS_56"

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.34-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_56"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_56) |  |  |

6.2.3.4.3.35	Message contents exceptions for network signalled value "NS_07"

1.	Information element additionalSpectrumEmission is set to NS_07. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.35-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_07"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_07) |  |  |

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable Table from table 6.2.3.5-1 to Table 6.2.3.5-36. The allowed A-MPR values specified in Table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

-------- Skipped unchanged clauses --------

Table 6.2.3.5-36: UE Power Class 3 test requirements (NS_56) for band n24

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 2 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 3 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 4 | 23 | 0.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 5 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 6 | 23 | 1 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 7 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 8 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 9 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 10 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 11 | 23 | 2 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 12 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 13 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 15 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 16 | 23 | 2.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 17 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 18 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 19 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 20 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 21 | 23 | 4.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 22 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 23 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 24 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 25 | 23 | 4.5 | 4 | 0 | 18.5 | 3.5 | 3 | 25+TT | 15-TT |
| 26 | 23 | 0.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 27 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 28 | 23 | 0.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 29 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 30 | 23 | 0.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 32 | 23 | 0.5 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 33 | 23 | 0.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 34 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 35 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 36 | 23 | 0.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 37 | 23 | 1 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 38 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 39 | 23 | 1 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 40 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 41 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 42 | 23 | 0 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 43 | 23 | 1 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 44 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 45 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 46 | 23 | 0 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 47 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 48 | 23 | 2 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 49 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 50 | 23 | 2 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 51 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 52 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 53 | 23 | 1 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 54 | 23 | 2 | 2 | 0 | 21 | 2 | 3 | 25+TT | 18-TT |
| 55 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 56 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 57 | 23 | 1 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 58 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 59 | 23 | 2.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 61 | 23 | 2.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 62 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 63 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 64 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 65 | 23 | 2.5 | 2 | 0 | 20.5 | 2.5 | 3 | 25+TT | 17.5-TT |
| 66 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 67 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 68 | 23 | 2.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 69 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 70 | 23 | 4.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 71 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 72 | 23 | 4.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 73 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 74 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 75 | 23 | 4.5 | 4 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 76 | 23 | 4.5 | 2 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 77 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 78 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 79 | 23 | 4.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 80 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 81 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 82 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 83 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 84 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 85 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 86 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 87 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 88 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 89 | 23 | 3 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 90 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 91 | 23 | 3.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 92 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 93 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 94 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 95 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 96 | 23 | 6.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 97 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 98 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 99 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 100 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 101 | 23 | 3 | 12 | 0 | 9 | 6 | 3 | 25+TT | 5-TT |
| 102 | 23 | 3 | 8 | 0 | 11 | 5 | 3 | 25+TT | 10-TT |
| 103 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 104 | 23 | 1.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 105 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 106 | 23 | 1.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 107 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 108 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 109 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 110 | 23 | 1.5 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 111 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 112 | 23 | 3 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 113 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 114 | 23 | 3 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 115 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 116 | 23 | 3 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 117 | 23 | 2 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 118 | 23 | 3 | 2 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 119 | 23 | 3 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 121 | 23 | 2 | 5 | 0 | 18 | 4 | 3 | 25+TT | 14-TT |
| 122 | 23 | 3 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 123 | 23 | 3.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 124 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 125 | 23 | 3.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 126 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 127 | 23 | 3.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 128 | 23 | 3.5 | 4 | 0 | 19 | 3.5 | 3 | 25+TT | 15.5-TT |
| 129 | 23 | 3.5 | 2 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 130 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 131 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 132 | 23 | 3.5 | 5 | 0 | 18 | 3.5 | 3 | 25+TT | 14.5-TT |
| 133 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 134 | 23 | 6.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 135 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 136 | 23 | 6.5 | 8 | 0 | 15 | 5 | 3 | 25+TT | 10-TT |
| 137 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 138 | 23 | 6.5 | 6 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 139 | 23 | 6.5 | 4 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 140 | 23 | 6.5 | 2 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 141 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 142 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 143 | 23 | 6.5 | 5 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 144 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-37: UE Power Class 3 test requirements (NS_07) for band n13

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 2 | 23 | 0 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 3 | 23 | 0 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 4 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 5 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 6 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 7 | 23 | 0 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 8 | 23 | 0 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 9 | 23 | 1 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 10 | 23 | 0 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 11 | 23 | 0 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 12 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 13 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 15 | 23 | 0 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 16 | 23 | 0 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 17 | 23 | 2 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 18 | 23 | 1 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 19 | 23 | 1 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 20 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 21 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 22 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 23 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 24 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 25 | 23 | 2.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 26 | 23 | 2.5 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 27 | 23 | 2.5 | 9 | 0 | 14 | 5 | 3 | 25+TT | 19-TT |
| 28 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 29 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 30 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 31 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 32 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 33 | 23 | 4.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 34 | 23 | 4.5 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 35 | 23 | 4.5 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 36 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 37 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 38 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 39 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 40 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 41 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 42 | 23 | 1.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 43 | 23 | 1.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 44 | 23 | 1.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 45 | 23 | 1.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 46 | 23 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 47 | 23 | 1.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 48 | 23 | 1.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 49 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 50 | 23 | 2 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 51 | 23 | 2 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 52 | 23 | 2 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 53 | 23 | 2 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 54 | 23 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 55 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 56 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 57 | 23 | 3.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 58 | 23 | 3.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 59 | 23 | 3.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 60 | 23 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 61 | 23 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 62 | 23 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 63 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 64 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 65 | 23 | 6.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 66 | 23 | 6.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 67 | 23 | 6.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 68 | 23 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 69 | 23 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 70 | 23 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 71 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 72 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

For the UE which supports inter-band NR CA configuration, inter-band NR-DC configuration, SUL configuration or inter-band EN-DC configuration, ΔTIB,c as specified in 6.2A.4.0.2 for NR CA, 6.2B.4.0.2 for NR-DC, clause 6.2C.2 for SUL, or TS 38.521-3 [14] clause 6.2B.4.2 for EN-DC applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations for CA, NR-DC, SUL or EN-DC, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2A.4.0.2, 6.2B.4.0.2, 6.2C.2 in this specification and 6.2B.4.2 in TS 38.521-3 [14] for the applicable operating bands.

-------- End of changes --------