3GPP TSG-RAN5 Meeting #99	R5-233030r1

# Incheon, Korea, May 22nd – 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0938 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to test applicability and side condition of beam correspondence |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The beam correspondence requirements are specifed for PC3 in Rel-15 and Rel-16. Based on UE’s capability, different test cases are expected to be excuted as below table. BC Without UL-BeamSweeping; SSB/CS-RS based enhanced BC; Rel-15; Rel-16. PC3 only; PC3 only. Support; Support; N/A; 6.2.1.x_1(EIRP+Spherical). Not support; 6.2.1.x(EIRP+Spherical); 6.2.1.x(EIRP+Spherical). Not Support; Support; N/A; 6.2.1.x_1(EIRP+Spherical) +6.6.2(BC tolerance). Not support; 6.2.1.x(EIRP+Spherical)+6.6.1(BC tolerance); 6.2.1.x(EIRP+Spherical)+6.6.1(BC tolerance). For PC1, PC2 and PC4, beam correspondence requirements are not specified, therefore only 6.2.1.x will always be excuted. The test applicability of existing test cases are not clear enough. PC1, PC2 and PC4 are not excluded from 6.2.1.x_1 and 6.6.2. Considering PC5, PC6 and PC7 would be introduced since Rel-17, it would be helpful to clearly indicate the applicable power classes for each test case. |
| Summary of change: |  | Update the test applicability of MOP and BC test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test applicability is not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1, 6.2.1.1_1, 6.2.1.2, 6.2.1.2_1, 6.6.1, 6.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Removing ‘either’ and ‘neither’ from applicability statements. |  |  |  |  |  |  |  |  |

<Start of Changes>

6.2.1.1	UE maximum output power - EIRP and TRP

Editor’s note: The following aspects are either missing or not yet determined:

-	Measurement Uncertainties and Test Tolerances are FFS for power class 1 FR2b, 2 and 4.

-	The test case is incomplete for band n259.

6.2.1.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2.1.1.2	Test applicability

This test case applies to all types of release 15 NR UEs, and all types of release 16 and forward NR Power Class 1, Power Class 2 and Power Class 4 UEs.

This test case also applies to all types of release 16 and forward NR Power Class 3 UEs not supporting CSI-RS based or SSB-based enhanced beam correspondence.

<Unchanged Text Skipped>

6.2.1.1_1	UE maximum output power - EIRP and TRP (Rel16 and forward)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	Same as in 6.2.1.1

6.2.1.1_1.1	Test purpose

Same as 6.2.1.1.1

6.2.1.1_1.2	Test applicability

This test case applies to all types of NR Power Class 3 UEs release 16 and forward supporting SSB-based or CSI-RS based enhanced beam correspondence.

6.2.1.1_1.3	Minimum conformance requirements

Same as 6.2.1.1.3 including UE multi-band relaxation factors defined for Rel-16 and forward UEs supporting power class 3, power class 5, power class 6 or power class 7.

6.2.1.1_1.4	Test description

<Unchanged Text Skipped>

6.2.1.2	UE maximum output power - Spherical coverage

Editor’s note: The following aspects are either missing or not yet determined:

- Measurement Uncertainties and Test Tolerances are FFS for power class 1, 2 and 4.

- The test case is incomplete for band n259.

6.2.1.2.1	Test purpose

To verify that the spatial coverage of the UE in expected directions is acceptable.

6.2.1.2.2	Test applicability

This test case applies to all types of release 15 NR UEs, and all types of release 16 and forward NR Power Class 1, Power Class 2 and Power Class 4 UEs.

This test case also applies to all types of release 16 and forward NR Power Class 3 UEs not supporting CSI-RS based or SSB-based enhanced beam correspondence.

6.2.1.2.3	Minimum conformance requirements

Minimum conformance requirements are defined in clause 6.2.1.1.3.

<Unchanged Text Skipped>

6.2.1.2_1	UE maximum output power - Spherical coverage (Rel16 and forward)

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

-	Same as in 6.2.1.2

6.2.1.2_1.1	Test purpose

Same as 6.2.1.2.1.

6.2.1.2.2	Test applicability

This test case applies to all types of NR Power Class 3 UE release 16 and forward supporting SSB-based or CSI-RS based enhanced beam correspondence.

6.2.1.2_1.3	Minimum conformance requirements

Same as 6.2.1.1_1.3

<Unchanged Text Skipped>

6.6.1	Beam correspondence - EIRP

Editor’s note: The following aspects are either missing or not yet determined:

- The test case is incomplete for band n259.

6.6.1.1	Test purpose

To verify the UE’s ability to select a suitable beam for UL transmission based on DL measurements with or without relying on UL beam sweeping within the range prescribed by the specified nominal maximum output power and beam correspondence tolerance.

6.6.1.2	Test applicability

This test case applies to all types of NR Power Class 3 UE release 15 that do not support beam correspondence without UL beam sweeping.

This test case applies to all types of NR Power Class 3 UE release 16 and forward that do not support SSB-based or CSI-RS based enhanced beam correspondence and do not support beam correspondence without UL beam sweeping.

<Unchanged Text Skipped>

6.6.2	Enhanced Beam correspondence – EIRP

6.6.2.1	Test purpose

To verify the UE’s ability to select a suitable beam for UL transmission based on DL measurements with or without relying on UL beam sweeping within the range prescribed by the specified nominal maximum output power and beam correspondence tolerance.

6.6.2.2	Test applicability

This test case applies to all types of NR UE Power Class 3 release 16 and forward that support CSI-RS or SSB based beam correspondence and do not support beam correspondence without UL beam sweeping.

< End of Changes >