| 3GPP TS 36.521-4 V0.2.0 (2023-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Satellite access Radio Frequency (RF) and performance Conformance Testing (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	9

4	General	10

4.1	Relationship between minimum requirements and test requirements	10

4.2	Applicability of minimum requirements	10

4.3	Specification suffix information	11

4.4	Relationship with core specifications	11

5	Operating bands and channel arrangement	11

5.1	General	11

5.2	Operating bands	11

5.2A	Operating bands for UE category M1	12

5.2B	Operating bands for category NB1 and NB2	12

5.3	Channel bandwidth	12

5.3A	Channel bandwidth for category M1	12

5.3B	Channel bandwidth for category NB1 and NB2	13

5.4	Channel arrangement	14

5.4A	Channel arrangement for category M1	14

5.4A.1	Channel spacing	14

5.4A.2	Channel raster, carrier frequency and EARFCN	14

5.4A.3	TX–RX frequency separation	15

5.4B	Channel arrangement for category NB1 and NB2	15

5.4B.1	Channel spacing	15

5.4B.2	Channel raster, carrier frequency and EARFCN	15

5.4B.3	TX–RX frequency separation	16

6	Transmitter characteristics	16

6.1	General	16

6.2	Transmit power	16

6.2A	Transmit power for category M1	16

6.2A.1	UE maximum output power for category M1	16

6.2A.2	UE maximum output power reduction for category M1	18

6.2A.3	UE additional maximum output power reduction for category M1 UE	21

6.2A.4	Configured transmitted Power for category M1	24

6.2B	Transmit power for category NB1 and NB2	27

6.2B.1	UE maximum output power for category NB1 and NB2	27

6.2B.2	UE maximum output power reduction for category NB1 and NB2	29

6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE	31

6.2B.4	Configured transmitted Power for category NB1 and NB2	34

6.3	Output power dynamics	37

6.3A	Output power dynamics for category M1	37

6.3A.1	UE Minimum output power for category M1	37

6.3A.2	Transmit OFF power for category M1	39

6.3A.3	ON/OFF time mask for category M1	40

6.3A.3.1	General ON/OFF time mask for category M1	40

6.3A.3.2	PRACH and SRS ON/OFF time mask for UE category M1	42

6.3A.3.2.1	PRACH time mask for UE category M1	42

6.3A.3.2.2	SRS time mask for UE category M1	45

6.3A.4	Power control for category M1	47

6.3A.4.1	Power Control Absolute power tolerance for UE category M1	47

6.3A.4.2	Power Control Relative power tolerance for UE category M1	50

6.3A.4.3	Aggregate power control tolerance for UE category M1	58

6.3B	Output power dynamics for category NB1 and NB2	62

6.3B.1	UE Minimum output power for category NB1 and NB2	62

6.3B.2	Transmit OFF power for category NB1 and NB2	64

6.3B.3	ON/OFF time mask for category NB1 and NB2	65

6.3B.3.1	General ON/OFF time mask for category NB1 and NB2	65

6.3B.3.2	NPRACH time mask for category NB1 and NB2	67

6.3B.4	Power Control for category NB1 and NB2	70

6.3B.4.1	Power Control Absolute power tolerance for category NB1 and NB2	70

6.3B.4.2 Power Control Relative power tolerance for category NB1 and NB2	74

6.3B.4.3 Aggregate power control tolerance for category NB1 and NB2	76

6.4	Transmit signal quality	81

6.4A	Transmit signal quality for category M1	81

6.4A.1	Frequency error for UE category M1	81

6.4A.2	Transmit modulation quality for category M1	81

6.4B	Transmit signal quality for category NB1 and NB2	81

6.4B.1	Frequency error for UE category NB1 and NB2	81

6.4B.2	Transmit modulation quality for Category NB1 and NB2	81

6.5	Output RF spectrum emissions	81

6.5A	Output RF spectrum emissions for category M1	82

6.5A.1	General	82

6.5A.2	Occupied bandwidth for category M1	82

6.5A.3	Out of band emission for category M1	82

6.5A.3.1	General	82

6.5A.3.2	Spectrum emission mask	82

6.5A.3.3	Additional Spectrum Emission Mask for category M1	82

6.5A.3.4	Adjacent Channel Leakage Ratio for category M1	82

6.5A.4	Spurious emission for category M1	82

6.5A.4.1	General	82

6.5A.4.2	Minimum requirements	82

6.5A.4.3	Spurious emission band UE co-existence	82

6.5A.4.4	Additional spurious emissions	83

6.5A.4.4.0	Minimum conformance requirements	83

6.5A.4.4.0.1	General	83

6.5A.4.4.0.2	Minimum requirement (network signalled value "NS_02N")	83

6.5A.4.4.0.3	Minimum requirement (network signalled value "NS_24")	83

6.5A.4.4.1	Additional spurious emissions for category M1	83

6.5B	Output RF spectrum emissions for category NB1 and NB2	83

6.5B.1	General	83

6.5B.2	Occupied bandwidth for category NB1 and NB2	83

6.5B.3	Out of band emission for category NB1 and NB2	83

6.5B.3.1	General	83

6.5B.3.2	Spectrum emission mask	83

6.5B.3.3	Additional Spectrum Emission Mask for category NB1 and NB2	83

6.5B.3.4	Adjacent Channel Leakage Ratio for category NB1 and NB2	83

6.5B.4	Spurious emission for category NB1 and NB2	84

6.5B.4.1	General	84

6.5B.4.2	Minimum requirements	84

6.5B.4.3	Spurious emission band UE co-existence	84

6.5B.4.4	Additional spurious emissions	84

6.5B.4.4.0	Minimum conformance requirements	84

6.5B.4.4.0.1	General	84

6.5B.4.4.0.2	Minimum requirement (network signalled value "NS_02N")	84

6.5B.4.4.0.3	Minimum requirement (network signalled value "NS_24")	84

6.5B.4.4.1	Additional spurious emissions for category NB1 and NB2	84

6.6	Transmit intermodulation	84

6.6A	Transmit intermodulation for category M1	84

6.6B	Transmit intermodulation for category NB1 and NB2	85

7	Receiver characteristics	85

7.1	General	85

7.2	Diversity characteristics	85

7.3	Reference sensitivity power level	85

7.3A	Reference sensitivity power level for UE category M1	85

7.3B	Reference sensitivity power level for UE category NB1 and NB2	85

7.4	Maximum input level	85

7.4A	Maximum input level for category M1	85

7.4B	Maximum input level for category NB1 and NB2	88

7.5	Adjacent Channel Selectivity (ACS)	89

7.5A	Adjacent Channel Selectivity for category M1	89

7.5B	Adjacent Channel Selectivity for category NB1 and NB2	93

7.6	Blocking characteristics	96

7.6A	Blocking characteristics for category M1	96

7.6A.1	General	96

7.6A.2	In-band blocking for category M1	96

7.6A.3	Out-of-band blocking for category M1	100

7.6A.4	Narrow band blocking for category M1	100

7.6B	Blocking characteristics for category NB1 and NB2	100

7.6B.1	General	100

7.6B.2	In-band blocking for category NB1 and NB2	100

7.6B.3	Out-of-band blocking for category NB1 and NB2	103

7.6B.4	Narrow band blocking for category NB1 and NB2	103

7.7	Spurious response	103

7.7A	Spurious response for category M1	103

7.7B	Spurious response for category NB1 and NB2	103

7.8	Intermodulation characteristics	103

7.8A	Intermodulation characteristics for category M1	103

7.8B	Intermodulation characteristics for category NB1 and NB2	103

7.9	Spurious emissions	104

7.9A	Spurious emissions for category M1	104

7.9B	Spurious emissions for category NB1 and NB2	105

8	Performance requirement	107

Annex A (reserved): Reserved	108

Annex B (informative): Change history	108

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part 1 of a multi-parts TS:

3GPP TS 36.521-1 [14]: Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification Radio transmission and reception; Part 1: Conformance Testing.

3GPP TS 36.521-2 [15]: Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification Radio transmission and reception; Part 2: Implementation Conformance Statement (ICS).

3GPP TS 36.521-3 [16]: Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification Radio transmission and reception; Part 3: Radio Resource Management (RRM) conformance testing.

3GPP TS 36.521-4: Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Satellite access Radio Frequency (RF) and performance Conformance Testing

# 1	Scope

The present document specifies the measurement procedures for the conformance test of E-UTRA User Equipment (UE) supporting satellite access operation that contains RF and Performance requirements. Conformance test for the support of RRM (Radio Resource Management) are specified in TS 36.521-3 [16].

The requirements are listed in different clauses only if the corresponding parameters deviate. More generally, tests are only applicable to those UEs that are intended to support the appropriate functionality. To indicate the circumstances in which tests apply, this is noted in the "definition" and "applicability" part of the test.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	[3]	3GPP TS 36.108: "Evolved Universal Terrestrial Radio Access (E-UTRA); Satellite Access Node (SAN) radio transmission and reception".

[3]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[4]	ITU-R Recommendation M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[5]	3GPP TS 36.307: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements on User Equipments (UEs) supporting a release-independent frequency band".

[6]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[7]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[8]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[9]	ITU-R Recommendation SM.329-10, "Unwanted emissions in the spurious domain".

[10]	[ANSI C63.26-2015, American National standard for Compliance Testing of Transmitters Used in Licensed Radio Services, Accredited Standards Committee C63 – Electromagnetic compatibility].

[11]	3GPP TS 36.102: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access".

[12]	3GPP TS 36.508: "Common test environments for User Equipment (UE)".

[13]	3GPP TS 36.509: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Special conformance testing functions for User Equipment (UE)".

[14]	3GPP TS 36.521-1: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance testing".

[15]	3GPP TS 36.521-2: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Implementation Conformance Statement (ICS)".

[16]	3GPP TS 36.521-3: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Radio Resource Management (RRM) conformance testing".

[17]	3GPP TR 36.904: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Derivation of test tolerances for User Equipment (UE) radio reception conformance tests".

[18]	3GPP TR 36.905: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Derivation of test points for radio transmission and reception conformance test cases".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Channel edge: The lowest and highest frequency of the carrier, separated by the channel bandwidth.

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Category NB1/NB2 stand-alone operation: category NB1/NB2 is operating standalone when it utilizes its own spectrum, for example the spectrum used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment.

Category NB1/NB2 guard band operation: category NB1/NB2 is operating in guard band when it utilizes the unused resource block(s) within a E-UTRA carrier’s guard-band.

Category NB1/NB2 in-band operation: category NB1/NB2 is operating in-band when it utilizes the resource block(s) within a normal E-UTRA carrier or within a normal NR carrier plus 15 kHz at each edge (and not within NR minimum guard band).

Geosynchronous Earth Orbit: Earth-centred orbit at approximately 35786 kilometres above Earth's surface and synchronised with Earth's rotation. A geostationary orbit is a non-inclined geosynchronous orbit, i.e. in the Earth’s equator plane.

Low Earth Orbit: Orbit around the Earth with an altitude between 300 km, and 1500 km.

Satellite: A space-borne vehicle embarking a bent pipe payload or a regenerative payload telecommunication transmitter, placed into Low-Earth Orbit (LEO), Medium-Earth Orbit (MEO), or Geostationary Earth Orbit (GEO).

Satellite Access Node: see definition in TS 36.108 [2].

sTTI: A transmission time interval (TTI) of either one slot or one subslot as defined in TS 36.211 [3] on either uplink or downlink.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔFRaster	Band dependent channel raster granularity

BWChannel	Channel bandwidth

F	Frequency

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer	Frequency of the interferer

FIoffset	Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

FC	Frequency of the carrier centre frequency

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FOOB	The boundary between the E-UTRA out of band emission and spurious emission domains.

LCtone	Transmission bandwidth which represents the length of a contiguous sub-carrier allocation expressed in units of tones

NDL 	Downlink EARFCN

NOffs-DL 	Offset used for calculating downlink EARFCN

NOffs-UL 	Offset used for calculating uplink EARFCN

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_alloc	Total number of simultaneously transmitted resource blocks in Channel bandwidth or Aggregated Channel Bandwidth.

Ntone	Transmission bandwidth configuration for category NB1 and NB2, expressed in units of tones.

Ntone 3.75kHz	Transmission bandwidth configuration for category NB1 and NB2 with 3.75 kHz sub-carrier spacing, expressed in units of tones.

Ntone 15kHz 	Transmission bandwidth configuration for category NB1 and NB2 with 15 kHz sub-carrier spacing, expressed in units of tones.

NUL	Uplink EARFCN.

PCMAX	The configured maximum UE output power.

PInterferer	Modulated mean power of the interferer

PPowerClass	PPowerClass is the nominal UE power (i.e., no tolerance).

PPowerClass_Default	PPowerClass_Default is the default nominal UE power (i.e., no tolerance) for the band.

PUMAX	The measured configured maximum UE output power.

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

ΔfOOB	Δ Frequency of Out Of Band emission

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

AWGN	Additive White Gaussian Noise

BW	Bandwidth

CW	Continuous Wave

DL	Downlink

EARFCN 	E-UTRA Absolute Radio Frequency Channel Number

E-UTRA 	Evolved UMTS Terrestrial Radio Access

EUTRAN	Evolved UMTS Terrestrial Radio Access Network

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

GEO	Geosynchronous Earth Orbit

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LEO	Low Earth Orbit

HD-FDD	Half- Duplex FDD

MEO	Medium Earth Orbit

MPR	Maximum Power Reduction

OCNG	OFDMA Channel Noise Generator

OFDMA	Orthogonal Frequency Division Multiple Access

OOB	Out-of-band

QAM	Quadrature Amplitude Modulation

RAN	Radio Access Network

RE	Resource Element

REFSENS	Reference Sensitivity power level

RF	Radio Frequency

UE	User Equipment

UL	Uplink

UMTS	Universal Mobile Telecommunications System

UTRA	UMTS Terrestrial Radio Access

UTRAN	UMTS Terrestrial Radio Access Network

# 4	General

## 4.1	Relationship between minimum requirements and test requirements

TS 36.102 [11] is a Single-RAT specification for satellite NR UE, covering RF characteristics and minimum performance requirements. Conformance to the TS 36.102 [11] is demonstrated by fulfilling the test requirements specified in the present document.

The Minimum Requirements given in TS 36.102 [11] makes no allowance for measurement uncertainty (MU). The present document defines test tolerances (TT). These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined by the various levels of “shared risk” principle as described below.

a)	Core specification value is not relaxed by any relaxation value (TT=0). For each single measurement, the probability of a borderline good UE being judged as FAIL equals the probability of a borderline bad UE being judged as PASS.

-	Test tolerances equal to 0 (TT=0) are considered in this specification.

b)	Core specification value is relaxed by a relaxation value (TT>0). For each single measurement, the probability of a borderline bad UE being judged as PASS is greater than the probability of a borderline good UE being judged as FAIL.

-	Test tolerances lower than measurement uncertainty and greater than 0 (0 < TT < MU) are considered in this specification.

-	Test tolerances up to measurement uncertainty (TT = MU) are considered in this specification which is also known as “Never fail a good DUT” principle.

c)	Core specification value is tightened by a stringent value (TT<0). For each single measurement, the probability of a borderline good UE being judged as FAIL is greater than the probability of a borderline bad UE being judged as PASS.

-	Test tolerances lower than 0 (TT<0) are not considered in this specification..

The “Never fail a good DUT” and the “Shared Risk” principles are defined in Recommendation ITU-R M.1545 [4].

## 4.2	Applicability of minimum requirements

a)	Minimum requirements are mandated to be met in all scenarios by UEs supporting the applicable UE category(ies) for which that requirement is specified. In TS 36.102 [11], only minimum requirements for UE categories of M1, NB1, and NB2 are specified.

b)	For UE category M1, the applicable minimum requirements in clauses 5, 6 or 7 are specified in the suffix A subclause where they differ from the requirements in the main subclause. Where suffix A does not exist for a requirement, the minimum requirement in the main subclause shall apply.

c)	For UE category NB1 and NB2, the applicable minimum requirements in clauses 5, 6 or 7 are specified in the Suffix B subclause, where they differ from the requirements in the main subclause. Where suffix B does not exist for a requirement, the minimum requirement in the main subclause shall apply.

d)	The reference sensitivity power levels defined in subclause 7.3 are valid for the specified reference measurement channels.

NOTE: Receiver sensitivity degradation may occur when:

1)	The UE simultaneously transmits and receives with bandwidth allocations less than the transmission bandwidth configuration (see Figure 5.3A-1 and Figure 5.3B-1), and

2)	Any part of the downlink transmission bandwidth is within an uplink transmission bandwidth from the downlink center subcarrier.

e)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal.

f)	The requirements related to subslot TTI and/or slot TTI shall apply only if UE supports multiple TTI patterns. And these requirements only apply to subslot and/or slot TTI configurations

g)	TS 36.307 [5] specifies which minimum requirements in the present document are applicable to UEs that conform to an earlier specification Release, and from which Release those requirements apply.

## 4.3	Specification suffix information

The following suffixes are defined at 2nd level for clauses 5, 6 and 7, as shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| A | Cat-M1 |
| B | NB1, NB2 |

The suffixes shall apply as defined in clause 4.2.

## 4.4	Relationship with core specifications

TS 36.102 [11] specifies the minimum RF and performance requirements for E-UTRA User Equipment (UE) operating satellite access. TS 36.108 [2] specifies the minimum RF and performance requirements of E-UTRA Satellite Access Node (SAN).

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future releases.

## 5.2	Operating bands

E-UTRA satellite access is designed to operate in the operating bands defined in Table 5.2-1.

Table 5.2-1 E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256 |  |  |  |  |  |  |  |

## 5.2A	Operating bands for UE category M1

UE category M1 is designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1 in both half duplex FDD mode and full-duplex FDD mode.

## 5.2B	Operating bands for category NB1 and NB2

Category NB1 and NB2 UE are designed to operate in the E-UTRA satellite access operating bands defined in Table 5.2-1.

Category NB1 and NB2 UE operate in HD-FDD duplex mode.

## 5.3	Channel bandwidth

This clause is reserved.

## 5.3A	Channel bandwidth for category M1

The requirements in present document are specified for the channel bandwidth listed in Table 5.3A-1.

Table 5.3A-1: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 |
| --- | --- |
| Transmission bandwidth configuration NRB | 6 |

Figure 5.3A-1 shows the relation between the Channel bandwidth (BWChannel) and the Transmission bandwidth configuration (NRB). The channel edges are defined as the lowest and highest frequencies of the carrier separated by the channel bandwidth, i.e. at FC +/- BWChannel /2.

Transmission

Center subcarrier (corresponds to DC in baseband) is not transmitted in downlink

Active Resource Blocks

Resource block

Transmission bandwidth configuration [NRB]

bandwidth [RB]

Channel bandwidth [MHz]

Figure 5.3A-1: Definition of channel bandwidth and transmission bandwidth configuration for one E-UTRA carrier

## 5.3B	Channel bandwidth for category NB1 and NB2

For category NB1 and NB2, requirements in present document are specified for the channel bandwidth listed in Table 5.3B-1.

Table 5.3B-1: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

Figure 5.3B-1 shows the relation between the Category NB1/NB2 channel bandwidth (BWChannel) and the Category NB1 /NB2 transmission bandwidth configuration (Ntone). The channel edges are defined as the lowest and highest frequencies of the carrier separated by the channel bandwidth, i.e. at FC +/- BWChannel /2.

Figure 5.3B-1: Definition of Channel Bandwidth and Transmission Bandwidth configuration

## 5.4	Channel arrangement

This clause is reserved.

## 5.4A	Channel arrangement for category M1

### 5.4A.1	Channel spacing

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between two adjacent E-UTRA carriers is defined as following:

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective E-UTRA carriers. The channel spacing can be adjusted to optimize performance in a particular deployment scenario.

### 5.4A.2	Channel raster, carrier frequency and EARFCN

The global frequency raster is defined for all frequencies. The granularity of the global frequency raster is 100 kHz, which means that the carrier centre frequency must be an integer multiple of 100 kHz. For each operating band, a subset of frequencies from the global frequency raster are applicable and forms a channel raster with a granularity ΔFRaster.

The carrier frequency in the uplink and downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143. The relation between EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL_low and NOffs-DL are given in Table 5.4A.2-1 and NDL is the downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL)

The relation between EARFCN and the carrier frequency in MHz for the uplink is given by the following equation where FUL_low and NOffs-UL are given in Table 5.4.2-1 and NUL is the uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL)

The applicable channel raster and EARFCNs for each operating band are specified in Table 5.4A.2-1.

For operating bands with a channel raster of 100 kHz, every EARFCN within the operating band shall be applicable for the channel raster, and the step size for the channel raster in Table 5.4A.2-1 is given as <1>. The broadcast parameter earfcn-LSB defined in TS 36.331 [6] may be used to assist the UE in synchronizing to the cell.

Table 5.4A.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 –<1>- 229375 | 1980 | 261844 | 261844 –<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 –<1>- 229075 | 1626.5 | 261504 | 261504 –<1>- 261843 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

### 5.4A.3	TX–RX frequency separation

The default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation is specified in Table 5.4A.3-1 for the TX and RX channel bandwidth defined in Table 5.3A.1-1.

Table 5.4A.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 256 | 190 MHz |
| 255 | -101.5 MHz |

## 5.4B	Channel arrangement for category NB1 and NB2

### 5.4B.1	Channel spacing

Nominal channel spacing for UE category NB1 and NB2 in stand-alone mode is 200 kHz.

### 5.4B.2	Channel raster, carrier frequency and EARFCN

The channel raster of UE category NB1/NB2 shall be as defined in clause 5.4A.2, and the channel raster per-frequency band shall be as defined in table 5.4A.2-1.

The carrier frequency of UE category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, -0.5, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.4A.2-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL)

The carrier frequency of UE category NB1/NB2 in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) as defined in Table 5.4A.2-1, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range of {-10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8, 9} for FDD. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.4A.2-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

NOTE 1:	Guard-band operation and in-band operation for NB-IoT are not supported in this version of the specification.

NOTE 2:	For the carrier including NPSS/NSSS for stand-alone operation, MDL = 0.

### 5.4B.3	TX–RX frequency separation

For UE category NB1/NB2 operation in stand-alone mode, the default TX-RX frequency separation shall be as specified in Table 5.4A.3-1 for the NB-IoT TX and RX channel bandwidth defined in Table 5.3B-1.

# 6	Transmitter characteristics

## 6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single transmit antenna. For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

The transient periods due to power steps, OFF/ON and ON/OFF transitions could occur at slot or symbol boundary with transients, on one or both sides of the boundary. The measurement period and whether to exclude the transient periods are specified in the respective sections below.

For testing of category NB1 in all operation bands, standalone is used as default operation mode unless otherwise stated by the test case.

## 6.2	Transmit power

## 6.2A	Transmit power for category M1

### 6.2A.1	UE maximum output power for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Annex F MU/TT is TBD

- Initial conditions and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

6.2A.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.2A.1.3	Minimum conformance requirements

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2A.1-1: UE Power Class

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 256 |  |  | 23 | +/-2 | 20 | +/-2 |
| 255 |  |  | 23 | +/-2 | 20 | +/-2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 6.2.A.

6.2A.1.4	Test description

6.2A.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on operating bands for satellite access that specified in sub-clause 5.2A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521-1[14] Annexe A.2. Configurations of PDSCH and MPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.2A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Ch BW | N/A for Max UE output power testing |  | Mod'n | RB allocation |
|  |  |  |  | HD-FDD |
| 1.4MHz |  |  | QPSK | 1 |
| 1.4MHz |  |  | QPSK | 2 |
| NOTE 1: Test Channel Bandwidth is checked separately for each E-UTRA satellite access band, the applicable channel bandwidths are specified in Table 5.3A-1. NOTE 2: The 1 RB allocation shall be tested at RB#0 with narrowband index 0 for low and mid range, RB #5 with max narrowband index for high range test frequency. NOTE 3: The RBstart of non-1RB allocation shall be RB #0 with narrowband index 0 for low and mid range, RB# (6 - RB allocation) with max narrowband index for high range test frequency. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2A.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.2A.4.3.

6.2A.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.2A.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

6.2A.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6.

6.2A.1.5	Test requirements

The maximum output power derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.5-1.

Table 6.2A.1.5-1: UE Power Class test requirements

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 256 |  |  | 23 | ±2.7 | 20 | ±2.7 |  |  |
| 255 |  |  | 23 | ±2.7 | 20 | ±2.7 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. |  |  |  |  |  |  |  |  |

### 6.2A.2	UE maximum output power reduction for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Annex F MU/TT is TBD

- Initial conditions and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

6.2A.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance covering configurations where a maximum power reduction is allowed in the UE.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2A.2.2	Test applicability

The requirements of this test apply in test case 6.5A.3.4 Adjacent Channel Leakage Power Ratio for UE category M1 to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.2A.2.3	Minimum conformance requirements

For UE Power Class 3 and 5 the allowed Maximum Power Reduction (MPR) for the maximum output power due to higher order modulation and transmit bandwidth configuration (resource blocks) is specified in Table 6.2A.2.3-1.

For subPRB allocation of category M1 UE of Power Class 3, there is no MPR applies.

Table 6.2A.2.3-1: Maximum Power Reduction (MPR) for Power Class 3 and 5

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) | MPR (dB) |
| --- | --- | --- |
|  | 1.4 MHz |  |
| QPSK | > 2 | ≤ 1 |
| QPSK | > 5 | ≤ 2 |
| NOTE: MPR only applicable for NRB ≥ 1 |  |  |

For PRACH, PUCCH and SRS transmissions, the allowed MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For each subframe, the MPR is evaluated per slot and given by the maximum value taken over the transmission(s) within the slot; the maximum MPR over the two slots is then applied for the entire subframe.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2A.4 apply.

The normative reference for this requirement is TS 36.102 [11] clause 6.2.A.

6.2A.2.4	Test description

6.2A.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.2A.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.2A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1 | 1.4MHz |  | QPSK | 2 |
| 2 | 1.4MHz |  | QPSK | 5 |
| 3 | 1.4MHz |  | QPSK | 6 |

Table 6.2A.2.4.1-2: Test Configuration Table, subPRB allocation

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | ½ (6 SCs) | 0 |
| High range |  |  |  |  |  |
| 2 | 1.4MHz |  | QPSK | ½ (6 SCs) | 3 |
| NOTE 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [3], 5.2.7. NOTE 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. NOTE 3: The SCstart shall be SC#0 and SC# (72 – RB allocation) of the narrowband, when RB allocation is defined as #SCs |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2A.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.2A.4.3.

7.	For UE supporting subPRB allocation, repeat step 1-6 with UL RMC according to Table 6.2A.2.4.1-2

6.2A.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.2A.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

6.2A.2.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the condition CEModeA.

6.2A.2.4.4	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2A.2.4.4-1

Table 6.2A.2.4.4-1: UE Power Class 3 and 5 test requirements

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2.0 | 25.7 | 20.3 | 0 | 20 | 2.0 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2.0 | 25.7 | 19.3 | 1 | 19 | 3.5 | 22.7 | 14.8 |
| 3 | 2 | 21 | 2.0 | 25.7 | 18.3 | 2 | 18 | 4.0 | 22.7 | 13.3 |
| NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.102 [11] clause 6.2A.4. |  |  |  |  |  |  |  |  |  |  |

Table 6.2A.2.4.4-1a: UE Power Class 3 test requirements for bands, subPRB allocation

| Configuration ID | Power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| 2 | 0 | 23 | 2.0 | 25.7 | 20.3 |
| NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.102 [11] clause 6.2A.4. |  |  |  |  |  |

### 6.2A.3	UE additional maximum output power reduction for category M1 UE

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Test Points analysis is TBD

- Addition to applicability spec is pending

- Test configuration is TBD

- Annex F MU/TT is TBD

6.2A.3.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2A.3.2	Test applicability

The requirements of this test apply in test case 6.5A.3.3 Additional Spectrum Emission Mask for network signalled values NS_02N and NS-24 to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

The requirements of this test apply in test case 6.5A.4.4 Additional Spurious Emissions for network signalled values NS_02N and NS_24 to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.2A.3.3	Minimum conformance requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2A.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2A.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2A.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | Table 5.3.1-1 | N/A |
| NS_02N | 6.5A.4.4.2 | 255 | Table 5.3.1-1 | N/A |
| NS_24 | 6.5A.4.4.3 | 256 | Table 5.3.1-1 | [3.5] |

For subPRB allocation, the allowed A-MPR values specified below in Table 6.2A.3-2 for category M1 UE are in addition to the allowed MPR requirements specified in subclause 6.2A.2.

Table 6.2A.3-2: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.5A.4.2 | Table 5.2-1 | N/A |
| NS_02N | 6.5A.4.4.2 | 255 | N/A |
| NS_24 | 6.5A.4.4.3 | 256 | [3.5] |

The normative reference for this requirement is TS 36.102 [11] subclause 6.2A.3

6.2A.3.4	Test description

6.2A.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in Table 6.2.4EA.4.1-1 to 6.2.4EA.4.1-20. The details of the uplink reference measurement channels (RMCs) are specified in TS36.521-1[14] Annex A.2. Configurations of PDSCH and MPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.2A.3.4.1-1: Test Configuration Table (network signalled value “NS_02N”)

FFS

Table 6.2A.3.4.1-1a: Test Configuration Table, subPRB allocation (network signalled value “NS_02N”)

FFS

Table 6.2A.3.4.1-2: Test Configuration Table (network signalled value "NS_24")

FFS

Table 6.2A.3.4.1-2a: Test Configuration Table, subPRB allocation (network signalled value "NS_24")

FFS

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2A.3.4.1-1 to Table 6.2A.3.4.1-2.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.2A.3.4.3.

6.2A.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via M-PDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2A.3.4.1-1 to Table 6.2A.3.4.1-2. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

4.	For UE supporting subPRB allocation, repeat step 1-3 with UL RMC according to applicable Table from Table 6.2A.3.4.1-1a to Table 6.2A.3.4.1-2a.

6.2A.3.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the condition CEModeA and with the following exceptions for each network signalled value.

6.2A.3.4.3.1	Message contents exceptions (network signalled value “NS_02N”)

1.	Information element additionalSpectrumEmission is set to NS_02N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for “NS_02N”

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2N |  |  |

6.2A.3.4.3.2	Message contents exceptions (network signalled value "NS_24")

1.	Information element additionalSpectrumEmission is set to NS_24. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2A.3.4.3.2-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_24"

| Derivation Path: TS 36.508 [12] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 24 (NS_24) |  |  |

6.2A.3.5	Test requirements

The maximum output power derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4EA.5-1 to Table 6.2.4EA.5-22. The allowed A-MPR values specified in Table 6.2.4EA-1 are in addition to the allowed MPR requirements specified in clause 6.2.3 EA. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5 EA.3-1 and 6.2.5EA.3-2 apply.

Table 6.2A.3.5-1: UE Power Class test requirements (network signalled value “NS_02N”)

FFS

Table 6.2A.3.5-1a: UE Power Class test requirements, subPRB allocation (network signalled value “NS_02N”)

FFS

Table 6.2A.3.5-2: UE Power Class test requirements (network signalled value "NS_24")

FFS

Table 6.2A.3.5-1a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_24")

FFS

### 6.2A.4	Configured transmitted Power for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Annex F MU/TT is TBD

- Initial conditions and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

6.2A.4.1	Test purpose

To verify the UE does not exceed the minimum between the PEMAX maximum allowed UL TX Power signalled by the E-UTRAN and the PUMAX maximum UE power the UE power class.

6.2A.4.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward that support satellite access operation.

6.2A.4.3	Minimum conformance requirements

The configured transmitted power requirements in clause 6.2.5 of TS 36.101 [7] shall apply, wherein

-	The Maximum output power requirements are specified in TS 36.102 [11] subclause 6.2A.1

-	The MPR requirements are specified in TS 36.102 [11] subclause 6.2A.2

-	The A-MPR requirements are specified in TS 36.102 [11] subclause 6.2A.3.

6.2A.4.4	Test description

6.2A.4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.2A.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.2A.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  | 1.4MHz |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |  |
| Ch BW | N/A for Configured UE transmitted Output Power test case |  | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1.4MHz |  |  | QPSK | 1 (Note 3), 2 (Note 4) | 0 |
| NOTE 1: Denote where in the channel bandwidth the narrowband shall be placed. Narrowband and narrowband index are defined in TS 36.211[3], 5.2.4. NOTE 2: The RBstart of partial RB allocation shall be RB#0 of the narrowband. NOTE 3: Only applicable for Power class 3. NOTE 4: Only applicable for Power class 5. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A, in Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to TS 36.521-1[14] Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2A.4.4.1-1

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.2A.4.4.3.

6.2A.4.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.2A.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send transmit uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach the Pumax level of the test point.

3.	Measure the mean power of the UE in the channel bandwidth for each test point in table 6.2A.4.5-1 for power class 3 and table 6.2A.4.5-2 for power class 5 according to the test configuration from Table 6.2A.4.4.1-1. The period of measurement shall be at least continuous duration of one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

6.2A.4.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the following exceptions

Table 6.2A.4.4.3-1: SystemInformationBlockType1-BR-r13: Test point 1 for power class 3

| Derivation Path: TS 36.508 [12] clause 4.4.3.2, Table 4.4.3.2-3A SystemInformationBlockType1-BR-r13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2A.4.4.3-2: SystemInformationBlockType1-BR-r13: Test point 2 for power class 3

| Derivation Path: TS 36.508 [12] clause 4.4.3.2, Table 4.4.3.2-3A SystemInformationBlockType1-BR-r13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2A.4.4.3-3: SystemInformationBlockType1-BR-r13: Test point 3 for power class 3

| Derivation Path: TS 36.508 [12] clause 4.4.3.2, Table 4.4.3.2-3A SystemInformationBlockType1-BR-r13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 |  |  |

Table 6.2A.4.4.3-4: SystemInformationBlockType1-BR-r13: Test point 1 for power class 5

| Derivation Path: TS 36.508 [12] clause 4.4.3.2, Table 4.4.3.2-3A SystemInformationBlockType1-BR-r13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2A.4.4.3-5: SystemInformationBlockType1-BR-r13: Test point 2 for power class 5

| Derivation Path: TS 36.508 [12] clause 4.4.3.2, Table 4.4.3.2-3A SystemInformationBlockType1-BR-r13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 7 |  |  |

Table 6.2A.4.4.3-6: SystemInformationBlockType1-BR-r13: Test point 3 for power class 5

| Derivation Path: TS 36.508 [12] clause 4.4.3.2, Table 4.4.3.2-3A SystemInformationBlockType1-BR-r13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 12 |  |  |

6.2A.4.5	Test requirements

The maximum output power measured shall not exceed the values specified in Table 6.2A.4.5-1 for power class 3 and Table 6.2A.4.5-2 for power class 5.

Table 6.2A.4.5-1: PCMAX configured UE output power for UE category M1 power class 3

|  | Channel bandwidth / maximum output power |
| --- | --- |
|  | 1.4MHz |
| Measured UE output power test point 1 | For carrier frequency f ≤ 3.0GHz: -10 dBm ± 7.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: -10 dBm ± 8.0 |
| Measured UE output power test point 2 | For carrier frequency f ≤ 3.0GHz: 10 dBm ± 6.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: 10 dBm ± 7.0 |
| Measured UE output power test point 3 | For carrier frequency f ≤ 3.0GHz: 15 dBm ± 5.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: 15 dBm ± 6.0 |

Table 6.2A.4.5-2: PCMAX configured UE output power for UE category M1 power class 5

|  | Channel bandwidth / maximum output power |
| --- | --- |
|  | 1.4MHz |
| Measured UE output power test point 1 | For carrier frequency f ≤ 3.0GHz: -10 dBm ± 7.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: -10 dBm ± 8.0 |
| Measured UE output power test point 2 | For carrier frequency f ≤ 3.0GHz: 7 dBm ± 7.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: 7 dBm ± 8.0 |
| Measured UE output power test point 3 | For carrier frequency f ≤ 3.0GHz: 12 dBm ± 6.7 For carrier frequency 3.0GHz < f ≤ 4.2GHz: 12 dBm ± 7.0 |

## 6.2B	Transmit power for category NB1 and NB2

### 6.2B.1	UE maximum output power for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Annex F MU/TT is TBD

- Initial conditions and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

6.2B.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2B.1.2	Test applicability

This test case applies to all types of NB-IoT UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.2B.1.3	Minimum conformance requirements

Category NB1 and NB2 UE Power Classes are specified in Table 6.2B.1.3-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is at least one sub-frame (1ms).

Table 6.2B.1.3-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |
| 255 | 23 | +/-2 | 20 | +/-2 |

The normative reference for this requirement is TS 36.102 [11] clause 6.2B.1.

6.2B.1.4	Test description

6.2B.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT NTN in clause 5.2B. All of these configurations shall be tested with applicable test parameters and are shown in table 6.2B.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521 Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521 Annex C.2.

Table 6.2B.1.4.1-1: Test Configuration Initial Conditions for FDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 2) |  | BPSK | 1@0 | 3.75 |
| 2 (Note 3) |  | BPSK | 1@47 | 3.75 |
| 3 (Note 2) |  | QPSK | 1@0 | 15 |
| 4 (Note 3) |  | QPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 3@3 | 15 |
| NOTE 1: Applicable to UE supporting UL multi-tone transmissions. NOTE 2: only applicable for low range. NOTE 3: only applicable for high range. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521 Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channel is set according to Table 6.2B.1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.2B.1.4.3.

6.2B.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via NPDCCH with DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.2B.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC (UE should be already transmitting PUMAX after Initial Conditions setting).

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms) for sub-carrier spacing of 15 kHz or one slot (2ms) excluding the 2304Ts gap when UE is not transmitting for sub-carrier spacing of 3.75 kHz. For TDD slots with transient periods are not under test. For Half-Duplex guard subframes are not under test.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.2B.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6.

6.2B.1.5	Test requirements

The maximum output power derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2B.1.5-1.

Table 6.2B.1.5-1: UE Power Class test requirement

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| 256 | 23 | +/-2 | 20 | +/-2 |
| 255 | 23 | +/-2 | 20 | +/-2 |

### 6.2B.2	UE maximum output power reduction for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Annex F MU/TT is TBD

- Initial conditions and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

6.2B.2.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance covering configurations where a maximum power reduction is allowed in the UE.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2B.2.2	Test applicability

The requirements of this test apply in test case 6.6.2.3F Adjacent Channel Leakage power Ratio for category NB1 and NB2 to all types of NB-IoT FDD UE release 17 and forward of category NB2 that support satellite access operation.

6.2B.2.3	Minimum conformance requirements

For UE category NB1 power class 3 and 5 the allowed Maximum Power Reduction (MPR) for the maximum output power given in Table 6.2B.1.3-1 is specified in Table 6.2B.2.3-1.

Table 6.2B.2.3-1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 3 and 5

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 6.2B.2

6.2B.2.4	Test description

6.2B.2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT NTN in clause 5.2B. All of these configurations shall be tested with applicable test parameters, and are shown in table 6.2B.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521 Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521 Annex C.2.

Table 6.2B.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75 |
| 2 |  | QPSK | 1@47 | 3.75 |
| 3 |  | BPSK | 1@0 | 15 |
| 4 |  | BPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 3@0 | 15 |
| 6 (Note 1) |  | QPSK | 3@3 | 15 |
| 7 (Note 1) |  | QPSK | 3@9 | 15 |
| 8 (Note 1) |  | QPSK | 6@0 | 15 |
| 9 (Note 1) |  | QPSK | 6@6 | 15 |
| 10 (Note 1) |  | QPSK | 12@0 | 15 |
| NOTE 1: Applicable to UE supporting UL multi-tone transmissions. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0 of TS 36.521-1[14].

4.	The UL Reference Measurement channels are set according to Table 6.2B.2.4.1-1.

5.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.2.3F.4.3.

6.2B.2.4.2	Test procedure

1.	SS sends uplink scheduling information via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.2B.2.4.1-1 and with scheduling pattern and repetitions according to TS 36.521-1[14] Annex A.2. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms) for 15 kHz sub-carrier spacing, or for 3.75 kHz sub-carrier spacing at least one slot (2 ms) excluding the 2304Ts gap when UE is not transmitting. For TDD slots with transient periods are not under test.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.2B.2.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6.

6.2B.2.4.4	Test requirements

The maximum output power derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2B.2.4-1.

Table 6.2B.2.4-1: Maximum Power Reduction test requirements Power Class 3 and 5

| Test ID | MPR (dB) | Power class 3 |  |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX,c (dBm) |  | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |  | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| 1 | 0 |  | 23 | 2.0 | 25.7 |  | 20.3 | 20 | 2.0 | 22.7 | 17.3 |
| 2 | 0 |  | 23 | 2.0 | 25.7 |  | 20.3 | 20 | 2.0 | 22.7 | 17.3 |
| 3 | 0 |  | 23 | 2.0 | 25.7 |  | 20.3 | 20 | 2.0 | 22.7 | 17.3 |
| 4 | 0 |  | 23 | 2.0 | 25.7 |  | 20.3 | 20 | 2.0 | 22.7 | 17.3 |
| 5 | 0.5 |  | 22.5 | 2.0 | 25.7 |  | 19.8 | 19.5 | 2.0 | 22.7 | 16.8 |
| 6 | 0 |  | 23 | 2.0 | 25.7 |  | 20.3 | 20 | 2.0 | 22.7 | 17.3 |
| 7 | 0.5 |  | 22.5 | 2.0 | 25.7 |  | 19.8 | 19.5 | 2.0 | 22.7 | 16.8 |
| 8 | 1 |  | 22 | 2.0 | 25.7 |  | 19.3 | 19 | 2.0 | 22.7 | 16.3 |
| 9 | 1 |  | 22 | 2.0 | 25.7 |  | 19.3 | 19 | 2.0 | 22.7 | 16.3 |
| 10 | 2 |  | 21 | 2.0 | 25.7 |  | 18.3 | 18 | 2.0 | 22.7 | 15.3 |
| NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5F |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.2B.3	UE additional maximum output power reduction for category NB1 and NB2 UE

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Annex F MU/TT is TBD

- Test requirement is TBD

- Test Points analysis is TBD

-Initial conditions and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

6.2B.3.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2B.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2B.3.2	Test applicability

The requirements of this test apply in test case 6.5A.4.4 Additional Spurious Emissions for network signalled values NS_02N and NS_24 to all types of NB-IOT NTN UE release 17 and forward of UE category NB1 and NB2 that support satellite access operation.

6.2B.3.3	Minimum conformance requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2B.1-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2B.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2B.3.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2B.2.

Table 6.2B.3.3-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | NB-IOT NTN Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.5B.4.2 | Table 5.2-1 | N/A |
| NS_02N | 6.5B.4.4.2 | 255 | N/A |
| NS_24 | 6.5B.4.4.3 | 256 | [3.5] |

The normative reference for this requirement is TS 36.102 [11] subclause 6.2B.3

6.2B.3.4	Test description

6.2B.3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NB-IOT NTN operating bands specified in sub-clause 5.2B. All these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in Table 6.2B.3.4.1-1 to 6.2B.3.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in TS36.521-1[14] Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.2B.3.4.1-1: Test Configuration Table (network signalled value “NS_02N”)

FFS

Table 6.2B.3.4.1-2: Test Configuration Table (network signalled value "NS_24")

FFS

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2B.3.4.1-1 to Table 6.2B.3.4.1-2.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause8.1.5. Message contents are defined in clause 6.2B.3.4.3.

6.2B.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via N-PDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2B.3.4.1-1 to Table 6.2B.3.4.1-2. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms) for 15 kHz sub-carrier spacing, or for 3.75 kHz sub-carrier spacing at least one slot (2 ms) excluding the 2304Ts gap when UE is not transmitting. For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

6.2B.3.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 and with the following exceptions for each network signalled value.

6.2B.3.4.3.1	Message contents exceptions (network signalled value “NS_02N”)

1.	Information element additionalSpectrumEmission is set to NS_02N. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.1-1: SystemInformationBlockType2: Additional spurious emissions test requirement for “NS_02N”

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2N |  |  |

6.2B.3.4.3.2	Message contents exceptions (network signalled value "NS_24")

1.	Information element additionalSpectrumEmission is set to NS_24. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2B.3.4.3.2-1: SystemInformationBlockType2: Additional spurious emissions test requirement for "NS_24"

| Derivation Path: TS 36.508 [12] clause 8.1.4.3, Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 24 (NS_24) |  |  |

6.2B.3.5	Test requirement

The maximum output power derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2B.3.5-1 to Table 6.2B.3.5-2. The allowed A-MPR values specified in Table 6.2B.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2B.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in TS.36.521-1[14] Table 6.2.5 F.3-1 and 6.2.5F.3-2 apply.

Table 6.2B.3.5-1: UE Power Class test requirements (network signalled value “NS_02N”)

FFS

Table 6.2B.3.5-2: UE Power Class test requirements (network signalled value "NS_24")

FFS

### 6.2B.4	Configured transmitted Power for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Annex F MU/TT is TBD

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

6.2B.4.1	Test purpose

To verify the UE does not exceed the minimum between the PEMAX maximum allowed UL TX Power signalled by the E-UTRAN and the PUMAX maximum UE power for the UE power class.

6.2B.4.2	Test applicability

This test case applies to all types of NB-IoT UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.2B.4.3	Minimum conformance requirements

For category M1 UE, the configured transmitted power requirements in clause 6.2.5F of TS 36.101 [7] shall apply, wherein

-	The Maximum output power requirements are specified in TS 36.102 [11] subclause 6.2B.1

-	The MPR requirements are specified in TS 36.102 [11] subclause 6.2B.2

-	The A-MPR requirements are specified in TS 36.102 [11] subclause 6.2B.3.

6.2B.4.4	Test description

6.2B.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.2B.4.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521-1[14] A.2.4. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.2B.4.4.1-1: Test Configuration Table for FDD & TDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75 |
| 2 |  | QPSK | 1@47 | 3.75 |
| 3 |  | QPSK | 1@0 | 15 |
| 4 |  | QPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 12@0 | 15 |
| NOTE 1: Applicable to UE supporting UL multi-tone transmissions. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A, in Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex H of TS 36.521-1 [14].

4.	The UL Reference Measurement channel is set according to Table 6.2B.4.4.1-1

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.2B.4.4.3.

6.2B.4.4.2	Test procedure

1.	SS sends uplink scheduling information for UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.2B.4.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Configure UE to transmit NPUSCH with the Pumax level of each test points.

3.	Measure the mean power of the UE in the channel bandwidth for each test point in table 6.2B.4.4-1 according to the test configuration from table 6.2B.4.4-1. The period of measurement shall be at least continuous duration of one sub-frame (1ms) for 15 KHz channel spacing, and at least a 2ms slot (excluding the 2304Ts gap when UE is not transmitting) respectively for the 3.75 KHz channel spacing. For TDD slots with transient periods are not under test.

6.2B.4.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 with the following exceptions:

Table 6.2B.4.4.3-1: SystemInformationBlockType1: Test point 1 for power class 3

| Derivation Path: TS 36.508 [12] clause 8.1.4, Table 8.1.4.3.2-3 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2B.4.4.3-2: SystemInformationBlockType1: Test point 2 for power class 3

| Derivation Path: TS 36.508 [12] clause 8.1.4, Table 8.1.4.3.2-3 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 10 |  |  |

Table 6.2B.4.4.3-3: SystemInformationBlockType1: Test point 3 for power class 3

| Derivation Path: TS 36.508 [12] clause 8.1.4, Table 8.1.4.3.2-3 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 15 |  |  |

Table 6.2B.4.4.3-4: SystemInformationBlockType1: Test point 1 for power class 5

| Derivation Path: TS 36.508 [12] clause 8.1.4, Table 8.1.4.3.2-3 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | -10 |  |  |

Table 6.2B.4.4.3: SystemInformationBlockType1: Test point 2 for power class 5

| Derivation Path: TS 36.508 [12] clause 8.1.4, Table 8.1.4.3.2-3 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 7 |  |  |

Table 6.2B.4.4.3: SystemInformationBlockType1: Test point 3 for power class 5

| Derivation Path: TS 36.508 [12] clause 8.1.4, Table 8.1.4.3.2-3 SystemInformationBlockType1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 12 |  |  |

6.2B.5	Test requirement

The maximum output power measured shall not exceed the values specified in Table 6.2B.5-1.

Table 6.2B.5-1: PCMAX configured UE output power for power class 3

|  | maximum output power |
| --- | --- |
| Measured UE output power test point 1 | For carrier frequency f ≤ 3.0GHz: -10 dBm ± 7.7 |
| Measured UE output power test point 2 | For carrier frequency f ≤ 3.0GHz: 10 dBm ± 6.7 |
| Measured UE output power test point 3 | For carrier frequency f ≤ 3.0GHz: 15 dBm ± 5.7 |

Table 6.2B.5-2: PCMAX configured UE output power for power class 5

|  | maximum output power |
| --- | --- |
| Measured UE output power test point 1 | For carrier frequency f ≤ 3.0GHz: -10 dBm ± 7.7 |
| Measured UE output power test point 2 | For carrier frequency f ≤ 3.0GHz: 7 dBm ± 6.7 |
| Measured UE output power test point 3 | For carrier frequency f ≤ 3.0GHz: 12 dBm ± 5.7 |

## 6.3	Output power dynamics

This clause is reserved.

## 6.3A	Output power dynamics for category M1

### 6.3A.1	UE Minimum output power for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3A.1.1	Test purpose

To verify the UE's ability to transmit with a broadband output power below the value specified in the test requirement when the power is set to a minimum value.

6.3A.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.3A.1.3	Minimum conformance requirements

The minimum output power is defined as the mean power in one sub-frame (1ms). The minimum output power shall not exceed the values specified in Table 6.3A.1-1.

Table 6.3A.1-1: Minimum output power

|  | Channel bandwidth / minimum output power / measurement bandwidth |
| --- | --- |
|  | 1.4MHz |
| Minimum output power | -40 dBm |
| Measurement bandwidth | 1.08 MHz |

The normative reference for this requirement is TS 36.102 [11] clause 6.3A.1.

Minimum output power test verifies the UE's ability to transmit with a broadband output power below the specified limit when the power is set to a minimum value. The broadband output power is defined as the power in the channel bandwidth, for all transmit bandwidth configurations (resource blocks).

An excess minimum output power potentially increases the Rise Over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3A.1.4	Test description

6.3A.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on operating bands for satellite access that specified in table 5.2-1. All these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3A.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521-1[14] Annex A.2. Configurations of PDSCH and MPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.3A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Ch BW | N/A for min output power test |  | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1.4MHz |  |  | QPSK | 1 |
|  |  |  |  | 2 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.3A.1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.3A.1.4.3.

6.3A.1.4.2	Test procedure

1.	The SS sends uplink scheduling information via MPDCCH DCI format 6-0A with TPC command 0dB for C_RNTI to schedule the UL RMC according to Table 6.3A.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuous uplink power control "down" commands in the uplink scheduling information to the UE to ensure that the UE transmits at its minimum output power.

3.	Measure the mean power of the UE in the associated measurement bandwidth specified in Table 6.3A.1.5-1 for the specific channel bandwidth under test. The period of measurement shall be the continuous duration of one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

6.3A.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6

6.3A.1.5	Test requirement

The minimum output power measured shall not exceed the values specified in Table 6.3A.1.5-1.

Table 6.3A.1.5-1: Minimum output power for UE category M1

|  | Channel bandwidth / minimum output power / measurement bandwidth |
| --- | --- |
|  | 1.4 MHz |
| Minimum output power | For carrier frequency f ≤ 3.0GHz: ≤ -39 dBm For carrier frequency 3.0GHz < f ≤ 4.2GHz: ≤ -38.7 dBm |
| Measurement bandwidth (Note 1) | 1.08 MHz |
| Note 1: Different implementations such as FFT or spectrum analyzer approach are allowed. For spectrum analyzer approach the measurement bandwidth is defined as an equivalent noise bandwidth. |  |

### 6.3A.2	Transmit OFF power for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3A.2.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

6.3A.2.2	Test applicability

The requirements of this test apply in test cases 6.3A.3.1 ON/OFF time mask and 6.3A.3.2 PRACH and SRS time mask to all types of E-UTRA UE release 17 and forward that support satellite access operation.

6.3A.2.3	Minimum conformance requirements

The minimum output power is defined as the mean power in one sub-frame (1ms). The minimum output power shall not exceed the values specified in Table 6.3A.2.3-1.

Table 6.3A.2.3-1: Minimum output power

|  | Channel bandwidth / minimum output power / measurement bandwidth |
| --- | --- |
|  | 1.4MHz |
| Minimum output power | -50 dBm |
| Measurement bandwidth | 1.08 MHz |

The normative reference for this requirement is TS 36.102 [11] clause 6.3A.2.

Transmit OFF power is defined as the mean power when the transmitter is OFF. The transmitter is considered to be OFF when the UE is not allowed to transmit or during periods when the UE is not transmitting a sub-frame. During DTX and measurements gaps, the UE is not considered to be OFF.

An excess transmits OFF power potentially increases the Rise over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3A.2.4	Test description

This test is covered by clause 6.3A.3.1 ON/OFF time mask and 6.3A.3.2 PRACH and SRS time mask.

6.3A.2.5	Test requirement

The minimum output power measured shall not exceed the values specified in Table 6.3A.2.5-1.

Table 6.3A.2.5-1: Minimum output power for UE category M1

|  | Channel bandwidth / minimum output power / measurement bandwidth |
| --- | --- |
|  | 1.4 MHz |
| Minimum output power | For carrier frequency f ≤ 3.0GHz: ≤ -48.5 dBm For carrier frequency 3.0GHz < f ≤ 4.2GHz: ≤ -48.2 dBm |
| Measurement bandwidth | 1.08 MHz |

### 6.3A.3	ON/OFF time mask for category M1

### 6.3A.3.1	General ON/OFF time mask for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3A.3.1.1	Test purpose

To verify that the general ON/OFF time mask meets the requirements given in TS 36.521-1[14] clause 6.3.4.1.5.

The time mask for transmit ON/OFF defines the ramping time allowed for the UE between transmit OFF power and transmit ON power.

Transmission of the wrong power increases interference to other channels or increases transmission errors in the uplink channel.

6.3A.3.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.3A.3.1.3	Minimum conformance requirements

The requirements for transmit ON/OFF time mask defined in clause 6.3.4 of TS 36.101 [7] shall apply.

6.3A.3.1.4	Test description

6.3A.3.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in clause 5.2A. All these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3A.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521-1[14] Annexes A.2. Configurations of PDSCH and MPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.3A.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  | 1.4 MHz |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Ch BW | N/A for General On/Off Time Mask test case |  | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1.4MHz |  |  | QPSK | 6 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.3A.3.1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.3A.3.4.3. Note that MPDCCH DCI format 6-0A sent after resetting uplink power with RRC Connection Reconfiguration, should have TPC command 0dB.

6.3A.3.1.4.2	Test procedure

1.	The SS sends uplink scheduling information via MPDCCH DCI format 6-0A with TPC command 0dB for C_RNTI to schedule the UL RMC according to Table 6.3A.3.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. For FDD: The UL assignment is such that the UE transmits on UL sub-frame 2 of every radio frame. For HD-FDD: The UL assignment is such that the UE transmits on UL sub-frame 5 of every radio frame.

2.	For FDD and HD-FDD: Measure the UE transmission OFF power during the sub-frame prior to the PUSCH subframe.

3.	Measure the output power of the UE PUSCH transmission during one sub-frame, excluding a transient period of 20 µs at the beginning of the subframe.

4.	Measure the UE transmission OFF power during one sub-frame following the PUSCH subframe, excluding a transient period of 20 µs at the beginning of the subframe.

6.3A.3.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the condition CEModeA and the following exceptions:

Table 6.3A.3.1.4.3-1: UplinkPowerControlCommon: Test point 1

| Derivation Path: TS 36.508 [12] clause 4.6.3, Table 4.6.3-25 UplinkPowerControlCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-DEFAULT ::= SEQUENCE { p0-NominalPUSCH | -105 | Test point 1 to verify a UE relative low initial power transmission |  |

Table 6.3A.3.1.4.3-2: PhysicalConfigDedicated

| Derivation Path: TS 36.508 [12] clause 5.5.1.2, Table 5.5.1.2.1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| UplinkPowerControlDedicated |  | UplinkPowerControlDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  |  | UplinkPowerControlDedicated-DEFAULT | See subclause 4.6.3 | RBC |

Table 6.3A.3.1.4.3-3: UplinkPowerControlDedicated

| Derivation Path: TS 36.508 [12] clause 4.6.3, Table 4.6.3-26 UplinkPowerControlDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-UE-PUSCH | 1 |  | SRB1 |
|  | 0 |  | RBC |
| } |  |  |  |

6.3A.3.1.5	Test requirement

The requirement for the power measured in steps (2), (3) and (4) of the test procedure shall not exceed the values specified in Table 6.3A.3.5-1.

Table 6.3A.3.1.5-1: General ON/OFF time mask

|  | Channel bandwidth / minimum output power / measurement bandwidth |
| --- | --- |
|  | 1.4 MHz |
| Transmit OFF power | For carrier frequency f ≤ 3.0GHz: ≤ -48.5 dBm For carrier frequency 3.0GHz < f ≤ 4.2GHz: ≤ -48.2 dBm |
| Transmission OFF Measurement bandwidth | 1.08 MHz |
| Expected Transmission ON Measured power | -14.8 dBm |
| ON power tolerance f ≤ 3.0GHz 3.0GHz < f ≤ 4.2GHz | ± 7.5dB ± 7.8dB |

### 6.3A.3.2	PRACH and SRS ON/OFF time mask for UE category M1

### 6.3A.3.2.1	PRACH time mask for UE category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3A.3.2.1.1	Test purpose

To verify that the PRACH time mask meets the requirements given in TS 36.521-1[14] clause 6.3.4.2.1.5.

The time mask for PRACH time mask defines the ramping time allowed for the UE between transmit OFF power and transmit ON power when transmitting the PRACH.

Transmission of the wrong power increases interference to other channels or increases transmission errors in the uplink channel.

6.3A.3.2.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.3A.3.2.1.3	Minimum conformance requirements

For the PRACH Power / Time mask defines the observation period for PRACH transmissions. The PRACH ON power is specified as the mean power over the PRACH measurement period excluding any transient periods. The measurement period for different PRACH preamble format is specified in Table 6.3A.3.2.1.3-1.

There are no additional requirements on UE transmit power beyond that which is required in clause 6.2 and clause 6.5

Table 6.3A.3.2.1.3-1: PRACH ON power measurement period

| PRACH preamble format | Measurement period (ms) |
| --- | --- |
| 0 | 0.9031 |
| 1 | 1.4844 |
| 2 | 1.8031 |
| 3 | 2.2844 |
| 4 | 0.1479 |

Figure 6.3A.3.2.1.3-1: PRACH ON/OFF time mask

The normative reference for this requirement is TS 36.101 [7] clause 6.3.4.2.1.

6.3A.3.2.1.4	Test description

6.3A.3.2.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in clause 5.2A. All these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3A.3.2.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521-1[14] Annexes A.2. Configurations of PDSCH and MPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.3A.3.2.1.4.1-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Environment (as specified in TS 36.508 [12] subclause 4.1) | Normal, TL/VL, TL/VH, TH/VL, TH/VH |
| Test Frequencies (as specified in TS 36.508 [12] subclause 4.3.1) | Mid range |
| Test Channel Bandwidths (as specified in TS 36.508 [12] subclause 4.3.1) | 1.4 MHz |
| PRACH preamble format |  |
|  | FDD |
| PRACH Configuration Index | 3 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

5.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2. Message contents are defined in clause 6.3A.3.2.1.4.3.

6.3A.3.2.1.4.2	Test procedure

1.	The SS shall signal a Random Access Preamble ID via a MPDCCH order to the UE and initiate a Non-contention based Random Access procedure.

2.	The UE shall send the signalled preamble to the SS.

3.	For FDD UE, the SS measure the UE transmission OFF power during the sub-frame preceding the PRACH preamble excluding a transient period of 20 µs according to Figure 6.3A.3.2.1.3-1.

4.	Measure the output power of the transmitted PRACH preamble according to Figure 6.3A.3.2.1.3-1.

5.	Measure the UE transmission OFF power, starting 20 µs after the PRACH preamble ends for a measurement period of 980 µs.

6.3A.3.2.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the following exceptions:

Table 6.3A.3.2.1.4.3-1: RACH-ConfigCommon-DEFAULT: PRACH measurement

| Derivation Path: TS 36.508 [12] clause 4.6.3, Table 4.6.3-12 RACH-ConfigCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| powerRampingParameters SEQUENCE { |  |  |  |
| powerRampingStep | dB0 |  |  |
| preambleInitialReceivedTargetPower | dBm-104 |  | PRACH Format 0 |
|  | dBm-112 |  | PRACH Format 4 |
| } |  |  |  |
| ) |  |  |  |

6.3A.3.2.1.5	Test requirement

The requirement for the power measured in steps (3), (4) and (5) of the test procedure shall not exceed the values specified in Table 6.3A.3.2.1.5-1.

Table 6.3A.3.2.1.5-1: PRACH time mask

|  | Channel bandwidth / Output Power [dBm] / measurement bandwidth |
| --- | --- |
|  | 1.4 MHz |
| Transmit OFF power | For carrier frequency f ≤ 3.0GHz: -48.5 dBm For carrier frequency 3.0GHz < f ≤ 4.2GHz: ≤ -48.2 dBm |
| Transmission OFF Measurement bandwidth | 1.08 MHz |
| Expected PRACH Transmission ON Measured power | -1 dBm |
| ON power tolerance f ≤ 3.0GHz 3.0GHz < f ≤ 4.2GHz | ± 7.5dB ± 7.8dB |

### 6.3A.3.2.2	SRS time mask for UE category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3A.3.2.2.1	Test purpose

Same test purpose as in TS 36.521-1[14] clause 6.3.4.2.2.1.

6.3A.3.2.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.3A.3.2.2.3	Minimum conformance requirements

Same minimum conformance requirements as in TS 36.521-1[14] clause 6.3.4.2.2.3.

6.3A.3.2.2.4	Test description

6.3A.3.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in clause 5.2A. All these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3A.3.2.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521-1 Annexes A.2 and A.3

Table 6.3A.3.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Environment (as specified in TS 36.508 [12] subclause 4.1) | Normal, TL/VL, TL/VH, TH/VL, TH/VH |
| Test Frequencies (as specified in TS 36.508 [12] subclause 4.3.1) | Mid range |
| Test Channel Bandwidths (as specified in TS 36.508 [12] subclause 4.3.1) | 1.4MHz |
| SRS configuration |  |
|  | FDD |
| srs-BandwidthConfig | bw7 |
| srs-SubframeConfig | sc3 |
| ackNackSRS-SimultaneousTransmission | FALSE |
| srsMaxUpPts | N/A |
| srs-Bandwidth | bw3 |
| srs-HoppingBandwidth | hbw3 |
| freqDomainPosition | 0 |
| Duration | TRUE |
| srs-ConfigIndex | 7 |
| transmissionComb | 0 |
| cyclicShift | cs0 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

5.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.3A.3.2.2.4.3. Note that MPDCCH DCI format 6-0A sent after resetting uplink power with RRC Connection Reconfiguration, should have TPC command 0dB.

6.3A.3.2.2.4.2	Test procedure

1.	For FDD UE, the SS measure the UE transmission OFF power during the 13 symbols preceding the SRS symbol excluding a transient period of 20 µs according to TS 36.521-1[14] Figure 6.3.4.2.2.3-1

2.	Measure the output power of the transmitted SRS according to TS 36.521-1[14] Figure 6.3.4.2.2.3-1 for FDD and HD-FDD UE and the transient periods are excluded from measurement accordingly.

3.	Measure the UE transmission OFF power during the sub-frame following the SRS under test, excluding a transient period of 20 µs according to TS 36.521-1[14] Figure 6.3A.3.2.2.3-1 for FDD and HD-FDD UE

6.3A.3.2.2.4.3	Message contents

Same message contents as in TS 36.521-1[14] clause 6.3.4.2.2.4.3 with the following exception.

Table 6.3A.3.2.2.4.3-1: SchedulingRequest-Config

| Derivation Path: 36.331[6] clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequest-Config-DEFAULT ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| sr-ConfigIndex | 33 |  | HD-FDD |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| HD-FDD | Half Duplex FDD cell environment |

6.3A.3.2.2.5	Test requirement

The requirement for the power measured in steps (1), (2) and (3) of the test procedure shall not exceed the values specified in Table 6.3A.3.2.2.5-1.

Table 6.3A.3.2.2.5-1: SRS time mask

|  | Channel bandwidth / Output Power [dBm] / measurement bandwidth |
| --- | --- |
|  | 1.4 MHz |
| Transmit OFF power | For carrier frequency f ≤ 3.0GHz: -48.5 dBm For carrier frequency 3.0GHz < f ≤ 4.2GHz: ≤ -48.2 dBm |
| Transmission OFF Measurement bandwidth | 1.08 MHz |
| Expected SRS Transmission ON Measured power | -2.6 dBm |
| ON power tolerance f ≤ 3.0GHz 3.0GHz < f ≤ 4.2GHz | ± 7.5dB ± 7.8dB |

### 6.3A.4	Power control for category M1

### 6.3A.4.1	Power Control Absolute power tolerance for UE category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3A.4.1.1	Test purpose

To verify the ability of the UE transmitter to set its initial output power to a specific value at the start of a contiguous transmission or non-contiguous transmission with a long transmission gap, i.e. transmission gap is larger than 20 ms.

6.3A.4.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.3A.4.1.3	Minimum conformance requirements

Absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20ms.

The minimum requirement on absolute power tolerance is given in Table 6.3A.4.1.3-1 over the power range bounded by the Maximum output power as defined in sub-clause 6.2A.1 and the Minimum output power as defined in sub clause 6.3A.1.

Table 6.3A.4.1.3-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal conditions | ± 9.0 dB |
| Extreme conditions | ± 12.0 dB |

The normative reference for this requirement is TS 36.102 [11] clause 6.3A.4.

6.3A.4.1.4	Test description

6.3A.4.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in clause 5.2A. All these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3A.4.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521-1[14] Annexes A.2. Configurations of PDSCH and MPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.3A.4.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Ch BW | N/A |  | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1.4MHz |  |  | QPSK | 6 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.3A.4.1.4.1-1.

5.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A. 2 AA. Message contents are defined in clause 6.3A.4.1.4.3. Note that MPDCCH DCI format 6-0A sent after resetting uplink power with RRC Connection Reconfiguration, should have TPC command 0dB.

6.3A.4.1.4.2	Test procedure

1.	SS sends uplink scheduling information via MPDCCH DCI format 6-0A with TPC command 0dB for C_RNTI to schedule the UL RMC according to Table 6.3A.4.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

2.	Measure the initial output power of the first subframe of UE PUSCH first transmission. The transient periods of 20us are excluded.

3.	Repeat for the two test points as indicated in section 6.3A.4.1.4.3. The timing of the execution between the two test points shall be larger than 20ms.

6.3A.4.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the following exceptions:

Table 6.3A.4.1.4.3-1: UplinkPowerControlCommon: Test point 1

| Derivation Path: TS 36.508 [12] clause 4.6.3, Table 4.6.3-25 UplinkPowerControlCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-DEFAULT ::= SEQUENCE { p0-NominalPUSCH | -105 | Test point 1 to verify a UE relative low initial power transmission |  |

Table 6.3A.4.1.4.3-2: UplinkPowerControlCommon: Test point 2

| Derivation Path: TS 36.508 [12] clause 4.6.3, Table 4.6.3-25 UplinkPowerControlCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-DEFAULT ::= SEQUENCE { p0-NominalPUSCH | -93 | Test point 2 to verify a UE relative high initial power transmission |  |

Table 6.3A.4.1.4.3-3: PhysicalConfigDedicated

| Derivation Path: TS 36.508 [12] clause 5.5.1.2, Table 5.5.1.2.1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| uplinkPowerControlDedicated |  | UplinkPowerControlDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  |  | UplinkPowerControlDedicated-DEFAULT | See subclause 4.6.3 | RBC |

Table 6.3A.4.1.4.3-4: UplinkPowerControlDedicated

| Derivation Path: TS 36.508 [12] clause 4.6.3, Table 4.6.3-26 UplinkPowerControlDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-UE-PUSCH | 1 |  | SRB1 |
|  | 0 |  | RBC |
| } |  |  |  |

6.3A.4.1.5	Test requirement

The requirement for the power measured in step (2) of the test procedure is not to exceed the values specified in Table 6.3A.4.1.5-1 and 6.3A.4.1.5-2.

Table 6.3A.4.1.5-1: Absolute power tolerance: test point 1

|  | UE Power Class / Expected output power (dBm) |  |
| --- | --- | --- |
|  | Class 3 | Class 5 |
| Expected Measured power Normal conditions | -14.8 dBm | -14.8 dBm |
| Power tolerance f ≤ 3.0GHz | ± 10.0dB | ± 10.0dB |
| Expected Measured power Extreme conditions | -14.8 dBm | -14.8 dBm |
| Power tolerance f ≤ 3.0GHz | ± 13.0dB | ± 13.0dB |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3A.1 |  |  |

Table 6.3A.4.1.5-2: Absolute power tolerance: test point 2

|  | Expected output power (dBm) |  |
| --- | --- | --- |
|  | Class 3 | Class 5 |
| Expected Measured power Normal conditions | -2.8 dBm | -2.8 dBm |
| Power tolerance f ≤ 3.0GHz | ± 10.0dB | ± 10.0dBm |
| Expected Measured power Extreme conditions | -2.8 dBm | -2.8 dBm |
| Power tolerance f ≤ 3.0GHz | ± 13.0dB | ± 13.0dB |
| Note 1: The upper power limit shall not exceed the maximum output power requirements defined in sub-clause 6.2A.1 |  |  |

### 6.3A.4.2	Power Control Relative power tolerance for UE category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3A.4.2.1	Test purpose

To verify the ability of the UE transmitter to set its output power relatively to the power in a target sub-frame relatively to the power of the most recently transmitted reference sub-frame if the transmission gap between these sub-frames is ≤ 20 ms.

6.3A.4.2.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.3A.4.2.3	Minimum conformance requirements

The UE shall meet the requirements specified in Table 6.3A.4.2.3-1.

To account for RF Power amplifier mode changes 2 exceptions are allowed for each of two test patterns. The test patterns are a monotonically increasing power sweep and a monotonically decreasing power sweep over a range bounded by the requirements of minimum power and maximum power specified in clauses 6.3A.1 and 6.2A.1. For these exceptions the power tolerance limit is a maximum of ±6.0 dB in Table 6.3A.4.2.3-1.

Table 6.3A.4.2.3-1: Relative Power Tolerance for Transmission (normal conditions)

| Power step P (Up or down) [dB] | All combinations of PUSCH and PUCCH transitions [dB] | All combinations of PUSCH/PUCCH and SRS transitions between sub-frames [dB] | PRACH [dB] |
| --- | --- | --- | --- |
| ΔP < 2 | ±2.5 (Note 3) | ±3.0 | ±2.5 |
| 2 ≤ ΔP < 3 | ±3.0 | ±4.0 | ±3.0 |
| 3 ≤ ΔP < 4 | ±3.5 | ±5.0 | ±3.5 |
| 4 ≤ ΔP ≤ 10 | ±4.0 | ±6.0 | ±4.0 |
| 10 ≤ ΔP < 15 | ±5.0 | ±8.0 | ±5.0 |
| 15 ≤ ΔP | ±6.0 | ±9.0 | ±6.0 |
| Note 1: For extreme conditions an additional ± 2.0 dB relaxation is allowed |  |  |  |

The power step (ΔP) is defined as the difference in the calculated setting of the UE Transmit power between the target and reference sub-frames with the power setting according to Clause 5.1 of TS 36.213. The error is the difference between ΔP and the power change measured at the UE antenna port with the power of the cell-specific reference signals kept constant. The error shall be less than the relative power tolerance specified in Table 6.3A.4.2.3-1.

The normative reference for this requirement is TS 36.102 [11] clause 6.3A.4.

6.3A.4.2.4	Test description

6.3A.4.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in clause 5.2A. All these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3A.4.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521-1[14] Annexes A.2. Configurations of PDSCH and MPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.3A.4.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  | Low range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Ch BW | N/A |  | Mod'n | RB allocation |
|  |  |  |  | FDD and HD-FDD |
| 1.4MHz |  |  | QPSK | See table 6.3A.4.2.5-1 6.3A.4.2.5-2 6.3A.4.2.5-3 |
| Note 1: The RBstart of partial RB allocation shall be RB#0. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.3A.4.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A. 2 AA. Message contents are defined in clause 6.3A.4.2.4.3.

6.3A.4.2.4.2	Test procedure

The procedure is separated in various subtests to verify different aspects of relative power control. The power patterns of the subtests are described in figure 6.3A.4.2.4.2-1.

Figure 6.3A.4.2.4.2-1: FDD ramping up test power patterns

Figure 6.3A.4.2.4.2-2: FDD ramping down test power patterns

Figure 6.3A.4.2.4.2-3: HD-FDD ramping up test power patterns

Figure 6.3A.4.2.4.2-4: HD-FDD ramping down test power patterns

Figure 6.3A.4.2.4.2-5: Alternating Test Power patterns

1.	Sub test: ramping up pattern

1.1	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the PUSCH. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH to the UE to ensure that the UE transmits PUSCH at -36.8dBm +/- 3.2 dB for carrier frequency f ≤ 3.0GHz.

1.2	Schedule the UE's PUSCH data transmission as described in Figure 6.3A.4.2.4.2-1 (FDD pattern A: sub-test is divided in 4 arbitrary radio frames with 10 active uplink sub-frames per radio frame)  and Figure 6.3A4..2.4.2-5 (HD-FDD pattern A: sub-test is divided in 14 arbitrary radio frames with 3 active uplink sub-frames per radio frame) with an uplink RB allocation as defined in tables 6.3A.4.2.5-1. On the MPDCCH DCI format 6-0A for the scheduling of the PUSCH the SS will transmit a +1dB TPC command. Note that the measurement need not be done continuously, provided that interruptions are whole numbers of frames, and TPC commands of 0dB are sent during the interruption.

1.3	Measure the power of PUSCH transmissions to verify the UE relative power control meet test requirements 6.3A.4.2.5. For power transients between subframes, transient periods of 40us between subframes are excluded. For ON/OFF or OFF/ON transients, transient periods of 20 us at the beginning of the subframe are excluded.

1.4	Repeat the subtest different pattern B, C to move the RB allocation change at different points in the pattern as described in Table 6.3A.4.2.5-1 to force bigger UE power steps at various points in the power range.

2.	Sub test: ramping down pattern

2.1	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the PUSCH. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH to the UE to ensure that the UE transmits PUSCH at +18.0dBm +/- 3.2 dB for carrier frequency f ≤ 3.0GHz with UE power class 3 or +15.0dBm +/- 3.2 dB for carrier frequency f ≤ 3.0GHz with UE power class 5 or +9.0 dBm +/- 3.2 dB for carrier frequency f ≤ 3.0GHz with UE power class 6.

2.2	Schedule the UE's PUSCH data transmission as described in Figure 6.3A.4.2.4.2-2 (FDD pattern A: sub-test is divided in 4 arbitrary radio frames with 10 active uplink sub-frames per radio frame) and Figure 6.3.5EA.2.4.2-6 (HD-FDD pattern A: sub-test is divided in 14 arbitrary radio frames with 3 active uplink sub-frames per radio frame)with an uplink RB allocation as defined in table 6.3A.4.2.5-2. On the MPDCCH DCI format 6-0A for the scheduling of the PUSCH the SS will transmit a -1dB TPC command. Note that the measurement need not be done continuously, provided that interruptions are whole numbers of frames, and TPC commands of 0dB are sent during the interruption.

2.3	Measure the power of PUSCH transmissions to verify the UE relative power control meet test requirements 6.3A.4.2.5. For power transients between subframes, transient periods of 40us between subframes are excluded. For ON/OFF or OFF/ON transients, transient periods of 20 us at the beginning of the subframe are excluded.

2.4	Repeat the subtest different pattern B, C to move the RB allocation change at different points in the pattern as described in Table 6.3A.4.2.5-2 to force bigger UE power steps at various points in the power range.

3.	Sub test: alternating pattern

3.1	SS sends uplink scheduling information for each UL HARQ process via P MPDCCH DCI format 6-0A for C_RNTI to schedule the PUSCH. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH to the UE to ensure that the UE transmits PUSCH at -10dBm +/- 3.2 dB for carrier frequency f ≤ 3.0GHz or at -10dBm +/- 3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz. The initial uplink RB allocation is defined as the smaller uplink RB allocation value specified in table 6.3A.4.2.5-3. The power level and RB allocation are reset for each sub-test.

3.2	Schedule the UE's PUSCH data transmission as described in Figure 6.3A.4.2.4.2-5 for 10 sub-frames (FDD ) and 20 sub-frames (HD-FDD) with an uplink RB allocation alternating pattern as defined in table 6.3A.4.2.5-3 while transmitting 0dB TPC command for PUSCH via the MPDCCH.

3.3. Measure the power of PUSCH transmissions to verify the UE relative power control meet test requirements specified in clause 6.3A.4.2.5. For power transients between subframes, transient periods of 40us between subframes are excluded. For ON/OFF or OFF/ON transients, transient periods of 20 us at the beginning of the subframe are excluded.

6.3A.4.2.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6.

6.3A.4.2.5	Test requirement

Each UE power step measured in the test procedure 6.3A.4.2.4.2 should satisfy the test requirements specified in Table 6.3A.4.2.5-1, thru 6.3A.4.2.5-3 for normal conditions; for extreme conditions an additional ± 2.0 dB relaxation is allowed.

To account for RF Power amplifier mode changes 2 exceptions are allowed for each of ramping up and ramping down test patterns. For these exceptions the power tolerance limit is a maximum of ±6.7 dB. If there is an exception in the power step caused by the RB change for all test patterns (A, B, C) then fail the UE.

Table 6.3A.4.2.5-1: Test Requirements Relative Power Tolerance for Transmission (normal conditions – Note 5) channel bandwidth 1.4MHz (ramping up)

| Sub-test (ramp up) | Uplink RB allocation | TPC command | Expected power step size (Up) | Power step size range (Up) | PUSCH |
| --- | --- | --- | --- | --- | --- |
|  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
| Subframes before RB change | Fixed = 1 | TPC=+1dB | 1 | ΔP < 2 | 1 ± (1.7) |
| RB change | Change from 1 to 6 RBs | TPC=+1dB | 8.78 | 4 ≤ ΔP < 10 | 8.78 ± (4.7) Note 2 |
| Subframes after RB change | Fixed = 6 | TPC=+1dB | 1 | ΔP < 2 | 1 ± (1.7) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 10 active uplink subframes Pattern B the position of RB uplink allocation change is after 20 active uplink subframes Pattern C the position of RB uplink allocation change is after 30 active uplink subframes Note 2: When Note 3 does not apply. Note 3: N/A Note 4: N/A Note 5: For extreme conditions an additional ± 2.0 dB relaxation is allowed. Note 6: The starting resource block shall be RB# 0. |  |  |  |  |  |

Table 6.3A.4.2.5-2: Test Requirements Relative Power Tolerance for Transmission (normal conditions – Note 5) channel bandwidth 1.4MHz (ramping down)

| Sub-test (ramp down) | Uplink RB allocation | TPC command | Expected power step size (down) | Power step size range (down) | PUSCH |
| --- | --- | --- | --- | --- | --- |
|  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
| Subframes before RB change | Fixed = 5 | TPC=-1dB | 1 | ΔP < 2 | 1 ± (1.7) |
| RB change | Change from 5 to 1 RBs | TPC=-1dB | 7.99 | 4 ≤ ΔP < 10 | 7.99 ± (4.7) Note 2 |
| Subframes after RB change | Fixed = 1 | TPC=-1dB | 1 | ΔP < 2 | 1 ± (1.7) |
| Note 1: Position of RB change: Pattern A the position of RB uplink allocation change is after 6 active uplink subframes Pattern B the position of RB uplink allocation change is after 16 active uplink subframes Pattern C the position of RB uplink allocation change is after 26 active uplink subframes Note 2: When Note 4 does not apply. Note 3: N/A Note 4: N/A Note 5: For extreme conditions an additional ± 2.0 dB relaxation is allowed. Note 6: The starting resource block shall be RB# 0. |  |  |  |  |  |

Table 6.3A.4.2.5-3: Test Requirements Relative Power Tolerance for Transmission
(normal conditions – Note 5) (Alternating pattern)

| Sub-test | Uplink RB allocation | TPC command | Expected power step size (Up or down) | Power step size range (Up or down) | PUSCH |
| --- | --- | --- | --- | --- | --- |
|  |  |  | ΔP [dB] | ΔP [dB] | [dB] |
| 1.4 MHz | Alternating 1 and 6 | TPC=0dB | 7.78 | 4 ≤ ΔP < 10 | 7.78 ± (6.7) Note 1,2 7.78 +8.2/-6.7 Note 3 7.78 +6.7/-8.2 Note 4 |
| Note 1: Test tolerance +/- 6.7 dB was selected to allow PA switch possible exceptions to occur. Note 2: When neither Note 3 nor Note 4 applies. Note 3: N/A Note 4: N/A. Note 5: For extreme conditions an additional ± 2.0 dB relaxation is allowed. Note 6: The starting resource block shall be RB# 0. |  |  |  |  |  |

### 6.3A.4.3	Aggregate power control tolerance for UE category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3A.4.3.1	Test purpose

To verify the ability of category M1 UE to maintain its power level in non-contiguous transmission in response to 0 dB TPC commands with respect to the first UE transmission, when the power control parameters specified in TS 36.213 are constant.

6.3A.4.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category M1 that support satellite access operation.

6.3A.4.3.3	Minimum conformance requirements

The category M1 FD-FDD UEs shall meet the requirements specified in Table 6.3A.4.3.3-1 for aggregate power control over the power range bounded by the minimum output power as defined in subclause 6.3A.1, the maximum output power as defined in subclause 6.2A.1, and the requirements for configured transmitted power are specified in subclause 6.2A.4.

The category M1 HD-FDD UEs and for continuous uplink transmissions of duration ≤ 64 ms, shall meet the requirements specified in Table 6.3A.4.3.3-1 for aggregate power control over the power range bounded by the minimum output power as defined in subclause 6.3A.1, the maximum output power as defined in subclause 6.2A.1, and the requirements for configured transmitted power are specified in subclause 6.2A.4.

Table 6.3A.4.3.3-1: Aggregate power control tolerance

| TPC command | UL channel | Aggregate power tolerance within 21 ms 2 |
| --- | --- | --- |
| 0 dB | PUCCH | ±2.5 dB |
| 0 dB | PUSCH | ±3.5 dB |
| NOTE 1: The UE transmission gap is 4 ms for full-duplex FDD. For UE of half-duplex FDD, the transmission gap is 9 ms. TPC command is transmitted via MPDCCH 4 subframes preceding each PUCCH/PUSCH transmission. NOTE 2: For UE of half-duplex FDD, the test interval is 41 ms. |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 6.3A.4.

6.3A.4.3.4	Test description

6.3A.4.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in clause 5.2A. All these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3A.4.3.4.1-1 and table 6.3A.4.3.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521-1[14] Annexes A.2 and A.3. The details of the OCNG patterns used are specified in TS 36.521-1[14] Annex A.5. Configurations of PDSCH and MPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.3A.4.3.4.1-1: Test Configuration Table Tx test cases UE Cat-M1: PUCCH sub-test

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  |  | Normal |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |
| Ch BW | Mod'n | RB allocation |  | FDD: PUCCH format = Format 1a |
|  |  | FDD |  |  |
| 5MHz | QPSK | 4 |  |  |
| Note 1: Downlink RB position shall be RBstart = 0 within the narrowband |  |  |  |  |

Table 6.3A.4.3.4.1-2: Test Configuration Table Tx test cases UE Cat-M1: PUSCH sub-test

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 4.1 |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 4.3.1 |  | Mid range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  | 1.4MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |  |  |
| Ch BW | N/A for PUSCH sub-test | Mod'n |  | RB allocation |  |
|  |  |  | FDD and HD-FDD |  |  |
| Low range |  |  |  |  |  |
| 1.4MHz |  | QPSK | 5 | 5 | 0 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL and DL Reference Measurement channels are set according to Table 6.3A.4.3.4.1-1 (PUCCH sub-test) and Table 6.3A.4.3.4.1-2 (PUSCH sub-test).

5.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 6.3A.4.1.4.3.

6.3A.4.3.4.2	Test procedure

The procedure is separated in various subtests to verify different aspects of relative power control. The power patterns of the subtests are described in figure 6.3A.4.3.4.2-1.

Figure 6.3.A.4.3.4.2-1: Test uplink transmission

1.	PUCCH sub test:

1.1	The SS transmits PDSCH via M-PDCCH DCI format 6-1A for C_RNTI to transmit the DL RMC according to Table 6.3A.4.3.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The transmission of PDSCH will make the UE send uplink ACK/NACK using PUCCH. Send the appropriate TPC commands for PUCCH to the UE to ensure that the UE transmits PUCCH at 0dBm +/- 3.2 dB for carrier frequency f ≤ 3.0GHz or at 0dBm +/- 3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

1.2. For FDD: Every 5 subframes transmit to the UE downlink PDSCH MAC padding bits as well as 0 dB TPC command for PUCCH via the MPDCCH to make the UE transmit ACK/NACK on the PUCCH with transmission gap of 4 subframes. For HD-FDD: Subframe #3 every 10 subframes transmit to the UE downlink PDSCH MAC padding bits as well as 0 dB TPC command for PUCCH via the MPDCCH to make the UE transmit ACK/NACK on the PUCCH with transmission gap of 9 ms after subframe #7. The downlink transmission is scheduled in the appropriate sub-frames to make the UE transmit PUCCH as described in figure 6.3A.4.3.4.2-1.

1.3. Measure the power of 5 consecutive PUCCH transmissions to verify the UE transmitted PUCCH power is maintained within 21 ms for FDD and within 41 ms for HD-FDD. The transient periods of 20us are excluded from the power measurement.

2.	PUSCH sub test:

2.1.	The SS sends uplink scheduling information via M-PDCCH DCI format 6-0A for C_RNTI to schedule the PUSCH. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. Send the appropriate TPC commands for PUSCH to the UE to ensure that the UE transmits PUSCH at 0dBm +/- 3.2 dB for carrier frequency f ≤ 3.0GHz or at 0dBm +/- 3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

2.2. For FDD: Every 5 subframes schedule the UE's PUSCH data transmission and transmit 0 dB TPC command for PUSCH via the MPDCCH to make the UE transmit PUSCH with 4 subframes gap. For HD-FDD: Subframe #6 every 10 subframes schedule the UE's PUSCH data transmission and transmit 0 dB TPC command for PUSCH via the MPDCCH to make the UE transmit PUSCH with transmission gap of 9 ms after subframe #6. The uplink transmission patterns are described in figure 6.3A.4.3.4.2-1.

2.3. Measure the power of 5 consecutive PUSCH transmissions to verify the UE transmitted PUSCH power is maintained within 21 ms for FDD and within 41 ms for HD-FDD. The transient periods of 20us are excluded from the power measurement.

6.3A.4.3.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6.

6.3A.4.3.5	Test requirement

The requirement for the power measurements made in step (1.3) and (2.3) of the test procedure shall not exceed the values specified in Table 6.3A.4.3.5-1. The power measurement period shall be 1 sub-frame excluding transient periods.

Table 6.3A.4.3.5-1: Power control tolerance

| TPC command | UL channel | Test requirement measured power within 21m 2 |
| --- | --- | --- |
| 0 dB | PUCCH | Given 5 power measurements in the pattern, the 2nd, 3rd, 4th, and 5th measurements shall be within ± 3.2 dB of the 1st measurement. |
| 0 dB | PUSCH | Given 5 power measurements in the pattern, the 2nd, 3rd, 4th, and 5th measurements shall be within ± 4.2 dB of the 1st measurement. |
| Note 1: The UE transmission gap is 4 ms for full-duplex FDD. For UE of half-duplex FDD, the transmission gap is 9 ms TPC command is transmitted via PDCCH 4 subframes preceding each PUCCH/PUSCH transmission. Note 2: For UE of half-duplex FDD MHz, the test interval is 41 ms. |  |  |

## 6.3B	Output power dynamics for category NB1 and NB2

### 6.3A.2	Transmit OFF power for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3A.2.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

6.3A.2.2	Test applicability

The requirements of this test apply in test cases 6.3A.3.1 ON/OFF time mask and 6.3A.3.2 PRACH and SRS time mask to all types of E-UTRA UE release 17 and forward that support satellite access operation.

6.3A.2.3	Minimum conformance requirements

The minimum output power is defined as the mean power in one sub-frame (1ms). The minimum output power shall not exceed the values specified in Table 6.3A.2.3-1.

Table 6.3A.2.3-1: Minimum output power

|  | Channel bandwidth / minimum output power / measurement bandwidth |
| --- | --- |
|  | 1.4MHz |
| Minimum output power | -50 dBm |
| Measurement bandwidth | 1.08 MHz |

The normative reference for this requirement is TS 36.102 [11] clause 6.3A.2.

Transmit OFF power is defined as the mean power when the transmitter is OFF. The transmitter is considered to be OFF when the UE is not allowed to transmit or during periods when the UE is not transmitting a sub-frame. During DTX and measurements gaps, the UE is not considered to be OFF.

An excess transmits OFF power potentially increases the Rise over Thermal (RoT) and therefore reduces the cell coverage area for other UEs.

6.3A.2.4	Test description

This test is covered by clause 6.3A.3.1 ON/OFF time mask and 6.3A.3.2 PRACH and SRS time mask.

6.3A.2.5	Test requirement

The minimum output power measured shall not exceed the values specified in Table 6.3A.2.5-1.

Table 6.3A.2.5-1: Minimum output power for UE category M1

|  | Channel bandwidth / minimum output power / measurement bandwidth |
| --- | --- |
|  | 1.4 MHz |
| Minimum output power | For carrier frequency f ≤ 3.0GHz: ≤ -48.5 dBm For carrier frequency 3.0GHz < f ≤ 4.2GHz: ≤ -48.2 dBm |
| Measurement bandwidth | 1.08 MHz |

### 6.3B.1	UE Minimum output power for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3B.1.1	Test purpose

To verify the UE's ability to transmit with a broadband output power below the value specified in the test requirement when the power is set to a minimum value.

6.3B.1.2	Test applicability

This test case applies to all types of E-UTRA UE release 17 and forward of UE category NB11 and NB2 that support satellite access operation.

6.3B.1.3	Minimum conformance requirements

For category NB1 and NB2 UE the single-tone and multi-tone transmission minimum output power requirement for the channel bandwidth is -40 dBm. For 3.75kHz sub-carrier spacing the minimum output power is defined as mean power in one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the minimum output power is defined as mean power in one sub-frame (1ms).

The normative reference for this requirement is TS 36.102 [11] clause 6.3B.1.

6.3B.1.4	Test description

6.3B.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2B. All of these configurations shall be tested with applicable test parameters and are shown in table 6.3B.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in TS 36.521[14] Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521[14] Annex C.2.

Table 6.3B.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 2) |  | BPSK | 1@0 | 3.75 |
| 2 (Note 3) |  | BPSK | 1@47 | 3.75 |
| 3 (Note 2) |  | QPSK | 1@0 | 15 |
| 4 (Note 3) |  | QPSK | 1@11 | 15 |
| 5 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions Note 2: only applicable for low range Note 3: only applicable for high range |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channel is set according to Table 6.3B.1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.1.4.3.

6.3B.1.4.2	Test procedure

1.	SS sends uplink scheduling information for UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to o Table 6.3B.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration of one sub-frame (1ms) for sub-carrier spacing of 15 kHz or one slot (2ms) excluding the 2304Ts gap when UE is not transmitting for sub-carrier spacing of 3.75 kHz. Half-Duplex guard subframes are not under test.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.3B.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 with the following exception.

Table 6.3B.1.4.3-1: NPDSCH-ConfigCommon-NB-DEFAULT

| Derivation Path: 36.508 clause 8.1.6.3, Table 8.1.6.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | -39 (dBm) |  | 15 kHz SCS Ntones = 1 |
|  | -50 (dBm) |  | 15 kHz SCS Ntones = 12 |
|  | -33 (dBm) |  | 3.75 kHz SCS |
| } |  |  |  |

Table 6.3B.1.4.3-2: UplinkPowerControlCommon-NB-DEFAULT

| Derivation Path: 36.508 clause 8.1.6.3. Table 8.1.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | -95 (-95 dBm) |  |  |
| alpha-r13 | al1 (1) |  |  |
| deltaPreambleMsg3-r13 | 4 |  |  |
| } |  |  |  |

6.3B.1.5	Test requirement

The minimum output power measured shall not exceed the value of -39 dBm for the channel bandwidth of category NB1 and NB2 UE.

### 6.3B.2	Transmit OFF power for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3B.2.1	Test purpose

To verify that the UE transmit OFF power is lower than the value specified in the test requirement.

6.3B.2.2	Test applicability

The requirements of this test apply in test cases 6.3B.3.1 General ON/OFF time mask and 6.3B.3.2 NPRACH time mask for all types of NB-IoT FDD UE release 17 and forward of UE category NB1 and NB2 that support satellite access operation.

6.3B.2.3	Minimum conformance requirements

For category NB1 and NB2 UE the transmit OFF power requirement for the channel bandwidth is -50 dBm. For 3.75 kHz sub-carrier spacing the transmit OFF power is defined as mean power in one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15 kHz sub-carrier spacing the transmit OFF power is defined as mean power in one sub-frame (1ms).

The normative reference for this requirement is TS 36.102 [11] clause 6.3B.2.

6.3B.2.4	Test description

This test is covered by clause 6.3B.3.1 General ON/OFF time mask for category NB1 and NB2 and 6.3B.3.2 NPRACH time mask for category NB1 and NB2.

6.3B.2.5	Test requirement

The requirement for the transmit OFF power shall not exceed the values of -48.5 dBm for the channel bandwidth of category NB1 and NB2.

### 6.3B.3	ON/OFF time mask for category NB1 and NB2

### 6.3B.3.1	General ON/OFF time mask for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3B.3.1.1	Test purpose

To verify that the general ON/OFF time mask meets the requirements given in 6.3B.3.1.5.

The time mask for transmit ON/OFF defines the ramping time allowed for the UE between transmit OFF power and transmit ON power.

Transmission of the wrong power increases interference to other channels or increases transmission errors in the uplink channel.

6.3B.3.1.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.3B.3.1.3	Minimum conformance requirements

E-UTRA general ON/OFF time mask in TS 36.521-1[14] subclause 6.3.4.1 applies for category NB1 and NB2 UE with an exception that for 3.75kHz sub-carrier spacing the transmit OFF power is defined as mean power in one slot (2ms) and for 15kHz sub-carrier spacing the transmit OFF power is defined as mean power in one sub-frame (1ms), excluding any transient periods. The ON power is defined as the mean power over one RU excluding any transient periods.

The normative reference for this requirement is TS 36.102 [11] clause 6.3B.3.1.

6.3B.3.1.4	Test description

This test is covered by clause 6.3B.3.1 General ON/OFF time mask for category NB1 and NB2 and 6.3B.3.2 NPRACH time mask for category NB1 and NB2.

6.3B.3.1.4.1	Initial conditions

Initial conditions are set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2B. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3B.3.1.4.1-1. The details of the uplink reference measurement channel (RMCs) are specified in TS 36.521-1[14] Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521-1 [14] Annex C.2.

Table 6.3B.3.1.4.1-1: Test Configuration Table for FDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 15kHz |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1 and C.3.0, and uplink signals according to TS 36.521-1 [14] Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channel is set according to Table 6.3B.3.1.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.3.1.4.3.

6.3B.3.1.4.2	Test procedure

1.	SS sends uplink scheduling information via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.3B.3.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. (UE should be already transmitting PUMAX after Initial Conditions setting)

2.	For HD-FDD : Measure the UE transmission OFF power as the mean power in one sub-frame (1ms)  prior to the NPUSCH RU, excluding a transient period of 20 µs at the beginning of the sub-frame.

3.	Measure the output power of the UE NPUSCH transmission as the mean power in one RU, excluding a transient period of 20 µs at the beginning of the RU.

4.	Measure the UE transmission OFF power as the mean power in one sub-frame (1ms)  following the NPUSCH RU, excluding a transient period of 20 µs at the beginning of the sub-frame.

6.3B.3.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 with the following exceptions.

Table 6.3B.3.1.4.3-1: P0-NominalNPUSCH-r13 configuration

| Derivation Path: 36.508 clause 8.1.6.3 Table 8.1.6.3-14: UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | -117 (dBm) |  |  |
| alpha-r13 | al1 (1) |  |  |
| deltaPreambleMsg3-r13 | 4 |  |  |
| } |  |  |  |

Table 6.3B.1.4.3-2: NPDSCH-ConfigCommon-NB-DEFAULT configuration

| Derivation Path: 36.508 clause 8.1.6.3 Table 8.1.6.3-4: NPDSCH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | 21 (dBm) |  |  |
| } |  |  |  |

6.3B.3.1.5	Test requirement

The requirement for the power measured in steps (2), (3) and (4) of the test procedure shall not exceed the values specified in Table 6.3B.1.5-1.

Table 6.3B.1.5-1: General ON/OFF time mask for category NB1 and NB2

|  | Channel bandwidth / minimum output power / measurement bandwidth |
| --- | --- |
|  | 200 kHz |
| Transmit OFF power | For carrier frequency f ≤ 3.0GHz: ≤ -48.5 dBm |
| Transmission OFF Measurement bandwidth | 180kHz |
| Expected Transmission ON Measured power | -11 dBm |
| ON power tolerance f ≤ 3.0GHz | ± 7.5 dB |

### 6.3B.3.2	NPRACH time mask for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3B.3.2.1	Test purpose

To verify that the NPRACH time mask meets the requirements given in 6.3B.3.2.5.

The time mask for NPRACH time mask defines the ramping time allowed for the UE between transmit OFF power and transmit ON power when transmitting the NPRACH.

Transmission of the wrong power increases interference to other channels or increases transmission errors in the uplink channel.

6.3B.3.2.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.3B.3.2.3	Minimum conformance requirements

The NPRACH ON power is specified as the mean power over the NPRACH measurement period excluding any transient periods as shown in Figure 6.3B.3.2.3-1. The measurement period for different NPRACH preamble format is specified in Table 6.3B.2.3-1.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2B and 6.5B

Table 6.3B.3.2.3-1: NPRACH ON power measurement period

| NPRACH preamble format | Measurement period (ms) |
| --- | --- |
| 0 | 5.6 |
| 1 | 6.4 |

Figure 6.3B.3.2.3-1: NPRACH ON/OFF time mask

The normative reference for this requirement is TS 36.102 [11] clause 6.3B.3.3.

6.3B.3.2.4	Test description

6.3B.3.2.4.1	Initial conditions

Initial conditions are set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2B. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3B.3.2.4.1-1. The details of the uplink reference measurement channel (RMCs) are specified in TS 36.521-1[14] Annex A.2.4.

Table 6.3B.3.2.4.1-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 | Normal |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 | Frequency ranges defined in Annex K.1.1 |
| NPRACH preamble format | 0 |
|  | 1 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1 and C.3.0, and uplink signals according to TS 36.521-1 [14] Annex H.1.1 and H.4.0.

4.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

5.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.3.2.4.3.

6.3B.3.2.4.2	Test procedure

1.	The SS shall set RS EPRE according to Table 6.3B.3.2.4.1-1.

2.	The SS send the paging and UE shall send a preamble to the SS.

3.	The SS measure the UE transmission OFF power during the sub-frame preceding the NPRACH preamble excluding a transient period of 20 µs according to Figure 6.3B.3.2.3-1.

4.	Measure the output power of the transmitted NPRACH preamble according to Figure 6.3B.3.2.3-1.

5.	Measure the UE transmission OFF power, starting 20 µs after the NPRACH preamble ends for a measurement period of 980 µs.

6.	Switches off and on the UE and ensures the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.3.2.4.3 with NPRACH Format 1.

7.	Repeat test with step 1-5.

6.3B.3.2.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 with the following exceptions.

Table 6.3B.3.2.4.3-1: RACH-ConfigCommon-NB-DEFAULT

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-8 RACH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| powerRampingParameters-r13 SEQUENCE { |  |  |  |
| powerRampingStep | dB0 | 0 dB |  |
| preambleInitialReceivedTargetPower | -dBm-120 | -120 dBm | NPRACH Format 0 |
|  | dBm-120 | -120 dBm | NPRACH Format 1 |
| } |  |  |  |
| ) |  |  |  |

Table 6.3B.3.2.4.3-2: NPDSCH-ConfigCommon-NB-DEFAULT

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-4 NPDSCH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | 24 (dBm) |  |  |
| } |  |  |  |

Table 6.3B.3.2.4.3-3: NPRACH-ConfigSIB-NB-DEFAULT

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-5 NPRACH-ConfigSIB-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-CP-Length-r13 | us66dot7 | 2048*Ts | NPRACH Format 0 |
|  | us266dot7 | 8192*Ts | NPRACH Format 1 |
| } |  |  |  |

6.3B.3.2.5	Test requirement

The requirement for the power measured in steps (2), (3) and (4) of the test procedure shall not exceed the values specified in Table 6.3B.1.5-1.

Table 6.3B.3.2.5-1: General ON/OFF time mask for category NB1 and NB2

|  | Channel bandwidth / minimum output power / measurement bandwidth |
| --- | --- |
|  | 200 kHz |
| Transmit OFF power | For carrier frequency f ≤ 3.0GHz: ≤ -48.5 dBm |
| Transmission OFF Measurement bandwidth | 180kHz |
| Expected Transmission ON Measured power | -11 dBm |
| ON power tolerance f ≤ 3.0GHz | ± 7.5 dB |

### 6.3B.4	Power Control for category NB1 and NB2

### 6.3B.4.1	Power Control Absolute power tolerance for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3B.4.1.1	Test purpose

To verify the ability of the UE transmitter to set its initial output power to a specific value at the start of a contiguous transmission or non-contiguous transmission with a long transmission gap, i.e., transmission gap is larger than 20 ms.

6.3B.4.1.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.3B.4.1.3	Minimum conformance requirements

The minimum requirement for absolute power tolerance is given in Table 6.3B.4.1.3-1 over the power range bounded by the Maximum output power as defined in subclause 6.2B and the Minimum output power as defined in subclause 6.3B

Table 6.3B.4.1.3-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |
| Extreme | ± 12.0 dB |

The normative reference for this requirement is TS 36.102 [11] clause 6.3B.3.4.

6.3B.4.1.4	Test description

6.3B.4.1.4.1	Initial conditions

Initial conditions are set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2B. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in Table 6.3B.4.1.4.1-1. The details of the uplink reference measurement channel (RMCs) are specified in TS 36.521-1[14] Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.3B.4.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] clause 8.1.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS36.508 [12] clause 8.1.3.1 |  |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |
|  | Modulation | Subcarriers | Modulation | Ntones | Subcarrier spacing |
| 1 | N/A |  | QPSK | 1@0 | 3.75 kHz |
| 2 |  |  | QPSK | 1@0 | 15 kHz |
| 3 (NOTE 1) |  |  | QPSK | 12@0 | 15 kHz |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1 and C.3.0, and uplink signals according to TS 36.521-1 [14] Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channel is set according to Table 6.3B.4.1.4.1-1.

5.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.4.1.4.3.

6.3B.4.1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.3B.4.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	The SS shall configure the UE to transmit according to Table 6.3B.4.1.4.3-1 and Table 6.3B.4.1.4.3-2.

3.	Measure the initial output power of the first resource unit of UE NPUSCH first transmission. The transient periods of 20us are excluded. The period of measurement shall be at least the continuous duration of one sub-frame (1ms) for 15 kHz sub-carrier spacing or one slot (2ms) excluding the 2304Ts gap when UE is not transmitting for 3.75 kHz sub-carrier spacing. Half-Duplex guard subframes are not under test.

4.	Release the connection through State 3A-NB.

5.	Modify system information elements according to Table 6.3B.4.1.4.3-3 and Table 6.3B.4.1.4.3-4 and notify the UE via paging message with SystemInformationModification included.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 using the new UL power control setting.

7.	SS sends uplink scheduling information for each UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.3B.4.1.4.1-1. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

8.	Measure the initial output power of the first resource unit of UE NPUSCH first transmission. The transient periods of 20us are excluded. The period of measurement shall be at least the continuous duration of one sub-frame (1ms) for 15 kHz sub-carrier spacing or one slot (2ms) excluding the 2304Ts gap when UE is not transmitting for 3.75 kHz sub-carrier spacing. Half-Duplex guard subframes are not under test. For TDD slots with transient periods are not under test.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.3B.4.1.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 with the following exceptions.

Table 6.3B.4.1.4.3-1: UplinkPowerControlCommon: Test point 1

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-14 UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | -124 (-124 dBm) | Test point 1 to verify a UE relative low initial power transmission |  |

Table 6.3B.4.1.4.3-2: NPDSCH-ConfigCommon-NB-DEFAULT: Test Point 1

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | 20 (dBm) |  |  |
| } |  |  |  |

Table 6.3B.4.1.4.3-3: UplinkPowerControlCommon: Test point 2

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-14 UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | -94 (-94 dBm) | Test point 2 to verify a UE relative high initial power transmission |  |

Table 6.3B.4.1.4.3-4: NPDSCH-ConfigCommon-NB-DEFAULT: Test Point 2

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | 3 (dBm) |  |  |
| } |  |  |  |

Table 6.3B.4.1.4.3-5: PhysicalConfigDedicated

| Derivation Path: TS 36.508 [12] clause 8.1.8.2, Table 8.1.8.2.1.6-1 PhysicalConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |
| uplinkPowerControlDedicated-r13 |  | UplinkPowerControlDedicated-NB-DEFAULT | See subclause 8.1.6.3 |  |

Table 6.3B.4.1.4.3-6: UplinkPowerControlDedicated-NB-DEFAULT

| Derivation Path: TS 36.508 [12] clause 8.1.6.3, Table 8.1.6.3-15 UplinkPowerControlDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-UE-NPUSCH-r13 | 0 | Default |  |
| } |  |  |  |

6.3B.4.1.5	Test requirement

The requirement for the power measured in steps (2), (3) and (4) of the test procedure shall not exceed the values specified in Table 6.3B.4.1.5-1 and 6.3B.4.1.5-2.

Table 6.3B.4.1.5-1: Absolute power tolerance: test point 1

|  | Channel bandwidth / expected output power (dBm) |  |  |
| --- | --- | --- | --- |
|  | Configuration ID 1 3.75 kHz (1 tone) | Configuration ID 2 15 kHz (1 tone) | Configuration ID 3 15 kHz (12 tones) |
| Expected Measured power Normal conditions | -25 dBm | -19 dBm | -8.2 dBm |
| Power tolerance f ≤ 3.0GHz | ± 10.0dB | ± 10.0dB | ± 10.0dB |
| Expected Measured power Extreme conditions | -25 dBm | -19 dBm | -8.2 dBm |
| Power tolerance f ≤ 3.0GHz | ± 13.0dB | ± 13.0dB | ± 13.0dB |
| Note 1: The lower power limit shall not exceed the minimum output power requirements defined in sub-clause 6.3B.1 |  |  |  |

Table 6.3B.4.1.5-2: Absolute power tolerance: test point 2

|  | Channel bandwidth / expected output power (dBm) |  |  |
| --- | --- | --- | --- |
|  | Configuration ID 1 3.75 kHz (1 tone) | Configuration ID 2 15 kHz (1 tone) | Configuration ID 3 15 kHz (12 tones) |
| Expected Measured power Normal conditions | -12 dBm | -6 dBm | 4.8 dBm |
| Power tolerance f ≤ 3.0GHz | ± 10.0dB | ± 10.0dB | ± 10.0dB |
| Expected Measured power Extreme conditions | -12 dBm | -6 dBm | 4.8 dBm |
| Power tolerance f ≤ 3.0GHz | ± 13.0dB | ± 13.0dB | ± 13.0dB |
| Note 1: The upper power limit shall not exceed the maximum output power requirements defined in sub-clause 6.2B.1 |  |  |  |

### 6.3B.4.2 Power Control Relative power tolerance for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3B.4.2.1	Test purpose

To verify the ability of the UE transmitter to set its output power relatively to the power in a target sub-frame relatively to the power of the most recently transmitted reference sub-frame if the transmission gap between these sub-frames is ≤ 20 ms.

6.3B.4.2.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.3B.4.2.3	Minimum conformance requirements

Category NB1 and NB2 UE relative power control requirement is defined for NPRACH power step values of 0, 2, 4 and 6 dB. For NPRACH transmission, the relative tolerance is the ability of the UE transmitter to set its output power relatively to the power of the most recently transmitted preamble. The measurement period for the NPRACH preamble is specified in Table 6.3B.3.2.3-1.

The requirements specified in Table 6.3B.4.2.3-1 apply when the power of the target and reference sub-frames are within the power range bounded by the Minimum output power as defined in subclause 6.3B.1 and the maximum output power as defined in subclause 6.2B.1

Table 6.3B.4.2.3-1: Relative power tolerance for category NB1 and NB2 NPRACH transmission (normal conditions)

| Power step P [dB] | NPRACH [dB] |
| --- | --- |
| ΔP = 0 | ±1.5 |
| ΔP = 2 | ±2.0 |
| ΔP = 4 | ±3.5 |
| ΔP = 6 | ±4.0 |
| NOTE: For extreme conditions an additional ± 2.0 dB relaxation is allowed. |  |

The power step (ΔP) is defined as the difference in the calculated setting of the UE Transmit power between the target and reference sub-frames. The error is the difference between ΔP and the power change measured at the UE antenna port with the power of the cell-specific reference signals kept constant. The error shall be less than the relative power tolerance specified in Table 6.3B.4.2.3-1.

The normative reference for this requirement is TS 36.102 [11] clause 6.3B.4.

6.3B.4.2.4	Test description

6.3B.4.2.4.1	Initial conditions

Initial conditions are set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2B. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3B.4.2.4.1-1. The details of the uplink reference measurement channel (RMCs) are specified in TS 36.521-1[14] Annex A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.3B.4.2.4.1-1: Test Configuration Table

| Initial Conditions |  |
| --- | --- |
| Test Environment as specified in TS 36.508 [12] clause 8.1.1 | Normal, TL/VL, TL/VH, TH/VL, TH/VH |
| Test Frequencies as specified in TS36.508 [12] clause 8.1.3.1 | Frequency ranges defined in Annex K.1.1 |
| NPRACH preamble format | 0 |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C.0, C.1 and C.3.0, and uplink signals according to TS 36.521-1 [14] Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.3B.4.2.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	Ensure the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.4.2.4.3.

6.3B.4.2.4.2	Test procedure

1.	The SS send the paging and UE shall send a preamble to the SS.

2.	NPRACH is set according to Table 6.3B.2.4.1-1.

3.	UE shall send a preamble to the SS.

4.	In response to the preamble, the SS shall transmit a random access response not corresponding to the transmitted random access preamble, or send no response.

5.	The UE shall consider the random access response reception not successful then re-transmit the preamble with the calculated NPRACH transmission power.

6.	Measure the output power of the transmitted NPRACH preamble according to Figure 6.3B.3.2.3-1. Note that the measurement does not need to be done for overall measurement range at one time. The measurement range can be divided into few ranges. Each range needs to overlap neighbouring one.

7.	Switches off and on the UE and ensure the UE is in State 3A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.4.2.4.3.

8.	Repeat test with step 1-6 as indicated in section 6.3B.4.2.4.3.

6.3B.4.2.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 with the following exceptions.

Table 6.3B.4.2.4.3-1: NPDSCH-ConfigCommon-NB-DEFAULT: NPRACH measurement

| Derivation Path: TS 36.508 [7] clause 8.1.6, Table 8.1.6.3-4 NPDSCH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | -22 (dBm) |  |  |
| } |  |  |  |

Table 6.3B.4.2.4.3-2: RACH-ConfigCommon-NB-DEFAULT: NPRACH measurement (Subtest 1: power step size ΔP = 2 dB)

| Derivation Path: TS 36.508 [7] clause 8.1.6, Table 8.1.6.3-8 RACH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleTransMax-CE-r13 | n50 |  |  |
| } |  |  |  |

Table 6.3B.4.2.4.3-3: RACH-ConfigCommon-NB-DEFAULT: NPRACH measurement (Subtest 2: power step size ΔP = 6 dB)

| Derivation Path: TS 36.508 [7] clause 8.1.6, Table 8.1.6.3-8 RACH-ConfigCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleTransMax-CE-r13 | n10 |  |  |
| powerRampingParameters-r13 SEQUENCE { |  |  |  |
| powerRampingStep | dB6 |  |  |
| } |  |  |  |
| } |  |  |  |

6.3B.4.2.5	Test requirement

Each UE power step measured in the test procedure 6.3B.4.2.4.2 should satisfy the test requirements specified in Table 6.3B.4.2.5-1 for normal conditions; for extreme conditions an additional ± 2.0 dB relaxation is allowed.

Table 6.3B.4.2.5-1: Relative power tolerance for category NB1 and NB2 NPRACH transmission (normal conditions – Note 1)

| Expected power step size (up) ΔP [dB] | NPRACH [dB] |
| --- | --- |
| ΔP = 2 | 2 ± (2.7) |
| ΔP = 6 | 6 ± (4.7) |
| Note 1: For extreme conditions an additional ± 2.0 dB relaxation is allowed. Note 2: Only UE output power measurements within the range -39.3 to 20.3 dBm for Power Class 3, or -39.3 to 16.8 dBm for Power Class 5 shall be considered in the pass/fail criteria. |  |

### 6.3B.4.3 Aggregate power control tolerance for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.3B.4.3.1	Test purpose

To verify the ability of a category NB1 and NB2 UE to maintain its output power in non-contiguous transmission with respect to the first UE transmission, when the uplink power control parameters as defined in TS 36.213 are constant and α is set to 0.

6.3B.4.3.2	Test applicability

This test case applies to all types of NB-IoT FDD UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.3B.4.3.3	Minimum conformance requirements

The UE shall meet the requirements specified in Table 6.3B.4.3.3-1 for aggregate power control over the power range bounded by the minimum output power as defined in subclause 6.3B.1 and the maximum output power as defined in subclause 6.2B.1.

Table 6.3B.4.3.3-1: Aggregate power control tolerance for HD-FDD

| UL channel | Aggregate power tolerance |  |
| --- | --- | --- |
|  | 15 kHz / 12 tones within 53 ms | 15 kHz / 1 tone within 104 ms |
| NPUSCH | ±3.5 dB |  |
| NOTE: For five consecutive UE transmissions the transmission gaps are 12 ms for 12 tone and 16 ms for single tone transmissions. Uplink scheduling grant is transmitted via NPDCCH eight subframes before NPUSCH transmission. |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 6.3B.4.

6.3B.4.3.4	Test description

6.3B.4.3.4.1	Initial conditions

Initial conditions are set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for NB-IoT in clause 5.2B. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 6.3B.4.3.4.1-1. The details of the uplink reference measurement channel (RMCs) are specified in TS 36.521-1[14] Annex A.2.4. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521-1[14] Annex C.2.

Table 6.3B.4.3.4.1-1: Test Configuration

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.1 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones, start position | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 15 |
| 2 |  | QPSK | 1@11 | 15 |
| 3 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1[14] Annex C and uplink signals according to Annex H.

4.	The UL Reference Measurement channel is set according to Table 6.3B.4.3.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1[14] Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.3B.4.3.4.3.

6.3B.4.3.4.2	Test procedure

The procedure is separated in two subtests to verify single tone and multi tone NPUSCH aggregate power control tolerance respectively. The uplink transmission patterns are described in figure 6.3B.4.3.4.2-1.

Figure 6.3B.4.3.4.2-1 Test uplink transmission for HD-FDD

For single tone NPUSCH transmission scenario:

1.  SS sends uplink scheduling information for each UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.3B.4.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.  Schedule the UE's NPUSCH data transmission for period containing SI message transmission according to TS 36.521-1[14] Annex A.2.4 and make the UE transmit NPUSCH with 32 or 48 slots (16 or 24 ms) gap for HD-FDD or 46 slots (23ms) gap for TDD. Uplink scheduling grant is transmitted via NPDCCH 9 subframes before NPUSCH transmission.

3.  Measure the power of 5 consecutive NPUSCH transmissions to verify the UE transmitted NPUSCH power is maintained within 112 ms for HD-FDD. The transient periods of 20us are excluded from the power measurement.

4.  Repeat step 2 and 3 for configuration ID 2 in Table 6.3B.4.3.4.1-1.

For 12 tones NPUSCH transmission scenario:

0.  SS release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 and configure the UE to the new UL power level with messages in Table 6.3B.4.3.4.3-2.

1.  SS sends uplink scheduling information for each UL HARQ process via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.3B.4.3.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.  Schedule the UE's NPUSCH data transmission for period where there is no transmission of SI messages according to TS 36.521-1[14] Annex A.2.4 and make the UE transmit NPUSCH with 28, 30 or 32 slots (14, 15 or 16 ms) gap for HD-FDD . Uplink scheduling grant is transmitted via NPDCCH 9 subframes before NPUSCH transmission.

3.  Measure the power of 5 consecutive NPUSCH transmissions to verify the UE transmitted NPUSCH power is maintained within 65 ms for HD-FDD. The transient periods of 20us are excluded from the power measurement.

6.3B.4.3.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6 with the following exceptions.

Table 6.3B.4.3.4.3-1: P0-NominalNPUSCH-r13 configuration for single tone 15 kHz scenario

| Derivation Path: 36.508 clause 8.1.6.3 Table 8.1.6.3-14: UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | 0 (0 dBm) |  |  |
| alpha-r13 | al0 (0) |  |  |
| deltaPreambleMsg3-r13 | 4 |  |  |
| } |  |  |  |

Table 6.3B.4.3.4.3-2: P0-NominalNPUSCH-r13 configuration for 12 tones 15 kHz scenario

| Derivation Path: 36.508 clause 8.1.6.3 Table 8.1.6.3-14: UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | -11 (-0.2 dBm) |  |  |
| alpha-r13 | al0 (0) |  |  |
| deltaPreambleMsg3-r13 | 4 |  |  |
| } |  |  |  |

6.3B.4.3.5	Test requirement

The requirement for the power measurements made in step 4 of the test procedure shall not exceed the values specified in Table 6.3B.4.3.5-1. The power measurement period shall be 1 resource unit excluding transient periods.

Table 6.3B.4.3.5-1: Power control tolerance

| UL channel | Test requirement measured power |
| --- | --- |
| NPUSCH | Given 5 power measurements in the pattern, the 2nd, 3rd, 4th, and 5th measurements shall be within ± 4.2 dB of the 1st measurement. |

## 6.4	Transmit signal quality

This clause is reserved.

## 6.4A	Transmit signal quality for category M1

### 6.4A.1	Frequency error for UE category M1

### 6.4A.2	Transmit modulation quality for category M1

## 6.4B	Transmit signal quality for category NB1 and NB2

### 6.4B.1	Frequency error for UE category NB1 and NB2

### 6.4B.2	Transmit modulation quality for Category NB1 and NB2

## 6.5	Output RF spectrum emissions

The output UE transmitter spectrum consists of the three components; the emission within the occupied bandwidth (channel bandwidth), the Out Of Band (OOB) emissions and the far out spurious emission domain.

Figure 6.5-1: Transmitter RF spectrum

## 6.5A	Output RF spectrum emissions for category M1

### 6.5A.1	General

The definitions in clause 6.5 shall apply.

### 6.5A.2	Occupied bandwidth for category M1

### 6.5A.3	Out of band emission for category M1

#### 6.5A.3.1	General

The out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an Adjacent Channel Leakage power Ratio.

#### 6.5A.3.2	Spectrum emission mask

#### 6.5A.3.3	Additional Spectrum Emission Mask for category M1

The additional spectrum emission mask is not applicable.

#### 6.5A.3.4	Adjacent Channel Leakage Ratio for category M1

### 6.5A.4	Spurious emission for category M1

#### 6.5A.4.1	General

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements inline with SM.329 [9] and E-UTRA operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5A.4.2	Minimum requirements

#### 6.5A.4.3	Spurious emission band UE co-existence

#### 6.5A.4.4	Additional spurious emissions

##### 6.5A.4.4.0	Minimum conformance requirements

##### 6.5A.4.4.0.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious

emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a

specific deployment scenario as part of the cell handover/broadcast message.

NOTE: In addition to the requirements below, additional UE region-specific emissions requirements for European are expected to be added once more information becomes available.

##### 6.5A.4.4.0.2	Minimum requirement (network signalled value "NS_02N")

##### 6.5A.4.4.0.3	Minimum requirement (network signalled value "NS_24")

##### 6.5A.4.4.1	Additional spurious emissions for category M1

## 6.5B	Output RF spectrum emissions for category NB1 and NB2

### 6.5B.1	General

The definitions in clause 6.5 shall apply.

### 6.5B.2	Occupied bandwidth for category NB1 and NB2

### 6.5B.3	Out of band emission for category NB1 and NB2

#### 6.5B.3.1	General

The out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an Adjacent Channel Leakage power Ratio.

#### 6.5B.3.2	Spectrum emission mask

#### 6.5B.3.3	Additional Spectrum Emission Mask for category NB1 and NB2

The additional spectrum emission mask for category NB1 and NB2 is not applicable.

#### 6.5B.3.4	Adjacent Channel Leakage Ratio for category NB1 and NB2

### 6.5B.4	Spurious emission for category NB1 and NB2

#### 6.5B.4.1	General

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements inline with SM.329 [9] and E-UTRA operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.5B.4.2	Minimum requirements

#### 6.5B.4.3	Spurious emission band UE co-existence

#### 6.5B.4.4	Additional spurious emissions

##### 6.5B.4.4.0	Minimum conformance requirements

##### 6.5B.4.4.0.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE: In addition to the requirements below, additional UE region-specific emissions requirements for European are expected to be added once more information becomes available.

##### 6.5B.4.4.0.2	Minimum requirement (network signalled value "NS_02N")

##### 6.5B.4.4.0.3	Minimum requirement (network signalled value "NS_24")

##### 6.5B.4.4.1	Additional spurious emissions for category NB1 and NB2

## 6.6	Transmit intermodulation

This clause is reserved.

## 6.6A	Transmit intermodulation for category M1

For category M1 UE, Tx intermodulation requirements are not applicable.

## 6.6B	Transmit intermodulation for category NB1 and NB2

# 7	Receiver characteristics

## 7.1	General

## 7.2	Diversity characteristics

## 7.3	Reference sensitivity power level

This clause is reserved.

## 7.3A	Reference sensitivity power level for UE category M1

## 7.3B	Reference sensitivity power level for UE category NB1 and NB2

## 7.4	Maximum input level

## 7.4A	Maximum input level for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

7.4A.1	Test purpose

Maximum input level tests the ability of category M1 UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to an e-NodeB.

7.4A.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward of category M1 that support satellite access operation.

7.4A.3	Minimum conformance requirements

This is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1) with parameters specified in Table 7.4A.3-1.

Table 7.4A.3-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -402 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: Reference measurement channel is TS 36.101 [7] Annex A.3.2: 64QAM, R=3/4 variant with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 of TS 36.101 [7]. |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.4A.

7.4A.4	Test description

7.4A.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands defined for CAT M1 in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and are shown in table 7.4A.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS36.521-1 [14] Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 7.4A.4.1-1: Test Configuration Table RX test cases UE Cat-M1

FFS

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.4A.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 7.4A.4.3.

7.4A.4.2	Test procedure

1.	SS transmits PDSCH via M-PDCCH DCI format 6-1A for C_RNTI to transmit the DL RMC according to Table 7.4A.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via M-PDCCH DCI format 0 for C_RNTI to schedule the UL RMC according to Table 7.4A.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value defined in Table 7.4A.5-1. Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power is within +0, - 3.4 dB of the target level in Table 7.4A.5-1 for carrier frequency f ≤ 3.0GHz or within +0, -4.0 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the Throughput measurement.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

7.4A.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the following exception.

Table 7.4A.4.3-1: UplinkPowerControlDedicated

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-UePUSCH | 0 |  |  |
| deltaMCS-Enabled | en0 |  |  |
| accumulationEnabled | TRUE |  |  |
| p0-uePUCCH | 0 |  |  |
| pSRS-Offset | 3 (-6 dB) |  |  |
| filterCoefficient | fc8 | larger filter length is used to reduce the RSRP measurement variation |  |
| } |  |  |  |

Table 7.4A.4.3-2: EPDCCH-Config-r11-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| mpdcch-NumRepetition-r13 | r1 |  |  |

7.4A.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS36.521-1 [14] Annex A.2.2, A.2.3 and A.3.2 with parameters specified in Table 7.4A.5-1.

Table 7.4A.5-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -402 +TT |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L with PCMAX_L as defined in clause 6.2A.4. NOTE 2: Reference measurement channel is TS36.521-1 [14] Annex A.3.2 64QAM R=3/4variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in TS36.521-1 [14] Annex A.5.1.1. |  |  |

## 7.4B	Maximum input level for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

7.4B.1	Test purpose

Maximum input level tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to an e-NodeB.

7.4B.2	Test applicability

This test case applies to all types of NB-IoT HD-FDD UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

7.4B.3	Minimum conformance requirements

This is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

Category NB1 and NB2 UE maximum input level requirement is -40 dBm. For this input level the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.3.2 of TS 36.101 [7].

The normative reference for this requirement is TS 36.102 [11] clause 7.4B.

7.4B.4	Test description

7.4B.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the set of E-UTRA operating bands defined for NB-IoT in clause 5.2.F. All of these configurations shall be tested with applicable test parameters, and are shown in table 7.4F.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS36.521-1 [14] Annexes A.2 and A.3 respectively. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 7.4B.4.1-1: Test Configuration Table

FFS

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.3 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C1 and C.3.0, and NPUSCH Format 2 is used to carry ACK/NACK on the uplink.

4.	The DL Reference Measurement channels are set according to Table 7.4B.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.4B.4.3.

7.4B.4.2	Test procedure

1.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Table 7.4B.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The UE will send the HARQ feedback based on information contain in DCI format N1.

2.	Set the Downlink signal level to the value defined in Table 7.4B.5-1.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to TS36.521-1 [14] Annex G.2.

7.4B.4.3	Message contents

Message contents are according to TS 36.508 [12] clause 8.1.6.

7.4B.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS36.521-1 [14] Annex A.3.2 with parameters specified in Table 7.4B.5-1.

Table 7.4B.5-1: Maximum input level for category NB1 and NB2

| Rx Parameter | Units | Maximum input level test requirement |
| --- | --- | --- |
| Power in Transmission Bandwidth Configuration | dBm | For carrier frequency f ≤ 3.0GHz: -40 +TT |

## 7.5	Adjacent Channel Selectivity (ACS)

## 7.5A	Adjacent Channel Selectivity for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

7.5A.1	Test purpose

Adjacent channel selectivity tests the ability of category M1 UE to receive data with a given average throughput for a specified reference measurement channel, in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when other e-NodeB transmitters exist in the adjacent channel.

7.5A.2	Test applicability

This test applies to all types of E-UTRA UE release 17 and forward of category M1 that support satellite access operation.

7.5A.3	Minimum conformance requirements

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a E-UTRA signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The UE shall fulfil the minimum requirement specified in Table 7.5A.3-1 for all values of an adjacent channel interferer up to –40 dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5A.3-2 and Table 7.5A.3-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 [7] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 [7] Annex A.5.1.1). For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.5A.3-1: Adjacent channel selectivity

|  |  | Channel bandwidth |
| --- | --- | --- |
| Rx Parameter | Units | 1.4 MHz |
| ACS | dB | 33.0 |

Table 7.5A.3-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |
| --- | --- | --- | --- |
|  |  | 1.4 MHz |  |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |
| PInterferer | dBm | REFSENS +45.5dB |  |
| BWInterferer | MHz | 1.4 |  |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 [7] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 7.5A.3-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -71.5 |
| PInterferer | dBm | -40 |
| BWInterferer | MHz | 1.4 |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 [7] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.5A.

7.5A.4	Test description

7.5A.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands defined for CAT M1 in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.5A.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS36.521-1 [14] Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS36.521-1 [14] Annex A.5. Configurations of PDSCH and MPDCCH before measurement are specified in TS36.521-1 [14] Annex C.2.

Table 7.5A.4.1-1: Test Configuration Table RX test cases UE Cat-M1

FFS

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.4 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.5A.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 7.5A.4.3.

7.5A.4.2	Test procedure

1.	SS transmits PDSCH via M-PDCCH DCI format 6-1A for C_RNTI to transmit the DL RMC according to Table 7.5A.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via M-PDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 7.5A.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level to the value as defined in Table 7.5A.5-2 (Case 1). Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power is within +0, - 3.4 dB of the target level in Table 7.5A.5-2 (Case 1) for carrier frequency f ≤ 3.0GHz or within +0, -4.0 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the Throughput measurement.

4.	Set the Interferer signal level to the value as defined in Table 7.5A.5-2 (Case 1) and frequency below the wanted signal, using a modulated interferer bandwidth as defined in Annex D of the present document.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

6.	Repeat steps from 3 to 5, using an interfering signal above the wanted signal in Case 1 at step 4.

7.	Set the Downlink signal level to the value as defined in Table 7.5A.5-3 (Case 2). Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power is within +0, - 3.4 dB of the target level in Table 7.5A.5-3 (Case 2) for carrier frequency f ≤ 3.0GHz or within +0, -4.0 dB of the target level for carrier frequency 3.0GHz < f ≤ 4.2GHz, for at least the duration of the Throughput measurement.

8.	Set the Interferer signal level to the value as defined in Table 7.5A.5-3 (Case 2) and frequency below the wanted signal, using a modulated interferer bandwidth as defined in Annex D of the present document.

9.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

10.	Repeat steps from 7 to 9, using an interfering signal above the wanted signal in Case 2 at step 8.

11.	Repeat for applicable channel bandwidths and operating band combinations in both Case 1 and Case 2.

7.5A.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the following exception.

Table 7.5A.4.3-1: UplinkPowerControlDedicated

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-UePUSCH | 0 |  |  |
| deltaMCS-Enabled | en0 |  |  |
| accumulationEnabled | TRUE |  |  |
| p0-uePUCCH | 0 |  |  |
| pSRS-Offset | 3 (-6 dB) |  |  |
| filterCoefficient | fc8 | larger filter length is used to reduce the RSRP measurement variation |  |
| } |  |  |  |

7.5A.5	Test requirement

The throughput Rav shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS36.521-1 [14] Annex A.2.2, A.2.3 and A.3.2 with parameters specified in table 7.5A.5-2, and also under the conditions specified in table 7.5A.5-3.

Table 7.5A.5-1: Adjacent channel selectivity

|  |  | Channel bandwidth |
| --- | --- | --- |
| Rx Parameter | Units | 1.4 MHz |
| ACS | dB | 33.0 |

Table 7.5A.5-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |
| --- | --- | --- | --- |
|  |  | 1.4 MHz |  |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |
| PInterferer | dBm | REFSENS +45.5dB |  |
| BWInterferer | MHz | 1.4 |  |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in clause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in TS36.521-1 [14] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in TS36.521-1 [14] Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 and 7.3A-2 should be used as REFSENS for the power in Transmission Bandwidth Configuration and PInterferer. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 7.5A.5-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -71.5 |
| PInterferer | dBm | -40 |
| BWInterferer | MHz | 1.4 |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L with PCMAX_L as defined in clause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in TS36.521-1 [14] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in TS36.521-1 [14] Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. |  |  |

## 7.5B	Adjacent Channel Selectivity for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

7.5B.1	Test purpose

Adjacent channel selectivity tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel, under conditions of ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area when other e-NodeB transmitters exist in the adjacent channel.

7.5B.2	Test applicability

This test case applies to all types of E-UTRA HD-FDD UE release 17 and forward of category NB1 and NB2 that support satellite access.

7.5B.3	Minimum conformance requirements

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a E-UTRA signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

Category NB1 and NB2 UE shall fulfil the minimum requirement specified in Table 7.5B.3-1 for all values of an adjacent channel interferer up to -40 dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5B.3-1 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2.

Table 7.5B.3-1: Adjacent channel selectivity parameters for category NB1 and NB2

| ACS1 test Parameters |  |  |
| --- | --- | --- |
| Interferer | GSM (GMSK) | E-UTRA |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + 14 dB |  |
| Interferer signal power (PInterferer ) / dBm | REFSENS + 42 dB | REFSENS + 47 dB |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | ±200 kHz | ±2.5 MHz |
| ACS2 test Parameters |  |  |
| Interferer | GSM (GMSK) | E-UTRA |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | -68 dBm | -73 dBm |
| Interferer signal power (PInterferer ) / dBm | -40 dBm |  |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | ±200 kHz | ±2.5 MHz |

The normative reference for this requirement is TS 36.102 [11] clause 7.5B.

7.5B.4	Test description

7.5B.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands defined for CAT NB1 and NB2 in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.5B.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in TS36.521-1 [14] Annexes A.2 and A.3 respectively. The details of the OCNG patterns used are specified in TS36.521-1 [14] Annex A.5. Configurations of PDSCH and MPDCCH before measurement are specified in TS36.521-1 [14] Annex C.2.

Table 7.5B.4.1-1: Test Configuration Table

FFS

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.4 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.5B.4.1-1.

5	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5. Message contents are defined in clause 7.5B.4.3.

7.5B.4.2	Test procedure

1.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Table 7.5B.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The UE will send HARQ feedback based on information contained in DCI format N1.

2.	Set the Downlink signal level to the value defined for ACS1, GSM in Table 7.5B.5-1. For steps 2 to 5 and 6 to 9, use the default message contents.

3.	Set the Interferer signal level to the value defined for ACS1, GSM in Table 7.5B.5-1, with frequency below the wanted signal according to table 7.5B.5-1, using a modulated interferer bandwidth as defined in Annex D.2 of the present document.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

5.	Repeat steps 2 to 4, using an interfering signal above the wanted signal at step 3.

6.	Set the Downlink signal level to the value defined for ACS1, E-UTRA in Table 7.5B.5-1.

7.	Set the Interferer signal level to the value defined for ACS1, E-UTRA in Table 7.5B.5-1, with frequency below the wanted signal according to table 7.5B.5-1, using a modulated interferer bandwidth as defined in Annex D.2 of the present document.

8.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

9.	Repeat steps 6 to 8, using an interfering signal above the wanted signal at step 7.

10.	Release the connection through State 3A-NB.

11.	Modify system information elements according to Table 7.5B.4.3-1 and notify the UE via paging message with SystemInformationModification included.

12.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [7] clause 8.1.5 using the new UL power control setting.

13.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Table 7.5B.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The UE will send HARQ feedback based on information contained in DCI format N1.

14.	Set the Downlink signal level to the value defined for ACS2, GSM in Table 7.5B.5-1. For steps 14 to 17 and 18 to 21, use message contents with the exceptions defined in Table 7.5B.4.3-1.

15.	Set the Interferer signal level to the value defined for ACS2, GSM in Table 7.5B.5-1, with frequency below the wanted signal according to table 7.5B.5-1, using a modulated interferer of 5MHz bandwidth defined in Annex D.2 of the present document.

16.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

17.	Repeat steps 14 to 16, using an interfering signal above the wanted signal at step 15.

18.	Set the Downlink signal level to the value defined for ACS2, E-UTRA in Table 7.5B.5-1.

19.	Set the Interferer signal level to the value defined for ACS2, E-UTRA in Table 7.5B.5-1, with frequency below the wanted signal according to table 7.5B.5-1, using a modulated interferer of 5MHz bandwidth defined in Annex D.2 of the present document.

20.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G.2.

21.	Repeat steps 18 to 20, using an interfering signal above the wanted signal at step 19.

7.5B.4.3	Message contents

Message contents are according to TS 36.508 [7] clause 8.1.6 with the following exceptions:

Table 7.5B.4.3-1: UplinkPowerControlCommon-NB-DEFAULT: ACS2

| Derivation Path: TS 36.508 [7] clause 8.1.6.3, Table 8.1.6.3-14 UplinkPowerControlCommon-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { p0-NominalNPUSCH-r13 | -70 |  |  |

7.5B.5	Test requirement

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS36.521-1 [14] Annex A.3.2 with parameters specified in Table 7.5B.5-1.

Table 7.5B.5-1: Test parameters for Adjacent channel selectivity, category NB1 and NB2

| ACS1 test Parameters |  |  |
| --- | --- | --- |
| Interferer | GSM (GMSK) | E-UTRA |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + 14 dB |  |
| Interferer signal power (PInterferer ) / dBm | REFSENS + 42 dB | REFSENS + 47 dB |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | ±200 kHz | ±2.5 MHz |
| ACS2 test Parameters |  |  |
| Interferer | GSM (GMSK) | E-UTRA |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | -68 dBm | -73 dBm |
| Interferer signal power (PInterferer ) / dBm | -40 dBm |  |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | ±200 kHz | ±2.5 MHz |

## 7.6	Blocking characteristics

This clause is reserved.

## 7.6A	Blocking characteristics for category M1

### 7.6A.1	General

### 7.6A.2	In-band blocking for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

7.6A.2.1	Test Purpose

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

The lack of in-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6A.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 17 and forward of category M1 that support satellite access operation.

7.6A.2.3	Minimum Conformance Requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.101 [7] Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in TS 36.101 [7] Annex A.5.1.1) with parameters specified in Tables 7.6A.2.3-1 and 7.6A.2.3-2. For operating bands with an unpaired DL part (as noted in Table 5.2A-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6A.2.3-1: In band blocking parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| BWInterferer | MHz | 1.4 |
| FIoffset, case 1 | MHz | 2.1+0.0125 |
| FIoffset, case 2 | MHz | 3.5+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 [7] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 7.6A.2.3-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256, 255 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.6A.2.

7.6A.2.4	Test Description

7.6A.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for category M1 in clause 5.2A. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 7.6A.2.4.1-1. The details of the downlink and uplink reference measurement channels (RMCs) are specified in TS 36.521-1 [14] Annexes A.2 and A.3. The details of the OCNG patterns used are specified in TS 36.521-1 [14] Annex A.5. Configurations of PDSCH and MPDCCH before measurement are specified in TS 36.521-1 [14] Annex C.2.

Table 7.6A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] subclause 4.1 |  |  | [Normal] |  |  |
| Test Frequencies as specified in TS36.508 [12] subclause 4.3.1 |  |  | [Mid range] |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | 1.4MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| ID | Mod'n | RB allocation |  | Mod'n | RB allocation |
| 1 | TDB | TDB |  | TDB | TDB |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508[12] Annex A Figure A.4 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [14] Annex C0, C.1 and C.3.1, and uplink signals according to TS 36.521-1 [14] Annex H.1 and H.3.1.

4.	The UL and DL Reference Measurement channels are set according to Table 7.6A.2.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [14] Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 7.6A.2.4.3.

7.6A.2.4.2	Test Procedure

1.	SS transmits PDSCH via MPDCCH DCI format 6-1A for C_RNTI to transmit the DL RMC according to Table 7.6A.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The SS sends one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in TS 36.521-1 [14] Annex A.5.1.1/A.5.2.1.

2.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 7.6A.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the parameters of the signal generator for an interfering signal below the wanted signal in Case 1 according to Tables 7.6A.2.5-1 and 7.6A.2.5-2.

4.	Set the Downlink signal level to the value as defined in Table 7.6A.2.5-1. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6A.2.5-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in [Table F.1.3-1] for the carrier frequency f and the channel bandwidth BW

-	Uplink power control window size = 1dB (UE power step size) + 1.0dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 36.101-1 [7], Table 6.3.5.2.1-1 and is 1.0dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in [Table F.1.3-1].

5.	Measure the average throughput for duration sufficient to achieve statistical significance according to TS 36.521-1 [14] Annex G.2.

6.	Repeat steps from 3 to 5, using an interfering signal above the wanted signal in Case 1 at step 3.

7.	Repeat steps from 3 to 6, using interfering signals in Case 2 at step 3 and 6. The ranges of case 2 are covered in steps equal to the interferer bandwidth. The test frequencies are chosen in analogy to Table 7.6A.2.4.2-1.

Table 7.6A.2.4.2-1: Example for interferer frequencies

|  | Lower frequency | Upper frequency |
| --- | --- | --- |
| Band 256 DL | 2170 MHz | 2200 MHz |
| Band 1 Midrange | 2185 MHz |  |
| Receive band wanted signal (BW 1.4MHz) | 2184.3 MHz | 2145.7 MHz |
| Interferer case 1 | 2182.1875 MHz | 2147.8125 MHz |
| Interferer case 2 (inner frequency) | 2180.7925 MHz | 2149.2075 MHz |
| Interferer case 2 (outer frequency) | 2167.1925 MHz | 2202.8075 MHz |
| Outer limit for in band blocking | 2155MHz | 2215MHz |
| Number of test frequencies case 2 | [10] | [10] |

7.6A.2.4.3	Message Contents

Message contents are according to TS 36.508 [12] subclause 4.6 with the condition CEModeA and the following exception.

Table 7.6A.2.4.3-1: UplinkPowerControlDedicated

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-UE-PUSCH | 0 |  |  |
| deltaMCS-Enabled | en0 |  |  |
| accumulationEnabled | TRUE |  |  |
| p0-UE-PUCCH | 0 |  |  |
| pSRS-Offset | 3 (-6 dB) |  |  |
| filterCoefficient | fc8 | larger filter length is used to reduce the RSRP measurement variation |  |
| } |  |  |  |

7.6A.2.5	Test Requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [14] Annex A.3.2 with parameters specified in Tables 7.6A.2.5-1 and 7.6A.2.5-2.

Table 7.6A.2.5-1: In band blocking parameters

| Rx parameter | Units | Channel bandwidth |
| --- | --- | --- |
|  |  | 1.4 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |
|  |  | 6 |
| BWInterferer | MHz | 1.4 |
| FIoffset, case 1 | MHz | 2.1+0.0125 |
| FIoffset, case 2 | MHz | 3.5+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3A-3 with PCMAX_L as defined in subclause 6.2.5 of TS 36.101 [7]. NOTE 2: The interferer consists of the Reference measurement channel specified in TS 36.101 [7] Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1 and set-up according to Annex C.3.1. NOTE 3: For DL category M1 UE, the reference sensitivity for category M1 in table 7.3A-1 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE 4: For DL category M1 UE, the parameters for the applicable channel bandwidth apply. |  |  |

Table 7.6A.2.5-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 256, 255 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

### 7.6A.3	Out-of-band blocking for category M1

### 7.6A.4	Narrow band blocking for category M1

## 7.6B	Blocking characteristics for category NB1 and NB2

### 7.6B.1	General

### 7.6B.2	In-band blocking for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

7.6B.2.1	Test Purpose

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

The lack of in-band blocking ability will decrease the coverage area when other e-NodeB transmitters exist (except in the adjacent channels and spurious response).

7.6B.2.2	Test Applicability

This test case applies to all types of E-UTRA UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

7.6B.2.3	Minimum Conformance Requirements

Category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in TS 36.101 [7] Annex A.3.2 with parameters specified in Table 7.6B.2.3-1.

Table 7.6B.2.3-1: In-band blocking parameters for category NB1 and NB2

| IBB1 test Parameters |  |
| --- | --- |
| Category NB1 or NB2 signal power (Pwanted) / dBm | REFSENS + 6 dB |
| Interferer | E-UTRA |
| Interferer signal power (PInterferer ) / dBm | - 56 dBm |
| Interferer bandwidth | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | +7.5 MHz + 0.005 MHz and -7.5 MHz - 0.005 MHz |
| IBB2 test Parameters |  |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + 6 dB |
| Interferer | E-UTRA |
| Interferer signal power (PInterferer ) / dBm | - 44 dBm |
| Interferer bandwidth | 5 MHz |
| Interferer offset range from category NB1 or NB2 channel edge | From +12.5 MHz to FDL_high + 15 MHz and From -12.5 MHz to FDL_low - 15 MHz |

The normative reference for this requirement is TS 36.102 [11] clause 7.6B.2.

7.6B.2.4	Test Description

7.6B.2.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions and test frequencies based on the subset of E-UTRA operating bands defined for category NB1 and NB2 in clause 5.2B. All of these configurations shall be tested with applicable test parameters, and are shown in Table 7.6B.2.4.1-1. The details of the downlink reference measurement channels (RMCs) are specified in TS 36.521-1 [14] Annex A.3. Configurations of NPDSCH and NPDCCH before measurement are specified in TS 36.521-1 [14] Annex C.2.

Table 7.6B.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] clause 8.1.1 |  |  | [Normal] |  |  |
| Test Frequencies as specified in TS36.508 [12] clause 8.1.3.1 |  |  | [Frequency ranges defined in TS 36.521-1 [14] Annex K.1.1] |  |  |
| Configuration ID | Downlink Configuration |  | Uplink Configuration |  |  |
|  | Modulation | Subcarriers | Modulation | Ntones | Subcarrier spacing |
| 1 | [QPSK] | [12] | [BPSK] | [1@0] | [15 kHz] |

1.	Connect the SS to the UE antenna connector as shown in TS 36.508 [12] Annex A, Figure A.4 using only the main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to TS 36.521-1 [14] Annex C.0, C.1 and C.2, and NPUSCH Format 2 is used to carry ACK/NACK on the uplink.

4.	The DL Reference Measurement channel is set according to Table 7.6B.2.4.1-1.

5.	Propagation conditions are set according to TS 36.521-1 [14] Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.6B.2.4.3.

7.6B.2.4.2	Test Procedure

1.	SS transmits NPDSCH via NPDCCH DCI format N1 for C_RNTI to transmit the DL RMC according to Table 7.6B.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC. The UE will send HARQ feedback based on information contained in DCI format N1.

2.	Set the downlink signal level according to the Table 7.6B.2.5-1.

3.	Set the parameters of the signal generator for an interfering signal below the wanted signal in IBB1 according to Table 7.6B.2.5-1.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to TS 36.521-1 [14] Annex G.2.

5.	Repeat steps from 3 to 4, using an interfering signal above the wanted signal in IBB1 at step 3.

6.	Repeat steps from 3 to 5, using interfering signals in IBB2 at step 3 and 5. The ranges of IBB2 are covered in steps equal to the interferer bandwidth. The test frequencies are chosen in analogy to Table 7.6B.2.4.2-1.

Table 7.6B.2.4.2-1: Example for interferer frequencies

|  | Lower frequency | Upper frequency |
| --- | --- | --- |
| Band 256 DL | 2170 MHz | 2200 MHz |
| Band 1 Midrange | 2185 MHz |  |
| Receive band wanted signal (BW 200KHz) | 2184.9 MHz | 2185.1 MHz |
| Interferer IBB1 | 2177.395 MHz | 2192.605 MHz |
| Interferer IBB2 | 2172.4 MHz | 2197.6 MHz |
| Outer limit for in band blocking | 2155MHz | 2215MHz |

7.6B.2.4.3	Message Contents

Message contents are according to TS 36.508 [7] clause 8.1.6.

7.6B.2.5	Test Requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in TS 36.521-1 [14] clause A.3.2 with parameters specified in Tables 7.6B.2.5-1.

Table 7.6B.2.5-1: In-band blocking parameters for category NB1 and NB2

| IBB1 test Parameters |  |
| --- | --- |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + 6 dB |
| Interferer | E-UTRA |
| Interferer signal power (PInterferer ) / dBm | - 56 dBm |
| Interferer bandwidth | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | +7.5 MHz + 0.005 MHz and -7.5 MHz - 0.005 MHz |
| IBB2 test Parameters |  |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + 6 dB |
| Interferer | E-UTRA |
| Interferer signal power (PInterferer ) / dBm | - 44 dBm |
| Interferer bandwidth | 5 MHz |
| Interferer offset range from category NB1 or NB2 channel edge | From +12.5 MHz to FDL_high + 15 MHz and From -12.5 MHz to FDL_low - 15 MHz |

### 7.6B.3	Out-of-band blocking for category NB1 and NB2

### 7.6B.4	Narrow band blocking for category NB1 and NB2

## 7.7	Spurious response

This clause is reserved.

## 7.7A	Spurious response for category M1

## 7.7B	Spurious response for category NB1 and NB2

## 7.8	Intermodulation characteristics

## 7.8A	Intermodulation characteristics for category M1

## 7.8B	Intermodulation characteristics for category NB1 and NB2

## 7.9	Spurious emissions

## 7.9A	Spurious emissions for category M1

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

7.9A.1	Test Purpose

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

Test verifies the UE's spurious emissions meet the requirements described in clause 7.9A.3.

Excess spurious emissions increase the interference to other systems.

7.9A.2	Test Applicability

This test case applies to all types of E-UTRA UE release 17 and forward that support satellite access operation.

7.9A.3	Minimum Conformance Requirements

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.9A.3-1

Table 7.9A.3-1: General receiver spurious emission requirements

| Frequency Band | Measurement Bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | -47 dBm |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.9.

7.9A.4	Test Description

7.9A.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.9A.4.1-1. The details of the downlink and uplink reference measurement channels (RMCs) are specified in [Annexes A.3 and A.2] respectively.

Table 7.9A.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] subclause 4.1 |  |  | [Normal] |  |  |
| Test Frequencies as specified in TS36.508 [12] subclause 4.3.1 |  |  | [Low range, Mid range, High range] |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [12] subclause 4.3.1 |  |  | [Highest] |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | Mod'n | RB allocation |  | Mod'n | RB allocation |
| 1 | N/A | 0 |  | N/A | 0 |

1.	Connect a spectrum analyzer (or other suitable test equipment) to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to [Annex C0, C.1 and C.3.1].

4.	The DL Reference Measurement channels are set according to Table 7.9A.4.1-1.

5.	Propagation conditions are set according to [Annex B.0].

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [12] clause 5.2A.2AA. Message contents are defined in clause 7.9A.4.3.

7.9A.4.2	Test Procedure

1.	Sweep the spectrum analyzer (or equivalent equipment) over a frequency range and measure the average power of spurious emission.

2.	Repeat step 1 for all E-UTRA Rx antennas of the UE.

7.9A.4.3	Message Contents

Message contents are according to TS 36.508 [12] subclause 4.6.

7.9A.5	Test Requirement

The measured spurious emissions derived in step 1), shall not exceed the maximum level specified in Table 7.9A.5-1.

Table 7.9A.5-1: General receiver spurious emission requirements

| Frequency Band | Measurement Bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | -47 dBm |  |

## 7.9B	Spurious emissions for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

7.9B.1	Test Purpose

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

Test verifies the UE's spurious emissions meet the requirements described in clause 7.9B.3.

Excess spurious emissions increase the interference to other systems.

7.9B.2	Test Applicability

This test case applies to all types of E-UTRA UE release 17 and forward that support satellite access operation.

7.9B.3	Minimum Conformance Requirements

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.9B.3-1

Table 7.9B.3-1: General receiver spurious emission requirements

| Frequency Band | Measurement Bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | -47 dBm |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.9.

7.9B.4	Test Description

7.9B.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.9B.4.1-1. The details of the downlink and uplink reference measurement channels (RMCs) are specified in [Annexes A.3 and A.2] respectively.

Table 7.9B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] subclause 8.1.1 |  |  | [Normal] |  |  |
| Test Frequencies as specified in TS36.508 [12] subclause 8.1.3.1 |  |  | [Low range, Mid range, High range] |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | Mod'n | RB allocation |  | Mod'n | RB allocation |
| 1 | N/A | 0 |  | N/A | 0 |

1.	Connect a spectrum analyzer (or other suitable test equipment) to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to [Annex C0, C.1 and C.3.1].

4.	The DL Reference Measurement channels are set according to Table 7.9B.4.1-1.

5.	Propagation conditions are set according to [Annex B.0].

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.9B.4.3.

7.9B.4.2	Test Procedure

1.	Sweep the spectrum analyzer (or equivalent equipment) over a frequency range and measure the average power of spurious emission.

2.	Repeat step 1 for all E-UTRA Rx antennas of the UE.

7.9B.4.3	Message Contents

Message contents are according to TS 36.508 [12] subclause 8.1.6.

7.9B.5	Test Requirement

The measured spurious emissions derived in step 1), shall not exceed the maximum level specified in Table 7.9B.5-1.

Table 7.9B.5-1: General receiver spurious emission requirements

| Frequency Band | Measurement Bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | -47 dBm |  |

# 8	Performance requirement

Annex A (reserved): Reserved

Annex B (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-03 | RAN5#98 | R5-230406 |  |  |  | TP to add Foreword and Introduction to TS 36.521-4 | 0.1.0 |
| 2023-03 | RAN5#98 | R5-230407 |  |  |  | TP to add clause 1-3 to TS 36.521-4 | 0.1.0 |
| 2023-03 | RAN5#98 | R5-230408 |  |  |  | TP to add clause 4 to TS 36.521-4 | 0.1.0 |
| 2023-03 | RAN5#98 | R5-230409 |  |  |  | TP to add clause 5 to TS 36.521-4 | 0.1.0 |
| 2023-03 | RAN5#98 | R5-230410 |  |  |  | Skeleton for TS 36.521-4 v0.1.0 | 0.1.0 |
| 2023-03 | RAN5#98 | R5-231824 |  |  |  | New addition of RX test case of Maximum input level for category M1 with NTN | 0.1.0 |
| 2023-03 | RAN5#98 | R5-231825 |  |  |  | New addition of RX test case of Maximum input level for category NB1 and NB2 with NTN | 0.1.0 |
| 2023-03 | RAN5#98 | R5-231826 |  |  |  | New addition of RX test case of Adjacent Channel Selectivity for category M1 with NTN | 0.1.0 |
| 2023-03 | RAN5#98 | R5-231827 |  |  |  | New addition of RX test case of Adjacent Channel Selectivity for category NB1 and NB2 with NTN | 0.1.0 |
| 2023-03 | RAN5#98 | R5-231868 |  |  |  | Adding new test cases for 36.521-4 transmit power of category M1 | 0.1.0 |
| 2023-03 | RAN5#98 | R5-231869 |  |  |  | Adding new test cases for 36.521-4 transmit power of category NB1 and NB2 | 0.1.0 |
| 2023-05 | RAN5#99 | R5-232367 |  |  |  | Introduction of new test case 7.6A.2 In-band blocking for category M1 | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232368 |  |  |  | Introduction of new test case 7.6B.2 In-band blocking for category NB1 and NB2 | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232369 |  |  |  | Introduction of new test case 7.9A Spurious emissions for category M1 | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232370 |  |  |  | Correction of title of TS 36.521-1 in clause 2 References | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232382 |  |  |  | Introduction of new test case 7.9B Spurious emissions for category NB1 and NB2 | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232521 |  |  |  | Editorial correction for some type error in 6.2A | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232522 |  |  |  | Adding test case 6.2B.3 for UE A-MPR for category NB1 and NB2 UE | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232523 |  |  |  | Adding test case 6.3A.1 for UE Minimum output power for category M1 | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232524 |  |  |  | Adding test case 6.3A.2 for Transmit OFF power for category M1 | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232525 |  |  |  | Adding test case 6.3A.3.1 for General ON/OFF time mask | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232526 |  |  |  | Adding test case 6.3A.3.2.1 for PRACH time mask | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232527 |  |  |  | Adding test case 6.3A.3.2.2 for SRS time mask | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232549 |  |  |  | Adding test case 6.3A.4.1 for Power Control Absolute power tolerance | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232550 |  |  |  | Adding test case 6.3A.4.2 Power Control Relative power tolerance | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232551 |  |  |  | Adding test case 6.3A.4.3 for Aggregate power control tolerance | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232552 |  |  |  | Adding test case 6.3B.1 for UE Minimum output power | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232553 |  |  |  | Adding test case 6.3B.2 for Transmit OFF power | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232554 |  |  |  | Adding test case 6.3B.3.1 General ON/OFF time mask | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232555 |  |  |  | Adding test case 6.3B.3.2 NPRACH time mask | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232556 |  |  |  | Adding test case 6.3B.4.1 Power Control Absolute power tolerance | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232557 |  |  |  | Adding test case 6.3B.4.2 Power Control Relative power tolerance | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232558 |  |  |  | Adding test case 6.3B.4.3 Aggregate power control tolerance | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232815 |  |  |  | Introduction of eMTC/NB-IoT NTN Output RF spectrum emissions TC 6.5 | 0.2.0 |
| 2023-05 | RAN5#99 | R5-233574 |  |  |  | Introduction of eMTC NTN Output RF spectrum emissions TC 6.5A | 0.2.0 |
| 2023-05 | RAN5#99 | R5-232817 |  |  |  | Introduction of NB-IoT NTN Output RF spectrum emissions TC 6.5B | 0.2.0 |
| 2023-05 | RAN5#99 | R5-233575 |  |  |  | Update of editor notes for IoT NTN TCs | 0.2.0 |