3GPP TSG WG RAN5 Meeting #99	R5-232382

# Incheon, South Korea, 22 – 26 May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new test case 7.9B Spurious emissions for category NB1 and NB2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 7.9B Spurious emissions for category NB1 and NB2 is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 7.9B Spurious emissions for category NB1 and NB2 was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 7.9B Spurious emissions for category NB1 and NB2 will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9B (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.8B	Intermodulation characteristics for category NB1 and NB2

7.9	Spurious emissions

7.9B	Spurious emissions for category NB1 and NB2

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

7.9B.1	Test Purpose

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

Test verifies the UE's spurious emissions meet the requirements described in clause 7.9B.3.

Excess spurious emissions increase the interference to other systems.

7.9B.2	Test Applicability

This test case applies to all types of E-UTRA UE release 17 and forward that support satellite access operation.

7.9B.3	Minimum Conformance Requirements

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.9B.3-1

Table 7.9B.3-1: General receiver spurious emission requirements

| Frequency Band | Measurement Bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | -47 dBm |  |

The normative reference for this requirement is TS 36.102 [11] clause 7.9.

7.9B.4	Test Description

7.9B.4.1	Initial Conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in Table 5.2-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.9B.4.1-1. The details of the downlink and uplink reference measurement channels (RMCs) are specified in [Annexes A.3 and A.2] respectively.

Table 7.9B.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] subclause 8.1.1 |  |  | [Normal] |  |  |
| Test Frequencies as specified in TS36.508 [12] subclause 8.1.3.1 |  |  | [Low range, Mid range, High range] |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |
|  | Downlink Configuration |  |  | Uplink Configuration |  |
| Test ID | Mod'n | RB allocation |  | Mod'n | RB allocation |
| 1 | N/A | 0 |  | N/A | 0 |

1.	Connect a spectrum analyzer (or other suitable test equipment) to the UE antenna connectors as shown in TS 36.508 [12] Annex A, Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 4.4.3.

3.	Downlink signals are initially set up according to [Annex C0, C.1 and C.3.1].

4.	The DL Reference Measurement channels are set according to Table 7.9B.4.1-1.

5.	Propagation conditions are set according to [Annex B.0].

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 7.9B.4.3.

7.9B.4.2	Test Procedure

1.	Sweep the spectrum analyzer (or equivalent equipment) over a frequency range and measure the average power of spurious emission.

2.	Repeat step 1 for all E-UTRA Rx antennas of the UE.

7.9B.4.3	Message Contents

Message contents are according to TS 36.508 [12] subclause 8.1.6.

7.9B.5	Test Requirement

The measured spurious emissions derived in step 1), shall not exceed the maximum level specified in Table 7.9B.5-1.

Table 7.9B.5-1: General receiver spurious emission requirements

| Frequency Band | Measurement Bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | -47 dBm |  |

8	Performance requirement

<< End of changes >>