3GPP TSG RAN5 Meeting #99	 	R5-233174

Incheon, South Korea, 22nd – 26th May 2023

Source:		Rohde & Schwarz

Title:	On the MU for n259

Agenda Item:		5.4.17

Document for:	Discussion and Endorsement

# Introduction

For the frequency range FR2c, the value of only few MU contributors has been specified so far. In this contribution, we continue the discussion of the MU for band n259.

# Discussion

In this paper, we focus our analysis on PC3, CHBW ≤ 400 MHz, IFF with a maximum device size of 40cm.

## Mismatch uncertainty

The return loss of microwave components decreases with frequency which results in an increased mismatch uncertainty. Based on our assessment, we propose a value of 1.7 dB as default for FR2c. For ACLR testcase, this value has to be multiplied by 2 as outlined in Ref. [1].

Proposal 1: Apply a mismatch uncertainty of 1.7 dB as default in the frequency range from 40.8 GHz to 44.3 GHz, CHBW ≤ 400 MHz, IFF and PC3. For ACLR specify a mismatch uncertainty of 2.4 dB.

# Assumptions

| ID | Description | Assumption |
| --- | --- | --- |
| #1 | Frequency ranges under consideration | FR2c |
| #2 | Size of QZ for IFF | 40 cm |
| #3 | UE power class | PC3 |
| #4 | Max EIRP | EIRP MAX= +43 dBm (rms) |
| #5 | Wanted signal PAPR | 13 dB |
| #6 | Temperature range of the test equipment | 20°C – 35°C |
| #7 | Channel bandwidth | 400 MHz |

# Conclusion

In summary, we have assessed the mismatch uncertainty in the frequency range of FR2c and make the following proposals:

Proposal 1: Apply a mismatch uncertainty of 1.7 dB as default in the frequency range from 40.8 GHz to 44.3 GHz, CHBW ≤ 400 MHz, IFF and PC3. For ACLR specify a mismatch uncertainty of 2.4 dB.

# References

- R5-194624, “On mismatch uncertainty for ACLR”, Rohde & Schwarz, 3GPP TSG RAN WG 5 Meeting #83, May 2019