3GPP TSG-RAN5 Meeting #99	R5-233437

# Incheon, Korea (Republic Of), 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1422 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of default configuration for IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - For GSO condition, as the velocity (VX, VY, VZ) of stateVectors are all zero, that is exactly a GEO, therefore the values of Position (X, Y, Z) in IE servingSatelliteInfo-r17 within SIB31(-NB) needs to be updated for GSO. - Errors in IE RadioResourceConfigCommonSIB-NB-DEFAULT need to be fixed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - SIB31(-NB) were updated for different satellite scenarios. - Errors were fixed in Table 8.1.6.3-9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing or error parts will exist in default test configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.3, 4.6.3, 8.1.4.3.3 and 8.1.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is the outcome of R5-232560 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-232560 -> R5-233437 |  |  |  |  |  |  |  |  |

[Start of change]

4.4.3.3	Common contents of system information blocks

-	SystemInformationBlockType2

[Skip unchanged text]

-	SystemInformationBlockType31

The IE SystemInformationBlockType31 contains satellite assistance information for the serving cell. SystemInformationBlockType31 is only signalled in an NTN cell.

Table 4.4.3.3-21: SystemInformationBlockType31

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  | GSO |
| positionX-r17 | -16976014 |  |  |
| positionY-r17 | 27636499 |  |  |
| positionZ-r17 | 0 |  |  |
| velocityVX-r17 | 0 |  |  |
| velocityVY-r17 | 0 |  |  |
| velocityVZ-r17 | 0 |  |  |
| } |  |  |  |
| orbitalParameters SEQUENCE { |  |  | NGSO |
| semiMajorAxis-r17 | FFS |  |  |
| eccentricity-r17 | FFS |  |  |
| periapsis-r17 | FFS |  |  |
| longitude-r17 | FFS |  |  |
| inclination-r17 | FFS |  |  |
| anomaly-r17 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 8243100 |  | GSO |
|  | FFS |  | NGSO |
| nta-CommonDrift-r17 | 0 |  | GSO |
|  | FFS |  | NGSO |
| nta-CommonDriftVariation-r17 | 0 |  | GSO |
|  | FFS |  | NGSO |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s240 |  |  |
| epochTime-r17 | Not present |  | GSO |
| epochTime-r17 SEQUENCE { |  |  | NGSO |
| startSFN-r17 | SFN of PCell |  |  |
| startSubFrame-r17 | 9 |  |  |
| } |  |  |  |
| k-Offset-r17 | 550 |  | GSO |
|  | FFS |  | NGSO |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

-	SystemInformationBlockType32

The IE SystemInformationBlockType32 contains satellite assistance information for prediction of discontinuous coverage. SystemInformationBlockType32 is only signalled in an NTN cell.

Table 4.4.3.3-22: SystemInformationBlockType32

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType32-r17 ::= SEQUENCE { |  |  |  |
| satelliteInfoList-r17 | FFS |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

[Skip unchanged text]

4.6.3	Radio resource control information elements

Table 4.6.3-0A: Void

[Skip unchanged text]

–	RadioResourceConfigCommonSIB-DEFAULT

Table 4.6.3-14: RadioResourceConfigCommonSIB-DEFAULT

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |
| bcch-Config | BCCH-Config-DEFAULT |  |  |
| pcch-Config | PCCH-Config-DEFAULT |  |  |
| prach-Config | PRACH-ConfigSIB-DEFAULT |  |  |
| pdsch-ConfigCommon | PDSCH-ConfigCommon-DEFAULT |  |  |
| pusch-ConfigCommon | PUSCH-ConfigCommon-DEFAULT |  |  |
| pucch-ConfigCommon | PUCCH-ConfigCommon-DEFAULT |  |  |
| soundingRS-UL-ConfigCommon | SoundingRS-UL-ConfigCommon-DEFAULT |  |  |
| uplinkPowerControlCommon | UplinkPowerControlCommon-DEFAULT |  |  |
| ul-CyclicPrefixLength | len1 |  |  |
| uplinkPowerControlCommon-v1020 | Not present |  |  |
|  | UplinkPowerControlCommon-v1020-DEFAULT |  | CA |
| rach-ConfigCommon-v1250 | Not present |  |  |
| pusch-ConfigCommon-v1270 | Not present |  |  |
| pusch-ConfigCommon-v1270 SEQUENCE { |  |  | UL64QAM |
| enable64QAM-v1270 | true |  |  |
| } |  |  |  |
| bcch-Config-v1310 | Not present |  |  |
| pcch-Config-v1310 | Not present |  |  |
|  | PCCH-Config-v1310-DEFAULT |  | CEmodeA, CEmodeB |
| freqHoppingParameters-r13 | Not Present |  |  |
|  | FreqHoppingParameters-r13-DEFAULT |  | CEmodeA, CEmodeB |
| pdsch-ConfigCommon-v1310 | Not present |  |  |
|  | PDSCH-ConfigCommon-v1310-DEFAULT |  | CEmodeA, CEmodeB |
| pusch-ConfigCommon-v1310 | Not present |  |  |
|  | PUSCH-ConfigCommon-v1310-DEFAULT |  | CEmodeA, CEmodeB |
| prach-ConfigCommon-v1310 | Not present |  |  |
|  | PRACH-ConfigSIB-v1310-DEFAULT |  | CEmodeA, CEmodeB |
| pucch-ConfigCommon-v1310 | Not present |  |  |
|  | PUCCH-ConfigCommon-v1310-DEFAULT |  | CEmodeA, CEmodeB |
| highSpeedConfig-r14 | Not present |  |  |
|  | HighSpeedConfig-r14-DEFAULT |  | HighSpeedEnhance |
| prach-Config-v1430 | Not present |  |  |
|  | PRACH-Config-v1430-DEFAULT |  | HighSpeedEnhance |
| pucch-ConfigCommon-v1430 | Not present |  |  |
| prach-Config-v1530 | Not present |  |  |
|  | PRACH-ConfigSIB-v1530-DEFAULT |  | EDT |
| ce-RSS-Config-r15 | Not present |  |  |
| wus-Config-r15 | Not present |  |  |
| highSpeedConfig-v1530 | Not present |  |  |
| uplinkPowerControlCommon-v1540 | Not present |  |  |
| wus-Config-v1560 | Not present |  |  |
| wus-Config-v1610 | Not present |  |  |
| highSpeedConfig-v1610 | Not present |  |  |
| crs-ChEstMPDCCH-ConfigCommon-r16 | Not present |  |  |
| gwus-Config-r16 | Not present |  |  |
| uplinkPowerControlCommon-v1610 | Not present |  |  |
| rss-MeasConfig-r16 | Not present |  |  |
| rss-MeasNonNCL-r16 | Not present |  |  |
| puncturedSubcarriersDL-r16 | Not present |  |  |
| highSpeedInterRAT-NR-r16 | Not present |  |  |
| pcch-Config-v1700 | Not present |  |  |
| ntn-ConfigCommon-r17 | Not present |  |  |
| ntn-ConfigCommon-r17 SEQUENCE { |  |  | NTN |
| ta-Report-r17 | enabled |  |  |
| t318-r17 | ms4000 |  |  |
| prach-TxDuration-r17 | Not present |  |  |
| prach-TxDuration-r17 SEQUENCE { |  |  | NGSO |
| prach-TxDuration-r17 |  | n64 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| pucch-TxDuration-r17 |  | Not present |  |  |  |  |  |
| pucch-TxDuration-r17 SEQUENCE { |  |  |  |  |  | NGSO |  |
| pucch-TxDuration-r17 |  | sf64 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| pusch-TxDuration-r17 |  | Not present |  |  |  |  |  |
| pusch-TxDuration-r17 SEQUENCE { |  |  |  |  |  | NGSO |  |
| pusch-TxDuration-r17 |  | n64 |  |  |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CA | For Carrier Aggregation test cases |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |
| HighSpeedEnhance | High speed test environment |
| UL64QAM | Used for UL 64QAM testing |
| EDT | For Early Data Transmission testing |
| NTN | Non-Terrestrial Networks test environment |
| NGSO | Non-Geo-Stationary Orbit test environment |

[Skip unchanged text]

8.1.4.3.3	NB-IoT Common contents of system information blocks

-	SystemInformationBlockType2-NB

[Skip unchanged text]

-	SystemInformationBlockType31-NB

The IE SystemInformationBlockType31-NB contains satellite assistance information for the serving cell. SystemInformationBlockType31-NB is only signalled in an NTN cell.

Table 8.1.4.3.3-10: SystemInformationBlockType31-NB

| Derivation Path: TS 36.331 [17], clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  | GSO |
| positionX-r17 | -16976014 |  |  |
| positionY-r17 | 27636499 |  |  |
| positionZ-r17 | 0 |  |  |
| velocityVX-r17 | 0 |  |  |
| velocityVY-r17 | 0 |  |  |
| velocityVZ-r17 | 0 |  |  |
| } |  |  |  |
| orbitalParameters SEQUENCE { |  |  | NGSO |
| semiMajorAxis-r17 | FFS |  |  |
| eccentricity-r17 | FFS |  |  |
| periapsis-r17 | FFS |  |  |
| longitude-r17 | FFS |  |  |
| inclination-r17 | FFS |  |  |
| anomaly-r17 | FFS |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 8243100 |  | GSO |
|  | FFS |  | NGSO |
| nta-CommonDrift-r17 | 0 |  | GSO |
|  | FFS |  | NGSO |
| nta-CommonDriftVariation-r17 | 0 |  | GSO |
|  | FFS |  | NGSO |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s240 |  |  |
| epochTime-r17 | Not present |  | GSO |
| epochTime-r17 SEQUENCE { |  |  | NGSO |
| startSFN-r17 | SFN of PCell |  |  |
| startSubFrame-r17 | 9 |  |  |
| } |  |  |  |
| k-Offset-r17 | 550 |  | GSO |
|  | FFS |  | NGSO |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

-	SystemInformationBlockType32-NB

The IE SystemInformationBlockType32-NB contains satellite assistance information for prediction of discontinuous coverage. SystemInformationBlockType32-NB is only signalled in a NTN cell.

Table 8.1.4.3.3-11: SystemInformationBlockType32-NB

| Derivation Path: TS 36.331 [17], clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType32-NB-r17 ::= SEQUENCE { |  |  |  |
| satelliteInfoList-r17 | FFS |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

[Skip unchanged text]

8.1.6.3	NB-IoT Radio resource control information elements

–	BCCH-Config-NB-DEFAULT

Table 8.1.6.3-1: BCCH-Config-NB-DEFAULT

[Skip unchanged text]

–	RadioResourceConfigCommonSIB-NB-DEFAULT

Table 8.1.6.3-9: RadioResourceConfigCommonSIB-NB-DEFAULT

| Derivation Path: TS 36.331 [17], clause 6.7.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |  |
| rach-ConfigCommon-r13 |  | RACH-ConfigCommon-NB-DEFAULT |  |  |
| bcch-Config-r13 |  | BCCH-Config-NB-DEFAULT |  |  |
| pcch-Config-r13 |  | PCCH-Config-NB-DEFAULT |  |  |
| nprach-Config-r13 |  | NPRACH-ConfigSIB-NB-DEFAULT |  |  |
| npdsch-ConfigCommon-r13 |  | NPDSCH-ConfigCommon-NB-DEFAULT |  |  |
| npusch-ConfigCommon-r13 |  | NPUSCH-ConfigCommon-NB-DEFAULT |  |  |
| dl-Gap-r13 |  | Not present |  |  |
| uplinkPowerControlCommon-r13 |  | UplinkPowerControlCommon-NB-DEFAULT |  |  |
| nprach-Config-v1330 |  | Not present |  |  |
| nprach-Config-v1450 |  | Not present |  |  |
| nprach-Config-v1530 | NPRACH-ConfigSIB-NB-v1530-DEFAULT |  | TDD |  |
| dl-Gap-v1530 | Not present |  |  |  |
| wus-Config-v1530 | Not present |  |  |  |
| nprach-Config-v1550 | NPRACH-ConfigSIB-NB-v1550-DEFAULT |  | TDD |  |
| gwus-Config-r16 | Not present |  |  |  |
| nrs-NonAnchorConfig-r16 | Not present |  |  |  |
| ue-SpecificDRX-CycleMin-r16 | Not present |  |  |  |
| ntn-ConfigCommon-r17 | Not present |  |  |  |
| ntn-ConfigCommon-r17 SEQUENCE { |  |  | NTN |  |
| ta-Report-r17 | enabled |  |  |  |
| t318-r17 | ms8000 |  |  |  |
| nprach-TxDurationFmt01-r17 | Not present |  |  |  |
| nprach-TxDurationFmt01-r17 SEQUENCE { |  |  | NGSO |  |
| nprach-TxDurationFmt01-r17 | n32 |  |  |  |
| } |  |  |  |  |
| nprach-TxDurationFmt2-r17 | Not present |  |  |  |
| nprach-TxDurationFmt2-r17 SEQUENCE { |  |  | NGSO |  |
| nprach-TxDurationFmt2-r17 | n8 |  |  |  |
| } |  |  |  |  |
| npusch-TxDuration-r17 | Not present |  |  |  |
| npusch-TxDuration-r17 SEQUENCE { |  |  | NGSO |  |
| npusch-TxDuration-r17 | ms128 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| NTN | Non-Terrestrial Networks test environment |
| NGSO | Non-Geo-Stationary Orbit test environment |

[End of change]