3GPP TSG RAN WG5 Meeting #2 5G-NR Adhoc	R5-181938

Taoyuan, Taiwan, 9 - 13 April 2018

Source:		Rohde & Schwarz

Title:	Discussion on test point selection for NR Frequency Error in FR1

Agenda Item:		4.1.16

Document for:	Endorsement

# Introduction

This contribution is a supplement to the test point analysis for Frequency error in RF1 [1].

# Discussion

Discussion paper [1] considers the selection of test environment, test frequency, channel bandwidth, subcarrier spacing and uplink configuration. In this discussion paper, we consider the modulation type. For the uplink configuration, we suggest a more straightforward approach.

In LTE, for frequency error only QPSK modulation is tested. While the modulation order does not impact frequency error, both DFT-s-OFDM and CP-OFDM shall be tested to ensure that the UE processes frequency correctly also in the new case of CP-OFDM.

Proposal 1: Test both modulations DFT-s-OFDM QPSK and CP-OFDM QPSK.

As the frequency error is not impacted by the allocation, we suggest outer_full allocation as a straightforward approach.

Proposal 2: Apply outer_full as RB allocation for the uplink.

For the downlink also a full allocation is proposed as the UE’s ability to synchronize its frequency to the downlink is indirectly tested in the REFSENS test case.

Proposal 3: Apply full allocation for the downlink configuration.

# Conclusion

In this paper, we have discussed additional aspects in the test point selection for the test case frequency error in FR1.

Proposal 1: Test both modulations DFT-s-OFDM QPSK and CP-OFDM QPSK.

Proposal 2: Apply outer_full as RB allocation for the uplink.

Proposal 3: Apply full allocation for the downlink configuration.

# References

- R5-181781, Discussion on test point selection for NR Frequency Error in FR1, KDDI, 3GPP TSG RAN5 RAN5 5G-NR Adhoc #2, Taoyuan, Taiwan, April 2018