3GPP TSG-RAN5 Meeting #3	R5-185918

# Jaipur, India, 8th Oct 2018 - 12th Oct 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0023 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to TS 38.521-3 Section 4 with LTE anchor details |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Tech. Netherlands B.V |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2018-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add LTE configuration details for anchor-agnostic approach added in core spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added section 4.6 and updated sections 6.1 and 7.1 to align with core spec guidance |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details in test spec and it will not be aligned with core spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4, 4.6, 6.1, 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with discussion paper R5-185916 |  |  |  |  |  |  |  |  |

## << Start of changes >>

## 4.4	Test points analysis

The information on test point analysis and test point selection including number of test points for each test case is shown in TR 38.905 [7] clause 4.3.

## 4.6	LTE configuration for EN-DC FR1 tests with anchor-agnostic approach

This section applies to EN-DC test cases where LTE anchor needs to be configured as per the anchor-agnostic approach outlined in section 6.1 and 7.1.

For baseline configuration, the E-UTRA carrier will be configured as defined in the equivalent standalone test in TS 36.521-1. However, the below exceptions defined in Table 4.6-1, 4.6-2, 4.6-3 and 4.6-4 are applied. This ensures that the anchor resources do not interfere with NR operation

Table 4.6-1: E-UTRA configuration for EN-DC FR1 tests using anchor agnostic approach

| Parameter | Value | Comments |
| --- | --- | --- |
| DL signal levels during connection setup |  | RS EPRE -85.0 dBm/15kHz |  | DL physical channels as defined in Annex C0, C.1, C.2 and Annex C.3 of TS 36.521-1 36.521-1 annex C.0 defines the default DL power level of RS EPRE to be -85dBm/15kHz. |  |
| UL Signal levels during connection setup | PUSCH Power 0 dBm | Attained by enabling open loop power control and setting up UL signal levels according to Annex H.0, H.2 and H.3 of TS 36.521-1 |
| DL/UL RMC after connection setup |  | 0 RB allocation on both DL and UL |  | Once the LTE link is established, then LTE Tx can be restricted by configuring 0 RB allocation on DL and UL, as well as disabling any UE measurement reports. (see Table 4.6-5) TimeAlignmentTimerDedicated IE to be set to infinity to ensure UE doesn’t look for TA adjustments (See Table 4.6-4) |  |
| CQI Reports and SRS | Disabled (See Table 4.6-2 and 4.6-3) | Disable periodic and aperiodic CQI reports to ensure none of these transmissions occur on the LTE uplink. Since LTE transmissions could easily exceed spurious emissions limits, tests that are intended to measure RF parametrics on the NR should simply avoid LTE transmit altogether. |
| Test Frequency | Mid | As defined in TS 36.508 for the LTE band under test |
| Bandwidth | 5 MHz | Supported by all LTE bands. |

Table 4.6-2 -CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |  |
| cqi-ReportPeriodic |  | NOT PRESENT |  |  |
| } |  |  |  |  |

Table 4.6-3: PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| soundingRS-UL-ConfigDedicated |  | Not present |  | RBC |
| } |  |  |  |  |

Table 4.6-4: MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated |  | Infinity |  |  |

<< Many sections unchanged >>

## 6.1	General

For Tx test cases the identified beam peak direction can be stored and reused for a device under test in various configurations/environments for the full duration of device testing as long as beam peak direction is the same.

Unless otherwise stated the transmitter, characteristics are specified at the antenna connector(s) of the UE for the bands operating on frequency range 1 and over the air of the UE for the bands operating on frequency range 2. The requirements for frequency range 1 and frequency range 2 can be verified separately. For the carrier in frequency range 1, requirements can be verified with NR FR2 link disabled. For the carrier in frequency range 2, requirements can be verified in OTA mode with LTE connecting to the network by OTA without calibration.

Unless otherwise stated, requirements for NR transmitter written in TS 38.101-1 and TS 38.101-2 apply and are assumed anchor agnostic. Requirements are verified under conditions where anchor resources do not interfere NR operation.

<< Many sections unchanged >>

## 7.1	General

For Rx test cases the identified beam peak direction can be stored and reused for a device under test in various configurations/environments for the full duration of device testing as long as beam peak direction is the same.

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE for the bands operating on frequency range 1 and over the air of the UE for the bands operating on frequency range 2. The requirements for frequency range 1 and frequency range 2 can be verified separately. For the carrier in frequency range 1, requirements can be verified with NR FR2 link disabled. For the carrier in frequency range 2, requirements can be verified in OTA mode with LTE connecting to the network by OTA without calibration.

The requirements defined in this clause are the extra requirements compared with the single carrier requirements defined in [2] and [3].

Unless otherwise stated, the UL and DL reference measurement channels are the same with the configurations specified in [2] and [3].

Unless otherwise stated, requirements for NR receiver written in TS 38.101-1 and TS 38.101-2 apply and are assumed anchor agnostic. Requirements are verified under conditions where anchor resources do not interfere NR operation.

## 7.2	Diversity characteristics

<< End of changes >>