3GPP TSG-RAN5 Meeting NR#5 AdHoc	R5-193472

# Xian, China, 9th – 12th April. 2019

Title:							Test configuration table for EN-DC TRx tests with CCs > 2

Source:			Anritsu

Agenda Item:			4.1.16

Document for:			Endorsement

# 1.	Introduction

Test cases with multiple-CCs are about to be added to the test specification TS 38.521-3. In [3], the test approach for EN-DC TRx with CCs > 2 is endorsed however it is still TBD how it should be implemented in the test case.  In this paper, the concrete test configuration table which implements the agreements in [3].

# 2.	Discussion

The agreed proposals in [3] are quoted as below.

Proposal 1 :  NR exception reqs are tested by configuring min. number of LTE CCs concerning that requirements.

Proposal 2 : LTE exception reqs are tested by configuring min. number of NR CCs concerning that requirements.

Proposal 3 : Test cases are defined with the number of CCs in the DC band combination (Option B1)

As is mentioned in [3], following should be clarified in some way in the test case for each EN-DC band combination and its test point.

Test configuration table for each EN-DC band combination needs to clarify followings for each test point.

- Which CCs among all the CCs in the EN-DC band combination should be configured and which are not.

- Whether the anchor agnostic approach apply

- Configurations for the configured CCs

- Requirements for the “tested” CCs

In LTE CA, we had test configuration tables as below for both with and without Wgap. This format worked in LTE for CC<=5.

Figure 1 Test configuration table in LTE CA

Figure 2 Test configuration table in LTE CA(with Wgap)

When it comes to the EN-DC with #CC > 2 test case, we need to consider the following aspects:

- The number of CC could become bigger than 5  (Same will happen for LTE CA, though)

- We need to clarify 1-4 as listed above in some way

- The header of CCs ( “PCC”, “SCC1”, “SCC2” …) would need to specify whether they are in MCG or SCG, and it depends on the number of LTE/NR CCs in each EN-DC band combination

- WGap can happen between any combination of CCs : e.g. PCC(MCG) - PCC(SCG), PCC(MCG) - SCC1(SCG), SCC1(MCG) - SCC1(SCG), PCC(MCG)-SCC1(MCG), PCC(SCG)-SCC1(SCG) etc….

iii) and  iv) would make it difficult to adopt the same table format used in LTE CA. The proposed format is show in Table 1 for the case of #CCs = 4. We now propose to list parameters of one CC within single row. Note that we can use the same table for with WGap and without WGap with this proposal. The one drawback of adopting this format is that the table height will become longer than LTE CA table, but we think it is better than become too wide.

Table 1 Proposed table format

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] subclause 4.1 |  |  |  |  |  | NC |  |  |  |
| Test Frequencies as specified in TS38.508-1 [6] subclause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NRB” column |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ WGap | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-nYD Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | PCC(S) | nY | Highest | Lowest NRB_agg | Low/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
|  | SCC1(S) | nY | Highest |  | Low/CC2 | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
|  | SCC2(S) | nY | Highest |  | Low/CC3 | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| 2 | PCC(M) | X | N/A | Lowest | Mid | QPSK | QPSK | REFSENS | All RBs |
|  | PCC(S) | nY | Highest | Lowest NRB_agg | Low/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
|  | SCC1(S) | nY | Highest |  | Low/CC2 | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
|  | SCC2(S) | nY | Highest |  | Low/CC3 | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| Default Test Settings for a DC_XA-nYA-nZA-nRA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | PCC(S) | nY | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
|  | SCC1(S) | nZ | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
|  | SCC2(S) | nR | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| 2 | PCC(M) | X | N/A | Lowest | Mid | QPSK | QPSK | REFSENS | All RBs |
|  | PCC(S) | nY | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
|  | SCC1(S) | nZ | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
|  | SCC2(S) | nR | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| Default Test Settings for a DC_(n)XAA-nYC Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | Lowest NRB_agg | High/CC1 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | PCC(S) | nX | Highest |  | High/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
|  | SCC1(S) | nY | Highest | Lowest NRB_agg | High/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
|  | SCC2(S) | nY | Highest |  | High/CC2 | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| 2 | PCC(M) | X | N/A | Lowest NRB_agg | High/CC1 | QPSK | QPSK | REFSENS | All RBs |
|  | PCC(S) | nX | Highest |  | High/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
|  | SCC1(S) | nY3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | SCC2(S) | nY3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Default Test Settings for a DC_XA-nXA-nYA-nZA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | Lowest NRB_agg | High with maxWGap | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(S) | nX | Highest |  |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
|  | PCC(S) | nY | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
|  | SCC2(S) | nZ | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| 2 | PCC(M) | X | N/A | Lowest NRB_agg | High with maxWGap | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(S) | nX | Highest |  |  | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
|  | PCC(S) | nY | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
|  | SCC2(S) | nZ | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| Default Test Settings for a DC_XC-nYC Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | Lowest NRB_agg | High/CC1 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(M) | X2 | N/A |  | High/CC2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | PCC(S) | nY | Highest | Lowest NRB_agg | High/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
|  | SCC1(S) | nY | Highest |  | High/CC2 | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| 2 | PCC(M) | X | N/A | Lowest NRB_agg | High/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
|  | SCC1(M) | X | N/A |  | High/CC2 | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
|  | PCC(S) | nY | Highest | Lowest NRB_agg | High/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
|  | SCC1(S) | nY | Highest |  | High/CC2 | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| Default Test Settings for a DC_XA-YA-nZA-nRA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(M) | Y3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | PCC(S) | nZ | Highest | Lowest | QPSK | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
|  | SCC1(S) | nR | Highest | Lowest | QPSK | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| 2 | PCC(M) | X | N/A | Lowest | Mid | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | PCC(S) | nZ | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
|  | SCC1(S) | nR | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| Default Test Settings for a DC_(n)XDA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | Lowest NRB_agg | High/CC1 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(M) | X2 | N/A |  | High/CC2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC2(M) | X2 | N/A |  | High/CC3 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(S) | nX | Highest |  | High/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| 2 | PCC(M) | X | N/A | Lowest NRB_agg | High/CC1 | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | X | N/A |  | High/CC2 | QPSK | QPSK | N/A | All RBs |
|  | SCC2(M) | X | N/A |  | High/CC3 | QPSK | QPSK | N/A | All RBs |
|  | SCC1(S) | nX | Highest |  | High/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | N/A | All RBs |
| Default Test Settings for a DC_XA-YA-ZA-nRA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(M) | Y3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | SCC2(M) | Z3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | PCC(S) | nR | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X | N/A | Lowest | Mid | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y | N/A | Lowest | Mid | QPSK | QPSK | REFSENS | All RBs |
|  | SCC2(M) | Z | N/A | Lowest | Mid | QPSK | QPSK | REFSENS | All RBs |
|  | PCC(S) | nR | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-YA-(n)ZAA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(M) | Y3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | SCC2(M) | Z | N/A | Lowest NRB_agg | High/CC1 | QPSK | QPSK | REFSENS | All RBs |
|  | PCC(S) | nZ | Highest |  | High/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X | N/A | Lowest | Mid | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | SCC2(M) | Z | N/A | Lowest NRB_agg | High/CC1 | QPSK | QPSK | REFSENS | All RBs |
|  | PCC(S) | nZ | Highest |  | High/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XD-nXA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | Lowest NRB_agg | TBD | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(M) | X2 | N/A |  | TBD | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC2(M) | X2 | N/A |  | TBD | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | PCC(S) | nX | Highest |  | TBD | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X | N/A | Lowest NRB_agg | TBD | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | X | N/A |  | TBD | QPSK | QPSK | REFSENS | All RBs |
|  | SCC2(M) | X | N/A |  | TBD | QPSK | QPSK | REFSENS | All RBs |
|  | PCC(S) | nX | Highest |  | TBD | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-YA-YA-nZA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | Lowest | Mid | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(M) | Y3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | SCC2(M) | Y3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | PCC(S) | nZ | Highest | Lowest | CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X | N/A | Lowest | Mid | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y | N/A | Lowest NRB_agg | High with maxWGap | QPSK | QPSK | REFSENS | All RBs |
|  | SCC2(M) | Y | N/A |  |  | QPSK | QPSK | REFSENS | All RBs |
|  | PCC(S) | nZ | Highest | Lowest | Mid | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. |  |  |  |  |  |  |  |  |  |
| NOTE 2: Anchor agnostic approach applies. Configurations as per section 4.6. |  |  |  |  |  |  |  |  |  |
| NOTE 3: Note configured |  |  |  |  |  |  |  |  |  |
| NOTE 4: X, Y and Z in this table correspond to different bands i.e. X != Y != Z |  |  |  |  |  |  |  |  |  |
| NOTE 5: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA |  |  |  |  |  |  |  |  |  |

Note that in Table 1, the contents are just example, and they needs to be determined from individual analysis for each TC and EN-DC band combination.

Proposal 1: Adopt the parameter table format as shown in Table 1 for EN-DC with #CCs > 2

# 3.	Conclusion

Proposal 1: Adopt the parameter table format as shown in Table 1 for EN-DC with #CCs > 2

# 4.	References

[1] R5-185666, “Test approach of EN-DC TRx tests”, Anritsu, RAN5#NR#3 AdHoc

[2] R5-186228, “	Discussion on applicability rules and test coverage for 38.521-3 test cases”, Qualcomm Inc., RAN5 NR#4 AdHoc

[3] R5-190978, “Test approach for EN-DC TRx tests with CCs > 2”, Anritsu, RAN5 NR#4 AdHoc

[4] R5-192602, “Resolving ambiguity in LTE anchor agnostic approach”, Anritsu, RAN5#82