3GPP TSG-RAN5 Meeting NR#5 AdHoc	R5-193228r1

# Xian, China, 9th – 12th April. 2019

Title:							Test points for Wideband Intermodulation for EN-DC including FR1 (3 CCs)

Source:			Anritsu

Agenda Item:			4.1.16

Document for:			Endorsement

# 1.	Introduction

This paper provides the analysis for test points for wideband intermodulation for EN-DC including FR1(3 CCs).

# 2.	Discussion

# 2.1	General consideration

Wideband intermodulation with 2CC (within FR1) is specified based on following principle and test points.

Table 1 Test parameters for wideband intermodulation EN-DC(FR1)

|  | Requirement | ENV | Freq | BW | NR SCS |
| --- | --- | --- | --- | --- | --- |
| Inter-band EN-DC | No exceptional requirement for both LTE and NR | NC | Mid | Highest | Highest |
| Intra-band non-contiguous EN-DC | No exceptional requirement for both LTE and NR | NC | MaxWGap | Highest NRBagg | Highest |
| Intra-band contiguous EN-DC | Exceptional requirement for both LTE and NR | NC | Mid | Highest NRBagg and Lowest NRBagg | Highest |

When it comes to 3CCs, we need to consider the mix of these cases. Table 1 shows the possible unique combination of various 3CC EN-DC configurations.

Table 2 Possible EN-DC Band Combination with 3CCs

| 3CC EN-DC Band combination | #LTE | #NR | Included sub-set EN-DC Config |  |  | Example in 38.101-3 |
| --- | --- | --- | --- | --- | --- | --- |
|  | CC | CC | Inter | Intra-NC | Intra-C |  |
| DC_YA_nXA-nXA | 1 | 2 | 2 | 0 | 0 |  |
| DC_XA_nYA-nZA | 1 | 2 | 2 | 0 | 0 | DC_1A_n28A-n78A |
| DC_XA_nXA-nYA | 1 | 2 | 1 | 1 | 0 | DC_3A_n3A-n77A |
| DC_(n)XAC | 1 | 2 | 0 | 0 | 1 |  |
| DC_XA_nXA-nXA | 1 | 2 | 0 | 2 | 0 |  |
| DC_(n)XAA_nYA | 1 | 2 | 1 | 0 | 1 |  |
| DC_(n)XAA_nXA | 1 | 2 | 0 | 1 | 1 |  |
| DC_XA_nXC | 1 | 2 | 0 | 1 | 0 |  |
| DC_YA_nXC | 1 | 2 | 1 | 0 | 0 | DC_1A_n77C |
| DC_XA-XA_nYA | 2 | 1 | 2 | 0 | 0 | DC_7A-7A_n78A |
| DC_XA-(n)XAA | 2 | 1 | 0 | 1 | 1 |  |
| DC_(n)XCA | 2 | 1 | 0 | 0 | 1 | DC_(n)41CA |
| DC_YA-(n)XAA | 2 | 1 | 1 | 0 | 1 | DC_2A-(n)71AA |
| DC_XC_nYA | 2 | 1 | 1 | 0 | 0 | DC_7C_n78A |
| DC_XA-YA_nXA | 2 | 1 | 1 | 1 | 0 |  |
| DC_XA-XA_nXA | 2 | 1 | 0 | 2 | 0 |  |
| DC_XA-YA_nZA | 2 | 1 | 2 | 0 | 0 | DC_1A-3A_n28A |
| DC_XC_nXA | 2 | 1 | 0 | 1 | 0 | DC_41C_n41A |

The test point for 3CCs case should be defined so that all the consisting sub-set EN-DC configs is tested with is associated parameters in Table 1.

Upon deciding test points, following observations and proposals in [3] should be considered.

From [3]

Observation 1 : NR non-exception reqs are tested with LTE agnostic approach, with only 1 LTE PCC configured using the LTE configuration in section 4.6 of 38.521-3 is enough.

Observation 2 : LTE non-exception reqs are tested in LTE 36.521-1.

Proposal 1 :  NR exception reqs are tested by configuring min. number of LTE CCs concerning that requirements.

Proposal 2 : LTE exception reqs are tested by configuring min. number of NR CCs concerning that requirements.

From [4]

Observation 1 : According to 4.5 in TS 38.521-3, Anchor agnostic approach apply for each EN-DC configuration. Leveraging the results from different EN-DC configuration will not be possible.

Observation 2 : In the 2CC EN-DC test case, the EN-DC configuration should be same as indicated in the TC name even when LTE anchor agnostic approach apply

In addition, for the case of CC>2, one more dimension of “CC mapping” (i.e. CC mapping to PCC/SCC) should be considered so that every band becomes PCC, as done in LTE CA tests.

For NR, regardless of the exceptional/non-exceptional (tested with LTE anchor agnostic), test point should be defined so that all NR band experience PCC(SCG).

Proposal 1 : Define test points so that all NR band experience PCC(SCG) regardless of the NR exceptional/non-exceptional(tested with LTE anchor agnostic)

Note  : Test points for NR non-exceptional requirement, this policy should match that in SA NR CA test points. We assume here that SA NR CA test points are defined with the same manner.

For LTE, same should apply but restricted to the LTE bands associated with exceptional requirement.

Proposal 2 : Define test point so that all LTE bands associated with LTE exceptional requirement experience PCC(MCG)

Note that depending on the TC, especially for higher order EN-DC, the number of TP can be too many if TP is defined straightforwardly. In such case reduction of test points by means of e.g. merging multiple test points to single test points (by e.g. change some parameters) as was done in LTE CA, will be possible.

Ovseravation1 : Reduction of test points can be considered if the combination generates too many test points by means of e.g. merging multiple test points to single test points (by e.g. change some parameters) as was done in LTE CA.

# 2.2	Default test points for wideband intermod for EN-DC including FR1(3 CCs)

Considering the aspects in Section 2.1, the default test points are analyzed and proposed as follow. Note that not all the possible combination in Table 2 is analyzed here, only those with actual EN-DC band combination defined in 38.101-3 is considered. For other combination, TP can be defined on-demand basis.

DC_XA-nYA-nZA, DC_XA-nYC, DC_XA-XA-nYA, DC_XC-nYA, DC_XA-YA-nZA

- Non exceptional requirements for LTE and NR.

- Test the non-exception NR requirement with anchor agnostic approach, only one LTE CC is configured as per section 4.6. For DC_XA-XA-nYA, DC_XC-nYA and DC_XA-YA-nZA, SCC(MCG) is totally not configured.

- Specify one test point referring back to 38.521-1 for the NR configuration.

DC_XC-nXA

- Non exceptional requirement for LTE and NR.

- Test the non-exception NR requirement with anchor agnostic approach, LTE CCs are configured as per section 4.6. 2 contiguous LTE CCs need to be configured as per observation 2 and 3 in [4].

- Specify one test point referring back to 38.521-1 for the NR configuration.

DC_(n)XCA

- Exceptional requirement for LTE and NR

- Two test points defined one for lowest NRBagg another for highest NRBagg

DC_XA-nXA-nYA

- Non exceptional requiremebt between XA and nYA, XA and nXA

- 1 test point for nX being PCC(S) , another for nY begin PCC(S) (as per Proposal 1), both TP applies anchor agnostic approach.

DC_YA-(n)XAA

- Non exceptional requirement between YA and nXA, exceptional requirement between XA and nXA

- Test points defined with band X in PCC in MCG( as per Proposal 2)

- 2 test points for lowest NRBagg and highest NRBagg

- Band Y is nothing to do with the exceptional requirement of LTE and NR being tested, then no need to be configured.

Table 3 shows the table implements above analysis.

Table 3 Proposed test points for wideband intermod within FR1 (3CCs)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [6] subclause 4.1 |  |  |  |  |  | NC |  |  |  |
| Test Frequencies as specified in TS38.508-1 [6] subclause 4.3.1 for different EN-DC bandwidth classes |  |  |  |  |  | For test frequencies refer to “Range” columns. For mapping within Band refer to “CC” columns |  |  |  |
| Test EN-DC bandwidth combination as specified in Table 5.3B.1.2-1 across bandwidth combination sets supported by the UE |  |  |  |  |  | Refer to "NRB” column |  |  |  |
| NR Test SCS as specified in Table 5.3.5-1 in TS 38.521-1[8] |  |  |  |  |  | Refer to "SCS" column |  |  |  |
| Network signalling value |  |  |  |  |  | NS_01 by default |  |  |  |
| Test Parameters for EN-DC Configurations |  |  |  |  |  |  |  |  |  |
| ID | CC (NOTE1) | Band | SCS | NRB | Range/ Wgap | UL MOD | DL MOD | UL Alloc | DL Alloc |
| Default Test Settings for a DC_XA-nYA-nZA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | PCC(S) | As per CA_nXA-nZA in 7.8A.2.1 of 38.521-1 [1] |  |  |  |  |  |  |  |
|  | SCC1(S) |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-nYC Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | PCC(S) | As per CA_nYC in 7.8A.2.1 of 38.521-1 [1] |  |  |  |  |  |  |  |
|  | SCC1(S) |  |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-XA-nYA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(M) | X3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | PCC(S) | As per 7.8.2.1 of 38.521-1[1] |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XC-nYA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(M) | X3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | PCC(S) | As per 7.8.2.1 of 38.521-1[1] |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XA-YA-nZA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(M) | Y3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | PCC(S) | As per 7.8.2.1 of 38.521-1[1] |  |  |  |  |  |  |  |
| Default Test Settings for a DC_XC-nXA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | Highest NRB_agg | TBD | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(M) | X2 | N/A |  | TBD | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | PCC(S) | nX | Highest |  | TBD | As per 7.8.2.1 of 38.521-1[1] |  |  |  |
| Default Test Settings for a DC_(n)XCA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X | N/A | Lowest NRB_agg | Mid/CC1 | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | X | N/A |  | Mid/CC2 | QPSK | QPSK | N/A | All RBs |
|  | PCC(S) | nX | Highest |  | Mid/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| 2 | PCC(M) | X | N/A | Highest NRB_agg | Mid/CC1 | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | X | N/A |  | Mid/CC2 | QPSK | QPSK | N/A | All RBs |
|  | PCC(S) | nX | Highest |  | Mid/CC1 | DFT-s-OFDM QPSK | CP-OFDM QPSK | REFSENS | All RBs |
| Default Test Settings for a DC_XA-nXA-nYA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X2 | N/A | Highest NRB_agg | High with maxWGap | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | PCC(S) | nX | Highest |  |  | As per CA_nXA-nYA in 7.8A.2.1 of 38.521-1 [1] |  |  |  |
|  | SCC1(S) | nY | Highest | Highest | Mid |  |  |  |  |
| 2 | PCC(M) | X2 | N/A | Highest NRB_agg | High with maxWGap | NOTE2 | NOTE2 | NOTE2 | NOTE2 |
|  | SCC1(S) | nX | Highest |  |  | As per CA_nXA-nYA in 7.8A.2.1 of 38.521-1 [1] |  |  |  |
|  | PCC(S) | nY | Highest | Highest | Mid |  |  |  |  |
| Default Test Settings for a DC_YA-(n)XAA Configuration |  |  |  |  |  |  |  |  |  |
| 1 | PCC(M) | X | N/A | Lowest NRB_agg | Mid/CC1 | QPSK | QPSK | REFSENS | All RBs |
|  | PCC(S) | nX | Highest |  | Mid/CC1 | QPSK | QPSK | REFSENS | All RBs |
|  | SCC1(M) | Y3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| 2 | PCC(M) | X | N/A | Highest NRB_agg | Mid/CC1 | QPSK | QPSK | REFSENS | All RBs |
|  | PCC(S) | nX | Highest |  | Mid/CC1 | QPSK | QPSK | N/A | All RBs |
|  | SCC1(M) | Y3 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: (M) and (S) indicate MCG and SCG respectively. |  |  |  |  |  |  |  |  |  |
| NOTE 2: Anchor agnostic approach applies. Configurations as per section 4.6. |  |  |  |  |  |  |  |  |  |
| NOTE 3: Note configured |  |  |  |  |  |  |  |  |  |
| NOTE 4: X, Y and Z in this table correspond to different bands i.e. X != Y != Z |  |  |  |  |  |  |  |  |  |
| NOTE 5: The band combinations with difference appearance order of bands/sub-blocks in the band combination string are not distinguished. E.g. DC_YA-(n)XAA represents the set of DC_YA-(n)XAA and DC_(n)XAA-YA |  |  |  |  |  |  |  |  |  |

Proposal 3 : Adopt default test points as specified in Table 3 for Wideband Intermodulation TCs for EN-DC in FR1 (3 CCs)

Note that for other cases with the same situation in terms of definition of exceptional requirement (exceptional for intra-C, non-exceptional for inter and intra-NC), the same test points can be used.

# Conclusion

Proposal 1 : Define test points so that all NR band experience PCC(SCG) regardless of the NR exceptional/non-exceptional(tested with LTE anchor agnostic)

Proposal 2 : Define test point so that all LTE bands associated with LTE exceptional requirement experience PCC(MCG)

Ovseravation1 : Reduction of test points can be considered if the combination generates too many test points by means of e.g. merging multiple test points to single test points (by e.g. change some parameters) as was done in LTE CA.

Proposal 3 : Adopt default test points as specified in Table 3 for Wideband Intermodulation TCs for EN-DC in FR1 (3 CCs)

# 4.	References

[1] R5-185666, “Test approach of EN-DC TRx tests”, Anritsu, RAN5#NR#3 AdHoc

[2] R5-186228, “	Discussion on applicability rules and test coverage for 38.521-3 test cases”, Qualcomm Inc., RAN5 NR#4 AdHoc

[3] R5-190978, “Test approach for EN-DC TRx tests with CCs > 2”, Anritsu, RAN5 NR#4 AdHoc

[4] R5-192602, “Resolving ambiguity in LTE anchor agnostic approach”, Anritsu, RAN5#82