3GPP TSG-RAN5 Meeting #100	R5-234101

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3891 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 8.1.6.1.3.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case 8.1.6.1.3.11 needs to be added according to the WP for NR_ENDC_SON_MDT_enh-UEConTest. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 8.1.6.1.3.11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.3.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-2 CR 0374 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

8.1.6.1.3.11	Radio Link Failure / Logging and reporting / Successful Handovers Reports

8.1.6.1.3.11.1	Test Purpose (TP)

(1)

with { UE received RRC reconfiguration meesage and sourceDAPS-FailureReporting is included in the successHO-Config with value ‘true’, detected radio link failure in source PCell during the DAPS handover while T304 was running }

ensure that {

when { UE receives RRCReestablishment message }

then { UE sends the RRCReestablishmentComplete message with successHO-InfoAvailable }

}

(2)

with { UE received RRC reconfiguration meesage and sourceDAPS-FailureReporting is included in the successHO-Config with value ‘true’, detected radio link failure in source PCell during the DAPS handover while T304 was running }

ensure that {

when { UE receives the UEInformationRequest message containing successHO-ReportReq }

then { UE sends the UEInformationResponse message with successHO-Report}

}

8.1.6.1.3.11.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331, clauses 5.3.7.5, 5.3.10.5, 5.3.5.8.3, 5.3.5.3, 5.7.10.3, and 5.7.10.6. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.3.7.5]

The UE shall:

…

…

1>	set the content of RRCReestablishmentComplete message as follows:

…

…

2>	if the UE has successful handover information available in VarSuccessHO-Report and if the RPLMN is included in plmn-IdentityList stored in VarSuccessHO-Report:

3>	include successHO-InfoAvailable in the RRCReestablishmentComplete message;

1>	submit the RRCReestablishmentComplete message to lower layers for transmission;

[TS 38.331, clause 5.3.10.3]

The UE shall:

1>	if any DAPS bearer is configured and T304 is running:

2>	upon T310 expiry in source SpCell; or

2>	upon random access problem indication from source MCG MAC; or

2>	upon indication from source MCG RLC that the maximum number of retransmissions has been reached; or

2>	upon consistent uplink LBT failure indication from source MCG MAC:

3>	consider radio link failure to be detected for the source MCG i.e. source RLF;

3>	suspend the transmission and reception of all DRBs and multicast MRBs in the source MCG;

3>	reset MAC for the source MCG;

3>	release the source connection.

…

[TS 38.331, clause 5.3.5.8.3]

The UE shall:

1>	if T304 of the MCG expires:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

…

2>	else:

3>	revert back to the UE configuration used in the source PCell;

3>	store the handover failure information in VarRLF-Report as described in the subclause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

[TS 38.331, clause 5.3.5.3]

The UE shall perform the following actions upon reception of the RRCReconfiguration, or upon execution of the conditional reconfiguration (CHO or CPC):

…

1>	set the content of the RRCReconfigurationComplete message as follows:

…

2>	if the UE has radio link failure or handover failure information available in VarRLF-Report and if the RPLMN is included in plmn-IdentityList stored in VarRLF-Report; or

…

3>	include rlf-InfoAvailable in the RRCReconfigurationComplete message;

…

1>	else (RRCReconfiguration was received via SRB1):

2>	submit the RRCReconfigurationComplete message via SRB1 to lower layers for transmission using the new configuration;

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if the successHO-ReportReq is set to true and if the UE has successful handover related information available in VarSuccessHO-Report and if the RPLMN is included in the plmn-IdentityList stored in VarSuccessHO-Report:

2>	if the successHO-Report in the VarSuccessHO-Report concerns a DAPS handover and if a PDCP PDU has been received from the source cell of the concerned HO and a non-duplicated PDCP PDU has been received from the target cell of the concerned HO:

3>	set upInterruptionTimeAtHO in VarSuccessHO-Report to include the time elapsed between the time of arrival of the last PDCP PDU received from the source cell of the concerned handover and the time of arrival of the first non-duplicate PDCP PDU received from the target cell of the concerned handover, as measured at the time of arrival of the first non-duplicate PDCP PDU received from the target cell;

2>	set the successHO-Report in the UEInformationResponse message to the value of successHO-Report in the VarSuccessHO-Report, if available;

2>	discard the VarSuccessHO-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

1>	else:

2>	submit the UEInformationResponse message to lower layers for transmission via SRB1.

[TS 38.331, clause 5.7.10.6]

The UE shall for the PCell:

…

1>	if sourceDAPS-FailureReporting is included in the successHO-Config before executing the last reconfiguration with sync and is set to true and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

2>	store the successful handover information in VarSuccessHO-Report and determine the content in VarSuccessHO-Report as follows:

3>	clear the information included in VarSuccessHO-Report, if any;

3>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e., includes the RPLMN);

3>	set the c-RNTI to the C-RNTI assigned by the target PCell of the handover;

3>	for the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied:

4>	set the sourceCellID in sourceCellInfo to the global cell identity and tracking area code, if available, of the source PCell;

4>	set the sourceCellMeas in sourceCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the source PCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	set the rsIndexResults in sourceCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

5>	set the rlf-InSourceDAPS in sourceCellInfo to true;

3>	for the target PCell indicated in the last applied RRCReconfiguration message including reconfigurationWithSync:

4>	set the targetCellID in targetCellInfo to the global cell identity and tracking area code, if available, of the target PCell;

4>	set the targetCellMeas in targetCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the target PCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	set the rsIndexResults in targetCellMeas to include all the available SSB and CSI-RS measurement quantities of the target PCell collected up to the moment the UE sends RRCReconfigurationComplete message;

4>	if the last applied RRCReconfiguration message including reconfigurationWithSync was included in the stored condRRCReconfig:

5>	set the timeSinceCHO-Reconfig to the time elapsed between the initiation of the execution of conditional reconfiguration for the target PCell and the reception of the last conditionalReconfiguration including the condRRCReconfig of the target PCell in the source PCell;

3>	if the ratio between the value of the elapsed time of the timer T304 and the configured value of the T304 timer, included in the last applied RRCReconfiguration message including the reconfigurationWithSync, is greater than thresholdPercentageT304 if included in the successHO-Config received before executing the last reconfiguration with sync:

4>	set t304-cause in shr-Cause to true;

4>	set the ra-InformationCommon to include the random-access related information associated to the random access procedure in the target PCell, as specified in clause 5.7.10.5;

3>	if the ratio between the value of the elapsed time of the timer T310 and the configured value of the T310 timer, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync:

4>	set t310-cause in shr-Cause to true;

3>	if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the T312 timer, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync:

4>	set t312-cause in shr-Cause to true;

3>	if sourceDAPS-FailureReporting included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync is set to true, and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running:

4>	set sourceDAPS-Failure in shr-Cause to true;

3>	for each of the measObjectNR, configured by the source PCell, in which the last RRCReconfiguration message including reconfigurationWithSync was applied:

4>	if measurements are available for the measObjectNR:

5>	if the SS/PBCH block-based measurement quantities are available:

6>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell or target PCell, ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE sends the RRCReconfigurationComplete message;

6>	for each neighbour cell included, include the optional fields that are available;

NOTE 1:	For the neighboring cells set included in measResultListNR in measResultNeighCells ordered based on the SS/PBCH block measurement quantities, the UE includes also the CSI-RS based measurement quantities, if available.

5>	if the CSI-RS measurement quantities are available:

6>	set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell and target PCell, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE sends the RRCReconfigurationComplete message;

6>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	For the neighboring cells set ordered based on the CSI-RS measurement quantities, the UE includes measurements only for the cells not yet included in measResultListNR in measResultNeighCells to avoid overriding SS/PBCH block-based ordered measurements.

3>	for each of the measObjectEUTRA, configured by the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied:

4>	if measurements are available for the measObjectEUTRA:

5>	set the measResultListEUTRA in measResultNeighCells to include the best measured cells ordered such that the cell with highest RSRP is listed first if RSRP measurement results are available, otherwise the cell with highest RSRQ is listed first, based on measurements collected up to the moment the UE sends the RRCReconfigurationComplete message;

5>	for each neighbour cell included, include the optional fields that are available;

3>	for each of the neighbour cells included in measResultNeighCells:

4>	if the cell was a candidate target cell included in the condRRCReconfig within the conditionalReconfiguration configured by the source PCell, in which the last RRCReconfiguration message including reconfigurationWithSync was applied:

5>	set the choCandidate to true in measResultNR;

3>	if available, set the locationInfo as in 5.3.3.7;

1>	release successHO-Config configured by the source PCell and thresholdPercentageT304 if configured by the target PCell.

The UE may discard the successful handover information, i.e., release the UE variable VarSuccessHO-Report, 48 hours after the last successful handover information is added to the VarSuccessHO-Report.

8.1.6.1.3.11.3	Test description

8.1.6.1.3.11.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is the Serving cell and NR Cell 2 is the Suitable neighbour intra-frequency cell.

-	System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.3 is used for NR Cells

UE:

-	None.

Preamble:

-	If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A on NR Cell 1 according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

-	Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

8.1.6.1.3.11.3.2	Test procedure sequence

Table 8.1.6.1.3.11.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1", and "T2" are applied at the points indicated in the Main behaviour description in Table 8.1.6.1.3.11.3.2-2.

Table 8.1.6.1.3.11.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | off | -94 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 |  |

Table 8.1.6.1.3.11.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | off | FFS |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS |  |

Table 8.1.6.1.3.11.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message containing reconfigurationWithSync to order UE to perform DAPS handover to NR Cell 2. DRB#n of the first PDU session is configured as DAPS bearer. | <-- | NR RRC: RRCReconfiguration | - | - |
| - | EXCEPTION: In parallel with step 2-9, parallel behaviour defined in table 8.1.6.1.3.11.3.2-3 is executed. | - | - | - | - |
| 2 | The SS changes the power level setting according to the row "T1". | - | - | - | - |
| 3 | Waiting for T310 expiring. |  |  |  |  |
| 4 | The SS changes the power level setting according to the row "T2".- |  |  |  |  |
| 5 | The UE transmits an RRCReestablishmentRequest message in NR Cell 1. | --> | NR RRC: RRCReestablishmentRequest | - | - |
| 6 | The SS transmits an RRCReestablishment message to resume SRB1 operation and re-activate security in NR Cell 1. | <-- | NR RRC: RRCReestablishment | - | - |
| 7 | Check: Does the UE transmits an RRCReestablishmentComplete message with successHO-InfoAvailable in NR Cell 1. | --> | NR RRC: RRCReestablishmentComplete | 1 | P |
| 8 | The SS transmits an RRCReconfiguration message to resume existing radio bearers on NR Cell 1. | <-- | NR RRC: RRCReconfiguration | - | - |
| 9 | The UE transmit an RRCReconfigurationtComplete message. | --> | NR RRC: RRCReconfigurationtComplete | - | - |
| 10 | The SS transmits an RRCReconfiguration message with condition DAPS_HO_ReleaseSource in NR Cell 2. | <-- | NR RRC: RRCReconfiguration | - | - |
| 11 | The UE transmit an RRCReconfigurationComplete message in NR Cell 2. | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 12 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 13 | Check: Does the UE transmit a UEInformationResponse message includingsuccessHO-Report in NR Cell 2? | --> | NR RRC: UEInformationResponse | 2 | P |

Table 8.1.6.1.3.11.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits Random Access Response to respond to the latest preamble on NR Cell 2. | <-- | Random Access Response | - | - |
| 2 | Check: Does UE transmit an RRCReconfigurationComplete message in NR Cell 2? | --> | NR RRC: RRCReconfigurationComplete | - | - |

8.1.6.1.3.11.3.3	Specific message contents

Table 8.1.6.1.3.11.3.3-1: SIB1 for NR Cell 1 (all steps)

| Derivation Path: TS 38.508-1 [4] table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| ue-TimersAndConstants SEQUENCE { |  |  |  |
| t310 | ms0 | 0 ms |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.11.3.3-2: RRCReconfiguration (step 1, Table 8.1.6.1.3.11.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.8.1-1A with Condition RBConfig_KeyChange |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| radioBearerConfig | RadioBearerConfig with conditions (SRB_NR_PDCP) and (DRBn AND DAPS_PDCP) |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig)Table 8.1.6.1.3.11.3.3-3 |  |
| OtherConfig-r17 SEQUENCE{ |  |  |  |
| SuccessHO-Config-r17 SEQUENCE{ |  |  |  |
| sourceDAPS-FailureReporting-r17 | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.11.3.3-3: CellGroupConfig (Table 8.1.6.1.3.11.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 with condition PCell_change and CFRA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| rlc-BearerToAddModList SEQUENCE (SIZE(1..maxLCH)) OF RLC-BearerConfig { | 3 entries |  |  |
| RLC-BearerConfig[1] | RLC-BearerConfig with conditions SRB1 | entry 1 |  |
| RLC-BearerConfig[2] | RLC-BearerConfig with conditions SRB2 | entry 2 |  |
| RLC-BearerConfig[3] | RLC-BearerConfig with conditions AM, DRBn | entry 3 |  |
| } |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon SEQUENCE { |  |  |  |
| physCellId | Physical Cell Identity of NR Cell 2 |  |  |
| } |  |  |  |
| rach-ConfigDedicated CHOICE { |  |  |  |
| uplink | RACH-ConfigDedicated | Table 8.1.6.1.3.11.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.11.3.3-4: RACH-ConfigDedicated (Table 8.1.6.1.3.11.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-129 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigDedicated ::= SEQUENCE { |  |  |  |
| cfra SEQUENCE { |  |  |  |
| occasions SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric | Table 8.1.6.1.3.11.3.3-5 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.11.3.3-5: RACH-ConfigGeneric (Table 8.1.6.1.3.11.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| preambleTransMax | n200 |  |  |
| } |  |  |  |

able 8.1.6.1.3.11.3.3-3: RRCReestablishment (step 6, Table 8.1.6.1.3.11.3.2-3)

| Derivation Path: TS 38.508-1, Table 4.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishment ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishment SEQUENCE { |  |  |  |
| nextHopChainingCount | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.11.3.3-4: RRCReestablishmentComplete (step 7, Table 8.1.6.1.3.11.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReestablishmentComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReestablishmentComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 | UE-MeasurementsAvailable-r16 with condition successHO |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.3.11.3.3-5: RRCReconfiguration (step 8, Table 8.1.6.1.3.11.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition REEST

Table 8.1.6.1.3.11.3.3-6: RRCReconfiguration (step 10, Table 8.1.6.1.3.11.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with Condition DAPS_HO_ReleaseSource

Table 8.1.6.1.3.11.3.3-7: UEInformationRequest (step 12, Table 8.1.6.1.3.11.3.2-3)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition successHO

Table 8.1.6.1.3.11.3.3-8: UEInformationResponse (step 13, Table 8.1.6.1.3.11.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r17 SEQUENCE { |  |  |  |
| successHO-Report-r17 SEQUENCE { |  |  |  |
| sourceCellInfo-r17 SEQUENCE { |  |  |  |
| sourcePCellId-r17 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| sourceCellMeas-r17 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| rlf-InSourceDAPS-r17 | true |  |  |
| } |  |  |  |
| targetCellInfo-r17 SEQUENCE { |  |  |  |
| targetPCellId-r17 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN ID of NR Cell 1 |  |  |
| cellIdentity-r16 | CGI of NR Cell 1 |  |  |
| trackingAreaCode-r16 | TAC of NR Cell 1 |  |  |
| } |  |  |  |
| targetCellMeas-r17 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell-r16 | Not checked |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE { |  |  |  |
| ssb-Index | 1 |  |  |
| ssb-Results SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells-r17 | Not present |  |  |
| locationInfo-r16 | Not checked |  |  |
| timeSinceCHO-Reconfig-r17 | Any allowed value |  |  |
| shr-Cause-r17 SEQUENCE { |  |  |  |
| sourceDAPS-Failure-r17 | true |  |  |
| ra-InformationCommon-r17 | Not checked |  |  |
| upInterruptionTimeAtHO-r17 | (0..1023) |  |  |
| c-RNTI-r17 | the value of the C-RNTI of the UE used in NR Cell 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |