3GPP TSG WG RAN5 Meeting #100	R5-234258

# Toulouse, FR, 21 – 25 Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding test case 6.5B.3.2 Spectrum emission mask |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-18 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Adding test case 6.5B.3.2 Spectrum emission mask for category NB1 and NB2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Complete test case for 6.5B.3.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of changes >>

#### 6.5B.3.2	Spectrum Emission Mask

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Addition to applicability spec is pending

- Initial condition and call setup procedure to support satellite access are TBD

- Message exceptions specific to satellite access is TBD

- Test Points analysis is TBD

- Test configuration is TBD

- Annex F MU/TT is TBD

6.5B.3.2.1	Test purpose

To verify that the power of the category NB1 and NB2 UE emission shall not exceed specified level for the specified channel bandwidth.

6.5B.3.2.2	Test applicability

This test case applies to all types of NB-IoT UE release 17 and forward of category NB1 and NB2 that support satellite access operation.

6.5B.3.2.3	Minimum conformance requirements

The spectrum emission mask of the category NB1 and NB2 UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned category NB1 or NB2 channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6. 5B.3.2.3-1 the spurious requirements in TS 36.521-1[14] sub-clause 6.6.3 are applicable.

The power of any category NB1 or NB2 UE emission shall not exceed the levels specified in Table 6.5B.3.2.3-1.

Table 6.5B.3.2.3-1: category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

In addition to the spectrum emission mask requirement in Table 6.5B.3.2.3-1 a category NB1 or NB2 UE shall also meet the applicable E-UTRA spectrum emission mask requirement in TS 36.521-1[14] sub-clause 6.6.2. E-UTRA spectrum emission requirement applies for frequencies that are Foffset away from edge of NB1 or NB2 channel edge as defined in Table 6.5B.3.2.3-2.

Table 6.5B.3.2.3-2: Foffset for category NB1 and NB2 UE spectrum emission mask

| Channel BW (MHz) | Foffset [kHz] |
| --- | --- |
| 1.4 | 165 |
| 3 | 190 |
| 5 | 200 |
| 10 | 225 |
| 15 | 240 |
| 20 | 245 |

NOTE: Foffset in Table 6.5B.3.2.3-2 is used to guarantee co-existence for guard-band operation.

The normative reference for this requirement is TS 36.102[11] clause 6.5B.3.2.

6.5B.3.2.4	Test description

6.5B.3.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NB-IOT NTN operating bands specified in sub-clause 5.2B.All of these configurations shall be tested with applicable test parameters shown in table 6.5B.3.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of NPDSCH and NPDCCH before measurement are specified in Annex C.2.

Table 6.5B.3.2.4.1-1: Test Configuration Table for FDD

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[12] subclause 8.1.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508[12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 |  | QPSK | 1@0 | 3.75kHz |
| 2 |  | QPSK | 1@47 | 3.75kHz |
| 3 |  | QPSK | 1@0 | 15kHz |
| 4 |  | QPSK | 1@11 | 15kHz |
| 5 (Note 1) |  | QPSK | 3@0 | 15kHz |
| 6 (Note 1) |  | QPSK | 3@3 | 15kHz |
| 7 (Note 1) |  | QPSK | 3@9 | 15kHz |
| 8 (Note 1) |  | QPSK | 6@0 | 15kHz |
| 9 (Note 1) |  | QPSK | 6@6 | 15kHz |
| 10 (Note 1) |  | QPSK | 12@0 | 15kHz |
| Note 1: The allowed MPR for maximum output power UE might apply is described in clause [6.2B.2.3]. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [12] Annex A, Figure A.3 using only main Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [12] subclause 8.1.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1.1 and H.4.0.

4.	The UL Reference Measurement channels are set according to Table 6.5B.3.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5. Message contents are defined in clause 6.5B.3.2.4.3.

6.5B.3.2.4.2	Test procedure

1.	SS sends uplink scheduling information via NPDCCH DCI format N0 for C_RNTI to schedule the UL RMC according to Table 6.5B.3.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC. (UE should be already transmitting PUMAX after Initial Conditions setting).

2.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Table 6.2B.2.4-1. The measurement duration is at least one sub-frame (1ms) for 15 KHz channel spacing, and at least a 2ms slot (excluding the 2304Ts gap when UE is not transmitting) respectively for the 3.75 KHz channel spacing.

3.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.5B.3.2.5-1 as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

NOTE 1:	For configuration IDs applicable to UE depending on UE capability in Test Configuration Table with different UL sub-carrier spacing, the SS shall release the connection through State 3A-NB and finally ensure the UE is in State 2A-NB with CP CIoT Optimisation according to TS 36.508 [12] clause 8.1.5 using the appropriate UL subcarrier spacing in Random Access Response message.

6.5B.3.2.4.3	Message contents

Message contents are according to TS 36.508 [12] subclause 8.1.6.

6.5B.3.2.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2B.2.4-1 as appropriate, and the power of any UE emission shall fulfil requirements in 6.5B.3.2.5-1 as applicable.

Table 6.5B.3.2.5-1: category NB1 and NB2 UE spectrum emission mask, category NB1/NB2 bands ≤ 3GHz

| ΔfOOB (kHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 -100 | (27.5+(F-0)×-3.5-27.5100-0) | 30 kHz |
| 100 - 150 | (-3.5+(F-100)×-6.5-(-3.5)150-100) | 30 kHz |
| 150 - 300 | (-6.5+(F-150)×-27.5-(-6.5)300-150) | 30 kHz |
| 300 - 500 | (-27.5+(F-300)×-33.5-(-27.5)500-300) | 30 kHz |
| 500 - 1700 | -33.5 | 30 kHz |
| Note 1: The limit shall be calculated for the measurement frequency F given in kHz, cantered in the measurement bandwidth. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 30 kHz filter is the inside of +15 kHz and -15 kHz, respectively. The filter shall be stepped to cover the whole range. |  |  |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	The minimum requirements in clause 6.5B.3.2.3 have been combined into table 6.5B.3.2.5-1 to reduce testing complexity and test time.

<< End of changes >>