3GPP TSG-RAN5 Meeting #100	R5-234500

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4755 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PIXIT for IoT NTN test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In order to differentiate whether the test shall be executed as legacy eMTC/NB-IoT case or under NTN configuration, as well as the satellite scenarios, specific PIXIT shall be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New PIXIT was added in Table 9.1-5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be able to distinguish the test configuration and scenarios. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

9.1	E-UTRAN PIXIT

Table 9.1-1: Common PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_AccessPointName | octetstring |  |  | Access Point Name, as defined in TS 23.003 [48] and used in TS 24.008 [20], clause 10.5.6.1 |
| px_AttachTypeTested | EUTRA_ATTACH_TESTED_Type | EPS_ATTACH_ONLY | EPS_ATTACH_ONLY, COMBINED_ATTACH | Attach Type to be tested, if UE supports both pc_Attach and pc_Combined_Attach |
| px_eAuthRAND | B128_Type | oct2bit(‘A3DE0C6D363E30C364A4078F1BF8D577’O) |  | Random Challenge |
| px_EllipsoidPointWithAltitude | O8_Type |  |  | Ellipsoid Point With Altitude value to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_HorizontalVelocity | O3_Type |  |  | Horizontal Velocity value to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_eJapanMCC_Band6 | NAS_Mcc | ‘442’H |  | Japan MCC code to be used for Band 6. The same value will be used for E-UTRA and Inter-RAT cells. Type is different to that defined in TS 34.123-3 [7] |
| px_PWS_CB_DataPage1 | charstring |  |  | ETWS or CMAS Page 1 warning data message |
| px_PWS_CB_DataPage2 | charstring |  |  | ETWS or CMAS Page 2 warning data message |
| px_PWS_CB_DataPage3 | charstring |  |  | ETWS or CMAS Page 3 warning data message |
| px_PWS_CB_DataPage4 | charstring |  |  | ETWS or CMAS Page 4 warning data message |
| px_PWS_CB_DataCodingScheme | bitstring |  |  | ETWS or CMAS data coding scheme of the alphabet/coding and the applied language [see TS 23.041] |
| px_IPv4_Address1_UE | charstring |  |  | IPv4 Address connected to PDN1 |
| px_IPv4_Address2_UE | charstring |  |  | IPv4 Address connected to PDN2 |
| px_IPv4_Address2a_UE | charstring |  |  | IPv4 Address connected to PDN2a |
| px_IPv4_Address3_UE | charstring |  |  | IPv4 Address connected to PDN3 |
| px_IPv4_Address1_NW | charstring |  |  | IPv4 Gateway Address in PDN1 |
| px_IPv4_Address2_NW | charstring |  |  | IPv4 Gateway Address in PDN2 |
| px_IPv4_Address3_NW | charstring |  |  | IPv4 Gateway Address in PDN3 |
| px_IPv4_Address1_RemoteUE | charstring |  |  | IPv4 Address of remote UE connected to PDN1 |
| px_IPv4_Address2_RemoteUE | charstring |  |  | IPv4 Address of remote UE connected to PDN2 |
| px_IPv4_Address3_RemoteUE | charstring |  |  | IPv4 Address of remote UE connected to PDN3 |
| px_IPv4_Address_HomeAgent | charstring |  |  | IPv4 Home Agent Address |
| px_IPv6_Address1_UE | charstring |  |  | IPv6 Address connected to PDN1 |
| px_IPv6_Address2_UE | charstring |  |  | IPv6 Address connected to PDN2 |
| px_IPv6_Address2a_UE | charstring |  |  | IPv6 Address connected to PDN2a |
| px_IPv6_Address3_UE | charstring |  |  | IPv6 Address connected to PDN3 |
| px_IPv6_Address1_NW | charstring |  |  | IPv6 Gateway Address in PDN1 |
| px_IPv6_Address2_NW | charstring |  |  | IPv6 Gateway Address in PDN2 |
| px_IPv6_Address3_NW | charstring |  |  | IPv6 Gateway Address in PDN3 |
| px_IPv6_Address1_RemoteUE | charstring |  |  | IPv6 Address of remote UE connected to PDN1 |
| px_IPv6_Address2_RemoteUE | charstring |  |  | IPv6 Address of remote UE connected to PDN2 |
| px_IPv6_Address3_RemoteUE | charstring |  |  | IPv6 Address of remote UE connected to PDN3 |
| px_IPv6_Address_HomeAgent | charstring |  |  | IPv6 Home Agent Address |
| px_ePrimaryFrequencyBand | integer | 1 |  | E-UTRA primary frequency band |
| px_SMS_ChkMsgReceived | boolean | true |  | Whether the operator can check an MT Short Message received |
| px_RATComb_Tested | RATComb_Tested_Type | EUTRA_UTRA | EUTRA_UTRA, EUTRA_GERAN, EUTRA_Only | This parameter represents the network RAT capability / preference and indicates which, if any is supported, RAT combination is to be tested. |
| px_SinglePLMN_Tested | SinglePLMN_Tested_Type | MultiPLMN | SinglePLMN, MultiPLMN, MultiPLMNinPrimaryBand, MultiPLMNinSecondaryBand | This parameter represents the network capability/preference to support multi PLMNs on the same test Band and indicates the preference of multi PLMNs or single PLMN test environment. This PIXIT shall be set to SinglePLMN when only one frequency is defined for the test band. |
| px_UE_CS_PS_UsageSetting_Tested | CS_PS_MODE | VOICE_CENTRIC | VOICE_CENTRIC, DATA_CENTRIC | Specifies which CS/PS mode is under test |
| px_UE_PS_UsageSetting_Tested | PS_MODE | VOICE_CENTRIC | VOICE_CENTRIC, DATA_CENTRIC | Specifies which PS mode is under test |
| px_UTRAN_ModeUnderTest | UTRAN_FDD_TDD | UTRAN_FDD | UTRAN_FDD, UTRAN_TDD | Specifies which radio access technology is being tested in UTRAN |
| px_TestLoopModeB_Delay | O1_Type | ‘5A’ |  | This parameter represents the IP_PDU_delay to be used for UE test loop mode B in test cases, where long delay may be needed e.g. because of user interaction. |
| px_IP_MTU_Size | integer | 65535 |  | MTU Size. This value is specific to the SS. |
| px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI | integer |  |  | A not supported UTRAN frequency band that is overlapping with a supported band (px_MFBI_UTRAN_FrequencyBand). This PIXIT shall be set also in accordance with the values of the 3 PIXIT items: px_UARFCN_D_High, px_UARFCN_D_Mid,px_UARFCN_L_Mid. Applied to MFBI test case scenario. |
| px_MFBI_UTRAN_FrequencyBand | integer |  |  | A supported UTRAN frequency band overlapping with px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI. Applied to MFBI test case scenario. |
| px_WLAN_PDN1_ToOffload | boolean | true |  | If set to true, the first PDN connection is offloaded to WLAN. If set to false, the second PDN connection is offloaded to WLAN. |
| px_AquireGNSS_MaxTime | integer | 60 |  | The maximum time (in seconds) taken for the UE to aquire GNSS and derive UTC time (and normally position) and then start to transmit or receive as appropriate |
| px_MobileDataOn | boolean | true |  | Specifes whether mobile data, associated with Internet PDN, is enabled on the DUT. |

Table 9.1-2: E-UTRAN PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_eTDDsubframeConfig | TDD_SubframeAssignment_Type | 1 |  | TDD uplink-downlink subframe configuration |
| px_ePrimaryBandChannelBandwidth | Dl_Bandwidth_Type | n25 |  | Channel bandwidth used on px_ePrimaryFrequencyBand |
| px_eSecondaryBandChannelBandwidth | Dl_Bandwidth_Type | n25 |  | Channel bandwidth used on px_eSecondaryFrequencyBand |
| px_EUTRA_CA_BandCombination | CA_BandCombination_Type | CA_1C |  | Band combination for CA test cases |
| px_EUTRA_DC_BandCombination | DC_BandCombination_Type | DC_1A-3A |  | Band combination for DC test cases |
| px_eMaxNumberROHC_ContextSessions | MaxNumberROHC_ContextSessions_Type | Cs16 |  | Maximum number of ROHC context sessions |
| px_MFBI_FrequencyBand | integer | 26 |  | A supported E-UTRA frequency band overlapping with px_OverlappingNotSupportedFrequencyBand_MFBI. Applied to MFBI test case scenario. |
| px_MFBI_BandChannelBandwidth | Dl_Bandwidth_Type | n25 |  | E-UTRAN channel bandwidth used on px_MFBI_FrequencyBand. Applied to MFBI test case scenario. |
| px_OverlappingNotSupportedFrequencyBand_MFBI | integer | 27 |  | A not supported E-UTRA frequency band that is overlapping with a supported band (px_MFBI_FrequencyBand). Applied to MFBI test case scenario. |
| px_OverlappingSupportedFrequencyBand_MFBI | integer | 26 |  | A supported E-UTRA frequency band overlapping with px_OverlappingNotSupportedFrequencyBand_MFBI. Applied to MFBI enhancement test case scenario. |
| px_NoTDD_LCR_onLTE | boolean | false |  | Only applicable to UE supporting (at least) LTE and LCR TDD. If set to true, the UE is configured not to report support of LCR TDD (IE utraTDD128 in UECapabilityInformation) when being camped on an LTE cell with no detectable LCR TDD cell in the environment |

Table 9.1-3: D2D PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_EllipsoidPointWithAltitude_ProSe_Area1 | O8_Type |  |  | Ellipsoid Point With Altitude value corresponding to geographical area 1 provided in EFPROSE_RADIO_COM to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_EllipsoidPointWithAltitude_ProSe_Area2 | O8_Type |  |  | Ellipsoid Point With Altitude value corresponding to geographical area 2 provided in EFPROSE_RADIO_COM to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_EllipsoidPointWithAltitude_NotProSe | O8_Type |  |  | Ellipsoid Point With Altitude value not corresponding to geographical area provided in EFPROSE_RADIO_COM to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_ProSeAnnApplicationIdentity1 | charstring | mcc345.mnc012.ProSeApp.Food.Restaurants.Italian |  | A ProSe Application ID with which data structure the UE is preconfigured and for which the UE can request and perform Direct Discovery Announcing. TS 23.003, section 24.2. |
| px_ProSeAnnApplicationIdentity2 | charstring | mcc345.mnc012.ProSeApp.mcptt |  | An MCPTT relevant ProSe Application ID with which data structure the UE is preconfigured and for which the UE can request and perform Group Member Discovery Announcement. TS 23.003, section 24.2. |
| px_ProSeMonApplicationIdentity1 | charstring | mcc300.mnc165.ProSeApp.*.Sports.Surfing |  | A ProSe Application ID with which data structure the UE is preconfigured and for which the UE can request and perform Direct Discovery Monitoring. TS 23.003, section 24.2 |
| px_ProSeMonApplicationIdentityNot1 | charstring | mcc208.mnc*.ProSeApp.Shops.Food.Wine |  | A ProSe Application ID with which data structure the UE is NOT preconfigured to request and perform Direct Discovery Monitoring. TS 23.003, section 24.2 |
| px_ProSe_TLS_CipherSuite | TLS_CIPHER_Type | TLS_PSK_WITH_3DES_EDE_CBC_SHA |  | TLS cipher suite to be used for ProSe communication |
| px_SL_AdditionalSupportedBandwidth | Ul_Bandwidth_Type | n25 |  | Supported sidelink bandwidth different from px_ePrimaryBandChannelBandwidth |

Table 9.1-4: EUTRA_NB PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_eSecondaryFrequencyBand | integer | 2 |  | E-UTRA secondary frequency band |
| px_NAS_CipheringAlgorithm | B3_Type | 001’B |  | NAS Ciphering Algorithm (eea1, see NOTE) |
| px_NAS_IntegrityProtAlgorithm | B3_Type | 001’B |  | NAS Integrity Algorithm (eia1, see NOTE) |
| px_RRC_CipheringAlgorithm | CipheringAlgorithm_r12 | eea1 |  | Ciphering Algorithm (see NOTE) |
| px_RRC_IntegrityProtAlgorithm | IntegrityProtAlgorithm | eia1 |  | Integrity Algorithm (see NOTE) |
| px_SMSTransport_CP_CIoT | boolean | false |  | UE configured to utilise the SMS service as transport mechanism for user data for Control Plane CIoT Optimisation |
| px_nonSMSTransport_CP_CIoT | boolean | false |  | UE configured to utilise non-SMS services as transport mechanism for user data for Control Plane CIoT Optimisation |
| NOTE: Unless specified otherwise in the test case prose, the null algorithm shall not be used for verification. |  |  |  |  |

Table 9.1-5: NTN PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_NTN_test | boolean | false |  | Applied to eMTC NTN or NB-IoT NTN test case scenario. |
| px_NTN_ScenarioUnderTest | NTN_ScenarioUnderTest | GSO | GSO, NGSO | NTN satellite scenario under test if UE supports NTN. |

9.2	MultiRAT PIXIT

[End of change]