3GPP TSG-RAN WG5 Meeting #100 	R5-234574

# Toulouse, France, 21st – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 0794 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update 38.905 for NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update 4.1.1.6 for test point selection for NR-U since some test cases need to follow specific TP analysis for NR-U. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Table 4.1.1.6-1 and Table 4.1.1.6-2 to track how TP is determined for each test case |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will not be aligned with the core specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

#### 4.1.1.6	FR1 shared spectrum channel access test cases

This section contains information on test point selection for shared spectrum channel access test cases in TS 38.521-1[2]. The general rule in this section apply to all test cases for shared spectrum channel access test cases. Separate analysis for each FR1 shared spectrum channel access test case is not provided, unless deemed necessary.

From the latest core requirement for NR band with shared spectrum channel access, the frequency range are 5150 MHz – 5925 MHz for n46, and 5925 MHz – 7125 MHz for n96. Both bands support power class 5 operations with various channel bandwidths.

For all Tx and Rx test requirements of NR unlicensed band, the selections of test point shall be the same as for regular NR band for Test Environment, Test frequency, Test channel bandwidths, Test Subcarrier Spacing, Test uplink/downlink modulations and Test RB configurations, unless shared spectrum channel access configuration specific test points are over-ruling.

Table 4.1.1.6-1: UE transmitter test point selection for FR1 shared spectrum channel access

| Subclause |  | Number of test points |  | Justification |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6.2F.1 UE maximum output power for shared spectrum channel access |  | 800 |  | Same as 6.2.1 |  | RAN5#98 |  |
| 6.2F.2 Maximum Output Power Reduction |  | 800 |  | 38.521-1_TP analysis_NR-U_6.2F.3_MPR_6.5F.2.2_SEM_6.5F.2.4.1_ACLR.zip |  | RAN5#100 |  |
| 6.4F.2.2 Carrier leakage |  | 3 |  | General rules of test point selection apply. Test points of TC 6.4.2.2 from TS 38.521-1 can be leveraged. |  | RAN5#97 |  |
| 6.4F.2.4 EVM equalizer spectrum flatness |  | 90 |  | General rules of test point selection apply. Test points of TC 6.4.2.4 from TS 38.521-1 can be leveraged. |  | RAN5#97 |  |
| 6.5F.1 Occupied bandwidth for shared spectrum channel access |  | TBD |  |
| 6.5F.2.2 Spectrum emission mask for operation with shared spectrum channel access | 800 | 38.521-1_TP analysis_NR-U_6.2F.3_MPR_6.5F.2.2_SEM_6.5F.2.4.1_ACLR.zip | RAN5#100 |
| 6.5F.2.3 Additional spectrum emission mask |  | TBD |  |
| 6.5F.2.4.1 Shared spectrum channel access ACLR | 800 | 38.521-1_TP analysis_NR-U_6.2F.3_MPR_6.5F.2.2_SEM_6.5F.2.4.1_ACLR.zip | RAN5#100 |
| 6.5F.2.4.2 Additional requirements for network signaled value "NS_29" |  | TBD |  |
| 6.5F.3.1 General spurious emissions | 27 | Test points of TC 6.5.3.1 from TS 38.521-1 can be leveraged. | RAN5#97 |
| 6.5F.3.3 Additional spurious emissions |  | TBD |  |
| 6.5F.4 Transmit intermodulation for shared spectrum channel access | 8 | “38.521-1_TPanalysis_6.5.4_TxIm_v3.zip” | RAN5#100 |

Table 4.4-2: UE receiver test point selection for FR1 shared spectrum channel access

| Subclause |  | Number of test points |  | Justification | Comments |  |
| --- | --- | --- | --- | --- | --- | --- |
| 7.3F.2 Reference sensitivity power level | 45 |  | General rules of test point selection apply. Test points of TC 7.3.2 from TS 38.521-1 can be leveraged. |  | RAN5#97 |  |
| 7.5F.1 Adjacent channel selectivity for shared spectrum channel access |  | 3 |  | General rules of test point selection apply. Test points of TC 7.5 from TS 38.521-1 can be leveraged. | RAN5#97 |  |
| 7.6F.2.1 In-band blocking for shared spectrum channel access | 3 | General rules of test point selection apply. Test points of TC 7.6.2 from TS 38.521-1 can be leveraged. | RAN5#97 |
| 7.6F.3 Out of band blocking | 3 | General rules of test point selection apply. Test points of TC 7.6.3 from TS 38.521-1 can be leveraged. | RAN5#97 |
| 7.7F.1 Spurious response for shared spectrum channel access | 7 | General rules of test point selection apply. Test points of TC 7. from TS 38.521-1 can be leveraged. | RAN5#97 |

<< End of changes >>