3GPP TSG-RAN5 Meeting #100	R5-234767

# Toulouse, France, Aug 21 - 25, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Annex H and Annex I in 36521-4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 16) Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) |  |
| Reason for change: |  | In the updated WP for RA5#100, Annex H and Annex I have been allocated to “Uplink Physical Channels” and “Handling requirements and tests for different releases and UE capabilities”, respectively, and Annex H and Annex I in 36521-4 need to be added accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Annex H and Annex I in 36521-4 have been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work Plan will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex H (new), Annex I (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

Annex H (normative):
Uplink Physical Channels

H.0	Uplink Signal Levels

Uplink signal power is a UE figure, which is configured by the Test System by means of:

RRC messages (IE-s), such as:

-	UplinkPowerControlCommon (-v1020, SCell-r10),

-	UplinkPowerControlDedicated (-v1020, SCell-r10),

-	Other IE-s affecting directly or indirectly the uplink power,

and L1/2 Power control commands (TPC).

The uplink power settings are specified in the test case.

Otherwise, the uplink power settings result from the default RRC messages described in 3GPP TS 36.508 [12], and appropriate TPC-s, which are sent to the UE to transmit with an UL power level necessary for maintaining the call during the test.

H.0.1	Uplink Signal Levels for NB1

Uplink signal power is a UE figure, which is configured by the Test System by means of:

RRC messages (IE-s), such as:

-	NPUSCH-ConfigCommon-NB

-	NPUSCH-ConfigDedicated-NB

-	UplinkPowerControl-NB

-	Other IE-s affecting directly or indirectly the uplink power,

The uplink power settings are specified in the test case.

Otherwise, the uplink power settings result from the default RRC messages described in 3GPP TS 36.508 [12], which are sent to the UE to transmit with an UL power level necessary for maintaining the call during the test.

H.1	General

This annex specifies the uplink physical channels that are needed for setting a connection and channels that are needed during a connection. Table H.1-1 describes the mapping of uplink physical channels and signals to physical resources for FDD.

Table H.1-1: Mapping of uplink physical channels and signals to physical resources for FDD

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| PRACH | Allowed for the parameter prach-Configuration Index provided by higher layers | Allowed for the parameter prach-FrequencyOffset provided by higher layers | Mapping rule is specified in TS 36.211 [8] Section 5.7.1 |
| DMRS | For PUCCH: Symbols 2 to 4 of each slot (PUCCH format: 1, 1a, 1b) Symbol 1 and 5 of each slot (PUCCH format: 2, 2a, 2b) For PUSCH: Symbol 3 of each slot | Uplink system bandwidth dependent. | Mapping rule of DMRS for PUCCH is specified in TS 36.211 [8] 5.5.2.2.2 Mapping rule of DMRS for PUSCH is specified in TS 36.211 [8] 5.5.2.1.2 |
| PUCCH | Slot 0 and 1 of each subframe | Each 12 subcarriers of both ends of the bandwidth | Mapping rule is specified in TS 36.211 [8] Section 5.4.3 |
| PUSCH | All remaining SC-FDMA symbols of each subframe not allocated to DMRS | RBs allocated according to Reference Measurement channel in Annex A.2 | Mapping rule is specified in TS 36.211 [8] Section 5.4.2 |
| SRS | Allowed for the cell-specific parameter srs-BandwidthConfig and the UE-specific parameter srs-Bandwidth provided by higher layers | Allowed for the cell-specific parameter srsMaxUpPt and the UE-specific parameter transmissionComb or transmissionComb-ap provided by higher layers | Mapping rule is specified in TS 36.211 [8] Section 5.5.3.2 |

H.1.1	General for NB1

This annex specifies the uplink physical channels that are needed for setting a connection and channels that are needed during a connection. Table H.1.1-1 describes the mapping of uplink physical channels and signals to physical resources for NB1 UE.

Table H.1.1-1: Mapping of uplink physical channels and signals to physical resources for NB1

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| NPRACH | Allowed for the parameter nprach-Configuration Index provided by higher layers | Allowed for the parameter nprach-SubcarrierOffset provided by higher layers | Mapping rule is specified in TS 36.211 [8] Section 10.1.6 |
| NPUSCH | All the SC-FDMA symbols of each subframe. | RUs allocated according to Reference Measurement channel in Annex A.2 | Mapping rule is specified in TS 36.211 [8] Section 10.1.3 |

H.2	Set-up

Table H.2-1 describes the uplink physical channels that are required for connection set up.

Table H.2-1: Uplink Physical Channels required for connection set-up

Physical Channel

PRACH

DMRS

PUCCH

PUSCH

H.2.1	Set-up for NB1

Table H.2.1-1 describes the uplink physical channels that are required for connection set up.

Table H.2.1-1: Uplink Physical Channels required for connection set-up

Physical Channel

NPRACH

NPUSCH

H.3	Connection

The following clauses describes the uplink physical channels that are transmitted during a connection i.e., when measurements are done.

H.3.0	Measurement of Transmitter Characteristics

As specified in the test case. Otherwise:

-	PUSCH + DMRS for PUSCH (and DMRS) measurements.

-	PUCCH + DMRS for PUCCH (and DMRS) measurements.

-	PRACH for PRACH measurements.

SRS for SRS measurements.

H.3.1	Measurement of Receiver Characteristics

As specified in the test case. Otherwise:

- PUSCH + DMRS for measurements with uplink interference configured.

- PUCCH + DMRS for measurements without uplink interference configured.

H.3.2	Measurement of Performance Requirements

As specified in the test case. Otherwise:

- PUCCH + DMRS for measurements without CSI feedback, or with CSI feedback in PUCCH mode.

- PUSCH + DMRS for measurements with CSI feedback in PUSCH mode.

H.4	Connection for NB1

The following clauses describes the uplink physical channels that are transmitted during a connection i.e., when measurements are done.

H.4.0	Measurement of Transmitter Characteristics

As specified in the test case. Otherwise:

-	NPUSCH for measurements.

-	NPRACH for PRACH measurements.

H.4.1	Measurement of Receiver Characteristics

As specified in the test case. Otherwise:

- NPUSCH (format 2) for measurements.

H.4.2	Measurement of Performance Requirements

As specified in the test case. Otherwise:

- NPUSCH (format 2) for measurements.

Annex I (informative):
Handling requirements and tests for different releases and UE capabilities

This annex gives guidance on how minimum requirements in different releases of 3GPP TS 36.102 [11] and different UE capabilities are handled in the specification 3GPP TS 36.521-4.

<< END OF CHANGES >>