3GPP TSG-RAN5 Meeting #100	R5-234959

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

Source:	MediaTek Inc.

Title:	Discussion on handling legacy eMTC test case for IoT NTN UE

# Document for:	Endorsement

Agenda Item:	6.3.23.5

1.	Introduction and Background

Internet of Things Non-Terrestrial Network (IoT NTN) was introduced in 3GPP Rel-17 to support NB-IoT and eMTC over satellite, re-using as much as possible legacy NB-IoT and eMTC features as defined in TS 36.763 [1] and TS 36.300 [2]:

[TS 36.763]

5.3	IoT NTN features

It is assumed that all cellular IoT features specified up to Rel-16 are supported for IoT NTN.

It is assumed that both NB-IoT multi-carrier operation and NB-IoT single-carrier operation are supported as a baseline.

[TS 36.300]

23.21.1	General

Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks (see clause 4.12) is only applicable to E-UTRA connected to EPC. UEs not supporting NTN are barred from accessing an NTN cell.

In NTN, only BL UEs, UEs in enhanced coverage and NB-IoT UEs with GNSS capability are supported in this release of the specification.

…

In this release of the specification, NTN is only applicable to FDD system.

At RAN5 #99, legacy eMTC/NB-IoT test case list applicable to IoT NTN was discussed but not finally concluded, see details in [3] [4]. This discussion paper aims to complete the eMTC case review and furtherly update the adapted legacy test cases for IoT NTN UE.

2.	Discussion

This discussion paper proposes an overall solution for re-using the existing eMTC test cases, which are applicable to IoT NTN only UE, with method defined as below.

  - TS 36.523-1 Test cases

Handle the applicable test cases, after scoping, as follows:

- For a legacy eMTC TC that is not applicable to IoT NTN only UE fully:

- no change

- For a legacy eMTC TC that is applicable to IoT NTN only UE fully:

- no change, or

- add a note around the test procedure to mention the difference for IoT NTN. One example is captured in CR R5-235079 [5], TC 9.2.1.1.22.

- For a legacy eMTC TC that is applicable to IoT NTN only UE partially:

- add a exception ‘NOT IoT NTN only’ around the test steps not applicable to IoT NTN only UE. One example is captured in CR R5-235079 [5], TC 6.1.2.3.

Proposal #1: For eMTC legacy test cases are full or partially applicable to UEs supporting eMTC over NTN, update the test cases according to NTN requirements. See R5-235079 [5] for details.

  - TS 36.523-2 Case applicability

In TS 36.523-2, create a new clause and add all applicable legacy eMTC test cases which needs to be executed for IoT NTN only UE.

For IoT NTN only UE, it is not able to access TN networks and other RAT capabilities, e.g., pc_GERAN, pc_UTRA, pc_1xRTT, pc_HRPD and pc_eWLAN, shall be set to False, therefore inter-RAT test cases shall be excluded.

As NTN is only applicable to FDD duplex mode, the TDD capability, e.g., pc_eTDD shall be set to False, test cases with TDD mode, e.g., applicability condition includes “and E-UTRA TDD” shall be excluded. And NTN only applicability to Category M1 UE with FDD duplex mode, test cases not applicable to M1, e.g., applicability condition with “NOT Category M1”, shall be excluded.

Similarly, other features considered to be not applicable to IoT NTN includes: MFBI, (e)MBMS/SC-PTM, ePDG, LWA/LAA, CA/DC, CoMP, ProSe, V2X and HST. The corresponding PICS shall be set to false for IoT NTN only UE.

Proposal #2: Add a new table for legacy eMTC test cases applicable to UEs supporting eMTC over NTN. See R5-235083 [6] for details.

3.	Proposal/Way Forward

In the contribution, we discuss the analysis of applicable legacy eMTC cases for IoT NTN with following proposals:

Proposal #1: For eMTC legacy test cases are full or partially applicable to UEs supporting eMTC over NTN, update the test cases according to NTN requirements. See R5-235079 [5] for details.

Proposal #2: Add a new table for legacy eMTC test cases applicable to UEs supporting eMTC over NTN. See R5-235083 [6] for details.

4.	References

[1]	TS 36.763: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN); (cl. 5.3)

[2]	TS 36.300: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2; (cl. 23.21.1)

[3] 	R5-233478: Discussion paper of handling legacy test case for IoT NTN UE, RAN5#99

[4]	R5-233495: Applicability of legacy NB-IOT testcases to NTN GSO capable UE, RAN5#99

[5] 	R5-235079: Correction to legacy eMTC cases for IoT NTN, MediaTek Inc.

[6] 	R5-235083: Applicable legacy eMTC cases for IoT NTN, MediaTek Inc.