3GPP TSG-RAN WG5 Meeting #100	R5-235131

# Toulouse, France, August 21 – 25, 2023

Agenda item:	7.4.4

Source: 	ZTE Corporation

Title:	Suggestions on the modification of SUL CA configuration list for PRD21

Document for:	Discussion and endorsement

# 1. Introduction

In RAN4#107 meeting, three CRs were approved in [1-3] for introducing new notation for single SUL CA combinations in R16, R17 and R18 respectively. Notations for single SUL CA combinations are revised as below for the three example band combinations.

| CA configuration | Existing notation | Proposed notation | Note |
| --- | --- | --- | --- |
| Intra-band contiguous | SUL_n41C-n80A | CA_n41C-n80A | For the two NUL carriers within CA_n41C-n80A, the SUL band n80 is corresponding to one of them in one cell. |
| Intra-band non-contiguous | SUL_n41(2A)-n99A | CA_n41(2A)-n99A | For the two NUL carriers within CA_n41(2A)-n99A, the SUL band n99 is corresponding to one of them in one cell. |
| Inter-band | CA_n1A_SUL_n78A-n80A | CA_n1A_n78A-n80A | For the two NUL carriers within CA_n1A_n78A-n80A, the SUL band n80 is corresponding to n78 in one cell. |

In this paper, we suggest the modification for SUL CA configuration list in PRD21.

2. Discussion

According to PRD21, there are four types of configurations related to SUL band, in which 33 configurations are SUL band combination in FR1, 17 configurations are SUL band combination with intra-band contiguous CA, 5 configurations are SUL band combinations with intra-band non-contiguous CA, and 26 configurations are SUL band combination with inter-band CA. Based on the WF in [1], the notation for the three types of “SUL FR1 with IB cont CA”, “SUL FR1 with IB non-cont CA” and “SUL FR1 with inter-band CA” should be modified accordingly.

Proposal 1: It is suggested to modify the notations for the configuration types of “SUL FR1 with IB cont CA”, “SUL FR1 with IB non-cont CA” and “SUL FR1 with inter-band CA” in the Excel file “PRD21 5G NR bands and CADC configurations list.xlsm” attached as below.

# 3. Conclusion

In this proposal, we discuss the notation modification of SUL CA configurations. Based on the discussion, we have the following proposal.

Proposal 1: It is suggested to modify the notations for the configuration types of “SUL FR1 with IB cont CA”, “SUL FR1 with IB non-cont CA” and “SUL FR1 with inter-band CA” in the Excel file “PRD21 5G NR bands and CADC configurations list.xlsm” attached as below.

# 4. References

- R4-2306546, WF on the notation for single SUL CA combinations, Apple

- R4-2309084, Rel-16 CR for 38.101-1: Single SUL CA combination notation modifications, Apple

- R4-2309085, Rel-17 CR for 38.101-1: Single SUL CA combination notation modifications, Apple

- R4-2309086, Rel-18 CR for 38.101-1: Single SUL CA combination notation modifications, Apple