3GPP TSG-RAN5 Meeting #100	R5-235179

# Toulouse, France, 21st Aug 2023 – 25th Aug 2023

Agenda item:	5.4.17

Source:	Keysight Technologies

Title:	Discussion on PDCCH RMC DCI format 1-1 payload

Document for:	Discussion and Endorsement

# Introduction

This contribution intends to clarify a discrepancy identified between RAN4 definition of PDCCH Reference Channel in terms of DCI bit size defined in TS 38.521-4 [1] Annex A.3.3 and the real DCI bit size exercised by RAN5 PDCCH demodulation test cases.

# Discussion

A discrepancy about DCI bit size defined by RAN4 in TS 38.521-4 [1] Annex A.3.3 (it comes directly from TS 38.101-4 [ 2]) and the real DCI bit size that PDCCH demodulation test cases are exercising.

Let’s use PDCCH demod 5.3.3.1.2 Test3 as example:

Table 5.3.3.1.2.3-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 4 | R.PDCCH. 1-2.2 FDD | TDLC300-100 | 2x4 Low | 1 | -1.9 |
| 2 | 10 | 48 | 2 | 8 | R.PDCCH. 1-2.5 FDD | TDLC300-100 | 2x4 Low | 1 | -4.5 |
| 3 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.2 FDD | TDLA30-10 | 2x4 Low | 1 | -1.0 |

Test 3 points out to R.PDCCH.1-1.2 FDD which is defined as follows in TS 38.521-4 [1] Annex A.3.3

Table A.3.3.1.1-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-1.1 FDD | R.PDCCH.1-1.2 FDD | R.PDCCH.1-1.3 FDD |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |  |
| CORESET frequency domain allocation |  | 48 | 48 | 48 |  |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 |  |  |  |
| Aggregation level |  | 4 | 4 | 8 |  |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 |  |  |  |
| Payload (without CRC) | Bits | 39 | 52 | 52 |  |  |  |

R.PDCCH.1-1.2 FDD defines 52 bits for DCI format 1_1. However, based on TS 38.521-4 [1] test configuration and TS 38.508-1 [3], DCI format 1_1 is not reaching 52 bits.

Doing an exercise on all DCI format 1_1 fields that can have variable bit size depending on configuration, it has been identified that there is a need for some unexpected configuration for PDCCH to achieve bit size defined in PDCCH RMC. Following DCI format 1_1 fields do not match with current TS 38.521-4 [1] and TS 38.508-1 [3] configuration:

- Bandwidth Part Indicator:

| Field |  | Bit size | Comment |
| --- | --- | --- | --- |
| carrier_id_bits: 0 |  | 0 | Not Present - Table 4.3.6.1.2.2-1 |
| id_for_dci_formats_bits: 1 (fixed) |  | 1 | Fixed according to 38.212 |
| bandwidth_part_ind_bits: 0 |  | 2 | Required 2 bits to meet with PDCCH RMC |

According to TS 38.212 [4]:

For demod test cases no additional BWP is configured. To have the bandwidth part indicator with bit size = 2, It is needed to configure 4 additional bandwidth parts.

- Time domain resource assignment:

| Field |  | Bit size | Comment |
| --- | --- | --- | --- |
| time_res_assignment_bits: 1 |  | 4 | Required 4 bits to meet with PDCCH RMC |

According to TS 38.212 [4]:

According to TS 38.508-1 [3]:

There are two entries for demod test cases then, log2(2) = 1 bit size

To have 4 bits, it would be needed 16 entries in the PDSCH-TimeDomainResourceAllocationList then, log2(16) = 4 bits.

- Transport block 2 related parameters:

| Field |  |  | Bit size | Comment |
| --- | --- | --- | --- | --- |
| mcs2_bits: 0 |  |  | 5 | It would need 5 bits |
| ndi2_bits: 0 |  |  | 1 | It would need 1 bit |
| rv2_bits: 0 |  |  | 2 | It would need 2 bits |

According to TS 38.212 [4[, these parameters are present is maxNrofCodeWordsScheduledByDCI = 2:

According to TS 38.508-1 [3]

And this value is not overwritten in section 5.4.2 for Demod test cases, so, the values for “Modulation and Coding scheme 2”, “New data indicator 2” and “Redundancy Version 2” should be equal to 0 bits.

To have the mcs2, ndi2 and rv2 set to desired bitsize value, it would be needed maxNrofCodeWordsScheduledByDCI = 2.

- Antenna Ports

| Field |  | Bit size |  | Comment |
| --- | --- | --- | --- | --- |
| antenna_ports_bits: 4 |  |  | 6 | It would required 6 bits |

According to TS 38.212 [4]:

From TS 38.508-1 [3], following values for dmrs-Type and maxLength are defined, as these parameters are not overridden in section 5.4.2 from TS 38.508-1[3] nor TS 38.521-4 [1] section 5.3:

So, table 7.3.1.2.2-1 applies à 4 bits to select up to 16 different values:

To have the AntennaPort bitsize = 6, we would need to select TS 38.212 [4] table 7.3.1.2.2-4 with dmrs-Type=2 and maxLenght = 2. Please, note that this will also imply to change the Number of DMRS CDM groups without data to 3, which it is not valid for rank1 test cases (from section TS 38.521-4 [1] Table A.3.3.1.1-3, number of MIMO layers for PDCCH demod test cases is select to 1):

- Transmission configuration indication:

| Field |  |  | Bit size | Comment |
| --- | --- | --- | --- | --- |
| transmission_configuration_ind_bits: 0 |  | 3 |  | It would be required 3 bits |

According to TS 38.212 [4]:

From TS 38.508-1 [3], tci-PresentInDCI is set to not present. This parameter is not overwritten in section 5.4.2 for Demod test cases.

To have transmission configuration indication with bitsize = 3, then the tci-PresentInDCI should be transmitted in Coreset.

Observation 1: To summarize, current parameterization is not aligned with the initial calculations for DCI Format 1-1 bitsize in PDCCH RMC definition in TS 38.521-4 [1] and TS 38.101-4 [2]. To have the desired values, some modifications in the parameterization for PDCCH test cases in both TS 38.101-4 [2] and TS 38.521-4 [1] should be introduced as indicated in this document and for some items the required configuration is not aligned with common PDCCH configuration defined in TS 38.101-4 [2] and in others add some unnecessary extra complexity in test configuration.

Proposal 1. To send LS to RAN4 asking for guidance about PDCCH RMC DCI bit size.

# Conclusion

The following observations and proposals were made in this contribution.

Observation 1: To summarize, current parameterization is not aligned with the initial calculations for DCI Format 1-1 bitsize in PDCCH RMC definition in TS 38.521-4 [1] and TS 38.101-4 [2]. To have the desired values, some modifications in the parameterization for PDCCH test cases in both TS 38.101-4 [2] and TS 38.521-4 [1] should be introduced as indicated in this document and for some items the required configuration is not aligned with common PDCCH configuration defined in TS 38.101-4 [2] and in others add some unnecessary extra complexity in test configuration.

Proposal 1. To send LS to RAN4 asking for guidance about PDCCH RMC DCI bit size.

# References

- TS 38.521-4, “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance”, v17.3.0 (2023-06)

- TS 38.101-4, “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance”, v17.9.0 (2023-06)

- TS 38.508-1, “User Equipment (UE) conformance specification;Part 1: Common test environment”, v17.9.0 (2023-06)

- TS 38.212, “NR; Multiplexing and channel coding”, v17.5.0 (2023-06)