3GPP TSG-RAN5 Meeting #100	R5-234483

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 3989 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update BWP TC 7.1.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. According to the discussion result in R5-234485., the option1 in the proposal has been chosen to simplify the TC. 2.There is only one dedicated PUCCH Resource(pucch-ResourceId=0)in BWP#1/#2/#3, HARQ ACK and SchedulingRequest can only use this resource. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Delete the 6RB BWP#1#2#3 configuration (including TRS configuration and 6RB CORESET configuration) from 7.1.1.8.1. 2. In dedicated BWP#1#2#3, the harqfeedback use pucch-RessourceId=0 and SchedulingRequest use pucch-ResourceId=0,too. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is possible that SS validated test case with a pc_bwp-WithoutRestriction=False UE but a pc_bwp-WithoutRestriction=True UE may fail for testing different branch of 7.1.1.8.1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

7.1.1.8.1	Bandwidth Part (BWP) operation UL/DL

Editors Note: Test case is only applicable to NR FR1 TDD bands and is FFS for NR FR1 FDD and NR FR2 bands due to open issues and alignment with respective PICS.

7.1.1.8.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives BandwidthPart-Config IE included in RRC Message received on SpCell (i.e. PSCell in case of EN-DC or PCell in case of SA) }

then { UE starts normal MAC operation in the FirstActive UL and DL Bandwidth part }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a DL DCI format 1_1 assigning a BWP different than the previously configured BWP }

then { UE starts normal MAC operation in the received new BWP }

}

(3)

with { UE in RRC_CONNECTED }

ensure that {

when { UE receives a UL DCI format 0_1 assigning a BWP different than the previously configured BWP }

then { UE starts normal MAC operation in the received new BWP }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE determines that a RACH Procedure is triggered in SpCell (i.e. PSCell in case of EN-DC or PCell in case of SA) and PRACH occasions are not configured }

then { UE initiates the PRACH procedure in the initial BWP }

}

(5)

with { UE in RRC_Connected State with defaultDownlinkBWP configured }

ensure that {

when { UE bwp-InactivityTimer expires }

then { UE performs BWP switching to a BWP indicated by the defaultDownlinkBWP }

}

(6)

with { UE in RRC_Connected State with defaultDownlinkBWP configured and Active BWP is different than defaultDownlinkBWP and bwp-InactivityTimer is running }

ensure that {

when { UE receives UL assignment or DL grant addressed to its C-RNTI }

then { UE restarts the bwp-InactivityTimer }

}

<End of modified section 1>

<Start of modified section 2>

7.1.1.8.1.3	Test description

7.1.1.8.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

7.1.1.8.1.3.2	Test procedure sequence

Table 7.1.1.8.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS transmits RRCReconfiguration to configure the dedicated BWPs incl. the FirstActive BWP. (Note 1) (Note 4). | <-- | (RRCReconfiguration) | - | - |
| - | EXCEPTION: Steps 0Aa1 to 0Ab2 describe behaviour which depends on procedure parameters; the "lower case letter" identifies a step sequence that take place if a procedure parameter has a particular value. | - | - | - | - |
| 0Aa1 | IF Connectivity is EN-DC or NGEN-DC, the UE sends RRCReconfigurationComplete (Note 2). | --> | (RRCReconfigurationComplete) | - | - |
| 0Ab1 | IF Connectivity is NR, the SS allocates (transmitted in FirstActiveDownlinkBWP) an UL Grant with DCI format 0_1 indicating FirstActiveUplinkBWP (BWP#1). | <-- | UL Grant | - | - |
| 0Ab2 | Check: Does the UE send RRCReconfigurationComplete in the FirstActive BWP configured? (Note 2) (Note 3) (Note 5) | --> | (RRCReconfigurationComplete) | 1 | P |
| 1 | The SS transmits a valid MAC PDU containing RLC PDU in the configured FirstActive Downlink BWP configured. | <-- | MAC PDU | - | - |
| 2 | After 100ms from step 1, the SS allocates (transmitted in FirstActiveDownlinkBWP) an UL Grant. | <-- | UL Grant | - | - |
| 3 | Check: Does the UE transmit a MAC PDU including one RLC SDU in the FirstActive BWP configured? (Note 5) | --> | MAC PDU | 1 | P |
| 3A | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 3. | <-- | MAC PDU (RLC STATUS PDU) | - | - |
| 4 | Void | - | - | - | - |
| 5 | The SS indicates on PDCCH (transmitted in Downlink BWP#1) DL DCI format 1_1 with new BWP Id (IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD THEN BWP #0 ELSE IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD THEN = BWP #2) and transmits a MAC PDU containing RLC PDU on the newly configured BWP (i.e. IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD THEN Downlink BWP#0 ELSE IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD THEN Downlink BWP#2). | <-- | MAC PDU | - | - |
| 6 | After 100ms from step5, the SS allocates (transmitted in Downlink BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD ELSE in Downlink BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) an UL Grant (with DCI indicating BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD; ELSE BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD), sufficient for loopback of the RLC SDU from step 5 in a Slot. (Note 3) | <-- | UL Grant | - | - |
| 7 | Check: Does the UE transmit a MAC PDU including one RLC SDU in the configured BWP (i.e. Uplink BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD ELSE BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD)? (Note 5) | --> | MAC PDU | 2 | P |
| 7A | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 7. | <-- | MAC PDU (RLC STATUS PDU) | - |  |
| 8 | Void | - | - | - | - |
| 9 | The SS transmits a valid MAC PDU containing RLC PDU in the configured BWP (i.e. Downlink BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD; ELSE BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD). | <-- | MAC PDU | - | - |
| 10 | After 100ms from step 9 the SS indicates on PDCCH (transmitted in Downlink BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD ELSE BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD) UL DCI format 0_1 with new BWP Id (=IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD, THEN BWP #3 ELSE IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD THEN BWP #1) and allocates an UL Grant, sufficient for loopback of the RLC SDU from step 9 in a Slot. | <-- | UL Grant | - | - |
| 11 | Check: Does the UE transmit a MAC PDU including one RLC SDU in the configured BWP (i.e. (IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD, THEN BWP #3 ELSE IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD THEN BWP #1, for FDD and for TDD)? (Note 5) | --> | MAC PDU | 3 | P |
| 11AA | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDUs in step 11. | <-- | MAC PDU (RLC STATUS PDU) | - | - |
| 11A | The SS transmits a valid MAC PDU containing RLC PDU in the configured BWP (IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD, THEN BWP #3 ELSE IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD THEN BWP #1 for TDD and for FDD). | <-- | MAC PDU | - | - |
| 12 | After 100ms from step 11A the SS indicates PDCCH order on CSS for contention-based random access (transmitted in Downlink BWP (IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD, THEN BWP #3 ELSE IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD THEN BWP #1 for TDD and for FDD). | <-- | PDCCH Order (ra-PreambleIndex = '000000'B) | - | - |
| 13 | Check: Does the UE send PRACH Preamble in the initial BWP (UL BWP#0)? | --> | PRACH Preamble | 4 | P |
| 13A | The SS transmits (in Downlink BWP #0) a MAC PDU addressed to UE RA-RNTI, containing RAR with matching RAPID in MAC sub header. | <-- | Random Access Response | - | - |
| 13B | The UE sends (in UL BWP#0) a msg3 in the grant associated to the received Random Access Response. | --> | msg3 (C-RNTI MAC CONTROL ELEMENT) | - | - |
| 13C | SS schedules (in Downlink BWP#0) PDCCH transmission for UE C-RNTI and allocates UL grant sufficient for the UE to loop back the data received at step 11a. | <-- | Contention Resolution | - | - |
| 13D | Check: Does the UE transmit a MAC PDU including one RLC SDU in the initial BWP (i.e. Uplink BWP#0)? (Note 5) | --> | MAC PDU | 4 | P |
| 13E | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 13. | <-- | MAC PDU (RLC STATUS PDU) | - |  |
| 14-15 | Void | - | - | - | - |
| 16 | The SS indicates on PDCCH (transmitted in Downlink BWP#0) DL DCI format 1_1 with BWP Id (= BWP #1) and transmits a MAC PDU containing RLC PDU on the configured BWP (i.e. Downlink BWP#1). | <-- | MAC PDU | - | - |
| 17 | After 200 ms from step 16, the SS transmits another valid MAC PDU containing RLC PDU in the active BWP (i.e. Downlink BWP#1). | <-- | MAC PDU | - | - |
| 18 | After 200 ms from step 17, the SS allocates (transmitted in Downlink BWP#1) an UL Grant (with DCI indicating BWP#1), sufficient for loopback of a MAC PDU containing both RLC SDUs from steps 16 and 17 in a Slot. (Note 3) | <-- | UL Grant | - | - |
| 19 | Check: Does the UE transmit a MAC PDU containing both RLC SDUs in the active BWP (i.e. Uplink BWP#1)? (Note 5) | --> | MAC PDU | 6 | P |
| 19A | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 19. | <-- | MAC PDU (RLC STATUS PDU) | - |  |
| 20 | The SS waits 1000 ms from step 18 to ensure that the bwp-InactivityTimer expired and then transmits a valid MAC PDU containing RLC PDU in the BWP with defaultDownlinkBWP-Id (= Downlink BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD ELSE BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD). | <-- | MAC PDU | - | - |
| 21 | The SS allocates (transmitted in the defaultDownlinkBWP, i.e. Downlink BWP#2) an UL Grant (with DCI indicating BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD ELSE BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD), sufficient for loopback of the RLC SDU from step 20 in a Slot. (Note 3) | <-- | UL Grant | - | - |
| 22 | Check: Does the UE transmit a MAC PDU in Uplink BWP#2 IF pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD ELSE BWP#0 IF pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD (= BWP Id of the defaultDownlinkBWP)? (Note 5) | --> | MAC PDU | 5 | P |
| 22A | SS transmits a MAC PDU containing an RLC STATUS PDU acknowledging the reception of the AMD PDU in step 22. | <-- | MAC PDU (RLC STATUS PDU) | - |  |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. Note 3: In paired spectrum (= FDD), the switching of Downlink BWP and Uplink BWP can happen independently. Whereas in TDD, the switching of BWP for Downlink and Uplink is always at the same time instance. Currently, the scope of the Test Purposes (TP) is considered to not cover checking of a BWP deviation which results from non-synchronized Downlink and Uplink BWP switching in FDD. Note 4: After the preamble the UE is in RRC_CONNECTED, therefore SRBs and DRBs are already established. The RRCReconfiguration message in step 1 shall not contain any elements like e.g. "rlc-BearerToAddModList" whose value(s) remain unchanged since the preamble. The sole purpose of the RRCReconfiguration message in step 1 is to configure BWPs and related fields for switching of BWPs. Note 5: When the UE does not use the expected BWP for the UL transmission the SS shall not receive the data what implicitly fails the test case. |  |  |  |  |  |

7.1.1.8.1.3.3	Specific message contents

Table 7.1.1.8.1.3.3-0: Conditions for specific message contents

| Condition | Explanation |
| --- | --- |
| BWP#1 | Bandwidth part 1 |
| BWP#2 | Bandwidth part 2 |
| BWP#3 | Bandwidth part 3 |

Table 7.1.1.8.1.3.3-0A: PDSCH-TimeDomainResourceAllocationList (Preamble)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList ::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF PDSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | S=2, L=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | 1 |  | pc_bwp-SwitchingDelay_Type1 AND SCS_15KHz |
|  | 6 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_15KHz |
|  | 2 |  | pc_bwp-SwitchingDelay_Type1 AND SCS_30KHz |
|  | 6 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_30KHz |
|  | 3 |  | pc_bwp-SwitchingDelay_Type1 AND SCS_60KHz |
|  | 9 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_60KHz |
|  | 6 |  | pc_bwp-SwitchingDelay_Type1 AND SCS_120KHz |
|  | 18 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_120KHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 72 | Start symbol(S)=2, Length(L)=6 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-0B: SchedulingRequest-Config (Preamble)

| Derivation Path: 38.508-1 [4], Table 4.6.3-155 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| sr-TransMax | n64 |  |  |

Table 7.1.1.8.1.3.3-1: RRCReconfiguration (step 0)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 (see also Note 4 in Table 7.1.1.8.1.3.2-1) |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| secondaryCellGroup |  | CellGroupConfig |  | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  | NR |
| masterCellGroup |  | CellGroupConfig |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.8.1.3.3-1A: CellGroupConfig (Table 7.1.1.8.1.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
|  | ServCellIndex |  | EN-DC |
| spCellConfigCommon | Not present |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig-Dedicated | Table 7.1.1.8.1.3.3-2 |  |
| } |  |  |  |
| reportUplinkTxDirectCurrent-v1530 | true |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2: ServingCellConfig-Dedicated (Table 7.1.1.8.1.3.3-1A)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| tdd-UL-DL-ConfigurationDedicated | Not present |  |  |
| downlinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) BWP-Downlink { | 3 entries |  |  |
| BWP-Downlink[1] | BWP-Downlink-BWP-N with condition BWP#1 | entry 1 |  |
| BWP-Downlink[2] | BWP-Downlink-BWP-N with condition BWP#2 | entry 2 | pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD |
| BWP-Downlink[3] | BWP-Downlink-BWP-N with condition BWP#3 | entry 3 | pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD |
| } |  |  |  |
| firstActiveDownlinkBWP-Id | 1 |  |  |
| bwp-InactivityTimer | ms750 |  |  |
| defaultDownlinkBWP-Id | 2 |  | pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD |
|  | 0 |  | pc_bwp_sameNumerology_upto2_FR1_TDD and band under test is FR1 TDD |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToReleaseList | Not present |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF BWP-Uplink { | 3 entries |  |  |
| BWP-Uplink[1] | BWP-Uplink-BWP-N with condition BWP#1 | entry 1 |  |
| BWP-Uplink[2] | BWP-Uplink-BWP-N with condition BWP#2 | entry 2 | pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD |
| BWP-Uplink[3] | BWP-Uplink-BWP-N with condition BWP#3 | entry 3 | pc_bwp_sameNumerology_upto4_FR1_TDD and band under test is FR1 TDD |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 |  |  |
| pusch-ServingCellConfig | Not present |  |  |
| } |  |  |  |
| pdcch-ServingCellConfig | Not present |  |  |
| pdsch-ServingCellConfig | Not present |  |  |
| csi-MeasConfig | Not present |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2A: BWP-Downlink-BWP-N (Table 7.1.1.8.1.3.3-2 and Table 7.1.1.8.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Downlink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP#1 |
|  | 2 |  | BWP#2 |
|  | 3 |  | BWP#3 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | 6600 | Note 2 | BWP#1,2,3 and 5MHz |
|  | 7975 | Note 3 | BWP#1 |
|  | 9625 | Note 3 | BWP#2 |
|  | 11275 | Note 3 | BWP#3 |
|  | 12925 | Note 4 | BWP#1 and 100MHz |
|  | 14575 | Note 4 | BWP#2 and 100MHz |
|  | 16225 | Note 4 | BWP#3 and 100MHz |
| } |  |  |  |
| pdcch-ConfigCommon | Not present | no cell specific configuration for dedicated BWP |  |
| pdsch-ConfigCommon | Not present | no cell specific configuration for dedicated BWP |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| pdcch-Config CHOICE { |  |  |  |
| setup | PDCCH-Config-BWP-N with condition BWP#N |  |  |
| } |  |  |  |
| pdsch-Config CHOICE { |  |  |  |
| setup | PDSCH-Config-BWP-N with condition BWP#N |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: Void Note 2: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=25 and RBStart=0 for BWP#1,2,3 Note 3: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=30,36,42 and RBStart=0 for BWP#1,2,3 Note 4: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=48,54,60 and RBStart=0 for BWP#1,2,3 |  |  |  |

Table 7.1.1.8.1.3.3-2B: PDCCH-Config-BWP-N (Table 7.1.1.8.1.3.3-2A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE (SIZE (1..3)) OF ControlResourceSet { | 2 entries |  |  |
| ControlResourceSet[1] | ControlResourceSet-BWP-N with condition BWP#N | entry 1 |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE (SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace-BWP-N with condition BWP#N | entry 1 |  |
| SearchSpace[2] | SearchSpace-CSS-BWP-N with condition BWP#N | entry 2 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2C: PDSCH-Config-BWP-N (Table 7.1.1.8.1.3.3-2A)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-100 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| PDSCH-TimeDomainAllocationList::= SEQUENCE { | PDSCH-TimeDomainResourceAllocationList |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2CA: PDSCH-TimeDomainResourceAllocationList (Table 7.1.1.8.1.3.3-2C)

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainAllocationList::= SEQUENCE { | 2 entries |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  | entry 1 |  |
| k0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 53 | Start symbol(S)=2, Length(L)=12 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 |  |
| k0 | 1 |  | pc_bwp-SwitchingDelay_Type1 AND SCS_15KHz |
|  | 6 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_15KHz |
|  | 2 |  | pc_bwp-SwitchingDelay_Type1 AND SCS_30KHz |
|  | 6 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_30KHz |
|  | 3 |  | pc_bwp-SwitchingDelay_Type1 AND SCS_60KHz |
|  | 9 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_60KHz |
|  | 6 |  | pc_bwp-SwitchingDelay_Type1 AND SCS_120KHz |
|  | 18 |  | pc_bwp_SwitchingDelay_Type2 AND SCS_120KHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 72 | Start symbol(S)=2, Length(L)=6 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2D: ControlResourceSet-BWP-N (Table 7.1.1.8.1.3.3-2B)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 9 |  | BWP#1 |
|  | 10 |  | BWP#2 |
|  | 11 |  | BWP#3 |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2E: SearchSpace-BWP-N (Table 7.1.1.8.1.3.3-2B)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-162 with condition USS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 37 |  | BWP#1 |
|  | 38 |  | BWP#2 |
|  | 39 |  | BWP#3 |
| controlResourceSetId | 9 |  | BWP#1 |
|  | 10 |  | BWP#2 |
|  | 11 |  | BWP#3 |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2F: BWP-Uplink-BWP-N (Table 7.1.1.8.1.3.3-2 and Table 7.1.1.8.1.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-Uplink ::= SEQUENCE { |  |  |  |
| bwp-Id | 1 |  | BWP#1 |
|  | 2 |  | BWP#2 |
|  | 3 |  | BWP#3 |
| bwp-Common SEQUENCE { |  |  |  |
| genericParameters SEQUENCE { |  |  |  |
| locationAndBandwidth | 6600 | Note 2 | BWP#1,2,3 and 5MHz |
|  | 7975 | Note 3 | BWP#1 |
|  | 9625 | Note 3 | BWP#2 |
|  | 11275 | Note 3 | BWP#3 |
|  | 12925 | Note 4 | BWP#1 and 100MHz |
|  | 14575 | Note 4 | BWP#2 and 100MHz |
|  | 16225 | Note 4 | BWP#3 and 100MHz |
| } |  |  |  |
| rach-ConfigCommon | Not present | No cell specific configuration for dedicated BWP |  |
| pusch-ConfigCommon | PUSCH-ConfigCommon-BWP-N |  |  |
| pucch-ConfigCommon | PUCCH-ConfigCommon-BWP-N |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| pucch-Config CHOICE { |  |  |  |
| setup | PUCCH-Config-BWP-N |  |  |
| } |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup | PUSCH-Config-BWP-N |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1:Void Note 2: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=25 and RBStart=0 for BWP#1,2,3 Note 3: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=30,36,42 and RBStart=0 for BWP#1,2,3 Note 4: According to TS 38.214 [21] clause 5.1.2.2.2 with =275, LRBs=48,54,60 and RBStart=0 for BWP#1,2,3 |  |  |  |

| Condition | Explanation |
| --- | --- |
| 5MHz | According to TS 38.508-1 [4] clause 6.2.3.1 with CBW=5Mhz |
| 100MHz | According to TS 38.508-1 [4] clause 6.2.3.1 with CBW=100Mhz |

Table 7.1.1.8.1.3.3-2G: PUCCH-Config-BWP-N (Table 7.1.1.8.1.3.3-2F)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-112 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-Config::= SEQUENCE { |  |  |  |
| resourceSetToAddModList SEQUENCE (SIZE (1..4)) OF PUCCH-ResourceSet { | 1 entry |  |  |
| PUCCH-ResourceSet[1] SEQUENCE { |  | entry 1 |  |
| pucch-ResourceSetId | 0 |  |  |
| resourceList SEQUENCE (SIZE (1..32)) OF PUCCH-ResourceId { | 1 entry |  |  |
| PUCCH-ResourceId[1] | 0 | entry 1 |  |
| } |  |  |  |
| maxPayloadSize | 256 |  |  |
| } |  |  |  |
| } |  |  |  |
| resourceToAddModList SEQUENCE (SIZE (1..128)) OF PUCCH-Resource { | 1 entry |  |  |
| PUCCH-Resource[1] SEQUENCE { |  | entry 1 |  |
| pucch-RessourceId | 0 |  |  |
| startingPRB | 0 |  | BWP#1 |
|  | 0 |  | BWP#2 |
|  | 0 |  | BWP#3 |
| intraSlotFrequencyHopping | disabled |  |  |
| secondHopPRB | Not Present |  |  |
| format CHOICE { |  |  |  |
| format0 SEQUENCE { |  |  |  |
| initialCyclicShift | 0 |  |  |
| nrofSymbols | 2 |  |  |
| startingSymbolIndex | 10 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| schedulingRequestResourceToAddModList SEQUENCE (SIZE (1..maxNrofSR-Resources)) OF SchedulingRequestResourceConfig { | 1 entry |  |  |
| SchedulingRequestResourceConfig[1] | SchedulingRequestResourceConfig | entry 1 Table 7.1.1.8.1.3.3-2J |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2H: PUSCH-Config-BWP-N (Table 7.1.1.8.1.3.3-2F)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-118 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| pusch-TimeDomainAllocationList | PUSCH-TimeDomainResourceAllocationList | TS 38.508-1 [4] Table 4.6.3-122 |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-2I: Physical layer parameters for DCI format 1_1 (Steps 5-22A, Table 7.1.1.8.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PUCCH resource indicator | PUCCH-ResourceId= 0 | “000” |  |

Table 7.1.1.8.1.3.3-2J: SchedulingRequestResourceConfig (Table 7.1.1.8.1.3.3-2G)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-157 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SchedulingRequestResourceConfig ::= SEQUENCE { |  |  |  |
| resource | 0 |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-3: PUCCH-ConfigCommon-BWP-N (Table 7.1.1.8.1.3.3-2F)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-113 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| pucch-ResourceCommon | Not Present |  |  |
| pucch-GroupHopping | enable |  |  |
| hoppingId | Not present |  |  |
| p0-nominal | Not Present |  |  |
| } |  |  |  |

Table 7.1.1.8.1.3.3-4: SearchSpace-CSS-BWP-N (Table 7.1.1.8.1.3.3-2B)

| Derivation Path: TS 38.508-1[4], Table 4.6.3-162 with condition CSS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 30 |  | BWP#1 |
|  | 31 |  | BWP#2 |
|  | 32 |  | BWP#3 |
| controlResourceSetId | 9 |  | BWP#1 |
|  | 10 |  | BWP#2 |
|  | 11 |  | BWP#3 |
| } |  |  |  |

Table 7.1.1.8.1.3.3-5: Void

Table 7.1.1.8.1.3.3-6: Void

Table 7.1.1.8.1.3.3-7: Void

Table 7.1.1.8.1.3.3-8: Void

Table 7.1.1.8.1.3.3-9: Void

Table 7.1.1.8.1.3.3-10: Void

<End of modified section 2>

<Start of modified section 3>

7.1.1.8.1	Bandwidth Part (BWP) operation UL/DL

Editors Note: Test case is only applicable to NR FR1 TDD bands and is FFS for NR FR1 FDD and NR FR2 bands due to open issues and alignment with respective PICS.

Editors Note: Test case is only applicable to pc_bwp_WithoutRestriction =False and is FFS for pc_bwp_WithoutRestriction =True.

7.1.1.8.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives BandwidthPart-Config IE included in RRC Message received on SpCell (i.e. PSCell in case of EN-DC or PCell in case of SA) }

then { UE starts normal MAC operation in the FirstActive UL and DL Bandwidth part }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives a DL DCI format 1_1 assigning a BWP different than the previously configured BWP }

then { UE starts normal MAC operation in the received new BWP }

}

(3)

with { UE in RRC_CONNECTED }

ensure that {

when { UE receives a UL DCI format 0_1 assigning a BWP different than the previously configured BWP }

then { UE starts normal MAC operation in the received new BWP }

}

(4)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE determines that a RACH Procedure is triggered in SpCell (i.e. PSCell in case of EN-DC or PCell in case of SA) and PRACH occasions are not configured }

then { UE initiates the PRACH procedure in the initial BWP }

}

(5)

with { UE in RRC_Connected State with defaultDownlinkBWP configured }

ensure that {

when { UE bwp-InactivityTimer expires }

then { UE performs BWP switching to a BWP indicated by the defaultDownlinkBWP }

}

(6)

with { UE in RRC_Connected State with defaultDownlinkBWP configured and Active BWP is different than defaultDownlinkBWP and bwp-InactivityTimer is running }

ensure that {

when { UE receives UL assignment or DL grant addressed to its C-RNTI }

then { UE restarts the bwp-InactivityTimer }

}

<End of modified section 3>

<Start of modified section 4>

7.1.1.8.1.3	Test description

7.1.1.8.1.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 with the following differences:

- if pc_bwp_WithoutRestriction= True, the Resource block allocation for DCCH/DTCH transmissions (specified in TS 38.523-3 [3] 7.1.2.2.3.1) is not applied for BWP #1, BWP #2 and BWP #3. DCCH/DTCH could not be transmitted in even SFN slot#0(reserved for SSB#1) and slot#2(reserved for SIB1).

<End of modified section 4>