3GPP TSG-RAN5 Meeting #100	R5-235320

# Toulouse, France, 21st Aug 2023 – 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0546 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update test case 7.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Align with the outcome of the Tdoc R5-225274 to update IMS TCs with NG.114 default EVS configuration A2. Align with NG.114 default configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been updated to use the generic procedure A.4.1a. The test case has been updated to use preconditions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The aligment to use default EVS configuration cannot be completed. The test case applicability cannot be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0325 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-233906 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

<Start of first modified section>

## 7.1	MTSI MO Voice Call / 503 Service Unavailable / 5GS

7.1.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and UE having sent an initial INVITE request for MO Voice call }

ensure that {

when { UE receiving a 503 Service Unavailable response containing a Retry-After header indicating a period of 20 seconds }

then { UE does not reattempt the request until after the indicated period }

}

(2)

Void

7.1.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229, clause 5.1.3.1]:

Upon receiving a 503 (Service Unavailable) response to an initial INVITE request containing a Retry-After header, then the originating UE shall not automatically reattempt the request until after the period indicated by the Retry-After header contents.

[TS 24.229, clause 6.1.2]:

An INVITE request generated by a UE shall contain a SDP offer and at least one media description. This SDP offer shall reflect the calling user's terminal capabilities and user preferences for the session.

…

NOTE 2:	If the originating UE does not use the precondition mechanism (see subclause 5.1.3.1), it will not include any precondition information in the SDP message body.

7.1.3	Test description

7.1.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is configured to use preconditions.

Preamble:

-	UE is in state 1N-A and registered to IMS

7.1.3.2	Test procedure sequence

Table 7.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to attempt an IMS voice call | - | - | - | - |
| 2-7 | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| - | EXCEPTION: In parallel with Step 8, parallel behaviour defined in table 7.1.3.2-2 takes place | - | - | - | - |
| 8 | Step1 of Annex A.4.1a takes place. | --> | INVITE | - | - |
| 9 | SS sends 503 (Service Unavailable) with Retry-After header indicating a period of 20 seconds. | <-- | 503 Service Unavailable | - | - |
| 10 | UE acknowledges the reception of 503 (Service Unavailable) message. | --> | ACK | - | - |
| 11 | The SS starts timer t_Waits=20s. | - | - | - | - |
| 12 | Check: Does the UE reattempt the INVITE request? | --> | INVITE | 1 | F |
| 13 | The SS waits for expiry of t_Waits. | - | - | - | - |
| - | EXCEPTION: Steps 14a1 to 14a15 happen if the UE choses to reattempt the INVITE request, i.e., these steps are optional. The SS waits at most 30 more seconds for such INVITE request before it terminates the test case. | - | - | - | - |
| 14a1-14a5 | Steps 1 to 5 of Annex A.4.1a take place. | - | - | - | - |
| 14a6 | Step 10 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] is performed. | - | - | - | - |
| - | EXCEPTION: In parallel to steps 14a7 and 14a8 below, step 14a9 occurs. | - | - | - | - |
| 14a7-14a8 | Steps 11-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 14a9-14a15 | steps 6 to 12 of Annex A.4.1a take place | - | - | - | - |
| 14b1-14b11 | Void | - | - | - | - |

Table 7.1.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

7.1.3.3	Specific message contents

Table 7.1.3.3-1: 503 Service Unavailable (step 9, table 7.1.3.2-1)

| Derivation path: TS 34.229-1 [2], Annex A.4.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Retry-After |  |  |  |  |
| delta-seconds |  | 20 |  |  |

<End of modified section>