3GPP TSG-RAN5 Meeting #100	R5-235953

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1442 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common test environments for IOT-NTN testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Common test environ for IoT-NTN testing is currently missing in the spec |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added common test environ for IoT-NTN testing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-234238r1 R5-235953 |  |  |  |  |  |  |  |  |

#### 5.5.1.3	SRB and DRB combinations

##### 5.5.1.3.1	Combinations on DL-SCH and UL-SCH

5.5.1.3.1.1	SRB1 and SRB2 for DCCH + n x AM DRB + m x UM DRB, where n=1 and m=0

This SRB and DRB combination is setup in UE Registration procedure and the Generic Radio Bearer Establishment with UE Test Mode Activated using specific message content - the default RRCConnectionReconfiguration message with condition SRB2-DRB(n, m).

## 5.6	Test environment for NTN testing

This section defines the test environment which applies to all RF and Demod test cases executed for eMTC and NB-IoT capable NTN UEs, unless otherwise specified.

### 5.6.1	UE location

UE shall determine its location during the test using any of the following means.

1.	UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test.

2.	Other options such as providing UE location via AT command are not precluded.

UE location provided to the UE before the start of the test is as follows:

Longitude: depends on the ephemeris generated files

Latitude: depends on the ephemeris generated files

Altitude: depends on the ephemeris generated files

### 5.6.2	Ephemeris Generation

This section provides the means to generate Ephemeris files for GSO and NGSO orbits.

#### 5.6.2.0	Assumptions and Steps for Ephemeris generation

1.	Ensure the minimum elevation angle of the SAN as seen by the UE is atleast 30 deg

2.	FFS

#### 5.6.2.1	Ephemeris Generation for GSO

FFS

#### 5.6.2.2	Ephemeris Generation for NSGO

FFS

#### 5.6.3	IoT NTN message contents for RF Tx/Rx, Demodulation tests

### 5.6.3.1	EphemerisInfo-r17 values in SystemInformationBlockType31

The EphemerisInfo-r17 values in SIB31 shall be periodically signaled to the UE during test based on the Ephemeris files generated in clause 5.6.2

### 5.6.4	Satellite propagator model

During the test, TE shall use Eckstein-Hechler satellite propagator model for all tests unless specified otherwise.