3GPP TSG-RAN5 Meeting #100	R5-234152r1

# Toulouse, France, 21 – 25 Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-2 | CR | 0375 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of applicability for new test case 11.3.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2023-7-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The new test case following was added in TS 38.523-1. The corresponding applicability and condition statements needs to be added. 11.3.12: UAC / Access Identity 0 / AC7 / 0% access probability / Uplink user data transfer |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of applicability for new test case 11.3.12 and condition for this test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The applicability for new test case 11.3.12 and condition for this test case will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 CR3897 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 234152r1: change WIC from “TEI15_Test, 5GS_NR_LTE-UEConTest” to “TEI15_Test” |  |  |  |  |  |  |  |  |

[Start of change]

## 4.1	Protocol conformance test cases applicability

Table 4.1-5a: Applicability of Protocol conformance Multi-layer test cases, ref. TS 38.523-1 [2]

| Clause | TC Title | Release | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |
| 11 | Multi-layer and Services |  |  |  |
| 11.1 | 5GS / EPS Fallback |  |  |  |
| 11.1.1 | MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.1a | MO MMTEL enhanced voice service call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success | Rel-15 | C173 | UEs supporting 5G Core and E-UTRA and NG.114 v2.0 |
| 11.1.2 | MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode without N26 interface / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.3 | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.3a | MO MMTEL enhanced voice service call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / Success | Rel-15 | C173 | UEs supporting 5G Core and E-UTRA and NG.114 v2.0 |
| 11.1.4 | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode with N26 interface / E-UTRAN cell selection using cell status barred / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.5 | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode without N26 interface / E-UTRAN cell selection using cell status reservation / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS Voice (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and EPS fallback |
| 11.1.6 | MT MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode without N26 interface / Success | Rel-15 | C54 | UEs supporting 5G Core and E-UTRA and EPS IMS (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") Voice and EPS fallback |
| 11.1.7 | Emergency call setup from NR RRC_IDLE / Emergency Services Fallback to EPS with redirection / Single registration mode with N26 interface / Success | Rel-15 | C47 | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and Emergency Services Fallback in NR connected to 5GCN |
| 11.1.8 | MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / voiceFallbackIndication | Rel-16 | C95 | UEs supporting 5G Core and E-UTRA and EPS IMS (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") Voice and EPS fallback and voiceFallbackIndication |
| 11.1.9 | MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / voiceFallbackIndication | Rel-16 | C95 | UEs supporting 5G Core and E-UTRA and EPS IMS (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") Voice and EPS fallback and voiceFallbackIndication |
| 11.2 | 5G-SRVCC |  |  |  |
| 11.2.1 | 5G-SRVCC from NG-RAN to 3GPP UTRAN | Rel-16 | C127 | UEs supporting 5G Core and UTRA and NR to UTRA-FDD CELL_DCH CS handover |
| 11.3 | Unified Access Control (UAC) |  |  |  |
| 11.3.1 | UAC / Access Identity 0 / 0% access probability / MTSI MO speech call / SMSoIP | Rel-15 | C78 | UEs supporting 5G Core and Initiating session and MTSI speech and SMS over IP |
| 11.3.1a | UAC / Access Identity 0 / 0% access probability / Uplink user data transfer / RRC_INACTIVE | Rel-15 | C109A | UEs supporting 5G Core and RRC_INACTIVE and UE's usage setting as data centric |
| 11.3.2 | UAC / Access Identity 0 / 0% access probability / Paging for MT access/Emergency call | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.3.3 | UAC / Access Identity 0 / AC8 / RRC_INACTIVE / RNA update / RRC resume | Rel-15 | C109 | UEs supporting 5G Core and RRC_INACTIVE |
| 11.3.4 | UAC / Access Identity 0 / Registration procedure for mobility and periodic registration update / Barring per PLMN / Implicit AC barring list | Rel-15 | C21 | UEs supporting 5G Core |
| 11.3.5 | UAC / Access Identity 1 / New cell not in the country of its HPLMN/EHPLMN 0% access probability / MPS indicator / HPLMN/0%/100% accessibility AC5 / MMTEL-Video call | Rel-15 | C79 | UEs supporting 5G Core and Initiating session and MTSI video |
| 11.3.6 | UAC / Access Identity 2 / New cell not in the country of its HPLMN/EHPLMN 0% access probability / MCS indicator / HPLMN/0%/100% accessibility AC7 / RRC_INACTIVE | Rel-15 | C21 | UEs supporting 5G Core |
| 11.3.6a | UAC / Access Identity 2 / MCS indicator / SNPN / 0% / 100% accessibility AC7 / RRC_INACTIVE | Rel-16 | C231 | UEs supporting 5G Core and SNPN and configuration of access identities in the list of subscriber data |
| 11.3.7 | UAC / Access Identity 11..15 / High priority access / HPLMN/0% accessibility AC2 / Emergency call | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.3.8 | UAC / Access Identity 0 / NR RRC_IDLE / Cell re-selection while T390 is running | Rel-15 | C21 | UEs supporting 5G Core |
| 11.3.9 | UAC / Access Identity 0 / ODAC / PLMN / RPLMN / not EPLMN | Rel-15 | C21 | UEs supporting 5G Core |
| 11.3.9a | UAC / Access Identity 0 / ODAC / SNPN / RSNPN / new SNPN | Rel-16 | C131 | UEs supporting 5G Core and SNPN |
| 11.3.10 | UAC / Access Identity 0 / AC9 / 0% access probability / SIP Re-registration | Rel-16 | C198 | UEs supporting 5G Core and IMS security |
| 11.3.12 | UAC / Access Identity 0 / AC7 / 0% access probability / Uplink user data transfer | Rel-15 | C21 | UEs supporting 5G Core |
| 11.4 | Emergency Services |  |  |  |
| 11.4.1 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / Network failing the authentication check (5G AKA) | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.1a | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Utilising emergency number stored on the USIM / New emergency PDU session / PEIPS assistance information | Rel-17 | C224 | UEs supporting 5G Core and PEI |
| 11.4.2 | 5GMM-DEREGISTERED.LIMITED-SERVICE / Emergency call / Utilisation of emergency numbers stored on the ME / Initial registration for emergency services / Handling of forbidden PLMNs | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.3 | 5GMM-DEREGISTERED.NO-SUPI / Emergency call / Utilisation of emergency numbers stored on the ME / Initial registration for emergency services | Rel-15 | C238 | UEs supporting 5G Core and emergency services in NR connected to 5GCN and test execution with No USIM |
| 11.4.4 | 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE T3346 running / Emergency call establishment / 5GMM-REGISTERED.NORMAL-SERVICE / Emergency call establishment before T3396 expiry | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.5 | 5GMM-REGISTERED.LIMITED-SERVICE / 5GMM-IDLE / Emergency call establishment and release / Handling of 5GS forbidden tracking areas for roaming | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.6 | 5GMM-REGISTERED.NON-ALLOWED-SERVICE / Emergency call establishment and release / Handling of non-allowed tracking areas | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.7 | Handling of Local and Extended emergency numbers / Mobility | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.8 | Handling of Local and extended emergency numbers / Switch-off and maximum local numbers storage | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.9 | 5GMM-DEREGISTERED.LIMITED-SERVICE No suitable cells in tracking area / Emergency call establishment and release | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.10 | Void |  |  |  |
| 11.4.10a | 5GMM-REGISTERED.NORMAL-SERVICE / N26 interface not supported / N1 mode to S1 mode transfer of an existing emergency PDU session | Rel-15 | C85B | UEs supporting 5G core and Emergency PDU session transfer from N1 mode to S1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and IMS voice over NR |
| 11.4.11 | 5GMM-REGISTERED.NORMAL-SERVICE / N26 interface not supported / S1 mode to N1 mode transfer of an existing emergency PDN connection | Rel-15 | C85A | UEs supporting 5G core and Emergency PDN connection transfer from S1 mode to N1 mode when network does not support N26 interface, and, E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") and emergency services in NR connected to 5GCN |
| 11.4.12 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / Disabling N1 mode / Emergency call establishment over EPS / Success | Rel-15 | C176 | UEs supporting 5G Core and E-UTRA and EPS IMS emergency call (VoLTE in GSMA PRD IR.92: "IMS Profile for Voice and SMS") |
| 11.4.13 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call / obtaining new IP address different than the IP address | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.4.14 | 5GMM-REGISTERED.NORMAL-SERVICE / 5GMM-IDLE / Emergency call /Deregistration upon emergency registration expiration | Rel-15 | C92 | UEs supporting 5G Core and emergency services in NR connected to 5GCN |
| 11.5 | eCall over IMS |  |  |  |
| 11.5.1 | eCall Only mode / T3444 / eCall inactivity procedure / Removal of eCall only restriction after an eCall over IMS / 5GS to EPS | Rel-16 | C170 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |
| 11.5.2 | eCall Only mode / T3445 / eCall inactivity procedure / Removal of eCall only restriction after a call to URI for test service / 5GS to EPS | Rel-16 | C171 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |
| 11.5.3 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall over EPS / eCall failure if EPS and CS domain are not available | Rel-16 | C197 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.4 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported on 5GS / RACH failure in NR cell / eCall over EPS | Rel-16 | C197 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.5 | eCall Only mode / Limited service state / Call to URI for test service should not be attempted / eCall over IMS should be attempted / 5GS | Rel-16 | C174 | UEs supporting 5G Core and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and capable of triggering a Test eCall |
| 11.5.6 | eCall capable / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported / eCall using the CS domain / emergency call over IMS if eCall using the CS domain is not available / UTRA | Rel-16 | C185 | UEs supporting 5G Core and IMS eCall type of emergency services over 5GS and Automatic type of eCall initiation and emergency services in NR connected to 5GCN |
| 11.5.7 | eCall Only mode / SRVCC Handover to CS domain / UTRAN / MSD Update / Success / 5GS | Rel-16 | C186 | UEs supporting 5G Core and UTRA and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation and NR to UTRA-FDD CELL_DCH CS handover |
| 11.5.8 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is supported / RACH failure in NR cell / eCall using the CS domain | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.9 | eCall only mode / Manual initiation / Emergency registration / Abnormal case / IM CN sends a 486 (Busy Here) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS | Rel-16 | C187 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Manual type of eCall initiation |
| 11.5.10 | eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 600 (Busy Everywhere) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.11 | eCall only mode / Automatic initiation / Emergency registration / Abnormal case / IM CN sends a 603 (Decline) / UE performs eCall in CS domain / UTRAN or GERAN / 5GS | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.12 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported on 5GS / eCall over EPS | Rel-16 | C197 | UEs supporting 5G Core and E-UTRA and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.5.13 | eCall over IMS / Manual initiation / MSD transfer Failure / UE performs eCall in CS domain after Timer expiry / UTRAN or GERAN / 5GS | Rel-16 | C189 | UEs supporting 5G Core and (UTRA OR GERAN) and eCall type of emergency services over 5GS and Manual type of eCall initiation |
| 11.5.14 | eCall Only mode / 5GS supports IMS voice over PS session / 5GS does not support emergency service / eCall using CS domain | Rel-16 | C188 | UEs supporting 5G Core and (UTRA OR GERAN) and IMS eCall Only type of emergency services over 5GS and Automatic type of eCall initiation |
| 11.6 | 3GPP PS Data Off |  |  |  |
| 11.6.1 | Data Off / MO Voice Call | Rel-15 | C162 | UEs supporting 5G Core and NG.114 v1.0 default configuration voice exempt and 3GPP PS data off and Initiating session and MTSI speech |
| 11.6.2 | Data Off / MO Video Call | Rel-15 | C172 | UEs supporting 5G Core and NG.114 v2.0 default configuration video exempt and 3GPP PS data off and Initiating session and MTSI video |
| 11.6.3 | Data Off / SMSoIP | Rel-15 | C162A | UEs supporting 5G Core and NG.114 v2.0 and 3GPP PS data off and Initiating session and SMS over IP |
| 11.7 | eDRX |  |  |  |
| 11.7.1 | eDRX / IDLE | Rel-17 | C210 | UEs supporting 5G Core and eDRX |
| 11.7.2 | eDRX / Inactive / RAN-initiated paging | Rel-17 | C210 | UEs supporting 5G Core and eDRX |
| 11.8 | Inter-system mobility between untrusted Non-3GPP and 3GPP system |  |  |  |
| 11.8.1 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from NR to N3IWF/5GC | Rel-15 | C276 | UEs supporting 5G Core and handover from 5G Core Network to 5G Core over non-3GPP Access Network and WLAN |
| 11.8.2 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from N3IWF/5GC to NR / UE in 5GMM-DEREGISTERED states | Rel-15 | C248 | UEs supporting 5G Core and handover from 5G Core over non-3GPP Access Network to 5G Core Network and WLAN |
| 11.8.3 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from E-UTRAN/EPC to N3IWF/5GC | Rel-15 | C277 | UEs supporting 5G Core and handover from EPC Network to 5G Core over non-3GPP Access Network and WLAN |
| 11.8.4 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from N3IWF/5GC to E-UTRAN/EPC | Rel-15 | C249 | UEs supporting 5G Core and handover from 5G Core over non-3GPP Access Network to EPC Network and WLAN |
| 11.8.5 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from 5GS to EPC/ePDG | Rel-15 | C208 | UEs supporting 5G Core and IMS and handover from 5G Core to EPC over non-3GPP Access Network and GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" and WLAN. |
| 11.8.6 | Inter-system mobility between untrusted Non-3GPP and 3GPP system/Handover from EPC/ePDG to 5GS/ UE in 5GMM-DEREGISTERED and EMM-DEREGISTERED states | Rel-15 | C237 | UEs supporting WLAN and GSMA PRD IR.51: "IMS Profile for Voice, Video and SMS over Wi-Fi" and handover from EPC over non-3GPP Access Network to 5G Core and IMS and 5G Core. |

[End of change]