# Title:	Test tolerance methodology for FR1 inter-frequency event triggered reporting tests for concurrent gaps

# Source:	Mediatek

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for RRM tests 6.6.18.1 and 6.6.18.2.

The test cases in question, from TS 38.533, are:

- 6.6.18.1 NR SA FR1 event-triggered reporting for concurrent gaps non-overlap with SSB-based measurements in both inter-frequency layers

- 6.6.18.2 NR SA FR1 event-triggered reporting for concurrent gaps partially-overlap with SSB-based measurements in both inter-frequency layers

This document describes the process to derive the test tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. 	Test Tolerances Methodology for 6.6.18.1 in TS 38.133

The test conditions are defined in the following extract from TS 38.133 v18.2.0, with updates to Io values, time periods and Test Requirements shown in yellow highlight.

#### A.6.6.18.1	SA event triggered reporting tests for FR1 concurrent gaps with non-overalpping scenario for SSB-based measurements in both inter-frequency layers

##### A.6.6.18.1.1	Test Purpose and Environment

The purpose of this test is to verify that the concurrent gaps capable UE makes correct reporting of events. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4.

In this test, there are three cells: NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as neighbour cell in FR1 on NR RF channel 2, and NR cell 3 as neighbour cell in FR1 on NR RF channel 3.  The test parameters are given in Tables A.6.6.18.1.1-1, A.6.6.18.1.1-2 and A.6.6.18.1.1-3.

Two measurement gap patterns (MeasGapId #1 and MeasGapId #2) are configured with the gap pattern ID #0 and #1 as defined in Table A.6.6.18.1.1-2. MeasGapId #2 is configured with a higher priority than MeasGapId #1. MeasGapId #1 and MeasGapId #2 are associated with the MOs for RF channel numbers #2 and #3, respectively.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used for both frequency layers. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2 and NR cell 3.

Table A.6.6.18.1.1-1: SA event triggered reporting tests for FR1-FR1

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: Target NR cells have the same SCS, BW and duplex mode as NR serving cells |  |

Table A.6.6.18.1.1-2: General test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gaps with fully non-overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2, 3 |  | Three FR1 NR carrier frequencies are used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cells 2 and 3 |  | NR cell 2 is on NR RF channel number 2. NR cell 3 is on NR RF channel number 3. |
| Gap Pattern Id |  | Config 1,2,3 | 0 for MeasGapId #1 1 for MeasGapId #2 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3 | 39 for MeasGapId #1 19 for MeasGapId #2 |  |  |
| A3-Offset | dB | Config 1,2,3 | -6 |  |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |  |
| CP length |  | Config 1,2,3 | Normal |  |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |  |
| Filter coefficient |  | Config 1,2,3 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cell 1 |  | Config 1, 2, 3 | 3s |  | Synchronous. |
| Time offset between serving and neighbour cell 2 |  | Config 1, 2, 3 | 5ms |  | Asynchronous. The timing of Cell 3 is 5ms later than the timing of Cell 1. |
| T1 | s | Config 1,2,3 | 5 |  |  |
| T2 | s | Config 1,2,3 | 1.5 | 1.5 |  |

Table A.6.6.18.1.1-3: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 concurrent gaps with fully non-overalpping scenario for SSB-based measurements in both inter-frequency layers

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  | 2 |  | 3 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  | NA |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | NA |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | NA |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | NA |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  | NA |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  | NA |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  | NA |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | SR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1 | CCR.1.1 FDD |  |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |  |
| SSB parameters |  |  | Config 1,2,3 | SSB.1 FR1 |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1,2,3 | SMTC.2 |  | SMTC.2 |  | SMTC.2 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  | -98 |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98 |  | -98 |  | -98 |  |
|  |  |  | Config 3 | -95 |  | -95 |  | -95 |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94 | -94 | -Infinity | -91 | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 | -Infinity | -88 | -Infinity | -88 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
|  |  | dB | Config 1,2,3 | 4 | 4 | -Infinity | 7 | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59 | -64.59 | -70.05 | -62.26 | -70.05 | -62.26 |
|  |  | dBm/38.16MHz | Config 3 | -58.49 | -58.49 | -63.94 | -56.15 | -63.94 | -56.15 |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |

##### A.6.6.18.1.2	Test Requirements

The UE shall send one Event A3 triggered measurement report for each neighboring cell, with a measurement reporting delay less than 920 ms for cell 2 and 1280ms for cell 3 from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

# 3. 	Discussion

The test case consists of 3 test configurations. Each test configuration has 2 time periods: T1 and T2.

To analyse the test case, it is helpful to show the values of SS-RSRP for NR Cell 1, NR Cell 2, and NR Cell 3 versus time.

In the accompanying spreadsheet only configurations 1 and 3 are considered. For other configurations, critical test parameters are identical with test configuration 1 and 3, thus the same level uncertainties and method of deciding test tolerances can be used.

Observations

During T1 only NR Cell 1 is present and the nominal Ês/Iot value is +4 dB for all configurations 1, 2 and 3.

During T2, for all configurations 1, 2 and 3, the SS-RSRP of NR Cell 2 and NR Cell 3 are both 3dB higher than NR Cell 1. The “Event A3 (Neighbour becomes offset (-6dB) better than SpCell)” condition is easily met, but the effect of uncertainties on the test verdict needs to be checked. Then a decision can be made on whether Test Tolerances need to be applied.

A detailed analysis of the test case is given in section 5 of this document.

# 4. 	Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Table A.6.6.18.1.1-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

b) Derived parameters

Several derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In this test case at the start of T2, NR Cell 2 and NR Cell 3 become detectable, and the UE is expected to detect and send an event triggered measurement report. The criterion is Event A3: Neighbour becomes amount of offset better than SpCell. The UE makes a measurement of NR Cell 2 (neighbour cell 1) relative to NR Cell 1 (PSCell) and compares it to the signalled A3 offset. The UE makes a measurement of NR Cell 3 (neighbour cell 2) relative to NR Cell 1 (PSCell) and compares it to the signalled A3 offset. Further steps are done to calculate SS-RSRP power difference including the offset:

- (NR Cell 2 SS-RSRP – NR Cell 1 SS-RSRP) – Ofn

- (NR Cell 3 SS-RSRP – NR Cell 1 SS-RSRP) – Ofn

c) Uncertainties

The SS provides AWGN, 3 NR inter-frequency cells to the UE. We propose to control the following parameters:

- Frequency 1 AWGN Absolute power, Noc ±1.5 dB averaged over BWConfig

- Frequency 1 AWGN Absolute power, Noc ±1.5 dB for PRBs #RBJ-RBJ+19

- Frequency 2 AWGN Absolute power, Noc ±1.5 dB averaged over BWConfig

- Frequency 2 AWGN Absolute power, Noc ±1.5 dB for PRBs #RBJ-RBJ+19

- Frequency 3 AWGN Absolute power, Noc ±1.5 dB averaged over BWConfig

- Frequency 3 AWGN Absolute power, Noc ±1.5 dB for PRBs #RBJ-RBJ+19

- Ratio of NR cell 1 signal / AWGN, Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of NR cell 1 signal / AWGN, Ês1 / Noc ±0.3 dB for PRBs #RBJ-RBJ+19

- Ratio of NR cell 2 signal / AWGN, Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of NR cell 2 signal / AWGN, Ês1 / Noc ±0.3 dB for PRBs #RBJ-RBJ+19

- Ratio of NR cell 3 signal / AWGN, Ês2 / Noc ±0.3 dB averaged over BWConfig

- Ratio of NR cell 3 signal / AWGN, Ês1 / Noc ±0.3 dB for PRBs #RBJ-RBJ+19

In this test the UE measures the SS-RSRP over specific Physical Resource Block (PRB) numbers # RBJ to # RBJ+19. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

Firstly, the inter frequency cells need to be detectable so that the requirement of inter-frequency measurement can apply. The condition, that an inter-frequency cell is detectable, is defined in TS 38.133 9.3.2.

### 9.3.2	Requirements applicability

The requirements in clause 9.3 apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in clauses 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.9 and 10.1.10 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.14 and 10.1.15 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding Band.

Secondly, for an inter-frequency cell, the conditions of SSB_RP and SSB Es/Iot in section Annex B.2.3 need to be fulfilled.

## B.2.3	Conditions for NR inter-frequency measurements

This clause defines the following conditions for NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.3-1 for FR1 NR cells.

The conditions are defined in Table B.2.3-2 for FR2 NR cells.

Table B.2.3-1: Conditions for inter-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_G, NR_TDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

Thirdly, the UE makes the (NR Cell 2 SS-RSRP – NR Cell 1 SS-RSRP) and (NR Cell 3 SS-RSRP – NR Cell 1 SS-RSRP) comparison with the A3 offset, so the UE SS-RSRP relative measurement accuracy needs to be considered. In addition, according to TS 38.133 clause 10.1.1, inter-frequency requirements apply for non-serving cell measurements on NR carrier frequencies, so the UE SS-RSRP relative measurement accuracy between any two inter neighbour cells also need to be considered.

### 10.1.1	Introduction

The requirements in clause 10.1 apply as follows:

-	intra-frequency requirements apply for PCell measurements in SA, NR-DC, or NE-DC operaion mode,

-	intra-frequency requirements apply for PSCell measurements in NR-DC or EN-DC operaion mode,

-	intra-frequency requirements apply for SCell measurements in SA operation mode with NR CA or any MR-DC operaion mode with NR CA,

-	inter-frequency requirements apply for non-serving cell measurements on NR carrier frequencies.

-	inter-frequency requirements apply for measurements from one cell on a frequency compared to the measurement from another cell on a different frequency.

##### 10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1

The relative accuracy of SS-RSRP in inter frequency case is defined as the RSRP measured from one cell on a frequency in FR1compared to the RSRP measured from another cell on a different frequency in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasInterFreq-r17 is configured.

The accuracy requirements in Table 10.1.4.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

-	|SSB_RP1dBm - SSB_RP2dBm| ≤ 27 dB

-	|Channel 1_Io -Channel 2_Io |  20 dB

Table 10.1.4.1.2-1: SS-RSRP Inter frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 4.5 | 6 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

In summary the SSB Es/Iot of NR Cell 1, NR Cell 2 and NR Cell 3 shall be above -4dB, the tightest requirement for Io of NR RF channel shall be between -117.5dBm/15kHz (-89.5dBm/9.36MHz) or -114.5dBm/30kHz (-83.5dBm/38.16MHz) and -50dBm/BWchannel for band group NR_FDD_FR1_H, and the tightest requirement for SSB_RP shall be above -121.5dBm/15kHz or -118.5dBm/30kHz for band group NR_FDD_FR1_H.

d) Controlled parameters critical to verdict

In many RRM test cases, and particularly inter-frequency multi-cell tests, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. These may be obvious, such as the difference in SS-RSRP between NR Cell 2 and NR Cell 1 which the UE uses to compare with the A3 offset and trigger the reporting. Some are less obvious such as the Es/Iot side condition range over which the UE meets the specified relative measurement accuracy. The 17 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Comment” column of section g) in the spreadsheet.

In general, all the values are listed for each time interval T1 and T2 but as NR Cell 2 and NR Cell 3 are not present during T1 those entries are greyed out. Analysis is performed for configuration 1 (BW 10MHz, SCS 15kHz, 52 RBs) and configuration 3 (BW 40MHz, SCS 30kHz, 106 RBs), the shaded cells, in the spreadsheet, show what changes with configuration.

In general, all the values are listed for each time interval T1 and T2. NR Cell 1, NR Cell 2 and NR Cell 3 are on different frequencies and do not interfere.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x) and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute level Noc would cause 1dB error in the Io on NR RF Channel 1, so the sensitivity factor is 1.000 during T1 and T2. However, the same error of 1dB in the absolute level Noc would cause no change to NR Cell 2 and NR Cell 3 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

The overall cell power Io is an exception, as the effect of Es/Noc uncertainty is diluted depending on the linear fraction of cell power making up the total. The sensitivity factor is made equal to the linear fraction of cell power on that frequency.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment and the UE RSRP relative measurement accuracy where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1 and T2.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE SS-RSRP reporting accuracy are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE SS-RSRP reporting accuracy. If all the uncertainties were combined root-sum square, the resulting smaller test limits could cause a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

During T1 the UE is required to select NR Cell 1 as PSCell. The uncertainties do not take NR Cell 1 Ês/Iot or SSB_RP power outside the allowed range, therefore no offsets are required.

During T2 the NR Cell 2 and NR Cell 3 becomes detective. UE is expected to perform inter-frequency measurement. The nominal conditions in TS 38.133 Table A.6.6.18.1.1-3 for NR Cell 2 and NR Cell 3 are the same, and give SS-RSRP values, as measured by the UE, of -94 dBm/15kHz (-91dBm/30kHz) and -91 dBm/15kHz (-88dBm/30kHz), which are 3 dB apart nominally.

The Event A3 measurement report trigger requirement in TS 38.331 clause 5.5.4.4 is “Event A3 (Neighbour becomes offset better than SpCell), inequality A3-1 (Entering condition) is: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off”.

The values in this equation are derived as follows:

- Mn is evaluated for neighbouring NR Cell 2 or NR Cell 3 at T2 and is nominally -91dBm/15kHz (-88dBm/30kHz)

- Ofn is the measurement object specific offset offsetMO of the reference signal of the neighbour cell. Its default value is specified as 0dB in TS 38.508-1 Table 4.6.3-76.

- Ocn is the cell specific offset cellIndividualOffset of the neighbour cell and set to zero if not configured for the neighbour cell.

- Hys is the hysteresis parameter for this event, specified as 0dB in TS 38.133 Table A.6.6.18.1.1-2.

- Mp is evaluated for SpCell NR Cell 1 at T2 and is nominally -94dBm/15kHz (-91dBM/30kHz)

- Ofp is the measurement object specific offset offsetMO of the SpCell. Its default value is specified as 0dB in TS 38.508-1 Table 4.6.3-76.

- Ocp is the cell specific offset cellIndividualOffset of the serving cell and set to zero if not configured for the neighbour cell.

- Off is the offset parameter for this event (i.e. A3-Offset), specified as -6dB in TS 38.133 Table A.6.6.18.1.1-2.

The equation therefore reduces to:

Mn > Mp + Off. Putting in values and rearranging gives -94dBm - (-91)dBm > -6dB for configuration 1/2 and -91dBm - (-88)dBm > -6dB for configuration 3.

This inequality is satisfied by 3dB under nominal conditions. The overall uncertainty for UE Measured (Cell 2 RSRP - Cell 1 RSRP) and (Cell 3 RSRP- Cell 1RSRP), taking into account both test system and UE measurement uncertainties, is ±6.66dB as calculated in step d) of the accompanying spreadsheet. The inequality is therefore also satisfied when uncertainties are included. No offsets are therefore needed for this requirement.

In addition, during T2 the Es/Iot, SSB_RP of all NR Cells and the Io on NR RF channel 1/2/3 are fulfilling the required side conditions taking into account the uncertainties, and therefore no offsets are required.

The check that all other controlled parameters meet their required range is done in step g). In theory it is possible for steps e) to g) to be iterative, or possibly even steps c) to g) to be iterative, but this test case can be done without such iteration.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b). In general, all the values are listed for each time interval T1 and T2 but as NR Cell 2 and NR Cell 3 is not present during T1 those entries are greyed out (the formulae are left in place for use by future test cases).

g) Check controlled parameters Min/Max

Using a format like that in step d), the nominal value of each controlled parameter is recalculated.

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (SS-RSRP difference, Es/Iot, SSB_RP and Io). It is not necessary to calculate all parameters during each time interval T1 and T2, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

With the uncertainty values and test tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 6.6.18.1 also contains configurations with different subcarrier spacings and different channel bandwidths. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- RBs in Channel bandwidth, section a) and associated comment

- Cell 1, Cell 2 and Cell 3 SSB_RP minimum requirement, section g) and associated Source/Reference

- Io minimum requirement, section g) and associated Source/Reference

# 6. Treatment of test cases 6.6.18.2

TS 38.533 also contains a group of inter-frequency event triggered reporting test cases. Since the test parameters critical for inter-frequency measurements are identical with that of 6.6.18.1, the same level uncertainties and method of deciding Test Tolerances can be used.