3GPP TSG-RAN5 Meeting #100	R5-233883

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2682 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 13.4.1.1 for Transmit Timing Accuracy test in normal coverage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For NB-IOT NTN, UE Transmit Timing Accuracy new test case 13.4.1.1 is missing in TS 36.521-3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case 13.4.1.1 is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.1.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-2 CR 1009 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

## 13.4	Timing and signalling characteristics for satellite access

### 13.4.1	UE transmit timing for satellite access

#### 13.4.1.1	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- Connection diagram with GNSS simulator is missing in TS36.508.

- GNSS simulator configuration is missing.

- Minimum confiormance requirements and test requirement contain [],

##### 13.4.1.1.1	Test purpose

The purpose of this test is to verify that the Category NB1 UE under normal coverage is capable of following the frame timing change of the connected eNodeB and that the UE initial transmits timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in TS 36.133 [4] clause 7.20A.

##### 13.4.1.1.2	Test applicability

This test applies to all types of NB-IoT HD-FDD UE release 17 and forward of UE Category NB1 that supports NTN.

##### 13.4.1.1.3	Minimum conformance requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in 3GPP TS 36.133 [4] clause 7.20A and shown in Table 13.4.1.1.3-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR. The reference point for the UE initial transmit timing control requirement shall be the downlink timing of the serving NB-IoT cell minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts. The downlink timing is defined as the time when the first detected path (in time) of the corresponding downlink frame is received from the serving NB-IoT cell. NTA_Ref for NPRACH is defined as 0. NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific (in Ts units) for other channels is the difference between UE transmission timing and the Downlink timing immediately after when the last timing advance in 3GPP TS 36.133 clause 7.22A was applied. NTA_Ref for other channels is not changed until next timing advance is received.

Table 13.4.1.1.3-1: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | [97]*TS |
| Note 1: TS is the basic timing unit defined in 3GPP TS 36.211 [9] |  |

When it is not the first transmission in a DRX cycle or there is no DRX cycle, and when it is the transmission for NPUSCH the UE shall, when no repetitions are configured on the uplink or the repetition period is R=1, be capable of changing the transmission timing according to the received downlink frame of the serving NB-IoT cell except when the timing advance in 3GPP TS 36.133 clause 7.22A is applied such that the UE transmission timing error shall be less than or equal to ±Te, where the timing error limit value Te is specified in table 13.4.1.1.3-1.

When no repetition period is configured, or the configured repetition period is R=1, all adjustments made to the UE uplink timing shall follow these rules:

1)	The maximum amount of the magnitude of the timing change, apart from a change of NTA, UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, in one adjustment shall be [58.33]*TS seconds.

2)	The minimum aggregate adjustment rate, apart from a change of NTA, UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be [7]*TS per [1]second.

3)	The maximum aggregate adjustment rate, apart from a change of NTA, UE-specific due to satellite position update and NTA,common between the previous transmission and the current transmission, shall be [58.33]*TS per [200]ms.

when a repetition is configured on the uplink for which R>1, the UE shall not adjust the uplink transmission timing autonomously during an ongoing repetition period other than at initial transmission or at the start of a transmission segment boundary, as defined above.

The normative reference for this requirement is 3GPP TS 36.133 [4] clause 7.20A and A.13.4.1.1.

##### 13.4.1.1.4	Test description

There is one NB-IoT SAN cell (Ncell 1), which is the active cell in the test. The transmit timing is verified by the UE transmitting NPUSCH.

Time alignment timers shall be set to “infinity” so that UL timing alignment is maintained during the test.

During the test, the GNSS System Simulator shall emulate and send the GNSS signal to the test UE. The UE shall be provided with the valid information about the SAN serving cells before the test.

13.4.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 13.4.1.1.4.1-1.

Table 13.4.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | GSO, HD-FDD duplex mode |
| 2 | NGSO, HD-FDD duplex mode |
| Note: If UE supports both NGSO and GSO, the test case Config 1 can be skipped if the UE passes test case Config 2. |  |

Test Environment: Normal, as defined in 3GPP TS 36.508 [7] clause 8.1.1.

Frequencies to be tested: According to Annex E table Annex E-4 and 3GPP TS 36.508 [7] clauses 8.1.3 and 8.1.4.2.

Channel Bandwidth to be tested: 200 KHz as defined in 3GPP TS 36.508 [7] clause 8.1.3.1.

1.	Connection diagram with GNSS simulator is TBD.

2.	The general test parameter settings are set according to Table 13.4.1.1.4.1-2.

3.	Propagation conditions are set according to Annex B clause B.0.

4.	Message contents are defined in clause 13.4.1.1.4.3.

5.	There is one cell specified in this test. Ncell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

Table 13.4.1.1.4.1-2: General Test Parameters for E-UTRAN HD-FDD UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| NB-IoT Operation mode |  |  | Standalone |
| DRX |  |  | OFF |
| Satellite information | Config 1 |  | SCC.1 |
|  | Config 2 |  | SCC.2 |
| NPRACH configuration |  |  | NPRACH.R-1 As specified in A.10.3-1 |
| NPDCCH repetition level |  |  | 1 |
| npdcch-StartSF-USS Note 1 |  |  | v8 |
| npdcch-NumRepetitions-r13 Note 1 |  |  | r1 |
| NPUSCH repetition level |  |  | 1 |
| Note 1: For further information see clause 6.7.3.2 in TS 36.331 [5]. |  |  |  |

13.4.1.1.4.2	Test procedure

The test consists of a single cell Ncell 1, the UE shall be fully synchronized to Ncell 1. The transmit timing accuracy is verified related to the downlink frame timing of Ncell 1. The downlink timing of Ncell 1 is changed and the changes in UE transmit timing are observed. The transmit timing is verified by the UE transmitting NPUSCH used as a measurement reference facilitating the SS timing estimation.

1.	Ensure the UE is in State 2A-NB with CP CIoT Optimisation according to 3GPP TS 36.508 [7] clause 8.1.5.

2.	Set the parameters according to Table 13.4.1.1.5-1. Propagation conditions are set according to Annex B clause B.1.

3.  SS sends uplink scheduling information via NPDCCH DCI format N0 for C_RNTI to schedule uplink subframes.

4.	The UE shall transmit NPUSCH, and the SS shall measure the UE transmit timing offset (nTs).

5.	Using the value of n measured in step 4, the SS calculates the adjustment of the downlink transmit timing for the Ncell 1:

-	if n < 0, by +(144 - |n|)TS compared to that in step 4.

-	 if n ≥ 0, by -(144 - |n|)TS compared to that in step 4.

6.	Before applying the downlink transmit timing adjustment, the SS sends NPDCCH including uplink grant, measures the timing of the resulting NPUSCH, and verifies that it is within the limits specified in Table 13.4.1.1.5-2 with respect to the first detected path (in time) of the corresponding downlink frame of NB-IoT cell 1. The SS also records the measured value for use in step 8.

7.	Immediately after Step 6, the downlink timing adjustment is performed monotonically in multiple steps of |∆T| ≤ 9×TS per 256ms until the above required total timing change is achieved, during which no grant is transmitted for the UE.

8.	Immediately after Step 7, the SS sends NPDCCH including uplink grant for NPUSCH transmission and immediately after receiving NPUSCH the test system repeatedly sends NPDCCH including uplink grant for NPUSCH transmission. The SS samples the UE Transmit Timing once per NPUSCH transmission. The SS shall check that the maximum time adjustment step size between the first NPUSCH transmission in step 8 and the NPUSCH transmission in step 6 is within Rule 1 as specified in clause 13.4.1.1.5. And using the first NPUSCH transmission in step 8 and subsequent NPUSCH transmissions, the SS shall check the difference in timing between one NPUSCH transmission to the next is within Rule 1 as specified in clause 13.4.1.1.5. To check that the minimum adjustment rate is within Rule 2 as specified in clause 13.4.1.1.5, the SS shall measure the change in NPUSCH transmission timing over a 1008ms sliding window, with step size 8ms. To check that the maximum adjustment rate is within Rule 3 as specified in clause 13.4.1.1.5, the SS shall measure the change in NPUSCH transmission timing over a 192ms sliding window, with step size 8ms. The three rules apply until the UE transmit timing offset is within the limits specified in Table 13.4.1.1.5-2 with respect to the first detected path (in time) of the corresponding downlink frame of Ncell 1.

9.	The SS shall check that the UE transmit timing offset stays within the limits specified in Table 13.4.1.1.5-2 with respect to the first detected path (in time) of the corresponding downlink frame of Ncell 1.

13.4.1.1.4.3	Message contents

Message contents are according to 3GPP TS 36.508 [7] clause 8.1.6 and clause 8.1.4.3 using condition "Standalone" and “NTN” with the following exceptions:

Table 13.4.1.1.4.3-1: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 3GPP TS 36.508 [7] clause 8.1.6.3, Table 8.1.6.3-3: NPDCCH-ConfigDedicated-NB-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-r13 | r1 | Set NPDCCH repetition to 1 |  |
| npdcch-StartSF-USS-r13 | V8 |  |  |
| npdcch-Offset-USS-r13 | zero |  |  |
| } |  |  |  |

##### 13.4.1.1.5	Test requirement

Table 13.4.1.1.5-1: Cell specific Test Parameters for E-UTRAN HD-FDD UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| RF Channel Number |  | 1 |
| BWchannel | kHz | 200 |
| NPDSCH parameter |  | R.18 HD-FDD |
| NPDCCH parameter |  | R.30 HD-FDD |
| NOCNG Patterns |  | NOP.3 FDD |
| NPBCH_RA | dB | -3 |
| NPBCH_RB |  |  |
| NPSS_RA |  |  |
| NSSS_RA |  |  |
| NPDCCH_RA |  |  |
| NPDCCH_RB |  |  |
| NPDSCH_RA |  |  |
| NPDSCH_RB |  |  |
| NOCNG_RA Note1 |  |  |
| NOCNG_RB Note1 |  |  |
|  | dBm/15 kHz | -88 |
|  | dB | 4 |
|  | dB | 4+TT |
| Antenna Configuration |  | 1x1 |
| Propagation condition | - | AWGN |
| Note 1 NOCNG shall be used such that the cell is fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

The UE transmit timing offset shall be within the requirements in Table 13.4.1.1.5-2.

Table 13.4.1.1.5-2: Te Timing Error Limit

| Downlink Bandwidth (MHz) | Te_ |
| --- | --- |
| 0.18 | ([97]+TT)*TS |
| NOTE 1: TS is the basic timing unit defined in 3GPP TS 36.211 [9] |  |

The reference point for the UE initial transmit timing control test requirement shall be the downlink timing minus NTA_Ref+NTAoffset+NTA,common+NTA, UE-specific×Ts.

The UE shall be capable of changing the transmission timing according to the received downlink frame. When the transmission timing error between the UE and the reference cell exceeds Te the UE is required to adjust its timing to within Te.

All adjustments made to the UE uplink timing shall follow three rules:

1)	The maximum amount of the magnitude of the timing change in one adjustment shall be ([58.33]+TT)*TS seconds.

2)	The minimum aggregate adjustment rate shall be ([7]+TT)*TS per [1]second.

3)	The maximum aggregate adjustment rate shall be ([58.33]+TT)*TS per [200]ms.

<< End of changes >>