GPP TSG-RAN5 Meeting #100	R5-233890

# Toulouse, France, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0485 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for NTN test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM JAPAN LLC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current version of the spec is missing PICS for NTN |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added PICS for NTN related UE capability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the NTN WP will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.6, A.4.3.7, A.4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Beginning of changes>

### A.4.3.6	Measurement Capabilities

Table A.4.3.6-1: UE Measurement Capabilities

| Item |  |  |  | UE Measurement Capabilities |  |  |  | Ref. |  |  |  | Release |  |  |  | Mnemonic |  |  |  | M |  |  |  | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release |  |  |  | Comments |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | Support NR measurements and events A triggered reporting |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_eventA_MeasAndReport |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  | Support two independent measurement gap configurations for FR1 and FR2 |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_independentGapConfig |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  | Support NR intra-frequency and inter-frequency measurements and at least periodical reporting |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_intraAndInterF_MeasAndReport |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  | Support CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [21], where CSI-RS resource is configured with an associated SS/PBCH |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  | Support inter-RAT E-UTRA measurements and events B triggered reporting |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_eventB_MeasAndReport |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  | Support SS-SINR measurents |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_ss_SINR_Meas |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_eutra_CGI_Reporting |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_nr_CGI_Reporting |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is configured. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_nr_CGI_Reporting_ENDC |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  | Support shorter measurement gap length (i.e. gp2 and gp3) for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC. |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp2_gp3_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  | Support NR supports gap pattern 4 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp4_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  | Support NR supports gap pattern 5 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp5_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  | Support NR supports gap pattern 6 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp6_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  | Support NR supports gap pattern 7 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp7_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  | Support NR supports gap pattern 8 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp8_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  | Support NR supports gap pattern 9 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp9_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  | Support NR supports gap pattern 10 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp10_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  | Support NR supports gap pattern 11 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  |  |  | 36.331, 6.3.6 |  |  |  | Rel-15 |  |  |  | pc_gp11_en_dc |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  | Support measurement gap pattern 2 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp2_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  | Support measurement gap pattern 3 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp3_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  | Support measurement gap pattern 4 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp4_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  | Support measurement gap pattern 5 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp5_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  | Support measurement gap pattern 6 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp6_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  | Support measurement gap pattern 7 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp7_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  |  | Support measurement gap pattern 8 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp8_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 26 |  |  |  | Support measurement gap pattern 9 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp9_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  | Support measurement gap pattern 10 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp10_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  |  |  | Support measurement gap pattern 11 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp11_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Support measurement gap pattern 12 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp12_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  | Support measurement gap pattern 15 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp15_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  |  |  | Support measurement gap pattern 16 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp16_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 32 |  |  |  | Support measurement gap pattern 17 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp17_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 34 |  |  |  | Support measurement gap pattern 18 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp18_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 35 |  |  |  | Support measurement gap pattern 19 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp19_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  | Support measurement gap pattern 20 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp20_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 37 |  |  |  | Support measurement gap pattern 21 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp21_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 38 |  |  |  | Support measurement gap pattern 22 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp22_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 39 |  |  |  | Support measurement gap pattern 23 configured by NR RRC. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_gp23_nr |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 40 |  |  |  | Support CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [21], where CSI-RS resource is configured without an associated SS/PBCH |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 41 |  |  |  | Support CSI-RS based Radio Link Monitoring for FR1 |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_CSI_RS_RLM_FR1 |  |  |  | Yes |  |  |  |  |  |  |  | If the UE supports this feature, the UE needs to report maxNumberResource-CSI-RS-RLM in its capability report. If the UE doesn’t support CSI-RS based RLM, it will not include this IE in its capability report. |  |  |  |
| 41a |  |  |  | Support CSI-RS based Radio Link Monitoring for FR2 |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_CSI_RS_RLM_FR2 |  |  |  | Yes |  |  |  |  |  |  |  | If the UE supports this feature, the UE needs to report maxNumberResource-CSI-RS-RLM in its capability report. If the UE doesn’t support CSI-RS based RLM, it will not include this IE in its capability report. |  |  |  |
| 42 |  |  |  | Support of E-UTRA RS-SINR measurements |  |  |  | 38.306, 4.2.10 |  |  |  | Rel-15 |  |  |  | pc_RS_SINR_MeasEUTRA |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 43 |  |  |  | Support of SFTD measurements between a E-UTRA PCell and an NR PSCell in FDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasPSCell_MRDC_FDD |  |  |  | No |  |  |  |  |  |  |  | The SFTD measurement support should be indicated in MRDC capabilities for EN-DC. The support needs to be declared for FDD and TDD separately |  |  |  |
| 44 |  |  |  | Support of SFTD measurements between a E-UTRA PCell and an NR PSCell in TDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasPSCell_MRDC_TDD |  |  |  | No |  |  |  |  |  |  |  | The SFTD measurement support should be indicated in MRDC capabilities for EN-DC. The support needs to be declared for FDD and TDD separately |  |  |  |
| 45 |  |  |  | Support of relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE |  |  |  | 38.306, 5.6 |  |  |  | Rel-16 |  |  |  | pc_Relaxed_Measurement |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 46 |  |  |  | Support of SFTD measurements between a E-UTRA PCell and an NR neighbour cell in FDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasNR_Cell_FDD |  |  |  | No |  |  |  |  |  |  |  | The support needs to be declared for FDD and TDD separately The SFTD measurement support can only be indicated in MRDC capabilities for EN-DC |  |  |  |
| 47 |  |  |  | Support of SFTD measurements between a E-UTRA PCell and an NR neighbour cell in TDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasNR_Cell_TDD |  |  |  | No |  |  |  |  |  |  |  | The support needs to be declared for FDD and TDD separately The SFTD measurement support can only be indicated in MRDC capabilities for EN-DC |  |  |  |
| 48 |  |  |  | Support of SFTD measurements between a NR PCell and an NR neighbour cell in FDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasNR_Neigh_FDD |  |  |  | No |  |  |  |  |  |  |  | The support needs to be declared for FDD and TDD separately |  |  |  |
| 49 |  |  |  | Support of SFTD measurements between a NR PCell and an NR neighbour cell in TDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasNR_Neigh_TDD |  |  |  | No |  |  |  |  |  |  |  | The support needs to be declared for FDD and TDD separately |  |  |  |
| 50 |  |  |  | Support of SFTD measurements between a NR PCell and an NR PSCell in FDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasPSCell_NRDC_FDD |  |  |  | No |  |  |  |  |  |  |  | The SFTD measurement support should be indicated in UE-NR-Capability |  |  |  |
| 51 |  |  |  | Support of SFTD measurements between a NR PCell and an NR PSCell in TDD |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasPSCell_NRDC_TDD |  |  |  | No |  |  |  |  |  |  |  | The SFTD measurement support should be indicated in UE-NR-Capability |  |  |  |
| 52 |  |  |  | Support of acquisition of CGI related information from a neighbouring intra-frequency or inter-frequency NPN CAG cell |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-16 |  |  |  | pc_nr_CGI_Reporting_NPN_r16 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 53 |  |  |  | Supports periodic EUTRA measurement and reporting. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_periodicEUTRA_MeasAndReport |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 54 |  |  |  | Support configuration of NR SSB measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request as specified in TS 38.331 [9] |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-16 |  |  |  | pc_idleInactiveNR_MeasReport |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 55 |  |  |  | Support configuration of E-UTRA measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request as specified in TS 38.331 [9] |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-16 |  |  |  | pc_idleInactiveEUTRA_MeasReport |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  | Support SRS-RSRP measurements between a NR Pcell and an interfering UE, upon network request as specified in 38.331 [9] |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-16 |  |  |  | pc_nr_CLI_Reporting_r16 |  |  |  | No |  |  |  |  |  |  |  | If the UE supports this feature, the UE needs to report maxNumberCLI-SRS-RSRP-r16 and maxNumberPerSlotCLI-SRS-RSRP-r16 If the UE doesn’t support CLI SRS-RSRP measurement, it will not include this IE in its capability report. |  |  |  |
| 56A |  |  |  | Support SRS-RSRP measurements and periodical reporting and measurement event triggering based on SRS-RSRP |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-16 |  |  |  | pc_cli_SRS_RSRP_Meas_r16 |  |  |  | No |  |  |  |  |  |  |  | If the UE supports this feature, the UE needs to report maxNumberCLI-SRS-RSRP-r16 and maxNumberPerSlotCLI-SRS-RSRP-r16 |  |  |  |
| 56B |  |  |  | Support CLI RSSI measurements and periodical reporting and measurement event triggering |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-16 |  |  |  | pc_cli_RSSI_Meas_r16 |  |  |  | No |  |  |  |  |  |  |  | If the UE supports this feature, the UE needs to report maxNumberCLI-RSSI-r16 |  |  |  |
| 57 |  |  |  | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NE-DC is configured. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_eutra_CGI_Reporting_NEDC |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 58 |  |  |  | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_eutra_CGI_Reporting_NRDC |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 59 |  |  |  | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NE-DC is configured. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_nr_CGI_Reporting_NEDC |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 60 |  |  |  | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_nr_CGI_Reporting_NRDC |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 61 |  |  |  | Supports performing eNB-configured SSB-based RRM measurements for EN-DC configured NR FR1 carrier(s) in RRC_IDLE and reporting them when requested by the eNB while resuming from RRC_IDLE or in RRC_CONNECTED, as specified in TS 36.331 [5]. |  |  |  | 36.306, 4.3.6.41 |  |  |  | Rel-16 |  |  |  | pc_nrIdleInactiveNRFR1_MeasReport |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 62 |  |  |  | Supports performing eNB-configured SSB-based RRM measurements for EN-DC configured NR FR2 carrier(s) in RRC_IDLE and reporting them when requested by the eNB while resuming from RRC_IDLE or in RRC_CONNECTED, as specified in TS 36.331 [5]. |  |  |  | 36.306, 4.3.6.42 |  |  |  | Rel-16 |  |  |  | pc_nrIdleInactiveNRFR2_MeasReport |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 63 |  |  |  | Support more than 1 per-UE measurement gap configurations. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_concurrentPerUE_OnlyMeasGap_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 64 |  |  |  | Support all concurrent gap combination configurations as specified in TS 38.133 [5] including support of more than 1 per-UE measurement gap configurations. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_concurrentPerUE_PerFRCombMeasGap_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 65 |  |  |  | Support the configurations of E-UTRAN measurement objectives associated with more than 1 concurrent measurement gaps. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_concurrentMeasGapEUTRA_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 66 |  |  |  | Support reporting of the NCSG and measurement gap requirement information for E-UTRA target bands in the UE response to a network configuration RRC message as specified in TS 38.331 [9]. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_eutra_NeedForGapNCSG_Reporting_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 67 |  |  |  | Support two independent measurement gap configurations for FR1 and FR2 for PRS measurement. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_independentGapConfigPRS_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 68 |  |  |  | Support NR-only NCSG patterns. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_ncsg_MeasGapNR_Patterns_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 69 |  |  |  | Support NCSG patterns. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_ncsg_MeasGapPatterns_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 70 |  |  |  | Support per-FR NCSG. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_ncsg_MeasGapPerFR_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 71 |  |  |  | Support performing measurement with NCSG based on flag deriveSSB-IndexFromCell-inter and meeting the following requirements that the scheduling restriction in FR2 serving cell during NCSG ML is on SSB symbol level. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_ncsg_SymbolLevelScheduleRestrictionInter_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 72 |  |  |  | Support reporting of the NCSG and measurement gap requirement information for SSB based measurement in the UE response to a network configuration RRC message. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_nr_NeedForGapNCSG_Reporting_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 73 |  |  |  | Support the preconfigured measurement gap with UE-autonomous mechanism for activation and deactivation. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_preconfiguredUE_AutonomousMeasGap_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 74 |  |  |  | Support the preconfigured measurement gap with network-controlled mechanism for activation and deactivation. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_preconfiguredNW_ControlledMeasGap_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 75 |  |  |  | Support of SFTD measurements between the NR PCell and a configured E-UTRA PSCell |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-15 |  |  |  | pc_SFTD_MeasPSCell_NEDC |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| xx |  |  |  | Support of location-based triggered measurement reporting (i.e., event D1) as specified in TS 38.331 [9]. |  |  |  | 38.306, 4.2.9 |  |  |  | Rel-17 |  |  |  | pc_eventD1_MeasReportTrigger_r17 |  |  |  | CY |  |  |  |  |  |  |  | If UE supports this feature then UE needs to support locationBasedCondHandover-r17 in any NTN band. |  |  |  |

<Unchanged Sections Omitted >

### A.4.3.7	General Capabilities

Table A.4.3.7-1: UE General Capabilities

| Item |  |  |  | UE General Capabilities |  |  |  | Ref. |  |  |  | Release |  |  |  | Mnemonic |  |  |  | M |  |  |  | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release |  |  |  | Comments |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | Support UL transmission via either MCG path or SCG path for the split SRB as specified in TS 37.340[20] |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-15 |  |  |  | pc_splitSRB_WithOneUL_Path |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  | Support UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340[20] |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-15 |  |  |  | pc_splitDRB_withUL_Both_MCG_SCG |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  | Support direct SRB between the SN and the UE as specified in TS 37.340[20] |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-15 |  |  |  | pc_srb3 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  | Support of reflective QoS |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-15 |  |  |  | pc_as_ReflectiveQoS |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  | Support of NAS reflective QoS |  |  |  | 24.501, 6.2.5.1.4.1, 9.11.4.1 |  |  |  | Rel-15 |  |  |  | pc_nas_ReflectiveQoS |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  | Support of SMS over NAS |  |  |  | 24.501, 5.5.1.2 |  |  |  | Rel-15 |  |  |  | pc_sms_over_NAS |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  | Support of CMAS message on NR |  |  |  | 38.331, 5.2.2.2.2 |  |  |  | Rel-15 |  |  |  | pc_CMAS_NR |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  | Support of ETWS message on NR |  |  |  | 38.331, 5.2.2.2.2 |  |  |  | Rel-15 |  |  |  | pc_ETWS_NR |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  | The UE supports additional UE-requested PDU establishment |  |  |  | 24.501, 6.4.1.5 |  |  |  | Rel-15 |  |  |  | pc_Additional_PDU_establishment |  |  |  | No |  |  |  |  |  |  |  | pc_ExpectedNoOfPDUSessionsAtRegistration +1 |  |  |  |
| 10 |  |  |  | The UE includes the SM PDU DN request container IE in the PDU SESSION ESTABLISHMENT REQUEST message |  |  |  | 24.501, 6.4.1.2 |  |  |  | Rel-15 |  |  |  | pc_SM_PDU_DN_RequestContainer |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  | Support of emergency services fallback in NR connected to 5GCN |  |  |  | 24.501 |  |  |  | Rel-15 |  |  |  | pc_NR_5GC_EmergencyService_fallback |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  | Support of EPS fallback |  |  |  | 24.501, |  |  |  | Rel-15 |  |  |  | pc_EPS_fallback |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  | Support of UE requested PDU session modification |  |  |  | 24.501, 6.4.2.2 |  |  |  | Rel-15 |  |  |  | pc_MO_PDU_Session_Modification |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  | Support of emergency services in NR connected to 5GCN |  |  |  | 24.501 |  |  |  | Rel-15 |  |  |  | pc_NR_5GC_EmergencyServices |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  |  |  | Support of voiceFallbackIndication |  |  |  | 38.306, 4.2.13 |  |  |  | Rel-16 |  |  |  | pc_voiceFallbackIndication |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  | Support provision of referenceTimeInfo |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-16 |  |  |  | pc_referenceTimeProvision_r16 |  |  |  | No |  |  |  |  |  |  |  | specifically for TSC (time sensitive communication) services |  |  |  |
| 17 |  |  |  | Support of RACS |  |  |  | 24.501, 9.11.3.1 |  |  |  | Rel-16 |  |  |  | pc_5GC_RACS |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  | Support of RRC message Segmentation in the UL |  |  |  | 38.306, 5.4 |  |  |  | Rel-16 |  |  |  | pc_NR_UL_Segmentation |  |  |  | No |  |  |  |  |  |  |  | UE supports segmenation of UECapabilityInformation message, IF size > maximum supported size of a PDCP SDU |  |  |  |
| 19 |  |  |  | Support of RRC_INACTIVE as specified in TS 38.331 [9]. |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-15 |  |  |  | pc_inactiveState |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  | Support of UE local release when the security check is successful but SOR Transparent container indicates ACK has been NOT requested |  |  |  | 23.122 clause C.2 |  |  |  | Rel-15 |  |  |  | pc_SOR_ACKNotReqLocalRel |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  | Support of RRC connection release with deprioritisation |  |  |  | 38.306, 5.3 |  |  |  | Rel-15 |  |  |  | pc_NR_RRC_Release_With_Deprioritisation |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  | Support of RRC connection establishment failure with temporary offset |  |  |  | 38.306, 5.3 |  |  |  | Rel-15 |  |  |  | pc_NR_RRC_ConEstFail_With_TempOffset |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  | Support of Closed Access Group |  |  |  | 24.501, 9.11.3.1 |  |  |  | Rel-16 |  |  |  | pc_CAG |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  | Support of Stand-alone Non-Public Network |  |  |  | 23.501, 3.1 |  |  |  | Rel-16 |  |  |  | pc_SNPN |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  |  | Support of test function SET UL MESSAGE for using a preconfigured UE capability container over NR |  |  |  | 38.509, 5.9 |  |  |  | Rel-16 |  |  |  | pc_Set_UE_Cap_Info_NR |  |  |  | No |  |  |  |  |  |  |  | This test function is mandatory for UEs supporting UL RRC segmentation and whose maximum UECapabilityInformation message size is less than the allowed maximum supported size of a PDCP SDU. |  |  |  |
| 26 |  |  |  | Support of network slice-specific authentication and authorization |  |  |  | 24.501, 9.11.3.1 |  |  |  | Rel-16 |  |  |  | pc_5GC_NSSAA |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  | Support of EAP-AKA’ as EAP method for network slice-specific authentication and authorization |  |  |  | 24.501, 5.4.7 |  |  |  | Rel-16 |  |  |  | pc_5GC_NSSAA_EAP_AKA_Prime |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  |  |  | Support reduced control plane latency as defined in TS 38.331 [9] |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-15 |  |  |  | pc_reducedCP_Latency |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Support of release preference assistance information |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-16 |  |  |  | pc_releasePreference_r16 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 30 |  |  |  | Support of user initiated SNPN reselection in automatic mode |  |  |  | 23.122 |  |  |  | Rel-16 |  |  |  | pc_UserInitiated_SNPN_Reselection |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  |  |  | Support of autonomous search function to detect CAG cells on serving and non-serving frequencies |  |  |  | 38.304, 5.2.4.10 |  |  |  | Rel-16 |  |  |  | pc_Autonomous_search_function_nr_CAG |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 32 |  |  |  | Support IMS voice over NR |  |  |  | 38.306, 4.2.13 |  |  |  | Rel-15 |  |  |  | pc_voiceOverNR |  |  |  | No |  |  |  |  |  |  |  | A UE supporting IMS voice over NR shall support: - IMS emergency call over NR, and - IMS voice over E-UTRA/EPC if it supports E-UTRA/EPC. |  |  |  |
| 33 |  |  |  | Support of V2X communication |  |  |  | 24.501, 9.11.3.1 |  |  |  | Rel-16 |  |  |  | pc_V2X |  |  |  | No |  |  |  |  |  |  |  | UE support V2X communication over NR-Uu and/or NR-PC5. |  |  |  |
| 34 |  |  |  | Support of V2X communication over NR-PC5 |  |  |  | 24.501, 9.11.3.1 |  |  |  | Rel-16 |  |  |  | pc_V2XCNPC5 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 35 |  |  |  | Support of Manufacturer assigned Radio Capability ID |  |  |  | 23.501, 5.9.10 |  |  |  | Rel-16 |  |  |  | pc_5GC_RACS_Manufacturer_URCID |  |  |  | No |  |  |  |  |  |  |  | UE support of Manufacturer assigned radio capability ID |  |  |  |
| 36 |  |  |  | Support of 3GPP PS data off |  |  |  | 24.501, 6.2.10 |  |  |  | Rel-15 |  |  |  | pc_PS_data_off |  |  |  | No |  |  |  |  |  |  |  | UE support of 3GPP PS data off |  |  |  |
| 37 |  |  |  | Support of Network Slice Simultaneous Registration Group |  |  |  | 24.501, 9.11.3.82 |  |  |  | Rel-17 |  |  |  | pc_5GC_NSSRG |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 38 |  |  |  | Support of slice reselection information in SIB and on RRC release for slice based cell reselection in RRC _IDLE and RRC INACTIVE |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-17 |  |  |  | pc_sliceInfoforCellReselection_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 39 |  |  |  | Support of reception of segmented DL RRC messages |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-16 |  |  |  | pc_NR_dl_DedicatedMessageSegmentation |  |  |  | No |  |  |  |  |  |  |  | The SS initiates the DL Dedicated Message Segment transfer procedure IF the encoded RRCReconfiguration or RRCResume message PDU size > maximum PDCP SDU size. |  |  |  |
| 40 |  |  |  | Support of unified access control configuration in the list of subscriber data, indicating for which access identities (see 3GPP TS 24.501 [64]) the ME is configured, when the MS accesses an SNPN |  |  |  | 23.122, 4.9.3.0, 24.501 4.5.2A |  |  |  | Rel-16 |  |  |  | pc_SNPN_access_control_configuration |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 41 |  |  |  | Support of polarization signalling in NR NTN |  |  |  | 38.306, 5.4 |  |  |  | Rel-17 |  |  |  | pc_Polarization_Signalling_NR_NTN |  |  |  | No |  |  |  |  |  |  |  | UE supports polarization signalling in NR NTN |  |  |  |
| 42 |  |  |  | Supports receiving paging early indication and UE subgrouping indication with UEID |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-17 |  |  |  | pc_pei_SubgroupingSupportBandList_r17 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 43 |  |  |  | Support of Rel-17 extended DRX cycle up to 10485.76 seconds |  |  |  | 38.306, 5.8 24.501, 5.3.16 |  |  |  | Rel-17 |  |  |  | pc_NR_eDRX |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 44 |  |  |  | Support of (re-)configuration of an SCG during the resume procedure. |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-16 |  |  |  | pc_resumeWithSCG_Config_r16 |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 45 |  |  |  | Support of slice-based RACH prioritisation |  |  |  | 38.306, 5.4 |  |  |  | Rel-17 |  |  |  | pc_Slice_RACH_Prioiritisation |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 46 |  |  |  | Support of slice-based RACH partitioning |  |  |  | 38.306, 5.4 |  |  |  | Rel-17 |  |  |  | pc_Slice_RACH_Partitioning |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 47 |  |  |  | Support of RACH prioritisation for Access Identity 1 |  |  |  | 38.306, 5.4 |  |  |  | Rel-17 |  |  |  | pc_AccId1_RACH_Prioiritisation |  |  |  | No |  |  |  |  |  |  |  | US supporting this shall also support MPS (Access ID 1) |  |  |  |
| 48 |  |  |  | Support of ATSSS and MA PDU session |  |  |  | 24.501, 6.4.1.2 |  |  |  | Rel-16 |  |  |  | pc_5GC_ATSSS |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| 49 |  |  |  | Support gNB-side RTT-based PDC |  |  |  | 38.306, 4.2.2 |  |  |  | Rel-17 |  |  |  | pc_gNB_SideRTT_BasedPDC_r17 |  |  |  | No |  |  |  |  |  |  |  | A UE supporting this feature shall also support rtt-BasedPDC-CSI-RS-ForTracking-r17 and/or rtt-BasedPDC-PRS-r17. |  |  |  |
| 50 |  |  |  | Support of user plane integrity protection with EPS |  |  |  | 24.301, 5.5.1 33.401, 7.3.3 |  |  |  | Rel-17 |  |  |  | pc_EPS_UPIP |  |  |  | No |  |  |  |  |  |  |  | A UE supporting this feature shall also support EN-DC |  |  |  |
| 51 |  |  |  | Support of UAS Services |  |  |  | 24.501, 3.1, 4.22 |  |  |  | Rel-17 |  |  |  | pc_UAS |  |  |  | No |  |  |  |  |  |  |  | A UE supporting UAS services |  |  |  |
| 52 |  |  |  | Support of accessing SNPN using credentials from a Credentials Holder |  |  |  | 23.501 3.2, 5.30.2.9 |  |  |  | Rel-17 |  |  |  | pc_accessing_SNPN_usingCH |  |  |  | No |  |  |  |  |  |  |  | UE supports access using credentials assigned by a Credentials Holder separate from the SNPN |  |  |  |
| 53 |  |  |  | Support of Onboarding Stand-alone Non-Public Network |  |  |  | 23.501 5.30.2.10 |  |  |  | Rel-17 |  |  |  | pc_onboarding_SNPN |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |
| xx |  |  |  | Support of relaxed cell reselection on GEO. |  |  |  | 38.306. 5.4 |  |  |  | REl-17 |  |  |  | pc_relaxed_cell_reselection_on_gei |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |

<Unchanged Sections Omitted >

### A.4.3.8	Mobility Capabilities

Table A.4.3.8-1: UE Mobility Capabilities

| Item |  |  | UE Mobility Capabilities |  |  | Ref. |  |  | Release |  |  | Mnemonic |  |  | M |  |  | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release |  |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | Support inter-RAT Handover to EUTRA connected to EPC |  |  | 38.306, 4.2.9 |  |  | Rel-15 |  |  | pc_interRAT_EUTRA_Handover |  |  | Yes |  |  |  |  |  |  |  |  |
| 2 |  |  | Support inter-frequency Handover from the corresponding duplex mode or from the corresponding frequency range. |  |  | 38.306, 4.2.9 |  |  | Rel-15 |  |  | pc_handoverInterF |  |  | Yes |  |  |  |  |  |  |  |  |
| 3 |  |  | Support Handover between FR1 and FR2 |  |  | 38.306, 4.2.9 |  |  | Rel-15 |  |  | pc_FR1toFR2_Handover |  |  | Yes |  |  |  |  |  |  |  |  |
| 4 |  |  | Support Handover between FDD and TDD |  |  | 38.306, 4.2.9 |  |  | Rel-15 |  |  | pc_FDDtoTDD_Handover |  |  | Yes |  |  |  |  |  |  |  |  |
| 5 |  |  | Support inter-RAT Handover to E-UTRA connected to 5GC |  |  | 38.306, 4.2.9 |  |  | Rel-15 |  |  | pc_interRAT_eLTE_Handover |  |  | Yes |  |  |  |  |  |  |  |  |
| 6 |  |  | Support inter-RAT Handover to NR FR1 TDD from EUTRA connected to EPC |  |  | 36.306, 4.3.34.9 |  |  | Rel-15 |  |  | pc_eutra_EPC_HO_ToNR_TDD_FR1_r15 |  |  | Yes |  |  |  |  |  |  |  |  |
| 7 |  |  | Support inter-RAT Handover to NR FR1 FDD from EUTRA connected to EPC |  |  | 36.306, 4.3.34.8 |  |  | Rel-15 |  |  | pc_eutra_EPC_HO_ToNR_FDD_FR1_r15 |  |  | Yes |  |  |  |  |  |  |  |  |
| 8 |  |  | Support inter-RAT Handover to NR FR2 TDD from EUTRA connected to EPC |  |  | 36.306, 4.3.34.11 |  |  | Rel-15 |  |  | pc_eutra_EPC_HO_ToNR_TDD_FR2_r15 |  |  | Yes |  |  |  |  |  |  |  |  |
| 9 |  |  | Support intra-frequency DAPS handover |  |  | 38.306, 4.2.7.5 |  |  | Rel-16 |  |  | pc_intraFreqDAPS_r16 |  |  | No |  |  |  |  |  | It is mandated if the UE supports asynchronous intra-frequency DAPS handover |  |  |
| 10 |  |  | Support inter-RAT Handover from NR to EN-DC |  |  | 38.306, 4.2.10 |  |  | Rel-16 |  |  | pc_interRAT_NR_ToENDC |  |  | CY |  |  |  |  |  | It is mandated if the UE supports EN-DC. |  |  |
| 11 |  |  | Support conditional handover |  |  | 38.306, 4.2.7.2 |  |  | Rel-16 |  |  | pc_condHandover_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 12 |  |  | Support conditional handover during re-establishment procedure when the selected cell is configured as candidate cell for condition handover |  |  | 38.306, 4.2.7.2 |  |  | Rel-16 |  |  | pc_condHandoverFailure_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 13 |  |  | Support 2 trigger events for same execution condition of conditional handover |  |  | 38.306, 4.2.7.2 |  |  | Rel-16 |  |  | pc_condHandoverTwoTriggerEvents_r16 |  |  | CY |  |  |  |  |  | It is mandated if the UE supports condHandover-r16. |  |  |
| 14 |  |  | Support inter-RAT Handover from NR to UTRA-FDD CELL_DCH CS |  |  | 38.306, 4.2.9 |  |  | Rel-16 |  |  | pc_handoverUTRA_FDD_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 15 |  |  | Support inter-frequency DAPS handover |  |  | 38.306, 4.2.7.4 |  |  | Rel-16 |  |  | pc_interFreqDAPS_r16 |  |  | No |  |  |  |  |  | It is mandated if the UE supports asynchronous inter-frequency DAPS handover or supports different SCSs in source PCell and inter-frequency target PCell in DAPS handover |  |  |
| 16 |  |  | UE supports asynchronous intra-frequency DAPS handover |  |  | 38.306, 4.2.7.5 |  |  | Rel-16 |  |  | pc_intraFreqAsyncDAPS_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 17 |  |  | UE supports asynchronous inter-frequency DAPS handover |  |  | 38.306, 4.2.7.5 |  |  | Rel-16 |  |  | pc_interFreqAsyncDAPS_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 18 |  |  | UE supports different SCSs in source PCell and inter-frequency target PCell in DAPS handover |  |  | 38.306, 4.2.7.5 |  |  | Rel-16 |  |  | pc_ inteFreqDiffSCS_DAPS_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 19 |  |  | Support conditional PSCell change |  |  | 38.306, 4.2.7.2 |  |  | Rel-16 |  |  | pc_condPSCellChange_r16 |  |  | No |  |  |  |  |  |  |  |  |
| 20 |  |  | Support handover from 5GS to EPC/ePDG |  |  | 24.302, 7.2.2.1 |  |  | Rel-15 |  |  | pc_HO_from_5GS_to_EPC_ePDG |  |  | No |  |  |  |  |  |  |  |  |
| 21 |  |  | Support handover from EPC/ePDG to 5GS |  |  | 23.502, 4.11.4.1 |  |  | Rel-15 |  |  | pc_HO_from_EPC_ePDG_to_5GS |  |  | No |  |  |  |  |  |  |  |  |
| 22 |  |  | Support Handover from EPS to 5GC-N3IWF |  |  | 23.502, 4.11.3.1 |  |  | Rel-15 |  |  | pc_HO_from_EPS_to_5GC_N3IWF |  |  | No |  |  |  |  |  |  |  |  |
| 23 |  |  | Support Handover from 5GC-N3IWF to EPS |  |  | 23.502, 4.11.3.2 |  |  | Rel-15 |  |  | pc_HO_from_5GC_N3IWF_to_EPS |  |  | No |  |  |  |  |  |  |  |  |
| 24 |  |  | Support Handover of a PDU Session procedure from untrusted non-3GPP to 3GPP access |  |  | 23.502, 4.9.2.1 |  |  | Rel-15 |  |  | pc_HO_from_5GC_N3IWF_to_5GC |  |  | No |  |  |  |  |  |  |  |  |
| 25 |  |  | Support Handover of a PDU Session procedure from 3GPP to untrusted non-3GPP access |  |  | 23.502, 4.9.2.2 |  |  | Rel-15 |  |  | pc_HO_from_5GC_to_5GC_N3IWF |  |  | No |  |  |  |  |  |  |  |  |
| xx1 |  |  | Supports location based conditional handover, i.e., CondEvent D1 |  |  | 38.306, 4.2.7.2 |  |  | Rel-17 |  |  | pc_locationBasedCondHandover_r17 |  |  | No |  |  |  |  |  | A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands. |  |  |
| xx2 |  |  | Support time based conditional handover, i.e., CondEvent T1 |  |  | 38.306, 4.2.7.2 |  |  | Rel-17 |  |  | pc_timeBasedCondHandover_r17 |  |  | No |  |  |  |  |  | A UE supporting this feature shall also indicate the support of condHandover-r16 for NTN bands and the support of nonTerrestrialNetwork-r17. UE shall set the capability value consistently for all FDD-FR1 NTN bands. |  |  |

<Unchanged Sections Omitted >

<End of Changes >